| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0186 |
| --- | --- |

| Title: | Flexible coding tools to significantly improve SCC performance in cloud and mobile computing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liping Zhao, Wenting Cai, Jing Guo, Tao Lin 1239 Siping Ave, Shanghai 200092, China | Tel: Email: | lintao@tongji.edu.cn |
| Source: | Tongji University |  |  |

_____________________________

# Abstract

This document reports that some coding tools using flexible basic copy unit can improve SCC performance by up to -38% BD-rate reduction in some typical SCC application scenarios in cloud and cloud-mobile computing.

# Introduction

Screen contents cover a very wide range of a variety of pictures with very different characteristics. Tools that use fixed size block as basic copy unit or use variable length string as basic copy unit but only limited to the current CU are not flexible to code general screen contents in cloud computing and cloud-mobile computing in very efficient way. This document provides a few common and typical screen snapshots in cloud computing and cloud-mobile computing. Coding tools that can handle a variety of patterns in a picture using flexible basic copy unit can achieve much better BD-rate reduction (up to -38%) than SCM40 SCC reference software.

# Three typical and commonly seen screen snapshots

The first snapshot (sc_HEVC_BD-rate_excel_1st1920x1080.yuv) is the HEVC BD-rate excel sheet we use in our daily work. Part of the snapshot is shown below.

The second snapshot (sc_HEVC_BD-rate_excel_2nd1920x1080.yuv) is the same HEVC BD-rate excel sheet but the data of each sequence are shown in different background color. Part of the snapshot is shown below.

The third snapshot (sc_scc_analyzer1920x1080.yuv) is a SCC bit-stream analyzer GUI that displays all coding modes (intra, inter, IBC, palette) using different colors, also displays CU grids, MV, and translucent image of the original picture being coded. Part of the snapshot is shown below.

All three snapshots are common and typical screen contents seen in cloud computing and cloud-mobile computing.

# Simulation results

The coding results of SCM40 vs ISC are as follows:

|  |  | SCM40 |  |  |  | ISC |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sc_HEVC_BD-rate_excel_1st1920x1080.yuv |  |  |  |  |  |  |  |  |  |  |  |  |
| FF IBC | 22 | 22515.4 | 55.2 | 54.0 | 54.0 | 20882.9 | 58.3 | 57.8 | 56.9 | -7.1% | -6.8% | -7.0% |
| vs | 27 | 21521.8 | 49.1 | 48.0 | 48.3 | 19943.5 | 50.1 | 49.1 | 49.2 |  |  |  |
| 2CTU ISC | 32 | 20075.5 | 42.2 | 41.0 | 41.4 | 19124.6 | 44.8 | 41.9 | 43.0 |  |  |  |
|  | 37 | 17933.3 | 34.8 | 36.9 | 35.9 | 17406.2 | 35.4 | 36.4 | 36.2 |  |  |  |
| 4CTU IBC | 22 | 77978.9 | 56.5 | 55.0 | 54.8 | 50709.6 | 60.6 | 58.9 | 57.7 | -30.3% | -30.3% | -29.1% |
| vs | 27 | 72444.0 | 50.2 | 48.3 | 48.7 | 49793.3 | 53.6 | 51.0 | 50.5 |  |  |  |
| 4CTU ISC | 32 | 64731.4 | 43.5 | 41.7 | 42.4 | 47927.5 | 46.0 | 42.4 | 44.3 |  |  |  |
|  | 37 | 53898.2 | 35.6 | 36.6 | 36.0 | 43849.9 | 38.4 | 38.1 | 36.4 |  |  |  |
| sc_HEVC_BD-rate_excel_2nd1920x1080.yuv |  |  |  |  |  |  |  |  |  |  |  |  |
| FF IBC | 22 | 32484.0 | 52.2 | 51.5 | 49.9 | 27364.8 | 53.3 | 52.2 | 50.4 | -12.2% | -11.0% | -12.0% |
| vs | 27 | 29248.3 | 46.1 | 45.6 | 43.9 | 25844.2 | 47.1 | 45.7 | 45.5 |  |  |  |
| 2CTU ISC | 32 | 25595.5 | 40.9 | 39.3 | 38.9 | 23664.5 | 42.0 | 39.7 | 39.3 |  |  |  |
|  | 37 | 21880.3 | 35.1 | 35.1 | 34.6 | 20698.6 | 36.8 | 35.7 | 35.3 |  |  |  |
| 4CTU IBC | 22 | 80384.6 | 53.3 | 52.1 | 51.9 | 46575.4 | 54.6 | 53.3 | 52.7 | -37.7% | -37.3% | -37.2% |
| vs | 27 | 72132.5 | 47.7 | 46.5 | 46.9 | 44194.1 | 49.1 | 47.0 | 47.6 |  |  |  |
| 4CTU ISC | 32 | 62831.0 | 41.1 | 40.2 | 41.1 | 41890.6 | 43.5 | 41.3 | 43.4 |  |  |  |
|  | 37 | 51353.3 | 34.9 | 35.0 | 35.4 | 38330.4 | 36.9 | 36.5 | 36.7 |  |  |  |
| sc_scc_analyzer1920x1080.yuv |  |  |  |  |  |  |  |  |  |  |  |  |
| FF IBC | 22 | 82477.4 | 55.4 | 56.0 | 55.9 | 68904.5 | 56.8 | 56.8 | 57.4 | -15.3% | -15.3% | -15.0% |
| vs | 27 | 75791.5 | 50.3 | 49.4 | 50.8 | 64734.2 | 51.7 | 50.0 | 51.7 |  |  |  |
| 2CTU ISC | 32 | 69098.4 | 43.6 | 45.0 | 46.8 | 60058.6 | 44.3 | 46.3 | 47.2 |  |  |  |
|  | 37 | 59060.2 | 38.9 | 37.6 | 41.5 | 53367.8 | 40.0 | 39.5 | 42.6 |  |  |  |
| 4CTU IBC | 22 | 94786.1 | 56.4 | 56.6 | 56.4 | 71784.5 | 57.6 | 57.5 | 58.0 | -22.3% | -22.3% | -22.1% |
| vs | 27 | 86639.0 | 51.0 | 49.8 | 51.1 | 67943.5 | 53.1 | 50.6 | 52.4 |  |  |  |
| 4CTU ISC | 32 | 78842.9 | 44.0 | 45.4 | 46.9 | 63427.7 | 45.0 | 47.2 | 47.8 |  |  |  |
|  | 37 | 67552.8 | 39.2 | 37.9 | 41.9 | 58006.6 | 40.8 | 40.8 | 43.3 |  |  |  |

# Recommendation

It is recommended to add these three snapshot pictures into SCC CTC.

It is recommended to adopt PPC (U0116) subject to the condition that if any serious problem is found between now and the next meeting, it is removed from the draft.

It is also recommended to have a CE to do further syntax harmonization of PPC with existing palette coding.

# Conclusion

Coding tools like Intra String Copy (ISC) or Palette with Pixel Copy (PPC) that can handle a variety of patterns in a picture using flexible basic copy unit can achieve much better BD-rate reduction (up to -38%) than SCM40 SCC reference software.

# References

- JCT-VC, “HM-16.4+SCM-4.0,” https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.4+SCM-4.0/

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” JCTVC-S1015, Strasbourg, Oct. 2014.

- Tao Lin, Kailun Zhou, Liping Zhao, Xianyi Chen, “Non-CE1: Enhancement to Palette Coding by Palette with Pixel Copy (PPC) Coding,”JCTVC-U0116, Warsaw, June 2015.

- Kailun Zhou, Tao Lin, “Non-CE1: Palette with Pixel Copy (PPC) decoder IC reference design,”JCTVC-U0171, Warsaw, Oct. 2015.

- Xianyi Chen, Tao Lin, “Performance gain of IBC FF and loss of IBC FF vs 4CTU,”JCTVC-U0172, Warsaw, June 2015.

- Tao Lin, Kailun Zhou, Liping Zhao, Xianyi Chen, “Non-CE1: Further harmonization of PPC with no-pixel-copy palette coding,”JCTVC-U0182, Warsaw, June 2015.

# Patent rights declaration(s)

Tongji University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).