| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0188 |
| --- | --- |

| Title: | New SCC test sequence for consideration |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Wang, Meng Xu, Fanyi Duanmu, Haoping Yu 2330 Central Expy Santa Clara, CA 95050 USA | Tel: Email: | +1 408-330-4844 wei.wang.sc@huawei.com |
| Source: | Huawei R&D USA |  |  |

_____________________________

# Abstract

This contribution proposes a new test sequence for the SCC CTC. This sequence consists of content with Chinese text editing. The coding results from SCM-4.0 demonstrate different statistics in the usage of coding tools, when compared with the existing SCC test sequences.

# New sequence description

The first frame of this new sequence is shown in Figure 1 below. This new sequence is available for review in this meeting.

The sequence is ChineseEditing_1920x1080_60_8bit_rgb. The majority of content in this sequence shows Chinese characters. It has typical screen content characteristics, such as moving texts, zoom in, zoom out, copy & paste, highlighting, etc., often seen in word editing applications.

This sequence has 600 frames. The original RGB sequence was generated by capturing the raw 24-bit RGB signals from a computer screen to hard drive. The screen display format was set to 1920x1080 at 60fps.

Figure 1 First frame of ChineseEditing, 1920x1080

# Coding tool evaluation

## Software

SCM-4.0 software was used in the evaluation.

The statistics of different modes in intra frames are collected under two test conditions. The first configuration is CTC [1] with full frame IBC search range. The second one is to use constrained 4x1 CTU IntraBC search range in CTC, i.e., IBC search range is set to 3 CTUs to the left of current CTU.

## Test material

The 1st frame of the ChineseEditing sequence is used to visualize the CU partition as well as mode distribution. The statistics were collected from the full sequence.

All existing CTC RGB sequences were also used in the tests.

## Simulation results

The CU partition and the coding mode distribution of 1st coded frame coded using SCM-4.0 with full-frame IBC search range are visualized in Figure 2. The result from coding the same frame with 4x1 CTU IBC search range is illustrated in Figure 3. These visualization tests were done using QP equal to 22. The colors representing different modes are shown in Table 1.

Table 1 Color representation for modes

| Color | Mode |
| --- | --- |
| Magenta | Intra |
| Cyan | IntraBC |
| Black | Palette |

Figure 2 First frame of ChineseEditing, full frame IBC

Figure 3 Frist frame of ChineseEditing, 4x1 IBC

The statistics of the pixel coverage and bits consumption percentage for full frame IBC and 4x1-CTU IBC are shown in Table 2Error: Reference source not found. We also collected the statistic information on tool usage from the coding results of all other existing SCC RGB sequences. The details of these data are given in Table 3 Table and Error: Reference source not found4.

Table 2 Mode statistics from ChineseEditing

| IBC range | Palette pixels | Palette bits | IBC pixels | IBC bits | Intra pixels | Intra bits |
| --- | --- | --- | --- | --- | --- | --- |
| Full frame | 47.82% | 67.77% | 29.57% | 22.61% | 22.61% | 9.61% |
| 4x1 | 58.41% | 77.09% | 16.06% | 13.16% | 25.53% | 9.74% |

Table 3 Mode statistics for full frame IBC from existing CTC sequences

| Category | Sequence | Palette pixels | Palette bits | IBC pixels | IBC bits | Intra pixels | Intra bits |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Text & graphics with motion | Flyinggraphicstext | 34.60% | 32.10% | 46.61% | 49.99% | 18.79% | 17.91% |
|  | Desktop | 34.64% | 54.97% | 42.56% | 31.50% | 22.80% | 13.53% |
|  | Console | 34.22% | 52.54% | 46.02% | 40.40% | 19.76% | 7.06% |
|  | WebBrowsing | 24.38% | 21.61% | 22.85% | 13.59% | 52.78% | 64.80% |
|  | Map | 54.18% | 70.70% | 7.92% | 8.81% | 37.90% | 20.50% |
|  | Programming | 19.17% | 33.52% | 25.75% | 35.23% | 55.09% | 31.25% |
|  | SlideShow | 11.70% | 27.09% | 12.22% | 12.81% | 76.08% | 60.09% |
| Mixed content | BasketballScreen | 13.91% | 17.16% | 20.90% | 22.16% | 65.20% | 60.68% |
|  | MissionControlClip2 | 4.70% | 12.14% | 18.13% | 24.22% | 77.17% | 63.64% |
|  | MissionControlClip3 | 8.17% | 12.72% | 24.64% | 29.93% | 67.18% | 57.35% |
| Animation | Robot | 1.70% | 0.92% | 3.67% | 4.91% | 94.63% | 94.18% |

Table 4 Mode statistics for 4x1 IBC from existing CTC sequences

| Category | Sequence | Palette pixels | Palette bits | IBC pixels | IBC bits | Intra pixels | Intra bits |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Text & graphics with motion | Flyinggraphicstext | 41.45% | 36.63% | 34.21% | 44.36% | 24.34% | 19.01% |
|  | Desktop | 57.06% | 71.13% | 22.52% | 18.56% | 20.42% | 10.31% |
|  | Console | 49.91% | 59.89% | 29.43% | 32.27% | 20.66% | 7.83% |
|  | WebBrowsing | 45.04% | 38.25% | 12.53% | 9.38% | 42.42% | 52.37% |
|  | Map | 57.20% | 74.37% | 5.30% | 5.53% | 37.50% | 20.11% |
|  | Programming | 25.30% | 41.04% | 19.33% | 28.80% | 55.37% | 30.16% |
|  | SlideShow | 14.69% | 29.88% | 9.37% | 9.46% | 75.94% | 60.66% |
| Mixed content | BasketballScreen | 16.91% | 23.66% | 14.67% | 18.86% | 68.41% | 57.48% |
|  | MissionControlClip2 | 12.28% | 28.69% | 15.36% | 24.88% | 72.36% | 46.43% |
|  | MissionControlClip3 | 17.25% | 27.38% | 19.93% | 29.29% | 62.82% | 43.33% |
| Animation | Robot | 1.70% | 0.85% | 3.00% | 4.30% | 95.30% | 94.84% |

# Observation

(1). Under full frame IBC condition, the percentage of average encoded pixel covered by Palette, IBC and Intra in Text & graphics with motion class are 30% : 29% : 41% for the existing sequences. The corresponding percentage of average encoded bits among Palette, IBC and Intra in Text & graphics with motion class are 42% : 28% : 31%.

(2). Under full frame IBC condition, the percentage of average encoded pixel covered by Palette, IBC and Intra in ChineseEditing are 48% : 30% : 22%. The corresponding percentage of average encoded bits among Palette, IBC and Intra are 68% : 23% : 9%.

(3). Under 4x1 IBC condition, the percentage of average encoded pixel covered by Palette, IBC and Intra in Text & graphics with motion class are 42% : 19% : 39% for the existing sequences. The corresponding percentage of average encoded bits among Palette, IBC and Intra in Text & graphics with motion class are 50% : 21% : 29%.

(4). Under 4x1 IBC condition, the percentage of average encoded pixel covered by Palette, IBC and Intra in ChineseEditing are 58% : 16% : 26%. The corresponding percentage of average encoded bits among Palette, IBC and Intra are 77% : 13% : 10%.

(5). Up to 30% more pixels were encoded in Palette mode in ChineseEditing sequence than in Text & graphics with motion class.

(6). As can be seen from above observation, Chinese text has quite different characteristics from existing test sequences. It is useful to test non-IBC tools.

# Recommendation

It is recommended to add this new sequence in SCC CTC.

# Reference

[1] H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding”, Doc. JCT-VC T1015, Geneva, CH, Feb. 2015.

# Patent rights declaration(s)

Huawei R&D USA may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).