| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U1000 |
| --- | --- |

| Title: | Meeting report of the 21st meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Warsaw, PL, 19–26 June 2015 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from chairs of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs |  |  |

_____________________________

- Summary

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its twenty-first meeting during 19–26 June 2015 at the Mariott Hotel Warsaw in Warsaw, Poland. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.14 of this document.

The JCT-VC meeting sessions began at approximately 0900 hours on Friday 19 June 2015. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately 12:40 hours on Friday 26 June 2015. Approximately 91 people attended the JCT-VC meeting, and approximately 150 input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11– one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

One primary goal of the meeting was to review the work that was performed in the interim period since the twentieth JCT-VC meeting in producing:

- The HEVC test model (HM) 16 improved encoder description (including RExt modifications) update 2;

- Verification test plan for format range extensions and interlaced video;

- The RExt reference software draft 2 and conformance testing draft 4, the latter combined with improved version 1 conformance testing;

- The SHVC reference software draft 1, conformance testing draft 2 and SHM test model 9;

- For HEVC screen content coding (SCC) extensions, the HEVC screen content coding test model 4, SCC draft text 3, and a document specifying common test conditions and software reference configurations for SCC experiments.

The other most important goals were to review the results from two Core Experiments on Screen Content Coding (CE1–2), and review other technical input documents. Reviewing the progress made towards definition of screen content coding tools was the most important topic of the meeting. Advancing the work on development of conformance and reference software for recently finalized HEVC extensions (RExt, SHVC) is also a significant goal. Further preparation of verification tests was conducted. Possible needs for corrections to version 2 were also considered.

In addition to one new experiment plan description in screen content coding, the JCT-VC produced 11 other particularly important output documents from the meeting:

- The HEVC test model (HM) 16 improved encoder description (including RExt modifications) update 3;

- The verification test report for format range extensions and interlaced video;

- For the format range extensions (RExt), the RExt reference software draft 3 and conformance testing draft 5, the latter combined with improved version 1 conformance testing;

- For the scalable extensions (SHVC), the SHVC reference software draft 2, conformance testing draft 3, SHVC test model 10 (SHM 10), and draft verification test plan for SHVC;

- For the HEVC screen content coding (SCC) extensions, the HEVC screen content coding test model 5 (SCM 5), SCC draft text 4, and a document specifying common test conditions and software reference configurations for SCC experiments.

For the organization and planning of its future work, the JCT-VC established 13 "ad hoc groups" (AHGs) to progress the work on particular subject areas. The next four JCT-VC meetings are planned for Thu. 15 – Wed. 21 Oct. 2015 under ITU-T auspices in Geneva, CH, Fri. 19 – Fri. 26 Feb. 2016 under WG 11 auspices in San Diego, US, Thu. 26 May – Wed. 1 June 2016 under ITU-T auspices in Geneva, CH, and Fri. 14 – Fri. 21 Oct. 2016 under WG 11 auspices in Chengdu, CN.

The document distribution site http://phenix.it-sudparis.eu/jct/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
https://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

- Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-VC are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its twenty-first meeting during 19–26 June 2015 at the Marriott Hotel Warsaw in Warsaw, Poland. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

## Meeting logistics

The JCT-VC meeting sessions began at approximately 0900 hours on Friday 19 June 2015. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately 1240 hours on Friday 26 June 2015. Approximately 91 people attended the JCT-VC meeting, and approximately 150 input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11– one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

Some statistics are provided below for historical reference purposes:

- 1st "A" meeting (Dresden, 2010-04):		188 people, 40 input documents

- 2nd "B" meeting (Geneva, 2010-07):		221 people, 120 input documents

- 3rd "C" meeting (Guangzhou, 2010-10):		244 people, 300 input documents

- 4th "D" meeting (Daegu, 2011-01):		248 people, 400 input documents

- 5th "E" meeting (Geneva, 2011-03):		226 people, 500 input documents

- 6th "F" meeting (Turin, 2011-07):		254 people, 700 input documents

- 7th "G" meeting (Geneva, 2011-11)		284 people, 1000 input documents

- 8th "H" meeting (San Jose, 2012-02)		255 people, 700 input documents

- 9th "I" meeting (Geneva, 2012-04/05)		241 people, 550 input documents

- 10th "J" meeting (Stockholm, 2012-07)		214 people, 550 input documents

- 11th "K" meeting (Shanghai, 2012-10)		235 people, 350 input documents

- 12th "L" meeting (Geneva, 2013-01)		262 people, 450 input documents

- 13th "M" meeting (Incheon, 2013-04)		183 people, 450 input documents

- 14th "N" meeting (Vienna, 2013-07/08)		162 people, 350 input documents

- 15th "O" meeting (Geneva, 2013-10/11)		195 people, 350 input documents

- 16th "P" meeting (San José, 2014-01)		152 people, 300 input documents

- 17th "Q" meeting (Valencia, 2014-03/04)	126 people, 250 input documents

- 18th "R" meeting (Sapporo, 2014-06/07)		150 people, 350 input documents

- 19th "S" meeting (Strasbourg, 2014-10)		125 people, 300 input documents

- 20th "T" meeting (Geneva, 2015-02)		120 people, 200 input documents

- 21st "U" meeting (Warsaw, 2015-06)		91 people, 150 input documents

Information regarding logistics arrangements for the meeting had been provided via the email reflector jct-vc@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jctvc-site/2015_06_U_Warsaw/ .

## Primary goals

One primary goal of the meeting was to review the work that was performed in the interim period since the nineteenth JCT-VC meeting in producing:

- The HEVC test model (HM) 16 improved encoder description (including RExt modifications) update 2;

- Verification test plan for format range extensions and interlaced video;

- The RExt reference software draft 2 and conformance testing draft 4, the latter combined with improved version 1 conformance testing;

- The SHVC reference software draft 1, conformance testing draft 2 and SHM test model 9;

- For HEVC screen content coding (SCC) extensions, the HEVC screen content coding test model 4, SCC draft text 3, and a document specifying common test conditions and software reference configurations for SCC experiments.

The other most important goals were to review the results from two Core Experiments on Screen Content Coding (CE1–2), and review other technical input documents. Reviewing the progress made towards definition of screen content coding tools was the most important topic of the meeting. Advancing the work on development of conformance and reference software for the recently finalized HEVC extensions (RExt, SHVC) was also a significant goal. Further preparation of verification tests was conducted. Possible needs for corrections to version 2 were also considered.

## Documents and document handling considerations

### General

The documents of the JCT-VC meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of entropy-coding contexts, number of context-coded bins, etc.) … "Decision (Compl.):".

This meeting report is based primarily on notes taken by the chairs and projected for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Tuesday, 9 June 2015.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Wednesday 10 June 2015 were considered "officially late".

Most documents in the "late" category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

At this meeting, we again had a substantial amount of late document activity, but in general the early document deadline gave a significantly better chance for thorough study of documents that were delivered in a timely fashion. The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

All contribution documents with registration numbers JCTVC-U0121 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). However, some documents in the "U0121+" range include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered on time but were uploaded late:

- (This case did not occur at this meeting)

The following technical design proposal contributions were both registered late and uploaded late:

- JCTVC-U0122 (a proposal document from Qualcomm relating to a CE on palette coding)

- JCTVC-U0127 (a proposal document from Qualcomm relating to a CE on palette coding)

- JCTVC-U0128 (a proposal from Nanjing Yunyan Tech. and Tsinghua Univ. of an SEI message for "green metadata")

- JCTVC-U0133 (a proposal from Qualcomm on palette coding)

- JCTVC-U0138 (a proposal from Apple and Qualcomm on chroma deblocking)

- JCTVC-U0142 (a proposal from Apple on intra block copy unification with inter coding)

- JCTVC-U0148 (a proposal from Huawei on palette coding)

- JCTVC-U0149 (a proposal from Huawei on palette coding)

- JCTVC-U0169 (a proposal from Qualcomm on palette coding)

- JCTVC-U0176 (a proposal from Qualcomm and MediaTek with a combination of techniques proposed in other contributions)

- JCTVC-U0180 (a proposal from Qualcomm and MediaTek with a combination of techniques proposed in other contributions)

- JCTVC-U0182 (a proposal from Tongji Univ. on palette coding with pixel string copying)

The following other documents not proposing normative technical content were registered on time but were uploaded late:

- JCTVC-U0038 (a proposal on various editorial improvements) [uploaded 06-19]

- JCTVC-U0041 (a document on 4:4:4 to 4:2:0 conversion) [uploaded 06-13]

- JCTVC-U0043 (a document on observations related to deblocking) [uploaded 06-16]

- JCTVC-U0047 (a document on HDR workflow) [uploaded 06-25]

- JCTVC-U0058 (a document on using SCC with multiple slices) [uploaded 06-17]

The following cross-verification reports were registered on time but were uploaded late: JCTVC-U0035 [uploaded 06-11], JCTVC-U0059 [uploaded 06-17], JCTVC-U0060 [uploaded 06-19], JCTVC-U0082 [uploaded 06-24], JCTVC-U0108 [uploaded 06-18], JCTVC-U0117 [uploaded 06-18].

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JCTVC-U0067, JCTVC-U0068, JCTVC-U0069, JCTVC-U0071, JCTVC-U0072, JCTVC-U0073, JCTVC-U0074, JCTVC-U0075, JCTVC-U0144, JCTVC-U0150 (later uploaded as JCTVC-U0188), JCTVC-U0151 (later uploaded as JCTVC-U0189), JCTVC-U0157, JCTVC-U0170, JCTVC-U0174, JCTVC-U0179.

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were to be rejected in the document management system, as has been agreed since the third meeting.

The following case did not occur at the 21st meeting: The initial uploads of the following contribution documents were rejected as a "placeholders" without any significant content and were not corrected until after the upload deadline:

- JCTVC-U0XXX (a proposal on … , corrected by a late upload on 06-XX)

- …

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the chairs).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Measures to facilitate the consideration of contributions

It was agreed that, due to the continuingly high workload for this meeting, the group would try to rely extensively on summary CE reports. For other contributions, it was agreed that generally presentations should not exceed 5 minutes to achieve a basic understanding of a proposal – with further review only if requested by the group. For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by a cross-check contributor.

### Outputs of the preceding meeting

The output documents of the previous meeting, particularly including the meeting report JCTVC-T1000, the improved HEVC Test Model 16 (HM16) JCTVC-T1002, the Verification Test Plan for RExt and interlaced coding with version 1, JCTVC-T1003, the RExt Reference Software Draft 2 JCTVC-T1011, the RExt Conformance Testing Draft 4 (including improved Version 1 Conformance Testing) JCTVC-T1012, the SHVC test model 9 (SHM9) JCTVC-T1007, the SHVC Conformance Testing Draft 2 JCTVC-T1008, the SHVC Reference Software Draft 1 JCTVC-T1013, the Screen Content Coding (SCC) Draft Text 3 JCTVC-T1005, the SCC test model 4 JCTVC-T1014, and the common test conditions for SCC (JCTVC-T1015) were approved. The HM reference software and its extensions for RExt, SHVC and SCC were also approved.

The group had initially been asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chairs asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

It was remarked that, in regard to software development efforts – for cases where "code cleanup" is a goal as well as integration of some intentional functional modification, it was emphasized that these two efforts should be conducted in separate integrations, so that it is possible to understand what is happening and to inspect the intentional functional modifications.

The need for establishing good communication with the software coordinators was also emphasized.

At some previous meetings, it had been remarked that in some cases the software implementation of adopted proposals revealed that the description that had been the basis of the adoption apparently was not precise enough, so that the software unveiled details that were not known before (except possibly for CE participants who had studied the software). Also, there should be time to study combinations of different adopted tools with more detail prior to adoption.

CE descriptions need to be fully precise – this is intended as a method of enabling full study and testing of a specific technology.

Greater discipline in terms of what can be established as a CE may be an approach to helping with such issues. CEs should be more focused on testing just a few specific things, and the description should precisely define what is intended to be tested (available by the end of the meeting when the CE plan is approved).

It was noted that sometimes there is a problem of needing to look up other referenced documents, sometimes through multiple levels of linked references, to understand what technology is being discussed in a contribution – and that this often seems to happen with CE documents. It was emphasized that we need to have some reasonably understandable description, within a document, of what it is talking about.

Software study can be a useful and important element of adequate study; however, software availability is not a proper substitute for document clarity.

Software shared for CE purposes needs to be available with adequate time for study. Software of CEs should be available early, to enable close study by cross-checkers (not just provided shortly before the document upload deadline).

Issues of combinations between different features (e.g., different adopted features) also tend to sometimes arise in the work.

## Attendance

The list of participants in the JCT-VC meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities

- Review of results of previous meeting

- Consideration of contributions and communications on project guidance

- Consideration of technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-VC and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-VC as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jctvc-site (JCT-VC contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-vc/index.html (JCT-VC general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director's IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur's group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the HEVC standard and its extensions, and for evaluating proposals for technology to be included in the design. After finalization of the draft (current version JCTVC-M1010), the software will be published by ITU-T and ISO/IEC as an example implementation of the HEVC standard and for use as the basis of products to promote adoption of the technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct/. For the first two JCT-VC meetings, the JCT-VC documents had been made available at http://ftp3.itu.int/av-arch/jctvc-site, and documents for the first two JCT-VC meetings remain archived there as well. That site was also used for distribution of the contribution document template and circulation of drafts of this meeting report.

JCT-VC email lists are managed through the site https://mailman.rwth-aachen.de/mailman/options/jct-vc, and to send email to the reflector, the email address is jct-vc@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JCT-VC participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Some terminology used in this report is explained below:

- ACT: Adaptive colour transform.

- Additional Review: The stage of the ITU-T "alternative approval process" that follows a Last Call if substantial comments are received in the Last Call, during which a proposed revised text is available on the ITU web site for consideration as a candidate for final approval.

- AHG: Ad hoc group.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component).

- AMVP: Adaptive motion vector prediction.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR).

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: See CPR or IBC.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- BV: Block vector (used for intra BC prediction).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCP: Cross-component prediction.

- CD: Committee draft – a draft text of an international standard for the first formal ballot stage of the approval process in ISO/IEC – corresponding to a PDAM for amendment texts.

- CE: Core experiment – a coordinated experiment conducted after the 3rd or subsequent JCT-VC meeting and approved to be considered a CE by the group (see also SCE and SCCE).

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CL-RAS: Cross-layer random-access skip.

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- Consent: A step taken in the ITU-T to formally move forward a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DAM: Draft amendment – a draft text of an amendment to an international standard for the second formal ballot stage of the approval process in ISO/IEC – corresponding to a DIS for complete texts.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC – corresponding to a DAM for amendment texts.

- DF: Deblocking filter.

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- EL: Enhancement layer.

- ET: Encoding time.

- FDAM: Final draft amendment – a draft text of an amendment to an international standard for the third formal ballot stage of the approval process in ISO/IEC – corresponding to an FDIS for complete texts.

- FDIS: Final draft international standard – a draft text of an international standard for the third formal ballot stage of the approval process in ISO/IEC – corresponding to an FDAM for amendment texts.

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC).

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- Last Call: The stage of the ITU-T "alternative approval process" that follows Consent, during which a proposed text is available on the ITU web site for consideration as a candidate for final approval.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures.

- MANE: Media-aware network elements.

- MC: Motion compensation.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JCT-VC).

- MV: Motion vector.

- NAL: Network abstraction layer (as in AVC and HEVC).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NSQT: Non-square quadtree.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F).

- OLS: Output layer set.

- PCP: Parallelization of context processing.

- PDAM: Proposed draft amendment – a draft text of an amendment to an international standard for the first formal ballot stage of the ISO/IEC approval process – corresponding to a CD for complete texts.

- POC: Picture order count.

- PoR: Plan of record.

- PPS: Picture parameter set (as in AVC and HEVC).

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPLM: Reference picture list modification.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SCC: Screen content coding.

- SCE: Scalability core experiment.

- SCCE: Screen content core experiment.

- SCM: Screen coding model.

- SD: Slice data; alternatively, standard-definition.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIMD: Single instruction, multiple data.

- SPS: Sequence parameter set (as in AVC and HEVC).

- TBA/TBD/TBP: To be announced/determined/presented.

- TE: Tool Experiment – a coordinated experiment conducted toward HEVC design between the 1st and 2nd or 2nd and 3rd JCT-VC meetings, or a coordinated experiment conducted toward SHVC design between the 11th and 12th JCT-VC meetings.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-VC).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WD: Working draft – a term for a draft standard that may sometimes be used loosely to refer to a draft standard prior to or at any actual stage of parent-level approval processes.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - LCU: (formerly LCTU) largest coding unit (name formerly used for CTU before finalization of HEVC version 1).

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntaxError: Reference source not found within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

## Liaison activity

The JCT-VC did not send or receive formal liaison communications at this meeting. However, see the notes relating to JCTVC-U0156.

## Opening remarks

Opening remarks included:

- Meeting logistics, review of communication practices, attendance recording, and registration and badge pick-up reminder

Primary topic areas were noted as follows:

- Screen content coding

- Corrigenda items for version 3 (see, e.g., the AHG2 report)

- Verification testing for interlaced video, RExt, & SHVC

- Reference software and conformance, RExt & SHVC

- Test model texts and software manuals

- Common test conditions for coding efficiency experiments

Unfinished (or less-than-optimally finished) deliverables

- SHM 9 was missing at the opening of the meeting, and some WG 11 deliverables were also missing (these issues were resolved in the first few days of the meeting)

Key deliverables from this meeting

- DAM on SCC

- DAM for RExt conformance

- DAMs for SHVC conformance & reference software

- Verification test report for RExt

- Verification test plan for SHVC

Two main tracks were followed for most meeting discussions:

- Track A (GJS): CE1 (palette), etc.

- Track B (JRO): CE2 (intra block copy), etc.

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0800–2000 hours, with coffee and lunch breaks as convenient. The meeting had been announced to start with AHG reports and continue with parallel review on Screen Content Coding CE work and related contributions during the first few days. Ongoing scheduling refinements were announced on the group email reflector as needed.

Some particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Fri. 19 June, 1st day

  - 0900–1300 JCT-VC opening and review of AHG reports and CE 1 summary [JRO & GJS]

  - 1430–2000 Tracks A (Palette) & B (IBC)

- Sat. 20 June, 2nd day

  - 0900–1330

    - Track A Palette U0097, U0109, U0116, U0149, U0036

    - Track B (IBC)

      - Remaining TBP U0142, U0172, some revisits

  - 1530–2000 Track A: U0084, U0114, VUI&SEI messages, and non-normative inputs

      - Remaining open included presentation of U0052 (on palette escape pixel coding), revisit of U0169 and U0090 (modified binarization for num_palette_indices_idc), revisit of U0112 (ambient viewing environment SEI message), high-level discussion of U0115 (video bitstream transition SEI message), presentation of U0128 (green metadata SEI message), revisit of U0095 (encoder speed-up), presentation of U0152 (R-lambda rate control), and presentation of new U0171 (palette with pixel copy)

- Sun. 21 June, 3rd day

  - 0900–1100 Track A: U0152 (R-lambda rate control), U0171 and U0116 (palette with pixel copy), SCC other (U0106 ACT at TU level, U0119 slice segment freeze signalling, U0138 deblocking; section 5.1.6)

    - Remaining open included revisit of U0152, U0119 slice segment freeze signalling, U0138 deblocking, other topics from Saturday.

  - 1130–1300 JCT-VC plenary

  - 1500–1645 Track A: U0052, U0119, U0138, SCC ballot comments m36255, U0128

    - Remaining open included revisit of U0128

- Mon. 22 June, 4th day

  - 16:30-18:00 Joint meeting with VCEG, MPEG Requirements, and MPEG Video

  - 18:00-21:00 Track B (with 2 BoG reviews)

- Tue. 23 June, 5th day

  - 11:00-13:00 Track A U0090/U0169/U0176, U0128

  - 17:30-18:00 Joint meeting with MPEG Systems and Video on dynamic resolution adaptation for video streaming

- Wed. 24 June, 6th day

  - 11:30-1300 Remainders

- Thu. 25 June, 7th day

  - 09:00-10:45 Remainders

  - 14:30-15:15 Joint meeting with VCEG, MPEG Requirements, MPEG Video, and JCT-3V

  - 16:00-18:00 Remainders

  - 18:30-20:00 Evening session on SCC performance analysis (U0051, U0058), Non-normative U0095 encoder speed-up, and U0152 R-Lambda (no presenter available for new input U0186)

- Fri. 26 June, 8th day

  - 09:00-12:40 Remainders (incl. SHVC verification testing U0037, U0040 bug fixing, new input U0186 for which no presenter was available Thu 06-25 20:00 or Fri 12:00 and so was not presented but is available for study), AHG status review for AHGs 3 (software), 4 (conformance), 9 (SCC complexity), 10 (test sequences), CEs (1), AHGs, CTCs (added test sequence), software plan, outputs

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized and categorized into "tracks" (A, B, or P) for "parallel session A", "parallel session B", or "Plenary" review, as follows. Discussions on topics categorized as "Track A" were primarily chaired by Gary Sullivan, whereas discussions on topic categorized as "Track B" were primarily chaired by Jens-Rainer Ohm. Some plenary sessions were chaired by both co-chairmen, and others were chaired by Gary Sullivan. Chairing of other discussions is noted for particular topics. (Note: Allocation to tracks was subject to changes)

- AHG reports (13) Track P (section 2)

- Project development status (15 + 10 HDR) Track P (section 3)

- SCC CE1: Palette mode improvements (14) Track A (section 4.1)

- SCC CE2: IBC memory access (8) Track B (section 4.2)

- Non-CE SCC (86) (section 5.1) with subtopics

  - CE1 Palette mode related (49) Track A (section 5.1.1)

  - CE2 Intra block copy related (1), Track B (section 5.1.2)

  - IBC/MC unification (28), Track B (section 5.1.3)

  - Other IBC related (5), Track B (section 5.1.4)

  - Tool complexity (0), Track P (section 5.1.5)

  - Other (4), Track A & Track P (section 5.1.6)

- High-level syntax (0) Track A (section 5.2)

- VUI and SEI messages (3+4HDR) Track A (section 5.3)

- Non-normative (6) Track A (section 5.4)

- Plenary discussions and BoG reports (2) Track P (section 6)

- Outputs & planning: AHG & CE plans, Conformance, Reference software, Verification testing, Chroma format, CTC (sections 7, 8, and 9)

NOTE – The number of contributions in each category, as shown in parenthesis above, may not be 100% precise.

# AHG reports (13)

The activities of ad hoc groups (AHGs) that had been established at the prior meeting are discussed in this section.

(Consideration of these reports was chaired by GJS & JRO on Friday 06-19, 0900-1100, except as noted.)

JCTVC-U0001 JCT-VC AHG report: Project management (AHG1) [G. J. Sullivan, J.-R. Ohm (AHG co-chairs)]

This document reports on the work of the JCT-VC ad hoc group on Project Management, including an overall status report on the project and the progress made during the interim period since the preceding meeting.

In the interim period since the 19th JCT-VC meeting, the following (12) documents had been produced:

- The HEVC test model (HM) 16 improved encoder description (including RExt modifications) update 2;

- Verification test plan for format range extensions and interlaced video;

- The RExt reference software draft 2 and conformance testing draft 4, the latter combined with improved version 1 conformance testing;

- The SHVC reference software draft 1, conformance testing draft 2 and SHM test model 9;

- For HEVC screen content coding (SCC) extensions, the HEVC screen content coding test model 4, SCC draft text 3, and a document specifying common test conditions and software reference configurations for SCC experiments.

- Finalized versions of two Core Experiment reports for SCC (CE1: Improvements of palette mode, and CE2: Intra block copy memory access.

Furthermore, two Core Experiments on screen content coding tools (CE1 and 2) were run. Advancing the work on development of conformance and reference software for HEVC extensions was also a significant goal.

The work of the JCT-VC overall had proceeded well and actively in the interim period with a considerable number of input documents to the current meeting. Active discussion had been carried out on the group email reflector (which had 1695 subscribers as of 2015-06-15), and the output documents from the preceding meeting had been produced.

Except as noted below, output documents from the preceding meeting had been made available at the "Phenix" site (http://phenix.it-sudparis.eu/jct/) or the ITU-based JCT-VC site (http://wftp3.itu.int/av-arch/jctvc-site/2015_06_U_Warsaw/), particularly including the following:

- The meeting report (JCTVC-T1000) [Posted 2015-06-19]

- The HM 16 improved encoder description update 2 (JCTVC-T1002) [Posted 2015-06-02]

- Verification test plan for interlaced video and format range extensions (JCTVC-T1003) [First posted 2015-02-19, last updated 2015-04-17]

- HEVC screen content coding draft 3 (JCTVC-T1005) [Posted 2015-04-05]

- SHVC Test Model 9 (JCTVC-T1007) [Posted 2015-06-20]

- SHVC Conformance Testing Draft 2 (JCTVC-T1008) [First posted 2015-03-27, last updated 2015-03-30]

- HEVC Reference Software for Format Range Extensions Profiles Draft 2 (JCTVC-T1011) [First posted 2015-04-15, last updated 2015-04-28]

- Range Extensions Conformance Testing Draft 4, and improved Version 1 Conformance Testing (JCTVC-T1012) [Posted 2015-04-20]

- Screen Content Coding Test Model 4 Encoder Description (JCTVC-T1014) [Posted 2015-06-01]

- Common SCC test conditions (JCTVC-T1015) [First posted 2015-03-13, last updated 2015-04-15]

- Description of Core Experiment 1 (CE1): Improvements of palette mode (JCTVC-T1101) [First posted 2015-02-22, last updated 2015-03-31]

- Description of Core Experiment 2 (CE2): Intra block copy memory access (JCTVC-T1102) [First posted 2015-02-23, last updated 2015-03-16]

The thirteen ad hoc groups and the three core experiments had made progress, and various reports from those activities had been submitted.

The different software modules (HM16.4, SHM9.0 and SCM4.0) had been prepared and released with appropriate updates approximately as scheduled. Both SHM and SCM are implemented as branches from HM16.

Since the approval of software copyright header language at the March 2011 parent-body meetings, that topic seems to be resolved.

Released versions of the software are available on the SVN server at the following URL:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/version_number,
where version_number corresponds to one of the versions described below – e.g., HM-16.4.

Intermediate code submissions can be found on a variety of branches available at:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/branch_name,
where branch_name corresponds to a branch (eg., HM-16.4-dev).

Various problem reports relating to asserted bugs in the software, draft specification text, and reference encoder description had been submitted to an informal "bug tracking" system (https://hevc.hhi.fraunhofer.de/trac/hevc). That system is not intended as a replacement of our ordinary contribution submission process. However, the bug tracking system was considered to have been helpful to the software coordinators and text editors. The bug tracker reports had been automatically forwarded to the group email reflector, where the issues were discussed – and this is reported to have been helpful. It was noted that contributions had generally been submitted that were relevant to resolving the more difficult cases that might require further review.

The ftp site at ITU-T is used to exchange draft conformance testing bitstreams. The ftp site for downloading bitstreams is http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/.

A spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available in the same directory. It includes the list of bitstreams, codec features and settings, and status of verification.

Approximately 100 input contributions to the current meeting had been registered as of the document deadline. A significant number of late-registered and late-uploaded contributions were noted, although most of these were cross-check documents.

A preliminary basis for the document subject allocation and meeting notes for the 21st meeting had been circulated to the participants by being announced in email, and was publicly available on the ITU-hosted ftp site.

JCTVC-U0002 JCT-VC AHG report: HEVC test model editing and errata reporting (AHG2) [B. Bross, C. Rosewarne (AHG co-chairs), M. Naccari, J.-R. Ohm, K. Sharman, G. J. Sullivan, Y.-K. Wang (AHG vice-chairs)]

This document reports the work of the JCT-VC ad hoc group on HEVC test model editing and errata reporting (AHG2) between the 20th meeting in Geneva, CH (February 2015) and the 21st meeting in Warsaw, PL (June 2015).

An issue tracker (https://hevc.hhi.fraunhofer.de/trac/hevc) was used in order to facilitate the reporting of errata with the HEVC documents.

The High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Update 2 of Encoder Description was produced as JCTVC-T1002. This document represented a refinement of the previous HM16 Improved Encoder Description document (JCTVC-S1002). In particular, numerous tool descriptions were improved and description of high level syntax and picture types was expanded. The resultant document provides a source of general tutorial information on HEVC Edition 1 and Range Extensions, together with an encoder-side description of the HM-16 software.

A discrepancy between the software and the HEVC Version 2 text has been identified that affects the CABAC alignment tool present in the high throughput profile. This error was first reported on the bug-tracker (ticket #1391), where it has been indicated that the software implements the intended function. A related contribution (JCTVC-U0031) has been submitted in relation to this discrepancy.

See the notes on JCTVC-U0031.

JCTVC-U0003 JCT-VC AHG report: HEVC HM software development and software technical evaluation (AHG3) [K. Suehring, K. Sharman, D. Flynn (AHG co-chairs)]

(Consideration of this report was chaired by GJS on Friday 06-26, 10:05-10:20.)

This report summarizes the activities of the AhG on HEVC HM software development and software technical evaluation that have taken place between the 20th and 21st JCT-VC meetings. Activities focused on software maintenance, i.e. code tidying and fixing bugs.

A brief summary of activities related to each mandate is given below.

- One change to the common test conditions (JCTVC-S0090) and one software change (JCTVC-T0196) were adopted at the last meeting. Two further software changes (JCTVC-T0216, JCTVC-T0225) were to be applied pending further consideration by the AHG3 chairs, but actually patches to just one (JCTVC-T0225) was supplied. This was duly considered and modified to address concerns by the chairs, and the patch was applied (r4487).

- In addition, numerous bug fixes and cleanups were addressed. The distribution of the software was made available through the SVN server set up at HHI, as announced on the JCT-VC email reflector, and http://hevc.info has been updated.

- The HM user manual had been updated and a version controlled copy was included in the doc directory of the repository. A PDF version had been produced and was included in the same location prior to each HM release.

- Version releases were reported as follows:

  - Version 16.4 was released on Mar. 9, 2015;

  - Version 16.5 was released on Apr. 21, 2015;

  - Version 16.6 was released on Jun. 18, 2015.

- There were a number of reported software bugs that remained in need of fixing.

- A mismatch between software and text for the High Throughput profile had been identified and a contribution had been filed to this meeting (JCTVC-U0031).

Additional details on the work done for each released version were provided in the report.

Substantial improvements of the software were reported.

The following were persistent bug reports where study was encouraged:

- High level picture types: IRAP, RASL, RADL, STSA: (Tickets #1096, #1101, #1333, #1334, #1346)

- Rate-control and QP selection – numerous problems with multiple slices (Tickets #1314, #1338, #1339 #1345)

- Field-coding (Tickets #1145, #1153)

- Decoded picture buffer (Tickets #1277, #1286, #1287, #1304)

- NoOutputOfPriorPictures processing (Tickets #1335, #1336, #1393)

- Additional decoder checks (Tickets #1367, #1383)

A recurring problem had been compiling the software for other architectures, such as ARM, where the C-type 'char' had been producing different results: on some compilers, 'char' is interpreted as unsigned, on others, 'char' is interpreted as signed. The HM 'Char' type maps directly to 'char'. To resolve this problem, a bug fix has been proposed whereby the HM 'Char' type is replaced with an 'SChar' type (C's signed char), which is comparable with the other types in the system, and a 'TChar' type (C's char) used for text. The patch was available and the editors wished to include this for the future HM16.7 release.

In addition, as described to the community at the last two JCT-VC meetings, alterations to remove the unused software hierarchy in the entropy coding sections of the code, and to remove terms such as CAVLC were being considered.

Recommendations of the AHG were to:

- Continue to develop reference software based on HM version 16.5 and improve its quality.

- Test reference software more extensively outside of common test conditions

- Add more conformance checks to the decoder to more easily identify non-conforming bitstreams, especially for profile and level constraints.

- Encourage people who are implementing HEVC based products to report all (potential) bugs that they are finding in that process.

- Encourage people to submit bitstreams that trigger bugs in the HM. Such bitstreams may also be useful for the conformance specification.

JCTVC-U0004 JCT-VC AHG report: HEVC conformance test development (AHG4) [T. Suzuki, J. Boyce, K. Kazui, A. K. Ramasubramonian, Y. Ye (AHG co-chairs)]

(Consideration of this report was chaired by GJS on Friday 06-26, 10:20-10:30.)

The ftp site at ITU-T is used to exchange bitstreams. The ftp site for downloading bitstreams is

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/.

The spreadsheet to summarize the status of bitstream exchange for conformance bitstream generation was available at this directory. It included the list of bitstreams, codec features and settings, and status of verification.

The guideline to generate the conformance bitstreams is summarized in JCTVC-O1010.

JCTVC-S1004 (output document from Strasbourg meeting) summarizes the defects of the current conformance bitstreams for HEVC v.1. After the Strasbourg meeting, the following problems were revised. All known problems were resolved. The revised bitstreams were uploaded at the following site, separating the bitstreams under ballot.

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/under_test/.

Two bitstreams to test SAO corner cases were provided and agreed to include in the RExt conformance modifications, however, the first version of the bitstream included bugs. Those were fixed and the latest bitstreams were made available as

- SAODBLK_A_Divx_3

- SAODBLK_B_Divx_3

Those were to be included in the next draft of RExt conformance.

The bitstreams that were originally planned to be generated were listed in the report, with highlighting for the bitstream cases that had not been generated yet. The highlighted cases included the following:

- Maximum number of motion vectors permitted per CTU

- Motion vectors that are random, with maximum page DRAM misses / minimize cache hits (according to memory bandwidth model, e.g., JCTVC-H007)

- Test collocated_ref_idx (single slice or multiple slice/picture with different reference lists)

- Bitstream with largest possible CPB delay (maximum number of coded bits per picture but within MinCR)

- High throughput binarization

- Maximum number of slices permitted for Profile & Level combination; Each slice has unique short_term_ref_pic_set(), ref_pic_list_modification() and pred_weight_table() with maximum num_long_term_pics

- CTU size is 64x64 with no cu splits, and only one coded transform unit in the lower right 4x4 pixel corner of the CTU with delaQp signalled  (as raised by P. Kaspenberg on the JCT reflector 12-Sept-2012)

- A wide range of values for pic_crop_left_offset, pic_crop_right_offset, pic_crop_top_offset, pic_crop_bottom_offset

- Inferred splits along non-CTU multiple picture borders

- Combinations of transform_skip_enabled_flag, transquant_bypass_enable_flag, and cu_transquant_bypass_flag

- Something to test a vast variety of POC signalling cases.

- pic_width_in_luma_samples = pic_height_in_luma_samples = (1 << Log2MinCbSize), with various CTU sizes.

For RExt conformance, the bit depths and chroma format to cover in RExt conformance are as follows.

- 4:0:0, 4:2:0, 4:2:2, 4:4:4

- 8, 10, 12, 16b

Profiles in RExt are as follows.

Monochrome 12, Monochrome 16, Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12, Main Intra, Main 10 Intra, Main 12 Intra, Main 4:2:2 10 Intra, Main 4:2:2 12 Intra, Main 4:4:4 10 Intra, Main 4:4:4 12 Intra, and Main 4:4:4 16 Intra profiles

The initial idea of bitstream features was summarized in a table. Some of the bitstreams had already been generated and were available at the following site.

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/RExt/.

Some of the missing features had already covered by other bitstreams. It was suggested that it may be OK to remove such features from the table, and that remaining missing bitstreams should be generated as soon as possible.

Volunteers that had been identified were listed.

The draft SHVC conformance plan was available in JCTVC-T1008, including instructions for generation of bitstreams.

The list of the 38 bitstream categories and their status was listed in section 2.6.1 of the report.

Volunteers were identified for the bitstreams in 37 categories. One categories that did not have an identified volunteer and additional volunteers were requested. The following 13 companies had volunteered to participate in generation of SHVC conformance bitstreams: BBC, Disney, Ericsson, ETRI, InterDigital, Fraunhofer HHI, Nokia, Qualcomm, Samsung, Sharp, Sony, Technicolor, and Vidyo.

At the start of the meeting, 58 bitstreams in 33 categories had made been available, with 4 categories not yet available.

JCTVC-U0005 JCT-VC AHG report: HEVC and range extensions verification test preparation (AHG5) [V. Baroncini, M. Karczewicz, M. Naccari, N. Ramzan, C. Rosewarne, T. K. Tan, J.-M. Thiesse, W. Wan (AHG co-chairs)]

(Consideration of this topic was chaired by GJS on Wednesday 06-24, 12:00-13:00.)

This report summarizes the Verification test preparation Ad-Hoc Group (AHG5) activities between the 20th JCT-VC meeting in Geneva, CH (February 2015) and the 21st JCT-VC meeting in Warsaw, PL (June 2015).

Verification test preparation Ad-Hoc Group (AHG5) had been asked to do the following:

- Make preparations for verification testing of HEVC for interlaced video content.

- Make preparations for verification testing of HEVC format range extensions.

The final version of the “Verification test plan for interlaced video and format range extensions” (JCTVC-T1003) had been produced and uploaded.

A request for financial support had been issued on the 27th of March 2015 in order to cover the expenses of the testing laboratory "Test & Write" led by Vittorio Baroncini.

The bitstreams corresponding to the tests specifications described in the document JCTVC-T1003 were generated by Canon.

For particular RExt cases, the subjective quality between different bit-rates was deemed too small for effective subjective testing. For these cases, encoding at lower bit-rates was performed.

The HEVC interlace and format range extensions subjective verification tests were performed by Vittorio Baroncini (Test & Write).

A report of the interlaced-video subjective verification tests was produced and uploaded as JCTVC-U0184.

A related contribution regarding result presentation is also noted (JCTVC-U0183).

The AHG5 members thanked the JCT-VC chairmen Gary Sullivan and Jens-Rainer Ohm for providing insight on the content of published documents and for publicizing the request for financial support as well as personally establishing the communication between some participating companies and the ad-hoc group members.

The AHG5 members also thanked the six companies whose financial support allowed the completion of the verification tests of interlace coding capability and Range Extension coding capability. These companies, which financially supported this activity, are:

- CBS Corporation

- The Fraunhofer Heinrich Hertz Institute (HHI)

- Microsoft Corporation

- Sony Corporation

- Cable Television Laboratories, Inc. (CableLabs)

- Fujitsu Laboratories Ltd.

The AHG recommended to approve the report of the interlaced video and format range extensions subjective verification tests (JCTVC-U0184).

See also the notes for JCTVC-U0184.

JCTVC-U0006 JCT-VC AHG report: SCC coding performance analysis (AHG6) [H. Yu (AHG chair), R. Cohen, A. Duenas, P. Lai, K. Rapaka, J. Xu (AHG vice-chairs)]

This report summarizes the activities of the JCT-VC ad hoc group on SCC coding performance analysis (AHG6) between the JCT-VC 20th meeting in Geneva, Switzerland, and the 21st meeting in Warsaw, Poland.

Per the decisions captured in the meeting notes JCTVC-T_Notes_d8, the following changes have been made in JCTVC-T1015 “Common Test conditions for screen content coding”:

- Added 10 new 4:2:0 sequences; they were converted by sub-sampling the original 4:4:4 test sequences with simple filters described in the meeting notes.

  - Uploaded these sequences to the ftp site for all test material

  - provided updated version of results reporting templates (Lossy and Lossless)

- Added 4x1 CTU IBC test conditions for 4:4:4 sequences; provided associated encoder settings as HashBasedIntraBlockCopySearchEnabled = 1; IntraBlockCopySearchWidthInCTUs = 3; IntraBlockCopyNonHashSearchWidthInCTUs = 1.

Three related contributions were noted:

- JCTVC-U0150: New SCC test sequence for consideration [W. Wang, M. Xu, F. Duanmu, H. Yu (Huawei)] (later uploaded as JCTVC-U0188)

- JCTVC-U0051: Comparison of Compression Performance of HEVC Test Model 16.4 and HEVC Screen Content Coding Extensions Test Model 4 with AVC High 4:4:4 Predictive profile [B. Li, J. Xu, G. J. Sullivan (Microsoft)]

- JCTVC-U0058: Compression Performance of HEVC Screen Content Coding Extensions Test Model 4.x with slices [C. Gisquet, G. Laroche, P. Onno (Canon)]

It was recommended to:

- Review the proposed new test material.

- Discuss further improvements to CTC

- Continue to evaluate the coding performance of the newly adopted tools and their interaction with the existing HEVC tools in the Main profile and range extensions.

JCTVC-U0007 JCT-VC AHG report: SCC extensions text editing (AHG7) [R. Joshi, J. Xu (AHG co-chairs), R. Cohen, S. Liu, Z. Ma, G. Sullivan, Y. Ye (AHG vice-chairs)]

This document reports on the work of the JCT-VC ad hoc group on SCC extensions text editing (AHG7) between the 20th JCT-VC meeting in Geneva, Switzerland (February 2015).and the 21st JCT-VC meeting in Warsaw, Poland (June 2015).

The third working draft for the High Efficiency Video Coding Screen Content Coding (HEVC SCC) extension was created as an output document following the decisions taken at the 20th JCT-VC meeting in Geneva, Switzerland (February 2015).

JCTVC-T1005 was produced by the editing ad hoc group (AHG) following the 20th JCT-VC meeting in Geneva, Switzerland. One significant change was that JCTVC-T1005 is based on HEVC v2 (without the 3D-HEVC extension yet integrated). Two versions JCTVC-T1005 were produced. The two are identical except that the editorial notes have been removed from the second version of the document. The text of JCTVC-T1005 (version 2) was submitted to ISO/IEC JTC1/SC29 for a ISO/IEC PDAM ballot (23008-2/PDAM 5).

List of changes with respect to JCTVC-S1005:

- Converted the document to use the complete HEVC version 2 text as its basis (without the 3D-HEVC extension yet integrated).

- Integrated palette escape colour coding (JCTVC-T0112/T0118)

- Integrated IBC unified with inter (JCTVC-T0227)

- Integrated Palette extension to non-4:4:4 format (JCTVC-T0072/T0109/T0120)

- Integrated intra MPM fix for SCC (JCTVC-T0049)

- Integrated disabling 64x64 palette (JCTVC-T0058)

- Integrated context reduction for coding run_type in palette (JCTVC-T0078)

- Integrated signalling delta maximum palette predictor size in SPS (JCTVC-T0134)

- Integrated HLS refinement for adaptive motion vector resolution (JCTVC-T0069)

- Integrated Inference for palette_escape_val_present_flag (JCTVC-T0183)

- Integrated palette predictor initialization entries in PPS (JCTVC-T0048)

- Integrated removal of palette sharing flag (JCTVC-T0064)

- Integrated EG0 for new palette entries (JCTVC-T0064)

- Integrated content light level information SEI message (JCTVC-T0101)

- Integrated clipping MV to 16-bit for AMVR (Notes of JCTVC-T0059)

- Integrated QP offset for different colour spaces (JCTVC-T0140)

- Integrated clipping before ACT (JCTVC-T0132)

The screen content coding test model 4 (SCM 4) (document JCTVC-T1014) was released on 1st, June 2015. The main changes were the description of unified signalling for intra block copy and inter modes.

Related contributions were noted as follows:

- JCTVC-U0038 proposes editorial improvements to address feedback and comments related to the SCC draft text 3. It also summarizes omissions (e.g., two forgotten SEI messages – the dependent RAP picture SEI message and the coded region completion SEI message) and known open issues (inclusion of errata and confusion between intra/inter and IBC).

- JCTVC-U0031, JCTVC-U0091 and JCTVC-U0099 point to mismatch between draft text 3 and SCM 4.0 software.

- JCTVC-U0036, JCTVC-U0055, and JCTVC-U0089, although containing normative aspects, propose constraints on syntax elements.

- JCTVC-U0092, JCTVC-U0110 are proposals for general editorial improvements/cleanups of the draft text.

The recommendations of the HEVC SCC extension draft text AHG are to:

- Approve the documents JCTVC-T1005 and JCTVC-T1014 as JCT-VC outputs

- Address the comments and feedback on SCC extensions text specification as appropriate

- Compare the HEVC SCC extensions document with the HEVC SCC extensions software and resolve any discrepancies that may exist, in collaboration with the SCC extension software development (AHG8)

- Review the related contributions

JCTVC-U0008 JCT-VC AHG report: SCC extensions software development (AHG8) [B. Li, K. Rapaka (AHG co-chairs), R. Cohen, P. Chuang, X. Xiu, M. Xu (AHG vice-chairs)]

This report summarizes the activities of Ad Hoc Group 8 on screen content extensions software (SCM) development that have taken place between the JCT-VC 20th meeting in Geneva, Switzerland, and the 21st meeting in Warsaw, Poland.

Multiple versions of the HM SCM software were produced and SCM4.0 was announced on the JCT-VC email reflector. The integration details and performance summary of these revisions are provided in the next subsections. The performance results of software revisions were observed to be consistent with the adopted techniques.

HM-16.4_SCM-4.0 was tagged on the SVN HHI repository on March 18th 2014. This release includes following adoptions:

- JCTVC-T0048: Palette_predictor_initializer at PPS-level

- JCTVC-T0058: Disallow palette mode for 64x64 CUs

- JCTVC-T0063: EG0 signalling for palette_num_signalled_entries

- JCTVC-T0064: Remove the palette sharing flag and its context

- JCTVC-T0065: Grouping Palette Indices

- JCTVC-T0069: AMVR high-level syntax cleanup

- JCTVC-T0072/T0109/T0120: Palette Coding for non-4:4:4 format content

- JCTVC-T0078: Remove context in palette run mode

- JCTVC-T0087: Palette Table Generation

- JCTVC-T0116: Encoder improvements on IBC search

- JCTVC-T0118/T0112: On escape colour coding

- JCTVC-T0121: Encoder operation to infer split_transform_flag

- JCTVC-T0132: ACT dynamic range control

- JCTVC-T0134: Use delta to signal palette predictor size

- JCTVC-T0140: QP offset for different colour spaces

- JCTVC-S0180/S0150/Meeting notes: Grouping escape at the end

- JCTVC-T0183: Add inference rule when palette_escape_val_present_flag is not present

- JCTVC-T0227: Intra block copy and inter signalling unification

- SW Bug fix:  For high bit-depth test configuration

- Meeting notes:  Double the palette and palette predictor size

- Ticket#1376: Incorrect search range for Nx2N

- Migration to HM-16.3, HM-16.4.

The main changes to the configuration files are:

- Addition of 4:2:0 configuration files as in JCTVC-T1015

- Temporal scalability is supported in the RA test conditions.

The following adoptions had not been integrated to SCM 4.0 (These do not impact CTC):

- JCTVC-T0048/T0055/T0056:  IntraBC constraint for multiple slices/tiles SW fix. These aspects are integrated to SCC branch targeting SCM 4.1

- JCTVC-S0043 – palette delta QP coding: During integration, it was observed that original software palette design (from SCM 2.0) does not properly handle changing QP's across CU's (a functionality that is available in HM using –d options). This issue was brought to attention of palette experts and investigation is ongoing.

The release was announced on the email reflector. The software can be downloaded at

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.4+SCM-4.0/

The performance of this version against HM-16.2+SCM-3.0 was described according to the common test conditions in JCTVC-S1015. For the lossy 4:4:4 configuration, it is reported that this version provides BD-rate reduction of 1.7%, 3.4% and 2.9% for RGB 1080p & 720p text and graphics category in AI/RA/LB configurations respectively and BD-rate reduction of 1.0%, 3.2% and 2.8% for YUV 1080p & 720p text and graphics category in AI/RA/LB configuration, respectively. For the lossless 4:4:4 configuration, it is reported that this version provides BD-rate reduction of 1.0%, −0.4% and −0.2% for RGB 1080p & 720p text and graphics category in AI/RA/LB configurations respectively and BD-rate reduction of 0.7%, −0.6% and −0.3% for YUV 1080p & 720p text and graphics category in AI/RA/LB configuration, respectively.

The tables below summarize BD-rate change for lossy and lossless 4:4:4 configurations respectively.

BD-Rate change in Lossy 4:4:4 configuration

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | −1.7% | −3.0% | −2.9% |
| RGB, mixed content, 1440p & 1080p | −1.1% | −2.4% | −2.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | −1.0% | −2.4% | −2.4% |
| YUV, mixed content, 1440p & 1080p | −1.2% | −2.5% | −2.2% |
| YUV, Animation, 720p | 0.0% | −0.2% | −0.3% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 116% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | −3.4% | −5.0% | −4.9% |
| RGB, mixed content, 1440p & 1080p | −1.2% | −3.3% | −3.3% |
| RGB, Animation, 720p | 0.2% | 0.0% | −0.2% |
| RGB, camera captured, 1080p | −0.1% | −0.1% | −0.3% |
| YUV, text & graphics with motion, 1080p & 720p | −3.2% | −5.6% | −5.9% |
| YUV, mixed content, 1440p & 1080p | −1.7% | −4.3% | −3.9% |
| YUV, Animation, 720p | 0.3% | −0.3% | −0.2% |
| YUV, camera captured, 1080p | −0.2% | 0.1% | −0.1% |
| Enc Time[%] | 87% |  |  |
| Dec Time[%] | 87% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | −2.9% | −4.4% | −4.2% |
| RGB, mixed content, 1440p & 1080p | −1.7% | −3.7% | −3.8% |
| RGB, Animation, 720p | 0.1% | −0.1% | −0.2% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | −2.8% | −4.6% | −5.1% |
| YUV, mixed content, 1440p & 1080p | −2.6% | −5.9% | −5.2% |
| YUV, Animation, 720p | 0.1% | −0.1% | −0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 85% |  |  |
| Dec Time[%] | 86% |  |  |

BD-Rate change in Lossless 4:4:4 configuration

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | −1.0% | −0.9% | −2.9% | 0.2% |
| RGB, mixed content, 1440p & 1080p | −1.1% | −0.9% | −2.3% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | −0.7% | −0.6% | −3.2% | 1.0% |
| YUV, mixed content, 1440p & 1080p | −0.5% | −0.4% | −1.6% | 0.3% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 111% |  |  |  |
| Dec Time[%] | 97% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.4% | −0.2% | −1.5% | 1.2% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.0% | −0.1% | 0.1% |
| RGB, Animation, 720p | 0.7% | 0.7% | 0.7% | 0.7% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.6% | −0.4% | −4.2% | 1.8% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.1% | −0.1% | 0.2% |
| YUV, Animation, 720p | 0.5% | 0.5% | 0.5% | 0.5% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 118% |  |  |  |
| Dec Time[%] | 103% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.2% | −0.5% | −2.4% | 0.8% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.3% | −0.7% | −3.7% | 1.3% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 126% |  |  |  |
| Dec Time[%] | 117% |  |  |  |

HM-16.4_SCM-4.0rc1 was tagged on the SVN HHI repository on March 13th 2015 before the release of HM-16.4_SCM-4.0 to allow proponents to cross-check integrations and interactions with other adoptions. Further minor cleanup and bug fix were done before tagging HM-16.4_SCM-4.0.

The release was announced on the email reflector. The software can be downloaded at https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.4+SCM-4.0rc1/

HM-16.3+SCM-3.2 has been tagged on HHI Server on February 25th 2015 and can be downloaded at https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.3+SCM-3.2/

The changes included in this release were:

- Updated SCM bit-depth support is enabled.

- Removal of macros related to previous integrations.

- Bug fix when high bit-depth support is enabled.

The JCT-VC issue tracker at https://hevc.hhi.fraunhofer.de/trac/hevc/ has been updated to allow bug reports to be entered for SCM, currently under milestone HM+SCC-5.0, version SCC-4.0.

Several changes were made after the release of SCM-4.0. Among them, the following changes have impact on CTC:

- r4450, fix ticket 1311, which has minor performance impact on 4:4:4 lossless coding (when ACT is enabled).

- r4451, Intra BC clean up to align with spec, which requires to signal RPLM for RA coding under CTC (when Intra BC is enabled). Additional bits are signalled at slice header.

Recommendations of the AHG were:

- Continue to develop reference software based on HM16.4_SCM4.0 and improve its quality and release HM16.4_SCM4.1.

- Remove macros introduced in previous versions before starting integration towards SCM-3.0 such as to make the software more readable.

- Continue merging with later HM versions.

Release of SCM 4.1 (based on HM 16.5) was expected to occur on 06-29, just after the current meeting.

JCTVC-U0009 JCT-VC AHG report: Complexity of SCC extensions (AHG9) [A. Duenas (AHG chair), M. Budagavi, R. Joshi, S.-H. Kim, P. Lai, W. Wang, X. Xiu (AHG vice-chairs)]

(Consideration of this report was chaired by GJS on Friday 06-26, 10:30-10:40.)

This document reports on the work of the JCT-VC ad hoc group on Complexity of SCC extensions (AHG9) between the 20th JCT-VC meeting in Geneva, Switzerland (February 2015) and the 21st JCT-VC meeting in Warsaw, Poland (June 2015).

Relevant contributions were noted by the AHG as follows:

- Document on CE1 Test B (JCTVC-U0050) was related to the complexity of index adjustment in palette mode. All the documents classified as CE2 were related to memory bandwidth issues in IBC.

- Other related documents included the following:

  - JCTVC-U0052: On the palette escape pixel coding

  - JCTVC-U0065: CE2-related: Intra block copy searching constraints for reducing worst case bandwidth

  - JCTVC-U0086: CE1-related: Simplification on coding NumPaletteIndices

  - JCTVC-U0090: CE1-related: Palette Mode Context and Codeword Simplification

  - JCTVC-U0093: Non-CE1: On Number of Palette Indices coding

  - JCTVC-U0097: Non-CE1: On maximum palette predictor size

  - JCTVC-U0099: Modified method for sending number of palette indices

  - JCTVC-U0100: On operation of DPB in Screen Content Coding

  - JCTVC-U0105: Non-CE1: Simplification on deriving NumPaletteIndices for palette mode

  - JCTVC-U0109: Non-CE1: Simplification for parsing the index of the most significant bit in the paletteRun binarization

  - JCTVC-U0148: Non-CE1: Context model unification for palette run type flags in HEVC SCC

  - JCTVC-U0149: Non-CE1: Index Map Splitting (IMS) Mode for Palette Coding in HEVC SCC

JCTVC-U0010 JCT-VC AHG report: Test sequence material (AHG10) [T. Suzuki, V. Baroncini, R. Cohen, T. K. Tan, S. Wenger, H. Yu (AHG co-chairs)]

(Consideration of this report was chaired by GJS on Friday 06-26, 10:30-10:40.)

A new SCC test sequence contribution was noted: JCTVC-U0150 (later uploaded as JCTVC-U0188): New SCC test sequence for consideration [W. Wang, M. Xu, F. Duanmu, H. Yu (Huawei)]

This contribution proposes a new test sequence for the SCC CTC. This sequence consists of content with Chinese text editing. The coding results from SCM-4.0 demonstrate different statistics in the usage of coding tools, when compared with the existing SCC test sequences.

The AHG report also listed the known test sequences that were available for use, including:

- HEVC v.1 CTC sequences (available on ftp://hevc@ftp.tnt.uni-hannover.de/testsequences/; login information available from the JCT-VC chairs for qualified participants)

- HEVC v.1 Verification Test sequences (reportedly to be available soon)

- HEVC Range Extensions CTC sequences (available on ftp://hevc@ftp.tnt.uni-hannover.de/testsequences/FrExt-candidate-sequences/; login information available from the JCT-VC chairs for qualified participants)

A number of the new sequences were kindly provided by the EBU. To use these materials, users should register with the EBU to gain access to the ftp server containing the master sequences:

http://tech.ebu.ch/testsequences/uhd-1_public_form

Copies of these sequences in appropriate formats have been made available on the uni-hannover site. However, so as to not hinder the EBU's aims, the zip files are password protected using the same password obtained from completing the form at the above URL.

- HEVC screen content sequences (available on ftp://hevc@ftp.tnt.uni-hannover.de/testsequences/FrExt-candidate-sequences/screen_content/;
login information available from the JCT-VC chairs for qualified participants)

The AHG recommended

- To continue to create the list of test sequences available for HEVC development including licensing statements

- To review all related contributions

- To continue to collect test materials

JCTVC-U0011 JCT-VC AHG report: SHVC test model editing (AHG11) [J. Chen (AHG chair), J. Boyce, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang, Y. Ye (AHG vice-chairs)]

This document reports the work of the JCT-VC ad hoc group on SHVC test model editing (AHG11) between the 20th JCT-VC meeting in Geneva, Switzerland (10–18 Feb. 2015) and the 21th JCT-VC meeting in Warsaw, Poland (19–26 June 2015).

This AHG also collected text problem reports.

During this period, the editorial team worked to improve the ISO/IEC 23008-2 document (HEVC version 2 specification text) prior to its publication. The following refinements and bug fixes have been included and forwarded to the SC 29 secretary for incorporation into the publication process:

- Fix: remove one inference rule of sub_layer_tier_flag, there are two inference rules for of sub_layer_tier_flag in current spec (this fix was already integrated into the published ITU-T HEVC version 2)

- Editorial fix: adding a missing "the" and removing an unnecessary and confusing “less than or” in the semantics of syntax element part_mode in 7.4.9.5 Coding unit semantics

- Fix: adding missing inference rule for the syntax element collocated_ref_idx

- Clarification fix: replacing "chroma residual samples" by "chroma prediction samples" in 8.5.3.3 Decoding process for inter prediction samples

- Fix: wrong the inputs (passing samples) of deblocking process invocation in 8.7.2.5.3 Decision process for luma block edges

- Fix: replacement of "buffering period" with "active parameter sets" and inserting a missing “access unit with" in an informative NOTE in D.3.3 (Picture timing SEI message semantics)

- Typo fix: fixing titles of clauses F.6, G.6, H.6 to correspond to titles in subclauses of clause 6

- Typo fix: missing changes from RExt specification in F.7.3.2.2.1 General sequence parameter set RBSP syntax

- Editorial fix: missing changes from RExt specification in F.3.7.6.1 General slice segment header syntax

- Minor typo fix in F.7.3.2.3.3 Picture parameter set multilayer extension syntax

- Fix: removal of "for each access unit" in the note on BaseLayerOutputFlag which is not access unit specific, in F.8.1.2 CVSG decoding process

- Fix: wrong index for the derivation of LayerInitializedFlag in F.8.1.3 Common decoding process for a coded picture

- Fix: changing " discardable_flag equal to 1" to " discardable_flag equal to 0" in one place in F.14.3.2.2 Buffering period SEI message semantics for multi-layer extensions

- Minor fix: wrong reference (F.8.5 -> G.8.4) in G.8.1.2 Decoding process for a coded picture with nuh_layer_id greater than 0

- Editorial fix: replacing “dependent layer” by "reference layer" in the Multiview Main and Scalable Main profile definition, since “dependent layer” is not a defined term

- Fix: Removed misleading redundant constraint expressions for SPS and PPS extension flag values in G.11.1.1 and H.11.1.1.

- Typo fix: index typo H.8.1.4.2 Resampling process of picture motion and mode parameters

- Typo fix: remove wrong brackets in equation H-85 and equation H-86

The editorial team did not work on the Scalable HEVC (SHVC) Test Model text during this period.

The AHG recommended to:

- Use SHVC bug-tracker (https://hevc.hhi.fraunhofer.de/trac/shvc) to report issues related to SHVC Draft and Test Model text.

- Compare the SHVC documents with the SHVC software and resolve any discrepancies that may exist, in collaboration with the SHVC Software AHG.

- Continue to improve the quality of the SHVC test model document.

JCTVC-U0012 JCT-VC AHG report: SHVC software development (AHG12) [V. Seregin, Y. He]

(Reviewed verbally prior to upload.)

This report summarized activities of the AHG12 on SHVC software development between 20th and 21th JCT-VC meetings.

SHM software could be downloaded at https://hevc.hhi.fraunhofer.de/svn/svn_SHVCSoftware/tags/

The software issues could be reported using bug tracker https://hevc.hhi.fraunhofer.de/trac/shvc

The latest version was SHM-9.0 and it was released with JCTVC-T1013 for PDAM.

SHM-9.0 is based on HM-16.2 (RExt), and the following software improvements have been made along with the migration:

- Fixes for tickets ## 65, 66, 67, 68, 69, 70, 71, 72, 73, 74, 76, 77, 78, 79

- JCTVC-P0123: alpha channel SEI

- Fix the invocation of layerId, layerIdx, layerIdc and the following convention is used

- layerId – absolute layer Id number

- layerIdx – layer index used in VPS

- layerIdc – layer index used to indicate layers included in a set, for example, several layers can be defined in VPS, however not all may be used for inter-layer prediction, so such subset is indexed with layerIdc

Anchor data and templates had been generated based on common test conditions JCTVC-Q1009 and attached to this report.

The AHG recommended to:

- Continue to develop reference software based on SHM-9.0 and improve its quality.

- Resolve identified software and working draft mismatches.

JCTVC-U0013 JCT-VC AHG report: SHVC verification testing (AHG13) [V. Baroncini, Y.-K. Wang, Y. Ye]

This AHG was established to prepare the plan for the verification testing of the SHVC specification.

The input contribution JCTVC-U0037 provides a draft SHVC verification test plan, including the following:

- Suggested timeline

- Suggested test cases covering 2-layer spatial 2x, 1.5x, SNR, and CGS test cases

- Suggested test sequences for each test case

- Provided preliminary coding results for SHVC and HEVC simulcast at the suggested rate points

Some open issues were identified and needed further discussion:

- It was suggested to compare SHVC with HEVC simulcast, and not compare with SVC. This was partly because

  - the number of test points would increase, making the test more difficult to perform

  - there may be some difficulty with the SVC software (e.g., low-delay B), and

  - there is an apparent higher interest in HEVC simulcast as the primary basis for comparison.

This was agreed.

- Issues with CGS test case, in particular lack of test sequences and lack of test equipment (e.g., display with WCG)

- Test point selection (BL and EL bit rates)

The AHG recommended

- To discuss JCTVC-U0037, and agree on a proposed draft SHVC verification test plan with possible refinements after discussion.

- To encourage participants to contribute test sequences suitable as subjective test material, especially material in the CGS test case.

- To start bitstream preparation following the agreed SHVC verification test plan.

# Project development, status, and guidance (25)

## Conformance test set development (0)

See the AHG report JCTVC-U0004 and outputs JCTVC-U1008 for SHVC and JCTVC-U1012 for RExt.

## Version 1/2 bug reports and cleanup (2)

See also JCTVC-U0137.

JCTVC-U0031 Discrepancy between Version 2 text and software for High Throughput Profile [K. Sharman, N. Saunders (Sony)]

(Consideration of this report was chaired by GJS on Wednesday 06-24, 11:30-12:00.)

A discrepancy between the software and the HEVC Version 2 text has been identified that affects the CABAC alignment tool present in the High Throughput profile. This error was first reported on the bug-tracker (ticket #1391), where it has been indicated that the software implements the intended function.

Results are presented to compare the implementation in the standardized text relative to the implementation in the software. The results indicate that the implementation in the standardized text is about 0.05% less efficient in terms of BD-rate for low bit rate conditions, and less than 0.01% increase in BD-rate and bit rates for high bit rate and lossless test conditions.

It was proposed for the JCT-VC to consider whether to amend the software or the standardized text to resolve this discrepancy.

It was suggested that the HM implements the originally-intended design.

Decision: Change the text to match the software.

- JCTVC-U0156 On 4:2:0 chroma positioning for Rec. ITU-R BT.2020 [C. Fogg (MovieLabs), G. J. Sullivan (Microsoft), A. M. Tourapis (Apple)] [late]

(Consideration of this report was chaired by GJS on Wednesday 06-24, 11:30-12:00.)

This contributions reports that Rec. ITU-R BT.2020 appears to prescribe a different chroma positioning for 4:2:0 sampling than what has been previously assumed in video coding work. Based on the chroma sampling location type numbering scheme found in AVC and HEVC, it is reported that, rather than using location type #0 (which is considered the default in AVC and HEVC and is prescribed in MPEG-2) or type #1 (which is considered the default in H.261, H.263, and JFIF), it appears that (in its Table 5) BT.2020 specifies the use of a 4:2:0 chroma sampling lattice with chroma sampling location type #2.

This interpretation is confirmed by a white paper that was just published by the Blu-ray Disc Forum: "White Paper Blu-ray Disc Read-Only Format – Coding constraints on HEVC video streams for BD-ROM Version 3.0", June 2015 (clause 2.7.3 Location of chroma samples), available at http://www.blu-raydisc.com/en/Technical/TechnicalWhitePapers/General.aspx.

Two actions are suggested: 1) Sending a liaison letter to ITU-R to ask them to confirm that this is the intended interpretation and to ask why it was decided to differ from the recent past practice in this regard, and 2) Studying the potential impact of this on the colour gamut scalability design in the SHVC scalable extensions of HEVC.

For further study and liaison consideration. After joint discussion (see section 6.2) and parent-level consideration, a liaison letter on this topic was sent by VCEG to ITU-R WP6C.

## SCC text development (4)

JCTVC-U0038 Proposed editorial improvements to HEVC Screen Context Coding Draft Text 3 [R. Joshi (Qualcomm), J. Xu (Microsoft), S. Liu (MediaTek), Y. Ye (InterDigital)] [late]

(Consideration of this topic was chaired by GJS on Sunday 06-21, 17:30-18:00.)

For the first section of the document:

In the discussion, it seemed that the indexing for palette_transpose_flag is not needed. The editors were also requested to review indexing for merge index and CBF in regard to indexing. In any case, this is just an editorial matter.

Decision (Ed.): Editorial input delegated to the editors.

Second section: Basis text, forgotten DRAP and coded region completion SEI messages, ticket #1388 text integration error (also see JNB comment).

Decision (Ed.): Agreed as editorial issues to be fixed by the editors.

Third section:

- IBC usage relationship to IRAP pictures? IRAP pictures allow P and B slices that use IBC, and it previously during the meeting, it was agreed to also allow traditional I slices when IBC is enabled.

- Is IBC referencing prohibited for biprediction? Addressed previously during the meeting, this has been fixed. Referencing for bipred (in either or both lists) will be allowed.

- Should the definition of I slice exclude IBC usage? There was previously a CU-level flag to select between IBC and ordinary intra, which was removed as part of the "unification" effort. See the notes for U0079. After the action taken in response to U0079, the definition of "I slice" excludes IBC, but an IRAP picture can contain I, P and B slices (in which the only picture that is used for reference is the current picture).

JCTVC-U0055 High-level syntax refinement in monochrome signalling [C. Gisquet, G. Laroche, P. Onno (Canon)]

(Consideration of this topic was chaired by GJS on Tuesday 06-23, 12:45-13:00.)

During the editorial integration of JCTVC-T0048, related to palette syntax in the PPS SCC extension, support for monochrome and bit depths higher than 8 bits was considered and added. However, it is asserted this syntax may cause nonsensical signalling. This contribution proposes a modification of the PPS-level syntax to make the monochrome information more generic to address this issue. Specifically, the flag monochrome_palette_flag is proposed to be removed and a monochrome_flag is proposed to be added, keeping the previous meaning of monochrome_palette_flag, but also impacting the signalling of residual adaptive colour transform syntax. It is asserted this solves such ambiguity as having monochrome palette entries while ACT is active.

In the discussion, it was commented that the change does not seem really necessary and makes the syntax more logically complicated.

No action was taken on this.

In the discussion, it was remarked that we have SPS enabling flags for most new features, but not for the ACT. However, it was also noted that CCP signalling is the same.

JCTVC-U0092 Draft Text modifications on palette [P. Lai, J. Ye, J. Kim, X. Xu, S. Liu, S. Lei (MediaTek)]

(Consideration of this topic was chaired by GJS on Tuesday 06-23, 13:00-13:10.)

This contribution presents modifications to the HEVC SCC Draft Text related to palette coding. The modifications are asserted to: 1) Fix errors in the draft text, 2) Resolve mismatches between SCM software and draft text, 3) Remove parts that are not referred to. In the second version of the document, two modifications are added to reflect other issues discussed during the meeting in relation to U0089.

For section 1.5, it was agreed that this constraint is needed. Editors may determine the way it is expressed.

Decision (Ed.): This is purely editorial and adding constraints that are necessary for sensibility. This is delegated to the editors.

JCTVC-U0110 Bug fixes and cleanup for the text of palette mode in HEVC SCC draft [Y.-J. Chang, J.-S. Tu, C.-C. Lin, C.-L. Lin (ITRI)]

(Consideration of this topic was chaired by GJS on Tuesday 06-23, 13:10-13:20.)

This contribution presents bug fixes and cleanup issues for the palette mode in HEVC screen content coding draft 3. It is reported that a mismatch of coordinate scales between PaletteEscapeVal[ cIdx ][ xC ][ yC ] and recSamples[ x ][ y ] occurrs when the chroma format is not 4:4:4. This contribution provides two options to fix this bug: the first option is by modifying the syntax, and the second option is by modifying the decoding process of the reconstruction. Additionally, ctxInc of palette_run_type_flag was reportedly assigned to an undefined number.

The first part concerns PaletteEscapeVal. In the discussion, it was said that U0038 may already take care of this.

The second part of the contribution was addressed in 1.4 of U0092.

Decision (Ed.): In any case, this is purely editorial, and is delegated to the editors.

## HEVC coding performance, implementation demonstrations and design analysis (5)

### HM performance (0)

See the notes for JCTVC-U0051 and section 5.4.

### RExt performance/verification testing (1)

JCTVC-U0183 Visualization of BD-rate interpolation curves for MOS and PSNR results [A. S. Dias, M. Mrak (BBC)]

(Consideration of this topic was chaired by GJS on Thursday 06-24, 17:15-17:20.)

This contribution provided a spreadsheet to assist analysis of subjective quality scores with special treatment of cases where there is limited overlap between the bit rate and MOS curves. This was used for the verification testing of RExt and interlaced video.

It was remarked that some previous contributions have considered that problem for PSNR-based analysis. Further study was encouraged to check the way this was done before in relation to what is provided in this contribution.

JCTVC-U0184 Interlaced video and format range extensions verification test results [G. Barroux (Fujitsu), C. Rosewarne (Canon), V. Baroncini (FUB)]

(Consideration of this topic was chaired by GJS on Wednesday 06-24, 12:00-13:00.)

This contribution provides the results of the HEVC interlace verification and the HEVC Range extensions verification tests.

The selected test sequences are first presented. The test conditions are then introduced, followed by a description of the test methods and logistic. Finally, the results are presented and analyzed.

The interlace verification tests showed that overall, HEVC offers a better encoding quality than AVC even when AVC uses twice the bit rate.

The Range extensions tests showed that HEVC consistently provides a better encoding quality than AVC for range extensions content even when AVC is used with double the bit-rate used with HEVC, and especially for 4:4:4 content.

The tests were operated under the conditions specified in JCTVC-T1003.

Performance curves were provided in the report based on the scheme described in JCTVC-U0183.

This contribution was used toward preparation of the output verification test report JCTVC-U1003.

### SHVC performance/verification test (1)

JCTVC-U0037 SHVC Verification Test Draft Plan [Y. He, Y. Ye (InterDigital), Hendry, Y.-K. Wang (Qualcomm)]

(Consideration of this topic was chaired by GJS on Friday 06-26, 09:45-10:05.)

See the notes for the AHG13 report, which contain details about the planned testing.

This document contains a proposed draft plan for the video verification test to be conducted to verify the coding performance of the SHVC Scalable Main and Scalable Main 10 profiles. It is suggested that a formal subjective evaluation be conducted comparing the SHVC scalable Main and Main 10 profiles to HEVC simulcast using the HEVC Main and Main 10 profiles, respectively. The suggested test covers a range of video resolutions from 540p to 4K, and various scalability cases, including spatial 1.5x, spatial 2x, SNR and colour gamut scalabilities.

The dates listed for milestones seemed more aggressive than may be feasible based on remarks made earlier in the meeting. Of course, getting work done sooner is better if feasible and if this does not result in needing to repeat work after changing a decision about what to do. The planned output of the meeting is for an AHG to prepare the final test plan, but it was not necessarily anticipated to actually be able to conduct the test before the next meeting.

Identifying additional test sequences for CGS testing was identified as needed.

This input was used toward preparation of the output draft verification test plan JCTVC-U1004.

### SCC performance, design aspects and test conditions (2)

(Consideration of this topic was chaired by Rajan Joshi on Thursday 06-25, 18:30-19:00.)

JCTVC-U0051 Comparison of Compression Performance of HEVC Test Model 16.4 and HEVC Screen Content Coding Extensions Test Model 4 with AVC High 4:4:4 Predictive profile [B. Li, J. Xu, G. J. Sullivan (Microsoft)]

This contribution reports a study of the relative objective (i.e. PSNR-based) compression performance of HEVC RExt Main 4:4:4 Profile, HEVC Screen Content Coding (SCC) Test Model 4 (SCM 4) and AVC High 4:4:4 Predictive Profile. It builds upon the prior work reported in JCTVC-G399, JCTVC-H0360, JCTVC-I0409, JCTVC-J0236, JCTVC-K0279, JCTVC-L0322, JCTVC-M0329, JCTVC-O0184, JCTVC-P0213, JCTVC-R0101, JCTVC-S0084, and JCTVC-T0042 – updating the results by using the latest available reference software (JM-18.6, HM-16.4, HM-16.4+SCM-4.0), profile and test model designs, and SCC common test conditions (CTC) test sequences. The overall results reportedly indicate that for screen content CTC sequences, the HEVC SCC Test Model 4 improves quite substantially over JM-18.6. For example, for RGB text and graphics with motion (TGM) 1080p & 720 sequences, it is reported that the BD-rate savings of the HEVC SCC Test Model 4 over JM-18.6 are 86%, 80%, and 78% for lossy AI, RA and LB configurations, respectively.

The reported BD-rate gains for 4:4:4 sequences are similar to the previously reported gains. Reported gains for the 4:2:0 sequences are larger. It is asserted that this is due to the addition of new 4:2:0 test sequences and enabling of palette mode for 4:2:0. The decoding times are reportedly not very accurate but were asserted to give an overall indication. The BD-rates were obtained using rate matching. Lossless gains are smaller in general. It was commented that there may be an error in lossless results (vs JM) for 4:2:0 sequences. HM16.4 used for the comparisons did not contain the encoder changes specific to screen content material that are present in HM 16.4 + SCM 4.0. One participant asked whether 10-bit sequences coded with JM-18.6 in 8-bit mode were handled similarly to HM 16.4 + SCM 4.0. The presenter was not sure about this.

JCTVC-U0058 Compression Performance of HEVC Screen Content Coding Extensions Test Model 4.x with slices [C. Gisquet, G. Laroche, P. Onno (Canon)] [late]

This contribution is a study of the relative objective (i.e. PSNR-based) compression performance of HEVC SCC Test Model 4.x (SCM 4.x) using a configuration of 1500 bytes per slice. At the time of writing, SCM4.1 is still being under development, but the present contribution is asserted to provide relevant results.

The modified anchor used a version of SCM4.x (revision 4482) with 1500 bytes per slice, where reference samples for intra block copying (IBC) are restricted to be within the current slice. A bug related to reference picture list management is also fixed in the modified anchor and the palette predictor initializer is reported set to 1.

The BD-rate performance of the modified anchor is reported for configurations where with one tool disabled at a time (ACT, IBC, palette). It is reported that for RGB text and graphics with motion (TGM) 1080p & 720 sequences for the AI configuration, the BD-rate losses due to disabling ACT, IBC and palette are 9.6%, 18.7%, and 45.2%, respectively. For YUV text and graphics with motion (TGM) 1080p & 720 sequences, the corresponding BD-rate losses were reported to be 0.2%, 23.0% and 44.1%, respectively. It was asserted that in contrast, without the use of slices, the BD-rate losses due to disabling IBC are much higher than disabling palette.

It was commented by the presenter that compared to palette and IBC, ACT adds more run-time complexity even in case of random access and low delay configuartions. A participant commented that this was partly due to the fact that efficient hash-based searches are used for inter and IBC whereas the ACT encoder search is exhaustive.

## Systems interfacing (2)

JCTVC-U0039 Overview of the High Efficiency Image File Format [M. M. Hannuksela, V. K. Malamal Vadakital, J. Lainema (Nokia)]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 19:00-19:30.)

This document presents an overview of the High Efficiency Image File Format (HEIF, ISO/IEC 23008-12). HEIF specifies the storage of individual images as well as image sequences into a container file. HEIF includes the storage specification of HEVC intra images and HEVC image sequences in which inter prediction is applied in a constrained manner. HEIF files are compatible with the ISO Base Media File Format (ISO/IEC 14496-12) and can also include other media streams, such as timed text and audio.

The tutorial overview information was appreciated.

JCTVC-U0137 On the use of AVC and HEVC "unspecified" NAL unit types in file formats and systems [Y.-K. Wang (Qualcomm), D. Singer (Apple), G. J. Sullivan (Microsoft)] [late]

(Consideration of this report was chaired by GJS on Wednesday 06-24, 11:30-12:00.)

This contribution contains the same content as in the MPEG input document m36547. It was submitted for information to the JCT-VC.

This contribution discusses the handling of data associated with the "unspecified" AVC and HEVC NAL unit type codes, particularly in 'transport environments' such as the file formats of AVC, HEVC and their extensions, and in RTP.

The "unspecified" NAL unit types refer to those NAL unit types that are identified as "unspecified" in the AVC and HEVC specifications (i.e., type codes 0 and 24..31 for AVC and type codes 48..63 in HEVC in a recent edition of each). The type codes of these NAL unit types are, for example, used for the aggregator and the extractor structures that were introduced during the development of the SVC file format, and later included in the MVC file format, the (draft) MVC+D file format, the HEVC file format, and the (draft) layered HEVC file format. These (draft) standards are all documented in the latest integrated ISO/IEC 14496-15 draft specification in the MPEG output document N 15182.

This contribution reports that, unfortunately, some uses of these type codes may not conform to the formatting requirements that are specified for the proper formatting of NAL units in the AVC and HEVC video coding specifications (in regard to start code emulation, emulation prevention patterns, and trailing zeros). Thus, from the perspective of the video coding specifications, such strings of data associated with these type codes should not be considered part of an ordinary video elementary stream in the sense of conformance to the AVC and HEVC specification of the NAL unit format, and this may cause interoperability problems if such data is fed to an ordinary video decoder that has not been designed specifically for operation within the system environment in which this data is used. If feasible, a preferable solution would be for system specifications (esp. future ones) to be designed to obey the constraints specified in the video coding standards for NAL unit formatting.

These issues were considered to be for further study. It may be desirable to add some warning notes into video coding standards, and this may require action to correct/clarify application-domain standard that use "unspecified" NAL unit type codes.

## Source video test material (2)

JCTVC-U0150 New SCC test sequence for consideration [W. Wang, M. Xu, F. Duanmu, H. Yu (Huawei)] [late]

(Consideration of this report was chaired by GJS on Thursday 06-25, 10:30-10:45.)

The contribution U0150 was marked as withdrawn, as the contributor thought it would save meeting time to not request its presentation. However, the contributor was assured that the input was appreciated and was asked to re-upload another copy as a new registration. This was uploaded as U0188.

- JCTVC-U0188 New SCC test sequence for consideration [W. Wang, M. Xu, F. Duanmu, H. Yu (Huawei)] [late]

(Consideration of this report was chaired by GJS on Thursday 06-25, 10:30-10:45.)

This contribution proposes a new test sequence for the SCC CTC. This sequence consists of content depicting Chinese text editing. The coding results from SCM-4.0 demonstrate different statistics in the usage of coding tools when compared with the existing SCC test sequences.

The contribution proposes a new test sequence with 600 frames in RGB 4:4:4 8 bit format.

It was commented that it would be nice to use this content, as it depicts Chinese rather than English alphabet characters. It depicts text editing.

YUV and 4:2:0 generation was requested.

Decision (CTC): Add to TGM category of CTC.

## HDR (10)

Discussion with the parent bodies was necessary to understand the relationship between these contributions and the MPEG HDR CfE and its results. See also section 6.2 of this report.

Some input contributions on SEI messages (JCTVC-U0032, JCTVC-U0033, JCTVC-U0048, JCTVC-U0098, and possibly JCTVC-U0112) also relate to this topic.

See also the notes for contribution JCTVC-U0040 on non-normative encoder improvement.

The contributions listed below in this section were available for study but were not reviewed in detail in the JCT-VC meeting.

JCTVC-U0041 On 4:4:4 to 4:2:0 conversion: Performance of downsampling and upsampling filters and MinMax and Closed Loop Filtering [A. M. Tourapis, Y. Su, D. Singer (Apple), J. Sole, D. Rusanovskyy, S. Lee, D. Rusanovskyy, A. Ramasubramonian, M. Karczewicz (Qualcomm), C. Fogg (Movielabs), A. Duenas, F. Bossen (NGCodec)] [late]

JCTVC-U0042 Clipping during 4:2:0 to 4:4:4 conversions [A. M. Tourapis, Y. Su, D. Singer (Apple), J. Sole, D. Rusanovskyy, M. Karczewicz (Qualcomm), C. Fogg (Movielabs), A. Duenas, F. Bossen (NGCodec)]

JCTVC-U0043 Deblocking in HEVC: Some observations from the HDR/WCG CfE [A. M. Tourapis, Y. Su, D. Singer (Apple), J. Sole, D. Rusanovskyy, M. Karczewicz (Qualcomm), C. Fogg (Movielabs), A. Duenas, F. Bossen (NGCodec)] [late]

JCTVC-U0044 Coding results of noise-reduced 4:2:0 and 4:4:4 HDR signals [C. Fogg (MovieLabs), K. Slavin (isovideo)]

JCTVC-U0045 High Dynamic Range with HEVC Main10 [D. Le Gall (Ambarella), A. Tourapis (Apple), W. Wan (Broadcom), C. Fogg (MovieLabs), A. Duenas (NG Codec), D. Rusanovskyy (Qualcomm)]

JCTVC-U0046 Studio requirements for next-generation video codecs [C. Fogg, J. Helman (MovieLabs)]

JCTVC-U0047 HDR Workflow precision and steps [C. Fogg (MovieLabs)] [late]

JCTVC-U0049 Wiener filter for HDR noise reduction pre-processing [A. Tourapis (Apple), C. Fogg (MovieLabs)]

JCTVC-U0085 Modulation-based HDR video coding with SDR backward compatibility [S. Lasserre, F. Le Leannec, E. François, T. Poirier (Technicolor)]

JCTVC-U0134 Upsampling and downsampling filters for HDR content [A. K. Ramasubramonian, D. Rusanovskyy, J. Sole, M. Karczewicz, S. Lee, D. Bugdayci Sansli, J. Chen (Qualcomm)] [late]

# Core experiments in SCC (21)

## CE1: Palette mode improvements (14)

(Consideration of this topic was chaired by GJS and JRO on Friday 06-19, 11:00-13:00 and by GJS 14:30-15:30.)

### CE1 summary and general discussion (1)

JCTVC-U0021 CE1: Summary report of CE on palette mode improvement [P. Lai, V. Seregin, X. Xiu, C. Gisquet (CE coordinators)]

CE1 Test B proposed to remove condition checks for index adjustment. In this contribution, 3 different options were tested. In Test 1, index adjustment was removed when previous pixel has COPY_ABOVE mode. In Test 2, index adjustment was removed when previous pixel has COPY_INDEX mode. Test 3 removes index adjustment for both cases. Since Test 1 and Test 2 limit index adjustment based on previously decoded run_type, run_type should be signalled before grouped indices are signalled. Test 1 shows average 0.1%, 0.0%, 0.0% loss under AI, RA and LB configuration respectively. Test 2 shows average 0.3%, 0.2%, 0.2% loss under AI, RA and LB configuration respectively. Test 3.1 which doesnot signal run_type before grouped indices shows average 0.3%, 0.1%, 0.1% loss under AI, RA and LB configuration respectively and Test 3.2 which signals run_type before grouped indices shows maximum 0.3%, 0.2%, 0.1% loss under AI, RA and LB configuration, respectively.

Test A – Simplification for index map coding in palette mode

Test A.1 – Extended copy above mode

Proponent: MediaTek and Qualcomm, JCTVC-U0061

Crosschecker: ITRI International, JCTVC-U0121

Description:

This test evaluates palette mode using the nearest row of the above CU or the nearest column of the left CU to predict the pixel values of the current CU. With palette indices grouped at the front, an index adjustment indication is proposed to be introduced to signal whether it is an actual max index value when the parsed index is the max index value minus 1.

Test A.2 –Extended copy above mode to the first line with grouping the palette_run_type_flag in front

Proponent: MediaTek, JCTVC-U0062

Crosschecker: Qualcomm, JCTVC-U0147

Description:

This test evaluates palette mode using the nearest row of the above CU or the nearest column of the left CU to predict the pixel values of the current CU. Syntax elements palette_run_type_flag are proposed to be grouped and moved in front of the grouped palette indices. (The syntax of the palette run types, the palette indices, the palette run lengths, and the escape pixels are grouped respectively.) Palette index range adjustment does not depend on above position.

Runtime numbers for test A.2 were suggested not to be accurate.

- Lossy, 4:4:4, full-frame IBC

| 4:4:4, full-frame IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All Intra | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, RGM, 1080p & 720p | −1.5% | −1.6% | −1.6% | −1.4% | −1.5% | −1.5% |
| RGB, mixed, 1440p & 1080p | −0.5% | −0.7% | −0.7% | −0.5% | −0.6% | −0.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, cam-captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM, 1080p & 720p | −1.9% | −1.9% | −2.0% | −1.7% | −1.9% | −1.9% |
| YUV, mixed, 1440p & 1080p | −0.7% | −1.2% | −1.3% | −0.6% | −1.1% | −1.2% |
| YUV, Animation, 720p | 0.0% | −0.1% | −0.1% | 0.0% | −0.1% | −0.1% |
| YUV, cam-captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% (cross-checker) |  |  | 134% |  |  |
| Dec Time[%] | 101% (cross-checker) |  |  | 130% |  |  |

| 4:4:4, full-frame IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Random Access | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, RGM, 1080p & 720p | −0.9% | −1.0% | −1.0% | −0.9% | −1.0% | −1.0% |
| RGB, mixed, 1440p & 1080p | −0.4% | −0.5% | −0.5% | −0.4% | −0.5% | −0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, cam-captured, 1080p | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| YUV, TGM, 1080p & 720p | −1.1% | −1.2% | −1.4% | −1.0% | −1.2% | −1.4% |
| YUV, mixed, 1440p & 1080p | −0.6% | −1.1% | −1.4% | −0.5% | −0.9% | −1.1% |
| YUV, Animation, 720p | −0.1% | −0.3% | −0.1% | −0.1% | −0.2% | −0.1% |
| YUV, cam-captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | −0.1% |
| Enc Time[%] | 99% (cross-checker) |  |  | 126% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 122% |  |  |

| 4:4:4, full-frame IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Low Delay B | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, RGM, 1080p & 720p | −0.9% | −0.8% | −0.8% | −0.8% | −0.8% | −0.9% |
| RGB, mixed, 1440p & 1080p | −0.2% | −0.4% | −0.3% | −0.2% | −0.5% | −0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% | −0.1% | 0.0% | 0.0% |
| RGB, cam-captured, 1080p | 0.0% | −0.1% | 0.0% | −0.1% | 0.0% | −0.1% |
| YUV, TGM, 1080p & 720p | −1.0% | −1.1% | −1.1% | −0.9% | −1.0% | −1.1% |
| YUV, mixed, 1440p & 1080p | −0.2% | −0.7% | −1.1% | −0.1% | −0.6% | −0.8% |
| YUV, Animation, 720p | 0.0% | 0.4% | 0.3% | 0.0% | −0.4% | 0.1% |
| YUV, cam-captured, 1080p | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 99% (cross-checker) |  |  | 128% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 118% |  |  |

- Lossy, 4:4:4, 4x1-CTU IBC

| 4:4:4, 4x1-CTU IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All Intra | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, RGM, 1080p & 720p | −1.8% | −1.8% | −1.9% | −1.7% | −1.8% | −1.8% |
| RGB, mixed, 1440p & 1080p | −0.6% | −0.8% | −0.9% | −0.6% | −0.8% | −0.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, cam-captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM, 1080p & 720p | −2.2% | −2.2% | −2.4% | −2.1% | −2.0% | −2.2% |
| YUV, mixed, 1440p & 1080p | −0.9% | −1.4% | −1.4% | −0.8% | −1.3% | −1.3% |
| YUV, Animation, 720p | 0.0% | −0.1% | −0.1% | 0.0% | −0.1% | −0.1% |
| YUV, cam-captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 106% (cross-checker) |  |  | 130% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 127% |  |  |

| 4:4:4, 4x1-CTU IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Random Access | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, RGM, 1080p & 720p | −1.1% | −1.2% | −1.2% | −1.0% | −1.1% | −1.1% |
| RGB, mixed, 1440p & 1080p | −0.5% | −0.6% | −0.6% | −0.4% | −0.6% | −0.6% |
| RGB, Animation, 720p | −0.1% | 0.0% | 0.0% | −0.1% | −0.1% | 0.0% |
| RGB, cam-captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM, 1080p & 720p | −1.3% | −1.5% | −1.7% | −1.2% | −1.4% | −1.5% |
| YUV, mixed, 1440p & 1080p | −0.6% | −1.2% | −1.4% | −0.5% | −0.9% | −1.1% |
| YUV, Animation, 720p | 0.0% | 0.2% | 0.0% | 0.0% | −0.1% | 0.0% |
| YUV, cam-captured, 1080p | 0.0% | −0.1% | 0.1% | 0.0% | −0.1% | 0.1% |
| Enc Time[%] | 106% (cross-checker) |  |  | 123% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 119% |  |  |

| 4:4:4, 4x1-CTU IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Low Delay B | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, RGM, 1080p & 720p | −0.9% | −1.0% | −0.9% | −0.8% | −1.0% | −0.9% |
| RGB, mixed, 1440p & 1080p | −0.2% | −0.3% | −0.4% | −0.3% | −0.4% | −0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | −0.1% | −0.1% | −0.1% | −0.1% |
| RGB, cam-captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM, 1080p & 720p | −1.0% | −1.0% | −1.3% | −0.9% | −1.0% | −1.0% |
| YUV, mixed, 1440p & 1080p | −0.3% | −1.2% | −0.9% | −0.1% | −1.0% | −1.2% |
| YUV, Animation, 720p | 0.0% | −0.1% | −0.1% | 0.0% | −0.1% | 0.2% |
| YUV, cam-captured, 1080p | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 103% (cross-checker) |  |  | 130% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 121% |  |  |

- Lossy, 4:2:0, full-frame IBC

| 4:2:0, full-frame IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All Intra | G/Y | B/U | R/V | G/Y | B/U | R/V |
| YUV, TGM, 1080p & 720p | −0.5% | −0.3% | −0.4% | −0.5% | −0.4% | −0.5% |
| YUV, mixed, 1440p & 1080p | −0.4% | −0.6% | −0.5% | −0.3% | −0.6% | −0.5% |
| YUV, Animation, 720p & 768p | −0.1% | 0.1% | 0.2% | 0.0% | 0.1% | 0.3% |
| Enc Time[%] | 104% (cross-checker) |  |  | 140% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 129% |  |  |

| 4:2:0, full-frame IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Random Access | G/Y | B/U | R/V | G/Y | B/U | R/V |
| YUV, TGM, 1080p & 720p | −0.4% | −0.2% | −0.2% | −0.3% | −0.1% | −0.2% |
| YUV, mixed, 1440p & 1080p | −0.2% | −0.5% | −0.6% | −0.1% | −0.4% | −0.3% |
| YUV, Animation, 720p & 768p | 0.0% | 0.2% | 0.0% | 0.0% | 0.2% | 0.2% |
| Enc Time[%] | 102% (cross-checker) |  |  | 133% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 123% |  |  |

| 4:2:0, full-frame IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Low Delay B | G/Y | B/U | R/V | G/Y | B/U | R/V |
| YUV, TGM, 1080p & 720p | −0.3% | −0.5% | −0.7% | −0.2% | 0.1% | −0.4% |
| YUV, mixed, 1440p & 1080p | −0.2% | −0.8% | −0.3% | −0.3% | −1.1% | −0.3% |
| YUV, Animation, 720p & 768p | −0.1% | −0.5% | 0.3% | 0.0% | −0.2% | 0.5% |
| Enc Time[%] | 101% (cross-checker) |  |  | 140% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 130% |  |  |

Discussion: A.2 was characterized to be "cleaner" than A.1, with somewhat less gain (0.1%-0.2%), with overall gain in TGM AI full-frame IBC RGB 1.4%, AI full-frame IBC YUV 1.7%.

It was reported that the runtime of A.1 or A.2 is about 2-3% slower than the anchor, partly due to having another pass for encoding decisions.

It was remarked that this adds the ability to predict from outside the current CU, which adds some complexity, as it changes the architecture. It was asked whether the 1.5% gain is worth the complication.

It was remarked that there is a related contribution U0096 with encoder-only modifications showing gains higher than this with similar runtime impact, and that encoding speed-up could be achived as well (e.g., by using early-termination detection).

It was remarked that other aspects of the design may have issues – e.g., B slice RPL and motion estimation quality. It is not clear, however, whether that has a real relationship to this.

No action was taken on this.

Test B – Simplification for index map coding in palette mode

Proponent: MediaTek, JCTVC-U0050

Crosschecker: Sharp, JCTVC-U0070

Description:

This test evaluates removing a dependency introduced by index range adjustment. In order to apply the methods to the current palette index coding in SCM4.0, syntax reordering with “grouping palette run_type flag at the front” is used in the tests. The following are presented:

Result 1:	Group run_type in front, no dependency from the above index

(remove index range adjustment when previous pixel is COPY_ABOVE mode).

Result 2:	Group run_type in front, no dependency from the left index

(remove index range adjustment when previous pixel is COPY_INDEX mode).

Results 3-1:	Without grouping run_type in front, completely remove index range adjustment.

Results 3-2:	Group run_type in front, completely remove index range adjustment.

Supplementary: Group run_type in front.

- Lossy, 4:4:4, full-frame IBC

- Lossy, 4:4:4, 4x1-CTU IBC

- Lossy, 4:2:0, full-frame IBC

It was said that the complexity reduction available in this test is rather small, and some significant loss occurs in the TGM category by doing so.

No action was taken on this.

Test C – Palette run-length coding scan order modifications

Test C.1 –Palette run-length coding with palette reverse scan order

- Proponent: InterDigital and Qualcomm, JCTVC-U0101

- Crosschecker: Canon, JCTVC-U0059

- Description:

This test evaluates the palette reverse scan as proposed in JCTVC-T0119. The proposed method signals one additional flag at CU level. If the flag is equal to 1, the palette indices of the current CU are scanned in reverse scan order; otherwise, the palette indices of the current CU are scanned in original scan order. The following illustrate the reverse scan order when horizontal scan is applied to one palette-coded CU.

Test C.2 – Palette run-length coding with rotated index scans (scan from bottom-right)

- Proponent: MediaTek, JCTVC-U0088

- Crosschecker: InterDigital, JCTVC-U0108

- Description:

This test evaluates rotated palette index scanning order as reported in JCTVC-T0174. A CU-level flag is used to indicate whether to apply the rotation. Constraint the rotation on CU size is also tested.

(a) Horizontal scan	  (b) Reverse horizontal scan (C.1)    (c) Rotated horizontal scan (C.2)

(a) Vertical scan	    (b) Reverse vertical scan (C.1)	(c) Rotated vertical scan (C.2)

RD2: Using the RDO method of CE.1 Test C.1 released on Apr. 8, 2015

RD3: Using the RDO method of CE.1 Test C.1 released on May 30, 2015

- Lossy, 4:4:4, full-frame IBC

- Lossy, 4:4:4, 4x1-CTU IBC

- Lossy, 4:2:0, full-frame IBC

This increases encoding complexity by checking more scan patterns. A gain of about 1% is shown for TGM AI. There is also some decoder complexity increase, for being capable of handling more patterns. For software decoders, the impact seems small – adding some logic or tables up to 32x32 in size (but perhaps not a small impact for hardware).

Reversing and rotating scan patterns don't have a big difference in coding efficiency.

It was remarked that a related contribution U0146 was submitted that shows more gain by combining U0096 (encoder-only changes with some increase in complexity) with the C.1 variation of test C, to obtain 2.1% gain. It was also remarked that in U0096, an encoder-only change without any test C change was reported to obtain a 1.8% gain. So in that context the difference in coding efficiency for the additional scan patterns was only 0.3%.

It was suggested to also review U0127 before making a decision on this.

See the notes on U0127 and U0096.

Test D – Modification of palette run coding

Proponent: Qualcomm, JCTVC-U0122

Crosschecker: MediaTek, JCTVC-U0117

Description:

If the current pixel is the first pixel in the line, a flag is signalled into the bitstream, indicating whether it ends at the last pixel in certain line. If so, the number of whole lines (L) the current run spans is coded into the bitstream using coefficient coding function (xWriteCoefRemainExGolomb) with order 0. Instead of directly signalling L, a mapping process is used to map maximal a feasible value to zero and shift L up by 1 if it is less than the maximum. Otherwise, it falls back to normal palette run -ength coding with an independent set of context models.

If the current pixel is not the first pixel in the line, a run length L is coded into the bitstream using xWriteCoefRemainExGolomb with order 0. Instead of signalling L directly, if the current run ends one line below the current line, a mapping procedure is used to reorder the candidate positions in the line according to their horizontal distance between themselves and the current pixel.

- Lossy, 4:4:4, full-frame IBC

- Lossy, 4:4:4, 4x1-CTU IBC

- Lossy, 4:2:0, full-frame IBC

The improvement of coding efficiency is smaller than with categories A or C.

It was said that a substantial amount of text would be needed for this change. The mapping process, in particular, involves a significant block of logic, and additional logic would be needed in the syntax as well.

No action was taken on this.

### CE1 primary contributions (7)

JCTVC-U0050 CE1 Test B: Simplification for index map coding in palette mode [J. Kim, J. Ye, P. Lai, S. Liu, S. Lei (MediaTek)]

JCTVC-U0061 CE1: Test A.1: Extended copy above mode to the first line with index adjustment bits [Y.-C. Sun, J. Kim, T.-D. Chuang, S. Liu, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek), V. Seregin, F. Zou, W. Pu, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-U0062 CE1: Test A.2: Extended copy above mode to the first line with grouping the palette_run_type_flag in front [Y.-C. Sun, J. Kim, T.-D. Chuang, S. Liu, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-U0088 CE1 Test C.2: Palette run-length coding with rotated index scans (scan from bottom-right) [P. Lai, J. Kim, S. Liu, S. Lei (MediaTek)]

JCTVC-U0101 CE1 Test C.1: Improved palette run-length coding with palette reverse scan order [X. Xiu, Y. Ye, Y. He (InterDigital), W. Pu, R. Joshi, M. Karczewicz, F. Zou, V. Seregin (Qualcomm)]

JCTVC-U0122 CE1 Test D: Modification of Palette Run Coding [M. Karczewicz, W. Pu, R. Joshi, F. Zou, V. Seregin (Qualcomm)] [late]

JCTVC-U0127 CE1: Additional results for CE1 subtest C [R. Joshi, M. Karczewicz, V. Seregin, W. Pu, F. Zou (Qualcomm)] [late]

(Consideration of this topic was chaired by GJS on Friday 06-19, 15:00-15:30.)

This was not originally planned in the CE, and was classified as information document.

Additional results for CE1 subtest C were provided to understand how much do the different aspects (encoder, scans) contribute to the overall gain. The scans without any encoder modifications to the way in which runs are calculated reportedly show BD-rate improvements in the range of 0.0% to 0.5% for both full frame IBC and 1×4 CTU IBC, All-Intra configurations. The BD-rate gains from encoder modifications to the way in which runs are calculated are in the range of 0.0% to 0.5% for full frame IBC and 0.0% to 0.6% for 1×4 CTU IBC, All-Intra configurations.

Thus, the gains shown in CE1 subtest C were shown to be a combination of two things:

- Additional scan patterns (normative)

- Improved run-type flag decisions and run value decisions (non-normative)

The gain from each of these sources is roughly half of the total gain shown.

See the notes on U0096, which includes a simplified form of the technique described in this contribution, and also includes some other techniques.

### CE1 cross checks (6)

JCTVC-U0059 CE1: Cross-check of Test C.1 from Interdigital/Qualcomm [C. Gisquet (Canon)] [late]

JCTVC-U0070 CE1: Cross check of Test B on Simplification for index map coding in palette mode [J. Zhao, S. H. Kim (Sharp)]

JCTVC-U0108 Cross-verification of CE1 Test C.2: Palette run-length coding with rotated index scans [X. Xiu, Y. He, Y. Ye (InterDigital)] [late]

JCTVC-U0117 Cross-check of JCTVC-U0122 CE1 Test D: Modification of Palette Run Coding [P. Lai, J. Ye, J. Kim (MediaTek)] [late]

JCTVC-U0121 CE1: Crosscheck of CE1 Test A.1 (JCTVC-U0061) [W. Digonnet, C.-C. Lin, J.-S. Tu, Y.-J. Chang, C.-L. Lin (ITRI)]

JCTVC-U0147 Cross check CE1 Test A.2: Extended copy above mode to the first line with grouping the palette_run_type_flag in front (U0062) [W. Pu (Qualcomm)] [late]

## CE2: Intra block copy memory access (8)

(Consideration of this topic was chaired by JRO on Friday 06-19, 14:30-16:00.)

### CE2 summary and general discussion (2)

JCTVC-U0022 CE2: Summary report for Core Experiment 2 on Intra block copy memory access [K. Rapaka, C. Gisquet, J. Lainema, X. Xu (CE coordinators)]

Aspects of this CE were summarized and considered as shown below.

JCTVC-U0053 CE2 Test 2: Intra block copy constraints on filtering

Proposal: Don't allow IBC when filtering (SAO/DBF) is enabled. This requires a flag enabling/disabling the filters at the CTU level (otherwise, when disabling it for the whole slice, performance would be too bad). Requires two-pass encoding.

JCTVC-U0034 CE2: Test 3 on intra block copy constraints on filtering (single-pass encoding decisions)

Proposal: Same as test 2, but inferring a decision from previous coded blocks based on statistics (by tendency, filtering is rather turned off for screen content, not for natural content).

During the discussion, it was also suggested by experts that, whereas DBF is less beneficial for screen content, SAO could be beneficial.

JCTVC-U0078 CE2 : Test 5 on intra block copy constraints on prediction.

Proposal: Disabling IBC from CTUs where 8x8 biprediction is used. 5.1 intends to trade the additional memory accesses for IBC (once filtered, once unfiltered) against the current worst case. This constraint does not apply to the current and previous CTU, which are assumed to be available in cache. 5.2 uses IBC only when bi-prediction is not used in current slice. 5.3 is similar, but allows joint usage of IBC and bipred, if the latter uses integer MC. 5.4 is similar to 5.1, allowing joint usage of IBC and bipred, if the latter uses integer MC.

Discussion

All methods come with losses for screen content classes

AI: Test 2 around 1%, Test 3 around 2%, test 5.x 0% (does not affect AI)

RA: Test 2 around 0.9%, Test 3 >2%, test 5.1 around 3%, 5.2/5.3 around 0.5%, test 5.4 around 2%

LDB: Test 2 around 1%, Test 3 around 3%, test 5.1 4-5%, test 5.2/5.3 0.7%, test 5.4 around 2.5%

It was pointed out that the results have larger variation over different sequences.

Average memory access reduction is reported.

Worst case memory bandwidth saving by the different methods needs to be reported. It should also be reported what the absolute memory consumption of the different test sequence classes are: The average numbers indicate no saving for some classes where IBC is apparently not used, but in fact these may use biprediction more frequently and be worse in memory access than some of the screen content classes.

It was generally agreed in the initial review that 5.2/5.3 seem to come with a relative moderate loss, whereas only require a change at the slice level to enable/disable certain combinations of IBC/bipred/integer MC. Before making a decision, a precise number about the saved memory bandwidth should be available.

(Further consideration of this topic was chaired by JRO as Track B Monday 06-22 at 18:00.)

After reviewing the BoG report U0175, it was confirmed that the methods are not increasing the worst-case memory bandwidth for the 4:4:4 case. However, unlike the approach of 5.2/5.3, where the text specification globally disables the joint usage of IBC and 8x8 bi-pred at the sequence level, it would be more desirable to have a solution that disallows the joint usage per slice (which could be expressed differently, e.g., that 8x8 bi-pred shall not be used when the current picture is in the active reference picture list of the current slice).

It was generally agreed that this would be an appropriate solution (per a bitstream restriction expressed in a profile specification), which comes with small loss in compression efficiency while not increasing worst case memory bandwidth.

Text was requested to be provided for both solutions 5.2 and 5.3; the BoG U0175 provided further analysis for both 4:4:4 and 4:2:0 cases.

(Further consideration of this topic was chaired by JRO and GJS on Thursday 06-25, 09:00-10:00.)

Text was provided in U0078v2 (JCTVC-T1005_profile_restriction_text) and was presented in the plenary. Some more editorial improvements were made during that meeting. It was also pointed out that the restriction should apply for using IBC in both L0 and L1. These issues are editorial and it is up to the discretion of the editors to make further alignments and improvements.

Text was provided and reviewed. Variable naming was discussed (which may be refined by the editor), and it was confirmed that we will allow the current picture to be in list 1.

The constraint is intended as a slice-level constraint (not a picture-level constraint).

A participant suggested that it would be good to find a way to syntactically prohibit violation of the intended constraint rather than making it just a bitstream conformance requirement. However, no proposal on this existed and therefore no action could be taken on it. Another participant said that may be complicated to accomplish. Further study of that possibility was encouraged. It was also remarked that, depending on how this is done, it might harm the goal of harmonization with the ordinary inter prediction design.

It was remarked that 8x8 IBC bipred does not violate the worst-case memory bandwidth, but may be prohibited with the expressed constraint. This can also be further studied.

The 5.2 proposed approach prohibits the combination of 8x8 bipred with IBC. The 5.3 proposed approach only prohibits this if the MV/IBC displacement vectors are not integers.

Decision: Adopt U0078 solution 5.3 (the less restrictive form of the constraint, to be applied in all SCC profiles).

Further study was encouraged regarding whether the constraint should be released when using IBC 8x8 bipred (which is not currently supported by the encoder), and whether a restriction via syntax rather than a semantics constraint would be better.

JCTVC-U0175 BoG report on worst case memory bandwidth assessment [K. Rapaka]

This BoG report was presented and discussed Monday 06-22 at 18:00 (chaired by JRO).

Both memory read and write are taken into account in the bandwidth calculation. For memory reading, bandwidth is calculated as number of reference samples fetched from the memory based on some memory patterns such as 4x2 etc. It is assumed that memory can be accessed and read based on the memory patterns, that the chroma storage is not interleaved and only integer number of blocks can be fetched.

The following analysis was performed in the BoG related to CE2:

- Analysis of increase in the memory bandwidth due to intra block copy – see table below.

| memory pattern | SCM4.0 Vs HEVC |
| --- | --- |
| 4x1 | 112.83% |
| 8x1 | 109.76% |
| 4x2 | 112.00% |
| 8x2 | 109.09% |
| 4x4 | 109.52% |

** These numbers assume that the additional write of unfiltered IBC samples is fully aligned.

Note from discussion in Track B: The assumption of alignment seems to be justified, as it is likely that the write is executed once per CTU. It is also noted that such a memory access per CTU is more regular and may be less critical than for reading where random access depending on BV occurs.

- Worst case per sample memory assessment of the CE2 Test under 4:4:4 chroma format and comparison against HEVC v1.

| Memory Patterns | CE2 Tests (per sample memory access) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Test 2 | Test3 | Test5.1 | Test 5.2 | CE2Test5.3 | CE2 Test 5.4 |
| 4x1 | 23.375 | 23.375 | 23.375 | 21.75 | 21.75 | 23.375 |
| 8x1 | 30.750 | 30.750 | 30.750 | 28.25 | 28.25 | 30.750 |
| 4x2 | 25.000 | 25.000 | 25.000 | 23 | 23 | 25.000 |
| 8x2 | 33.000 | 33.000 | 33.000 | 30 | 30 | 33.000 |
| 4x4 | 31.500 | 31.500 | 31.500 | 28 | 28 | 31.500 |
|  | CE2 Tests Vs HEVC V1 |  |  |  |  |  |
|  | Test 2 vs HEVC v1 | Test 3 vs HEVC v1 | Test 5.1 vs HEVC v1 | Test 5.2 vs HEVC v1 | Test 5.3 vs HEVC v1 | Test 5.4 vs HEVC v1 |
| 4x1 | 100.00% | 100.00% | 100.00% | 93.05% | 93.05% | 100.00% |
| 8x1 | 100.00% | 100.00% | 100.00% | 91.87% | 91.87% | 100.00% |
| 4x2 | 100.00% | 100.00% | 100.00% | 92.00% | 92.00% | 100.00% |
| 8x2 | 100.00% | 100.00% | 100.00% | 90.91% | 90.91% | 100.00% |
| 4x4 | 100.00% | 100.00% | 100.00% | 88.89% | 88.89% | 100.00% |

4:2:0 Chroma interpolation

- Increase in the bandwidth due to chroma interpolation when only IBC is used and no temporal referencing:

| Memory Patterns | SCM 4.0 – IBC only (4:2:0) (per sample memory access) | JCTVC-U0080/U0103/U077 – IBC only (4:2:0) (per sample memory access) |
| --- | --- | --- |
| 4x1 | 7.5 | 9.5 |
| 8x1 | 9 | 12.25 |
| 4x2 | 9 | 10.75 |
| 8x2 | 11 | 14 |
| 4x4 | 10.5 | 13.5 |

- Worst case per sample memory assessment of chroma interpolation methods (JCTVC-U0080/00103/U0077) under 4:2:0 chroma format and compared against SCM 4.0- 4:2:0.

| Memory Patterns | SCM 4.0 4:2:0 (per sample memory access) | JCTVC-U0080/U0103/U0077 (per sample memory access) |
| --- | --- | --- |
| 4x1 | 15.875 | 15.875 |
| 8x1 | 19.500 | 19.500 |
| 4x2 | 17.000 | 17.000 |
| 8x2 | 21.000 | 21.000 |
| 4x4 | 21.500 | 21.500 |

- Compare worst case SCM 4.0 – 4:4:4 chroma methods vs. chroma interpolation methods.

| Memory Patterns | SCM 4.0 – 4:4:4 (per sample memory access) | SCM 4.0 – 4:2:0 IBC (per sample memory access) |
| --- | --- | --- |
| 4x1 | 26.375 | 15.875 |
| 8x1 | 33.75 | 19.500 |
| 4x2 | 28 | 17.000 |
| 8x2 | 36 | 21.000 |
| 4x4 | 34.5 | 21.500 |

Basically, the worst case does not change for chroma interpolation, since 8x8 bi-pred still requires more memory accesses; the penalty of IBC is still caused by the duplicate write.

The worst case when adopting chroma interpolation for 4:2:0 without any restrictions would be the same as for current SCM (see under "a" above). However, the investigation does not answer whether a combination of chroma interpolation with CE2 (as per table in "b" above) would still be less or equal the worst case of version 1.

During the discussion, concerns were raised advocating that SCC using IBC should not increase worst case memory bandwidth relative to version 1 (in the 4:2:0 case) or RExt (in the 4:4:4 case). The latter is proven for the methods from CE2 by table b) (where it should be said that 5.2 and 5.3 are still at 100% because it could still always use 8x8 bi-pred. The breakout group was asked to make a follow-up analysis to see whether this is still true for the 4:2:0 case (with chroma interpolation on and off).

(Further consideration of this topic was chaired by JRO and GJS on Thursday 06-25, 09:00-10:00.)

A modified version of BoG report was presented. The tabulated data above has been adjusted to include the updated numbers.

Further BoG Discussion was held Tuesday 06-30 at 17.40.

It was asked whether additional configurations of memory patterns (e.g., different burst modes) need to be included. After discussion, it was concluded that the same patterns (4x2 burstsize 1 and 8x2 burstsize 1) as used in the HEVC v1 and SHVC methods analysis be considered. 8x8 bipred was confirmed to still be the worst case.

For IBC stand-alone without restrictions, the worst case memory bandwidth for adding intra block copy increases by 12.8% for 4:4:4 and 10% for 4:2:0 (without chroma interpolation). With restrictions "5.2" and "5.3", the worst case memory bandwidth is not increased (with or without chroma interpolation).

After analysis of the memory numbers in this document, the BoG concluded that the worst case memory bandwidth for the following cases does not cross the HEVC v1 worst case memory bandwidth limit:

- 4:4:4 + CE2 Test 2,3, 5.1,5.2,5.3,5.4

- 4:2:0 + CE2 Test 2,3, 5.1,5.2,5.3,5.4

- 4:2:0 + chroma interpolation methods in (U0077,U0080,U0103)

- 4:2:0 + chroma interpolation methods in (U0077,U0080,U0103) + CE2 Tests 5.2, 5.3

Conclusion from JCT-VC plenary:

The worst case memory bandwidth for IBC without restrictions increases by approximately 12.83% for 4:4:4, and by 10.43% for the 4:2:0 case (with or without chroma interpolation) (relative to RExt for 4:4:4, and version 1 for 4:2:0). When restrictions 5.2/5.3 are applied, the worst case memory bandwidth is not increased relative to previous versions of HEVC.

### CE2 primary contributions (3)

JCTVC-U0034 CE2: Test 3 on intra block copy constraints on filtering (single-pass encoding decisions) [J. Lainema, M. M. Hannuksela (Nokia)]

JCTVC-U0053 CE2 Test 2: Intra block copy constraints on filtering [G. Laroche, G. Malard, C. Gisquet, P. Onno (Canon)]

JCTVC-U0078 CE2: Test 5 on intra block copy constraints on prediction [K. Rapaka (Qualcomm)]

### CE2 cross checks (3)

JCTVC-U0035 CE2: Crosscheck of Test 2 on intra block copy constraints on filtering (frame level encoding decisions) [J. Lainema (Nokia)] [late]

JCTVC-U0060 CE2: Cross check of Test 5 on Intra block copy constraints on bi-prediction samples with use of local cache [P. Onno, G. Malard (Canon)] [late]

JCTVC-U0082 CE2: Cross check of CE2 Test 2: Intra block copy constraints on filtering [K. Rapaka (Qualcomm)] [late]

# Non-CE Technical Contributions (99)

## SCC coding tools (86)

### CE1 related (palette mode improvements) (49)

JCTVC-U0063 CE1-related: Colour-plane-based escape pixel coding [T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 15:30-16:00.)

In HEVC residual decoding, the decoder decodes the transform block (TB) one by one among three colour components. Therefore, in HEVC decoder architecture, only a coefficient buffer for single colour component is required in entropy decoder. In SCC palette mode coding, there is no residual to be decoded, so it is suggested that the residual coefficient buffer can be reused to store the palette index map information, including palette index and escape values. However, in SCM-4.0 escape pixel coding, the escape values of three components of one sample are coded together. It is thus reported that three coefficient buffers for three colour components are required in the worst case, which increases the implementation cost and complexity of palette mode. This contribution proposes a colour-plane-based escape pixel coding to reduce the buffering requirement of escape pixel coding. The escape values of the same colour component are proposed to be grouped together first and signalled group by group. The entropy decoder can then decode the escape values one colour component at a time, so that only one coefficient buffer for a single colour component is required. The experiment results reportedly show that there is no BD-rate change.

The only syntax change is a change of loop nesting – swapping the nesting of the loop for colour components and the loop for the number of escape-coded pixels.

The proponent pointed to IPCM as another example where spatial grouping of component groups is ordered in the same manner.

It was noted that in the 4:2:0 and 4:2:2 cases, the number of colour components varies from pixel to pixel, and remarked that this is part of the justification for sending the colour planes separately for the IPCM case. It was discussed whether it seems easier to perform the parsing if the data is grouped as proposed, but the answer did not seem entirely clear.

It was remarked that a smaller buffer could be used (at least theoretically) with the proposal since only a buffer large enough to hold a single component would be needed rather than a buffer big enough to hold all three colour components.

U0087 approach #2 proposes the same change. See the notes on U0087.

JCTVC-U0154 Cross check CE1-related: Colour-plane-based escape pixel coding (U0063) [W. Pu (Qualcomm)] [late]

JCTVC-U0087 Syntax Cleanup for Palette Mode [W. Pu, R. Joshi, T. Hsieh, M. Karczewicz, F. Zou, V. Seregin (Qualcomm)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 16:00-16:30.)

In the current HEVC SCC draft specification, new palette entries are signalled by grouping each colour component while the escape pixels are signalled by interleaving the three colour components. In this contribution, two unification methods are proposed to clean up the palette mode syntax.

Approach #1 is to swap the loops for sending the palette entries so that they are sent in component-interleaved form as is done with the escape-coded entries.

Approach #2 is to swap the nesting of the loops for sending the escape-coded entries so that they are sent in component-grouped order.

Approach #2 of this is the same as proposed in U0063.

It was suggested that approach #1 is more consistent with the current PPS palette predictor syntax.

It was suggested that approach #2 could use less memory if the decoder fills in the decoded values into the residual buffer(s) as it performs the decoding process (versus filling in an indexed array of buffered content that is scanned later to reconstruct the CU residual).

It was suggested that approach #2 is more consistent with how we do most of the design (e.g., transform blocks and DPCM residuals scanned in component-priority order).

It was not entirely clear whether it really matters, but one approach needed to be chosen.

Decision: Adopt approach #2 (and also swap the loop order to reflect this in the PPS).

JCTVC-U0064 CE1-related: Palette coding with inter-prediction [W. Zhu, K. Zhang, X. Zhang, S. Lei (MediaTek)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 18:15-18:30.)

This contribution presents a new palette mode named inter-palette, which combines the palette mode and the inter mode directly. When a CU is coded as the proposed inter-palette mode, the motion information is signalled in the same say as the inter mode with SIZE2Nx2N. Thus a prediction block can be generated by motion compensation. And samples in the CU are coded in the same way as in the palette mode, except that an additional copy method called 'COPY_INTER' is appended in addition to the existing INDEX and COPY_ABOVE schemes. If a sample uses COPY_INTER, it will copy the value of the corresponding sample in the prediction block directly. Experiment results reportedly show that the segmental prediction method can achieve 2.6%, 1.8% and 0.9% BD-rate savings under AI, RA, and LB conditions, respectively, with constrained intra block copy searching; and 2.0%, 1.3% and 0.7% BD-rate savings under AI, RA, and LB conditions, respectively, with full-frame intra block copy searching for “RGB, text & graphics with motion, 1080p & 720p sequences”.

It was commented that it seems late in the process to try to propose a substantially different additional coding mode type.

It was commented that the gain is also not so large, and encoder optimization was mentioned as a potential factor that could be affecting our current performance.

It was commented that U0116 may have some similarities, although it is referencing the current picture rather than a different reference picture.

No action was taken on this.

JCTVC-U0162 Cross-check of CE1-related: Palette coding with inter-prediction (JCTVC-U0064) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-U0066 CE1-related: Row-based copy pixel from neighbouring CU [T.-D. Chuang, Y.-C. Sun, J. Kim, Y.-W. Chen, S. Liu, Y.-W. Huang, S. Lei (MediaTek)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 18:30-18:45.)

In this contribution, a row-based pixel copy operation from the neighbouring CU is proposed to reduce the complexity relative to CE1 Test A1 and A2 while maintaining the coding gains. A NumCopyPixelRow is first signalled to indicate the number of rows copied from the neighbouring CU. The rest of the rows in the CU are coded by the original palette index map coding scheme in SCM-4.0. Experiment results reportedly show that, compared with SCM-4.0 with full-frame IntraBC search, 1.4%, 0.9%, and 1.0% BD-rate savings are shown for “YUV, text & graphics with motion, 1080p & 720p sequences” under AI, RA, and LB, respectively; 1.8%, 1.3%, and 1.0 % BD-rate savings are shown for “RGB, text & graphics with motion, 1080p & 720p sequences” under AI, RA, and LB, respectively.

Similar comments were made for this proposal as for U0064.

No action was take on this.

- JCTVC-U0168 Crosscheck of JCTVC-U0066: CE1-related: Row-based copy pixel from neighbouring CU [F. Zou (Qualcomm)] [late]

JCTVC-U0086 CE1-related: Simplification on coding NumPaletteIndices [J. Ye, J. Kim, S. Liu, P. Lai, W. Zhu, K. Zhang, T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 17:00-18:15.)

This contribution proposes to modify the derivation process of NumPaletteIndices, i.e. the number of palette indices signalled for the current block. Two methods are provided to reduce the condition checks in the derivation process of NumPaletteIndices. Experiment results reportedly average −0.1% to 0.1% luma BD rate changes across all testing configurations and classes by using the proposed methods, compared with the SCM4.0 anchor.

Contributions U0086 (methods 1 & 2), U0093, U0099, U0105 are closely related to each other.

It was commented that U0086 method 1 seems very simple and straightforward and understandable (NumPaletteIndices = (num_palette_indices_minus1 + 1)), although it does have a (tiny) loss in some cases.

It was commented that method 2 could have some penalty for encoders that don't check whether all palette entries are used.

Decision: Adopt U0086 method 1.

JCTVC-U0093 Non-CE1: On Number of Palette Indices coding [S.-H. Kim, K. Misra, J. Zhao, A. Segall (Sharp)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 17:10-18:15.)

In the current SCC draft, num_palette_indices_idc is partitioned into three non-contiguous sets, and the variable NumPaletteIndices is derived using an equation selected based on the set membership of num_palette_indices_idc. The existing derivation process was asserted to be complex. This contribution proposes the use of a fixed predictor (2 * MaxPaletteIndex) and a signalled residual to derive NumPaletteIndices. This approach was asserted to be much simpler. It was reported that the proposed simplification provides a maximum luma bit rate difference of 0.1% for all configurations, including AI, RA, and LD(B) for lossy and lossless, and for the 4:4:4 and 4:2:0 formats.

Contributions U0086 (methods 1 & 2), U0093, U0099, U0105 are closely related to each other.

The proposal replaces one syntax element with two, for which one of them is sometimes not sent. It was commented that this is a bit more complicated than some other approaches, and thus perhaps not the best.

JCTVC-U0125 Cross-verfication of JCTVC-U0093: Non-CE1: On Number of Palette Indices Coding [X. Xiu (InterDigital)] [late]

JCTVC-U0099 Modified method for sending number of palette indices [R. Joshi, W. Pu, M. Karczewicz, F. Zou, V. Seregin (Qualcomm)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 17:15-18:15.)

It was asserted that there is a mismatch between HEVC SCC text specification version 3 and SCM 4.0 software in the derivation of NumPaletteIndices. Furthermore a modification of the derivation of NumPaletteIndices is proposed. This modification removes one condition check and associated operations. Simulation results are provided to show that there is virtually no BD-rate change due to this modification.

The contribution notes that the current text has an error, which would need to be corrected if some other approach is not chosen.

Contributions U0086 (methods 1 & 2), U0093, U0099, U0105 are closely related to each other.

It was commented that this is a bit more complicated than other approaches, and thus perhaps not the best.

JCTVC-U0105 Non-CE1: Simplification on deriving NumPaletteIndices for palette mode [X. Xiu, Y. He, Y. Ye (InterDigital)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 17:25-18:15.)

This contribution proposes to modify the derivation of NumPaletteIndices, which specifies the number of palette indices signalled for the current block. Compared to the SCM-4.0 anchor, the proposed method reportedly has no impact on the average coding performance. It was asserted that the proposed modification simplifies the decoding of NumPaletteIndices.

Contributions U0086 (methods 1 & 2), U0093, U0099, U0105 are closely related to each other.

This proposal would add a constraint that disallows sending palette entries that are not used in the CU. It was commented that the current reference software sometimes does this, and that disallowing it might sometimes require an encoder to perform an extra pass to avoid letting this happen. It was therefore suggested not to choose this particular approach.

JCTVC-U0155 Crosscheck of JCTVC-U0105: Non-CE1: Simplification on deriving NumPaletteIndices for palette mode [W. Digonnet, C.-C. Lin, J.-S. Tu, Y.-J. Chang, C.-L. Lin (ITRI)] [late]

JCTVC-U0131 Crosscheck for simplification on coding numPaletteIndices (JCTVC-U0086) [W. Zhang, L. Xu, Y. Chiu (Intel)] [late]

JCTVC-U0090 CE1-related: Palette Mode Context and Codeword Simplification [J. Ye, J. Kim, S. Liu, P. Lai, W. Zhu, K. Zhang, T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 18:45-19:00.)

This contribution proposes to modify the palette coding process in three ways.

- First, the context for last_palette_run_type_flag is proposed to be removed, with last_palette_run_type_flag sharing the context with palette_run_type_flag. Experiment results of item 1 reportedly average −0.13% to 0.17% luma BD rate changes across all testing configurations and classes, compared with the SCM4.0 anchor. This was also proposed in U0148. It reduces the number of contexts by 1 and eliminates an apparently-useless need to treat the last palette run type differently. Decision: Adopt.

- Second, palette_transpose_flag is proposed to be signalled after the last_palette_run_type_flag to group the CABAC context coded bins together. Experiment results of item2 reportedly show no BD rate changes compared with the SCM4.0 anchor. This is part of what is contained in U0133. See the notes for U0133.

- Third, a modified binarization for num_palette_indices_idc is proposed based on a simplification of a derivation of the Rice parameter to avoid a division by 6 – changing it from c=2+Idx/6 to c=2+((Idx+4)>>3). Experiment results for item3 reportedly average −0.1% to 0.1% luma BD rate changes across all testing configurations and classes, compared with the SCM4.0 anchor. It was remarked that this is closely related to U0169. See the notes for U0169, and subsequently U0176.

JCTVC-U0126 Crosscheck report of JCTVC-U0090 [S.-H. Kim (Sharp)] [late]

JCTVC-U0148 Non-CE1: Context model unification for palette run type flags in HEVC SCC [M. Xu, W. Wang, F. Duanmu, H. Yu (Huawei)] [late]

(Consideration of this topic was chaired by GJS on Friday 06-19, 18:45-19:00.)

This proposal is the same as item 1 of U0090. See the notes for that contribution.

JCTVC-U0166 Cross-check of Non-CE1: Context model unification for palette run type flags in HEVC SCC (JCTVC-U0148) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-U0133 Comment on signalling the palette_transpose_flag after last_palette_run_type_flag (JCTVC-U0090) [R. Joshi, V. Seregin, M. Karczewicz, W. Pu, F. Zou (Qualcomm)] [late]

(Consideration of this topic was chaired by GJS on Friday 06-19, 19:00-19:10.)

In JCTVC-U0090, it is proposed that the palette_transpose_flag be signalled after last_palette_run_type_flag to group bypass bins together. It is asserted that when syntax elements related to palette delta QP or palette chroma QP offset are present, this does not result in grouping of more than one additional bypass bin. It is proposed that in addition to placing the palette_transpose_flag after last_palette_run_type_flag, the syntax elements related to palette delta QP and palette chroma QP offset be signalled after the palette_transpose_flag. It is asserted that due to this reordering, all the bins up to last_palette_run_type_flag are bypass-coded and grouped together.

Part of this proposal is also proposed in item 2 of U0090.

The basic motivation is to group together bypass-coded data in the palette mode syntax.

Decision: Adopt.

JCTVC-U0169 CE1 Related: Simplification of num_palette_indices_idc coding parameter derivation [F. Zou, V. Seregin, M. Karczewicz, W. Pu, R. Joshi (Qualcomm)] [late]

(Consideration of this topic was chaired by GJS on Friday 06-19, 19:10-19:20.)

This document presents a simplification method of Rice parameter derivation for numIndices in palette coding. The results were tested on top of SCM4.0 as well as on top of CE1 A.1 and A.2. Simulation results reportedly show that the proposed simplification does not impact the performance of SCM4.0 for AI full frame IBC.

This is closely related to item 3 of U0090.

Like U0090, a modified binarization for num_palette_indices_idc is proposed for simplification of the derivation of the Rice parameter, to avoid a division by 6 – changing it from c=2+Idx/6 to c=2+(Idx>>3).

Basically no effect on coding efficiency was reported.

It was remarked that since U0086 method 1 changed the values that will be coded by this, a new test should be run to identify whether the modified Rice parameter derivation would have a coding efficiency effect under the new circumstances. It was also suggested that using the modified non-normative encoding method U0096 in further testing would be desirable, just to make sure it does not affect anything.

Further discussion of this and U0090 occurred after obtaining additional test results for each. See the notes for U0176.

JCTVC-U0091 CE1-related: On the mismatch of SCM4.0 and HEVC SCC working draft [J. Ye, S. Liu, P. Lai, S. Lei (MediaTek)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 19:20-19:25.)

This contribution reports a mismatch between the SCM 4.0 software and HEVC SCC draft. This mismatch is about how “maxPaletteRun” is derived when palette mode is used. Changes to the draft text to match SCM 4.0 were provided, as well as fixes on SCM4.0 to match the draft text. Simulation results reportedly showed averages of −0.13% to 0.17% luma BD rate changes across all testing conditions and classes when the software was modified to align with the draft text.

Decision (BF): Change the text to match the software.

JCTVC-U0124 Crosscheck of JCTVC-U0091, CE1-related: On the mismatch of SCM4.0 and HEVC SCC working draft [R. Cohen (MERL)] [late]

JCTVC-U0089 Constraints on palette syntax elements [P. Lai, J. Kim, S. Liu, J. Ye, T.-D. Chuang, S. Lei (MediaTek)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 19:25-19:55.)

This contribution presents constraints on palette syntax elements.

- A constraint on the SPS parameter palette_max_size was proposed to require the palette_max_size to be in the range of 0 to 1 << (MaxTbLog2SizeY * 2). However, this does not seem fundamentally necessary. The maximum palette size would have some constraint such as a profile/level-specified constraint, and this seems sufficient.

- Decision (Ed.): Regarding constraining the sensibility of NumPaletteIndices, instead constrain the derived position to lie within the CU (or something mathematically equivalent).

- Regarding constraining num_signalled_palette_entries, no action seemed needed because of other constraints that are already specified.

- Decision (Ed.): Regarding palette_predictor_run, constrain the derived position within the palette predictor to not exceed the size of the palette predictor (or something mathematically equivalent).

Post-meeting note: These issues were further discussed during post-meeting draft text editing. It was noted that the truncated code used for representing the value of NumPaletteIndices would not allow improper values of NumPaletteIndices to be represented, which seems to resolve that issue. Double-checking of the draft text is encouraged to ensure that it expresses appropriate constraints, to the extent that such constraints are needed.

JCTVC-U0163 Cross-check of Non-CE1: Palette Run Hiding (JCTVC-U0094) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-U0094 Non-CE1: Palette Run Hiding [M. Karczewicz, W. Pu, R. Joshi, F. Zou, V. Seregin (Qualcomm)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 19:55-20:05.)

It was reported that in palette mode, the first or the last run in the block is generally longer than the average run length for the block. In this document, it was proposed to use a flag to indicate whether the first or the last run length is not coded in the bitstream. It was reported that for RGB and YUV text & graphics with motion, 0.2% and 0.3% BD-rate reduction were achieved, respectively, for the All-Intra configuration.

It was commented that although the idea seems interesting, it complicates the syntax and design while providing very little gain. No action was taken on this.

JCTVC-U0096 CE1 Related: Improved palette encoder [M. Karczewicz, F. Zou, V. Seregin, W. Pu, R. Joshi (Qualcomm)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 15:10-15:25.)

This document proposes encoder-only changes for palette mode relative to the current SCM-4.0 software. The changes include: 1) using squared error to derive the palette and for index assignment; 2) eliminating some of the palette entries if the related pixels can be represented by other palette indices; 3) merging of two consecutive runs and jointly optimizing the run values of index run and copy above run pair. Experiment results reportedly show that compared to SCM-4.0 anchor on 4:4:4 sequences, the proposed combination reportedly provides an average of 1.3% and 1.8% BD-rate savings for the category of text & graphics with motion, 1080p & 720p in both RGB and YCbCr colour formats, respectively, in AI with full-frame IBC, and 1.5% and 2.2% BD-rate savings for the category of text & graphics with motion, 1080p & 720p in both RGB and YCbCr colour formats, respectively, in AI with 4CTU IBC. For 4:2:0 sequences, the corresponding BD-rate savings of the proposed method was reportedly 1.2% for AI.

Part of this is similar to U0127, but in a somewhat simplified/sped-up form.

When tested in combination with normative changes from subtest C.1, additional gain was shown, but with most of the gain being from the non-normative aspects.

When compared to U0127, this contribution includes additional non-normative improvements, which provide additional gain.

The run-time is somewhat slower than the anchor – about 6-8% slower.

Decision (N-N): Adopt.

JCTVC-U0145 Cross-verification of JCTVC-U0096: Improved palette encoder [X. Xiu (InterDigital)] [late]

JCTVC-U0097 Non-CE1: On maximum palette predictor size [Y.-J. Chang, C.-C. Lin, C.-L. Lin, J.-S. Tu (ITRI)]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 09:00-10:00.)

Although the SCM4.0 sets the maximum palette predictor size as 128, there is no upper limitation for the maximum palette predictor size in the SCC draft text. This contribution presents four potential constraints and a proposed cleanup for the palette predictor size. The contribution also reports the rate-distortion evaluation of different maximum palette predictor sizes that are smaller than 128. It was reported that smaller values of the maximum palette predictor size, such as 80, 96 and 112, only change coding performance by 0.0-0.2% for All-Intra lossy coding and improve the coding time up to 1%.

It was remarked that the plan was to express limits in profile/level specifications; the lack of a currently-expressed limit is merely because we don't yet have a profile/level specification for SCC. The CTC establishes limits of 63 and 128 for the max palette size and max palette predictor size, respectively.

The contributor reported that the software currently requires recompilation to change these, as they are set by a macro rather than, or in addition to, a config parameter. Decision (SW): Fix this (if necessary and feasible) by managing the sizes in the config file so that experiments with different sizes (within reason) can be run without recompiling.

The contributor suggested having the supported max palette predictor size be up to 128 or 96 or twice the max palette size or 1.5 times the max palette size. It was commented that a fixed number seems like a simple and adequate approach, and there is no apparent need to make the constraint different for different max palette sizes.

An editorial error in the spec was also reported, with naming confusion between max_predictor_palette_size and PaletteMaxPredictorSize. Decsion (Ed.): Fix this.

Test results were reported with max palette predictor sizes of 112, 96, 80 (relative to the current 128 as anchor). For lossy coding, the penalties were small (average penalty in each class at most 0.3%). For lossless coding, the penalties were somewhat larger (average penalty in each class at most 0.7%).

It was remarked that we should just make the limit be 128 unless it seems like the complexity savings for reducing it would be significant.

JCTVC-U0143 Crosscheck report of JCTVC-U0097 [S.-H. Kim (Sharp)] [late]

JCTVC-U0109 Non-CE1: Simplification for parsing the index of the most significant bit in the paletteRun binarization [Y.-J. Chang, C.-L. Lin, C.-C. Lin, J.-S. Tu, W. Digonnet (ITRI)]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 10:00-10:20.)

In the current palette design, the pixel-level palette run value is derived from the combination of the index of the most significant bit (MSB) and the refinement bits, i.e., the syntax elements palette_run_msb_id_plus1 and palette_run_refinement_bits. The first five bins of palette_run_msb_id_plus1 are context-coded in the current design, and the other bins are coded in bypass mode. It was reported that larger CUs usually have more palette runs, such that a 32x32 CU coded by palette mode will result in slower CABAC throughput on the parsing of the palette run values than for the other CU sizes, due to dependency of context-coded bins. It was reported that the fourth and fifth bin have approximately equal occurring probability for being 0 or 1. The contribution proposes to change the derivation process of palette_run_msb_id_plus1 by using the bypass mode to code the fourth bin and the fifth bin. It is reported the BD-rate increases are 0.0-0.3% per class with an average across all classes smaller than 0.1%.

Results were provided for moving the bit plane switch-over by two bit positions, but not by moving it just one position.

It was commented that this does not affect the worst case, which is the case where the runs are all small. Larger runs are easier to handle than smaller runs, so making larger runs simpler doesn't help much. It was remarked that as a percentage of the overall total complexity of the decoding process, there may really be no measurable benefit, since it would only help when the mode is palette and the blocks are large and the runs are large, which are not what is likely to be what is consuming much of the total processing requirements of the decoding process.

No action was taken on this.

JCTVC-U0136 Cross-checking of JCTVC-U0109: Non-CE1: Simplification for parsing the index of the most significant bit in the paletteRun binarization [Y. He, X. Xiu, Y. Ye (InterDigital)] [late]

JCTVC-U0116 Non-CE1: Enhancement to Palette Coding by Palette with Pixel Copy (PPC) Coding [Tao Lin, Kailun Zhou, Liping Zhao, Xianyi Chen (Tongji)]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 10:20-11:15, and it was further discussed Sunday 06-21, 09:45-10:15.)

A "palette with pixel copy" (PPC) submode was proposed to be added to the existing palette mode. It is proposed that a CU either be coded by PPC or the current palette mode. The palette predictor is shared by the two modes. The palette in PPC is proposed to be used only for left-copy operation of the current palette mode. The search range for PPC was proposed to be either 2CTU (current and left one) or 4CTU (current and left three). The coding performance was reported under two configurations of search/reference range: (TC1) full frame (FF) IBC versus 2CTU PPC and (TC2) constrained 4CTU IBC vs 4CTU PPC. Using SCM40 as anchor, the coding results for the Y component of YUV TGM AI lossy coding were reported as −4.2% and −7.7%, respectively, for TC1 (full-frame IBC) and TC2 (4x1 IBC). The reported encoding time impact is a 5-6% increase.

The concept of the new mode/submode is similar to what we have previously studied as intra string copy (ISC). The proposed submode has alternating runs of strings of pixels that are coded by a simplified palette method and strings of pixels that are sample-copied strings.

There was a discussion of the application of this to 4:2:0 coding and how this would work. The contributor said that it was designed to function for 4:2:0 as well as 4:4:4. It was remarked that 4:2:0 test results may be missing from the contribution.

Relative to prior studies of ISC, this proposal has significantly more coding efficiency improvement and much lower encoding complexity. The additional improvement of coding efficiency was reportedly from making an effective combination of palette and ISC within a CU and from fixing some bugs.

The contributor noted that CUs that don't use the palette mode interrupt the palette prediction process, and said that allowing references to samples is a way of making accessible to a palette coded region the values in nearby regions that were coded by such other modes.

It was asked why the reported encoder complexity is not higher, due to the need to search for strings of samples to copy. The contributor said that the search is sped up by the use of a (small) hash table (based on the four MSBs of the sample value) for performing the search process. The hash concept is similar to what has been used in IBC testing.

A participant remarked that the worst-case memory access behavior of the proposed mode may be difficult to support.

The amount of text that would need to be added to support this is substantial (12 pages as proposed).

There is also an interaction between constrained intra prediction and the proposed mode.

Although the reported gain is interesting, given the late stage of the project, it seems very difficult to contemplate adding such a new mode. Adopting this would put our schedule at serious risk.

No action was recommended by track A. However, it was agreed that the JCT plenary reporting from Track A should highlight this topic.

(Further consideration of this topic was chaired by GJS and JRO on Sunday 06-21, 12:00-12:30.)

It was commented that, as proposed, decoders would need to have access to a cache that holds 4 CTUs in order to obtain the reported gain, which is a substantial requirement. Reducing the caching, e.g., from 4 CTUs to 2 CTUs, would reduce the amount of coding efficiency benefit. The proposal has a significant caching impact.

It was commented that the change is substantial (e.g., a new displacement offset coding similar to MV coding) and that other potential improvements of coding efficiency were not taken (e.g., for the goal of harmonizing IBC and inter). The stability of the specification is probably the most significant issue.

See also the notes for U0171, U0182, and U0186, and the joint discussion notes of section 6.2.

No action was taken on this.

JCTVC-U0189 Cross-check report of U0116: Enhancement to Palette Coding by Palette with Pixel Copy (PPC) Coding [W. Wang, M. Xu (Huawei)] [late]

U0151 was previously registered for a cross-check report, but was then accidentally marked as withdrawn. A new upload to fix this was requested. U0189 was provided as the replacement.

JCTVC-U0171 Non-CE1: Palette with Pixel Copy (PPC) decoder IC reference design [K. Zhou, T. Lin (Tongji)] [late]

(Consideration of this topic was chaired by GJS on Sunday 06-21, 09:45-10:15.)

This contribution is closely related to U0116. It describes a decoder IC implementation reference design for the proposed palette with pixel copy (PPC) mode. It was reported that it supports 4K decoding at 30 fps, with a working clock frequency of 330 MHz and a gate count of 6.2K gates, synthesized using 65nm CMOS technology standard library, with no extra SRAM reportedly needed. The memory bandwidth needed for the scheme was reported to be less than for some other parts of the processing.

In the review, a participant commented that some possibilities for parallelization did not seem to be fully accounted for in the comparison to other parts of the decoding process.

See also the notes for U0116, U0182, and U0186.

JCTVC-U0182 Non-CE1: Further harmonization of PPC with no-pixel-copy palette coding [T. Lin, K. Zhou, L. Zhao, X. Chen (Tongji)] [late]

(Consideration of this topic was chaired by GJS & JRO on Tuesday 06-23, 12:00-12:15.)

See also the notes for U0116, U0171, and U0186.

This contribution proposes further modification of the proposed palette with pixel copy (PPC) scheme that modifies its relationship to the prior (no-pixel-copy) palette coding scheme. The contributor asserted that after this modification, the only real difference between PPC and the prior palette coding is that PPC has one additional syntax element called "offset". Everything else, including binarization, was asserted to be the same in PPC and the prior palette coding.

It was asked why a separate submode is needed, rather than having a single mode that can do string copy as well as palette coding. This was said to be related to the way the choices are coded and the associated "redundancy removal" process that provides a more efficient syntax.

As previously noted, it was commented that the proposed PPC mode requires the decoder to store 4 CTUs in cache memory, which is a substantial memory storage need that is not a requirement for any other part of the design. It was said that this is not a small and simple change of the palette mode. No action was taken on this.

JCTVC-U0186 Flexible coding tools to significantly improve SCC performance in cloud and mobile computing [L. Zhao, W. Cai, J. Guo, T. Lin (Tongji Univ.)] [late]

This was a very late contribution. The contributor was not available to present the contribution Thu. evening or Fri. at 12:00 as the meeting was closing, so it was not presented. The contribution is available for study.

This document reports that some coding tools using a flexible basic copy unit can improve SCC performance by up to 38% BD-rate reduction in some SCC application scenarios that are asserted to be typical in cloud and cloud-mobile computing, based on some test results including some pictures not in the current SCC CTC. The contribution proposed adding some additional pictures to the SCC CTC and recommended to adopt PPC (U0116) subject to the condition that if any serious problem would be found between now and the next meeting, it would be removed from the draft. The contribution asserts that coding tools like Intra String Copy (ISC) or Palette with Pixel Copy (PPC) can better handle a variety of patterns in a picture using a flexible basic copy unit that is asserted to be able to achieve much better BD-rate reduction than the SCM40 SCC reference software on such test material.

See also the notes for U0116, U0171, U0182, and U0186.

No action was taken on this (as it was not presented).

JCTVC-U0146 CE1 Related: Combination of JCTVC-U0101 reverse scan and JCTVC-U0096 palette encoder improvement [V. Seregin, F. Zou, M. Karczewicz, W. Pu, R. Joshi (Qualcomm), X. Xiu, Y. Ye, Y. He (InterDigital)] [late]

See the notes on U0021.

JCTVC-U0149 Non-CE1: Index Map Splitting (IMS) Mode for Palette Coding in HEVC SCC [M. Xu, F. Duanmu, W. Wang, S. Minaee, H. Yu (Huawei)] [late]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 11:30-12:00.)

This contribution presents an index map coding scheme that involves splitting the index map into four equal-sized square sub index maps (SIMs). These SIMs would share the same palette table derived from the current CU and would be coded using existing index map coding approach (i.e., index mode and copy-above mode, respectively) with an adaptive scanning order. This proposal suggests improving the system performance by simultaneously processing the different SIMs in parallel. This index map splitting mode is applied on palette coded CUs with dimension ranging from 32x32 down to 8x8. Experiment results reportedly demonstrated a negligible BD-Rate performance loss (for lossy BD-Rate 1080p text & graphics content encoded using All intra configuration) but the contributor asserted a potential 4x processing efficiency improvement for index map encoding and decoding.

In the tested scheme, the splitting was not forced to be used – the encoder performed RDO decisions to determine whether to split the CU or not, and an extra flag was sent. The flag becomes a sort of palette sharing flag to signal palette sharing among the 4 pseudo-CUs.

It was asked why not to split the CU into smaller CUs (perhaps using the same palette) rather than introducing a new level of conceptual block hierarchy.

The parsing is sequential – that part is not parallelized, but the subsequent copying is proposed to be parallelized. It was commented that the ability to parallelize the copying is not so beneficial, such that the parsing is the main bottleneck issue, and that needs to be sequential with this scheme.

It was remarked that palettes and palette predictors are established during the parsing process, and within each CU the processing for the palette mode is separate and thus the copying process for multiple CUs that are coded in palette mode can already be parallelized.

Our current maximum CU size for palette mode coding is 32x32, and the contributor said they had done some testing of the coding efficiency impact of disabling some block sizes, but only by disabling the smallest block sizes rather than keeping the smaller block sizes and disabling the larger ones.

A participant reported that our prior decision to disable the 64x64 size had a coding efficiency penalty of about 0.3-0.4%. Disabling 32x32 also would presumably have a larger coding efficiency impact, but the amount was uknown.

No action was taken on this.

JCTVC-U0167 Cross-check of Non-CE1: Index Map Splitting (IMS) Mode for Palette Coding in HEVC SCC (JCTVC-U0149) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-U0036 On Zero Maximum Palette Size [K. Misra, S.-H. Kim (Sharp)]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 12:00-12:15.)

The current SCC draft design allows the maximum palette size to be set to 0. Zero palette sizes can be used to include blocks of quantized sample values within the bitstream as escape coded values.

When the maximum palette size is set to 0, this constraints the valid value for a subset of syntax elements. However, it was reported that the current SCC draft allows these syntax elements to take on invalid values. It was asserted that in such an event it is desirable to omit the inclusion of these syntax elements since the existing inference rules lead to valid values for the corresponding syntax elements. An alternative approach is to explicitly state conditional bitstream conformance requirements on the syntax element values.

The proposal concerns the high-level syntax.

When the palette_max_size is 0, it is proposed to not send the palette_max_predictor_size or to constrain its value. (Both of these are currently SPS-level information.) It was remarked that this change is not really necessary, as the current syntax will still function properly, although it may not seem very "clean" to send a non-zero value. Decision (BF/cleanup): Require palette_max_predictor_size to be equal to 0 when palette_max_size is 0.

It was proposed to condition the presence of palette_predictor_initializer_present_flag or to constrain its value based on the value of palette_max_size and palette_mode_enabled_flag. It was remarked that the presence conditioning would create a parsing dependency on PPS parsing using SPS syntax. Decision (BF/cleanup): Require palette_predictor_initializer_present_flag to be equal to 0 when palette_max_size or palette_mode_enabled_flag is 0.

JCTVC-U0052 On palette escape pixel coding [B. Li, J. Xu (Microsoft)]

(Consideration of this topic was chaired by K. Chono on Sunday 06-21, 15:00-15:40.)

Escape pixels are supported in the palette mode. However, the binarization process of escape pixels depends on the CU QP values. This introduces a parsing dependency between the palette escape pixel and the QP value. This document presents several methods to remove the parsing dependency between the palette escape pixel and the CU QP value.

Problem: Parsing dependency between the palette escape pixel and QP: the current binarization process of palette escape pixel depends on the CU QP value; this requires reconstruction of QP values for parsing palette escape pixel.

Three alternative methods are proposed:

Method 1: Use a constant QP value for palette escape pixels.

New QP values are introduced in the slice header to determine the order of Exp-Golomb binarization.

There is no BD-rate changes in CTC since QP is constant over the picture.

This method does not signal delta QP in palette mode CUs; the functionality to change the fidelity of pixel representations CU by CU is affected.

Method 2: QP-independent escape pixel binarization.

For the lossy case, a third order Exp-Golomb binarization is always used; for the lossless case, fixed-length coding is used.

This method reportedly has up to 0.3% coding loss in LDB.

This method changes only the binarization process; the QP is no longer used.

As in the current design, the fidelity of pixel representation can be changed CU by CU.

Method 3: Escape pixel binarization based on slice-level QP value.

For the lossy case, the slice QP value is used to determine the order of the Exp-Golomb binarization; for the lossless case, fixed-length coding is used.

This method reportedly has up to 0.3% coding loss in LDB.

This method changes only the binarization process; the slice QP is used instead of the CU QP.

As in the current design, the fidelity of pixel representation can be changed CU by CU.

Discussion:

The current design of palette escape pixel binarization was clarified. Palette escape pixel value is represented by TU binarization. Since the maximum palette escape pixel value changes according to the QP value, the QP is used in the parsing process.

It was questioned whether this dependency is really an issue since this parsing dependency occurs at the CU level. It was mentioned that in current design, the binarization process is changed coefficient by coefficient.

The proponent claimed that since the use of QP does not give significant gain, it is not straightforward to use the reconstructed QP only for palette escape pixel.

It was noted that a similar proposal (JCTVC-T0112) was presented in the last meeting.

An expert said that method 2 is a reasonable solution.

An expert commented that although TBC binarization is used in the current palette design, the method 2 (third order Exp-Golomb binarization) is acceptable.

Decision: Adopt Method 2.

JCTVC-U0153 Cross check: On the palette escape pixel coding (U0052) [W. Pu (Qualcomm)] [late]

JCTVC-U0084 On palette predictor initialization for Screen Content Coding [J. Ye, S. Liu, S. Lei (MediaTek)]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 15:45-16:10.)

This contribution proposes to allow signalling a palette predictor initialization in the SPS to reduce the overhead of signalling it in each PPS under some circumstances. When a proposed sps_palette_predictor_initializer_present_flag is equal to 1, a set of palette predictor initializers are proposed to be signalled in the SPS for the current CVS to use; otherwise, the entries in the palette predictor are initialized to 0. When pps_palette_predictor_initializer_present_flag is equal to 1, a set of palette predictor initializers are signalled in the PPS; otherwise, the entries in the palette predictor are derived from the SPS palette predictor initializers. Experiment results reportedly show similar BD rate performance by using the proposed method and the PPS palette predictor initializer scheme in SCM4.0 for the AI configuration. BD rate savings are observed when the proposed method is used for inter configurations.

The amount of data for these syntax elements is reported to be about 400 bytes.

Scaling list data was suggested as a similar example of SPS data with a PPS override.

It was asked whether we have anything similar in extensions.

It was noted that the reason for sending a new PPS may have nothing to do with wanting to send different palette initialization data, but as things currently stand, it would be necessary to send the palette initialization data whenever changing the PPS in any way.

Decision: Adopt.

JCTVC-U0130 Crosscheck for palette predictor initialization for Screen Content Coding (JCTVC-U0084) [W. Zhang, L. Xu, Y. Chiu (Intel)] [late]

JCTVC-U0114 Generate the palette predictor initializers with low delay [J. Ye, T.-D. Chuang, S. Liu, Y.-W. Huang, S. Lei (MediaTek)]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 15:30-15:45.)

This contribution proposes a way to generate the palette predictor initializers with low delay. In the current method, multipass operation on the whole current picture is performed for each picture.

In the proposed method, the first picture generates the palette predictor initializers from itself (as is done now), and for the rest of the pictures, the previous picture is used for generating the palette predictor initializers. Experiment results reportedly show similar BD rate performance by using the proposed method and the current scheme in SCM4.0 for AI, RA and LB configurations. Basically no difference in coding efficiency (and in fact perhaps a tiny improvement) was reported.

Decision (N-N SW): Adopt.

JCTVC-U0141 Crosscheck of Generate the palette predictor initializers with low delay (JCTVC-U0114) [C. Gisquet (Canon)] [late]

JCTVC-U0173 Crosscheck of Non-CE1: Enhancement to Palette Coding by Palette with Pixel Copy (PPC) Coding (JCTVC-U0116) [R.-L. Liao, C.-C. Chen, W. H. Peng, H.-M. Hang (NCTU/ITRI)] [late]

JCTVC-U0176 Non-CE1: Joint proposal of JCTVC-U0169 and JCTVC-U0090 Item 3 [F. Zou, V. Seregin, M. Karczewicz, W. Pu, R. Joshi (Qualcomm), J. Ye, J. Kim, S. Liu, P. Lai, W. Zhu, K. Zhang, T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek)] [late]

(Consideration of this topic was chaired by GJS on Tuesday 06-23, 11:00-11:30.)

This document presents a jointly proposed method for Rice parameter derivation of numIndices in palette coding. The results were tested on top of SCM4.0 + JCTVC-U0096 + JCTVC-U0086 Part1. Simulation results reportedly show that the proposed simplification has BD-rate impact of −0.1%, −0.1% for TGM RGB and YUV, respectively, in full frame IBC AI test conditions. And −0.1%, −0.1% for TGM RGB and YUV, respectively, in 4CTU IBC AI test conditions.

The Rice parameter is proposed to be changed from c=2+Idx/6 to c=3+(Idx>>3).

Decision (Cleanup/BF): Adopt (subject to completion of cross-check).

- JCTVC-U0177 Crosscheck for the Joint proposal of JCTVC-U0169 and JCTVC-U0090 Item 3 (JCTVC-U0176) [W. Zhang (Intel)] [late]

The cross-checker verbally confirmed most AI results; others were ongoing.

### CE2 related (intra block copy memory access) (1)

(Consideration of this topic was chaired by JRO on Friday 06-19, 16:15-16:30.)

JCTVC-U0065 CE2-related: Intra block copy searching constraints for reducing worst case bandwidth [T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

In HEVC-SCC, the IntraBC predictor comes from the unfiltered reconstructed samples of the current picture. The unfiltered reconstructed samples need to be accessed, which increases the worst-case memory bandwidth for HEVC-SCC. In this contribution, the worst case bandwidth of HEVC and HEVC-SCC is analyzed. Two IntraBC encoder searching constraints are proposed to reduce the worst case bandwidth.

In Method-1, a current picture is divided into 8×8 blocks. An 8×8 block cannot be referenced by IntraBC blocks if this 8×8 block is coded as an 8×8 bi-predicted block, two 4×8/8×4 uni-predicted blocks, or it belongs to a 16×16 CU that is coded by 4×16+12×16 bi-predicted blocks or 16×4+16×12 bi-predicted blocks, and the 8×8 block is not an IntraBC coded block and the use_integer_mv_flag is 0. In experiment results for method-1, no BD-rate losses are reported under AI configuration, 0.5% to 0.6% BD-rate losses are shown under RA configuration, and 0.7% to 1.0% BD-rate losses are reported under LB configuration.

In method-2, an additional constraint is added that if one MV in this 8×8 block is integer MV, this 8×8 block is allowed to be referenced by IntraBC blocks. In experiment results for method-2, no BD-rate losses are reported under AI configuration, 0.3% to 0.4% BD-rate losses are shown under RA configuration, and 0.2% to 0.6% BD-rate losses are reported under LB configuration.

With the proposed methods, the worst case bandwidth of the HEVC-SCC reportedly will be the same as the worst-case bandwidth of the current HEVC design.

This is a similar idea as in CE2 test 5, but this is in the form of an encoder/bitstream constraint.

The contribution did not contain draft text, but it was asserted that it would not be simple to specify the method as a bitstream constraint, and may not be easy for an encoder to implement it.

No action was taken on this.

### IBC/MC unification aspects (28)

(Consideration of this topic was chaired by JRO on Friday 06-19, 16:30-20:20, continued Saturday 06-20, 9:20-12:00.)

JCTVC-U0056 Unification of the Adaptive Motion Resolution signalling for both IBC and Inter modes [G. Laroche, G. Malard, C. Gisquet, P. Onno (Canon)]

This contribution is related to intra block copy and inter unification. In the current SCM4.0, the Intra block copy method is signalled as inter modes by setting the current frame as a reference frame. But the block vectors have systematically the full-pel resolution rather than what is done for motion vectors which have full-pel or sub-pel resolution according to the value of the use_integer_mv_flag in the slice header. In this contribution, it was proposed to unify the use of the adaptive motion vector resolution for IBC in order to unify the usage of full-pel or sub-pel vector for inter modes for all reference frames. There is no impact on coding efficiency over SCM4.0.

The approach is to signal the MV resolution separately at the slice level for each reference picture in each list. The benefit of this was not obvious, as it does not simplify the text, and introduces a parsing of additional flags at the slice header level.

JCTVC-U0159 Cross-check of JCTVC-U0056 on unification of the Adaptive Motion Resolution signalling for both IBC and Inter modes [K. Rapaka (Qualcomm)] [late]

JCTVC-U0057 On inter modes constraint for slices [G. Laroche, G. Malard, C. Gisquet, P. Onno (Canon)]

This contribution relates to inter prediction constraints. In SCM4.1, an inter block predictor from the current frame cannot come from a different slice than the current one. This constraint was not present in HEVC v1. The losses of coding efficiency of a slice configuration compared to the CTC is larger for screen content than for natural content and this may partly come from this constraint. This contribution proposes to enable or disable this constraint at the sequence or slice level. The BD rate results were reported on average, for SCC sequences and mixed content sequences, as −13.6%, −8.3%, −3.4% for AI, RA and LB configurations, respectively, when using slices limited to a maximum size of 1500 bytes.

The contribution refers to a configuration with fixed-size slices where the penalty is typically large. It is demonstrated that some of this penalty can be reduced (e.g., a 30% vs. 40% bit rate increase for all-intra case).

During the discussion, it was pointed out that allowing IBC across slices might introduce some additional problems with tiles. Furthermore, this restriction was introduced by purpose to enable more stability in intra coded pictures under lossy transmission, and such cases are not considered in the contribution.

It was also pointed out that by using dependent slices, such a functionality could already be implemented.

No action was taken on this.

JCTVC-U0160 Cross-check of JCTVC-U0057 on Inter modes constraint for slices [K. Rapaka (Qualcomm)] [late]

JCTVC-U0076 Unification of motion vector resolution for screen content coding [X. Xu, S. Liu, S. Lei (MediaTek)]

In SCM-4.0, it is intended to unify the intra block copy operation with inter coding. However, a discrepancy is asserted to exist in which the motion vectors pointing to the current reference picture are always using integer resolution while motion vectors pointing to temporal reference pictures can switch at the slice level between integer resolution and quarter-sample resolution. This contribution proposes to unify the resolutions of all motion vectors for the same slice. When the slice level use_integer_mv_flag is equal to 1, all motion vectors in this slice are proposed to use integer resolution; and when this flag is equal to 0, all motion vectors in this slice are proposed to use quarter-sample resolution. As a special case, if the current picture is the only reference picture available for the current slice, the use_integer_mv_flag for this slice is signalled or inferred to be 1. Experiment results reportedly average a 0.0 to 1.0% BD rate increase for RA and LB 4:4:4 lossy configurations by applying the proposed unification method. The BD rate change for AI is reportedly negligible.

The proposal saves approximately 6 lines in the specification text, but would need an additional bitstream constraint in the case of only using integer motion for IBC. Enforcing integer*4 motion vectors causes a BD rate increase (RA 1%) according to the results.

For the AI case, using integer MVs is signalled in the proposal (therefore no loss).

Other methods exist which achieve similar unification without losses (see the notes for U0081).

JCTVC-U0129 Crosscheck for unification of motion vector resolution for screen content coding (JCTVC-U0076) [W. Zhang, L. Xu, Y. Chiu (Intel)] [late]

JCTVC-U0077 Chroma motion vector derivation for intra block copy [X. Xu, S. Liu, S. Lei (MediaTek)]

In SCM-4.0, it is intended to unify intra block copy (IntraBC) with HEVC inter coding. However, the resolution of block vectors (i.e., motion vectors pointing to the current picture) is always full integer, for both luma and chroma components, while the resolution of motion vectors pointing to temporal reference pictures can switch between integer and quarter-sample displacements at the slice level. With this difference, the derivation process of chroma motion vectors for IntraBC blocks is different from that for blocks coded with HEVC inter mode. This contribution proposes several methods to remove this difference and unify the chroma motion vector derivation procedure. In method A, an extra few rows and columns of samples are restricted from being used as reference data when a motion vector points to the current picture to make sure the surrounding samples which may be used for interpolation are all readily available. In method B, the allowed range of block vectors remains the same as SCM-4.0, but extra rows and columns beyond the right and bottom boundaries of the reference area are padded by boundary or "prefixed" sample values. Experiment results reportedly show that:

- The proposed method A.1 reportedly has no change to 4:4:4 coding efficiency, and averages 2.3%, 1.2% and 0.8% BD-rate increase for RGB TGM 1080p & 720p in AI, RA and LB 4:2:0 lossy coding, respectively;

- The proposed method A.2 reportedly has no change to 4:4:4 coding efficiency, and averages2.8%, 1.6% and 0.8% BD-rate saving for RGB TGM 1080p & 720p in AI, RA and LB 4:2:0 lossy coding, respectively;

- The proposed method B.1 reportedly has no change to 4:4:4 coding efficiency, and averages3.2%, 1.9% and 1.0% BD-rate saving for RGB TGM 1080p & 720p in AI, RA and LB 4:2:0 lossy coding, respectively;

- The proposed method B.2 reportedly has no change to 4:4:4 coding efficiency, and averages3.2%, 1.9% and 0.9% BD-rate saving for RGB TGM 1080p & 720p in AI, RA and LB 4:4:4 lossy coding, respectively;

Chroma interpolation is used for cases of odd-valued integer motion vectors. The approach A uses padding as used at the picture boundary in version 1, and a block vector constraint for a 2-row/column "safety margin" at the CTU boundary (to avoid that samples from the current CTU are involved in IBC referencing). A1 is using the constraint for all even and odd cases (which gives loss), whereas A2 constrains only the odd-valued block vector cases which reportedly provides interesting gains. The cases B also implement padding at CTU boundaries (which appears undesirable because it has more complexity and adds IBC-specific operations which are not in the spirit of unification).

Most of the gains likely are more influenced from the CTU boundaries rather than the picture boundaries.

See further notes in the discussion of U0103.

JCTVC-U0080 Intra block copy chroma interpolation for Non-4:4:4 [K. Rapaka (Qualcomm)]

During the 20th JCTVC JCT-VC meeting held in Geneva, an alignment of Intra block copy (IBC) with inter signalling, as proposed by JCTVC-T0227, was adopted. In the unification framework, luma and chroma block vectors are restricted to have integer precision, and thus the prediction samples are generated without interpolation. However, for non-4:4:4 chroma formats, chroma block vectors may point to fractional sample positions. For these cases, in the current working draft, chroma block vectors are rounded to integer precision. In this contribution it is proposed to enable fractional chroma interpolation for IBC blocks. In the process, the IBC search region is reduced by interpolation samples when interpolation is applied. Compared to the SCM 4.0 anchor on 4:2:0 sequences, the proposed method reportedly provides average {Y, Cb, Cr} BD-rate savings for AI of {2.1%, 2.7%, 27%}, respectively, for the category YUV text & graphics with motion, 1080p & 720p. Additional results with encoder optimization to compute SAD based on interpolated chroma reference samples are included. With this modification, compared to the SCM 4.0 anchor on 4:2:0 sequences, the proposed method reportedly provides average {Y, Cb, Cr} BD-rate savings for AI of {2.5%, 3.1%, 3.0%}, respectively, for the category YUV text & graphics with motion, 1080p & 720p. Proposed specification text changes were provided in the contribution.

This proposal was roughly the same as JCTVC-U0077 A2, but instead of padding at the picture boundary, it also uses the 2-row/column margin, and therefore has slightly less gain.

See further notes in the discussion of U0103.

JCTVC-U0139 Cross check report of JCTVC-U0080: Intra block copy chroma interpolation for Non-4:4:4 [X. Xu, S. Liu (MediaTek)] [late]

JCTVC-U0103 On chroma derivation of intra block copy for non-4:4:4 video [X. Xiu, Y. Ye, Y. He (InterDigital)]

In this contribution, two methods are proposed to modify the chroma block vector derivation process in the unification of intra block copy in order to improve the coding performance of intra block copy for non-4:4:4 video. In the first method, the current chroma block vector clipping process is modified. In the second method, it is proposed to enable fractional chroma interpolation for intra block copy.

Compared to the SCM-4.0 anchor on 4:2:0 sequences, the first method reportedly provides average {Y, Cb, Cr} BD-rate savings for AI, RA and LB of {0.3%, 0.6%, 0.7%}, {0.1%, 0.3%, 0.5%} and {0.0%, 0.2%, 0.6%}, respectively, for the category "YUV text & graphics with motion, 1080p & 720p". For lossless coding, the corresponding reported BD-rate savings are 0.1%, 0.1% and 0.1% for AI, RA and LB, respectively.

Compared to the SCM-4.0 anchor on 4:2:0 sequences, the second method reportedly provides average {Y, Cb, Cr} BD-rate savings for AI, RA and LB of {2.5%, 3.1%, 3.0%}, {1.5%, 1.9%, 2.0%} and {0.8%, 1.4%, 1.6%}, respectively for the category "YUV text & graphics with motion, 1080p & 720p". For lossless coding, the corresponding reported BD-rate savings are 0.6%, 0.9% and 1.0% for AI, RA and LB, respectively.

The first method uses a different rounding for fractional MVs –rounding them towards zero.

The second method is identical with U0080 (although the proposed spec text is slightly different).

One expert pointed out that instead of the first method, another option could be adding an offset and applying conventional rounding (instead of needing absolute-value-based rounding).

General discussion and conclusions about U0077 A2, U0080, and U0103:

In terms of the impact on memory access, it cannot be argued that 4:4:4 is the worst case, because with same amount of reference picture memory, 4:2:0 could use double the picture size. However, the memory access should not be worse than in 8x8 bi-pred motion comp with quarter pel MVs, as the luminance vectors are still integers.

It was requested to consider whether SCC would have a 4:2:0 profile, or whether it would support 4:2:0 in a generic profile, and how important gains in 4:2:0 are. After discussion as noted elsewhere, a 4:2:0 profile is anticipated.

Following the discussion on one aspect of U0118 (releasing the BV value constraint, and using normal padding at picture boundaries), this unification is not necessarily desirable, and only gives small additional gain according to U0077 A2. The solution with the restriction of BVs by a 2 luma sample margin in each direction (regardless of whether it is a picture boundary or CTU boundary) is asserted to be the most beneficial solution. Side activity (of proponents of U0077, U0080, U0103) was requested to provide a unified text specification of this solution).

It was agreed to study this in a BoG side activity (K. Rapaka) on CE2 for worst case memory assessment, to confirm that it does not increase the worst case.

It was also agreed to discuss in the JCTVC plenary whether we would allow a higher processing complexity of IBC (as per chroma interpolation) in the case of 4:2:0 compared to 4:4:4. Note: The complexity would probably still be lower than in inter MC, but this might have consequences on worst-case complexity of intra-only profiles.

This issue was further discussed in JCT-VC plenary Sun 11:30-13:00. The results of the BoG side activity were not available yet. It was however suggested that it may not be necessary to strictly limit the worst case memory complexity to the level of that in version 1.

Further consideration of this topic was performed on Mon. 06-22 18:00 (chaired by JRO) after review of the BoG report U0175. The first report of the BoG did not analyze the combination of CE2 5.2/5.3 with chroma interpolation to give the proof that for the 4:2:0 case the worst case memory bandwidth would not be higher than for version 1 HEVC. Further BoG study seemed necessary, but if this is demonstrated, it was anticipated that the text as suggested in U0175v1 3.2.3 should be adopted.

Further discussion was suggested after further work of BoG.

(Further consideration of this topic was conducted with GJS & JRO Thursday 06-25 09:00-10:00.)

The new contribution U0185 was noted and discussed (see the notes for U0185).

The BoG report U0175 confirmed that the worst case memory access is not increased. Additional discussion was conducted Thu 9:00. It is unquestionable that additional implementation cost occurs when IBC is done in separate logic (which seems to be necessary at least when it is performed within the same CTU, see notes for U0185). However, considering the compression benefit, the additional implementation cost is justified.

Decision: Adopt approach of U0077 A2 / U0080 / U0103 (combined text in U0175).

JCTVC-U0158 Cross-check of JCTVC-U0103 on chroma derivation of intra block copy for non-4:4:4 video [K. Rapaka (Qualcomm)] [late]

JCTVC-U0185 Comments on intra block copy with interpolation filters [M. Zhou, T. Hellman (Broadcom)] [late]

(Consideration of this topic was chaired by GJS & JRO on Thursday 06-25 09:00-10:00.)

This document discusses how IBC (intra block copy) may be implemented on the top of an existing HEVC v1 decoder architecture, and why adding IBC interpolation filters is asserted to be costly as compared to the ones in MC. It was asserted that adding interpolation filters to IBC imposes a significant burden on the HEVC SCC decoder design, and the contributor recommended that the architecture implications of IBC fractional-valued vector support be carefully investigated in addition to its memory bandwidth impact.

In the contribution, it was expressed that a major difference between inter prediction and IBC is the possible dependency. When IBC refers to a neighbouring block, the inverse transform processing needs to be done before that data can be used. Therefore, a different building block may be required for IBC than for motion compensation.

Other experts pointed out that with a different implementation structure the problem may not exist. It is without question that the additional chroma interpolation requires more operations and memory access (from cache in the case of accessing within same CTU). However, it does not seem too difficult to implement, and gives a good compression benefit.

The contribution asserts that IBC is distinct from MC, and so it says MC interp is not naturally supported in that part of the design. The contributor especially expressed concern about the 8x4 / 4x8 block size cases.

From the discussion, it seemed that the practical difficulty of supporting chroma interpolation is implementation-dependent. However, it does have 2-5% improvement in coding efficiency (as measured by luma PSNR).

See also the notes on the related contributions U0077, U0080, U0103.

JCTVC-U0081 On unification of adaptive motion vector resolution [K. Rapaka (Qualcomm)]

During the 20th JCTVC JCT-VC meeting held in Geneva, the alignment of intra block copy (IBC) with inter signalling, as proposed by JCTVC-T0227, was adopted. Also, in the current draft text, adaptive motion vector resolution can be enabled using the syntax element use_integer_mv_flag. This contribution discusses several issues related to motion vector (MV) derivation and storage when IBC and use_integer_mv_flag are enabled. Several changes are proposed for a unification design to resolve the identified issues. The performance impact of proposed solutions reportedly seems non-significant – ranging from 0.0% to 0.1%.

Three methods are proposed:

- Method 1 stores quarter-sample-precisoin vectors in all cases. It unifies integer-valued motion comp and IBC, but moves the shift operation of integer motion compensation to the derivation stage (which was not originally intended when integer MC was adopted, since it was desired to keep the MV derivation stage untouched). Method 1also removes the clipping introduced at the last meeting, and removes the inconsistency in deblocking and TMVP operation.

- Method 2 uses storage of quarter-sample vectors for both BV and MV in the case of quarter-sample MC, and storage of integer vectors in the case of integer MC.

- Method 3 unifies integer-valued motion compensation and IBC in the style of the current integer motion compensation. It still requires the clipping operation and does not solve the deblocking problem.

Method 1 is identically proposed in U0107, and as method 3 in U0111.

Decision: Adopt Method 1.

JCTVC-U0140 Cross check report of JCTVC-U0081: On unification of adapative motion vector resolution [X. Xu, S. Liu (MediaTek)] [late]

JCTVC-U0107 On stored decoded motion vector resolution [X. Xu, S. Liu, S. Lei (MediaTek)]

In SCM-4.0, the decoded motion vectors that point to a temporal reference picture are stored in either quarter-pel resolution or integer resolution, depending on the slice level flag use_integer_mv_flag. These stored motion vectors are used later as predictors for motion vector prediction and for boundary strength (BS) decision in deblocking filter stage. Two issues are discussed in this contribution: 1) according to the contributor, the HEVC inter deblocking filter process should use motion vectors as input with quarter-sample resolution, and 2) when temporal motion vector prediction is used, and the vector resolution of the current picture is different from the predictor in the reference picture, the prediction is not formed in a consistent way. This contribution proposes to always store the decoded vectors using quarter-sample resolution. With this modification, the motion vectors that are inputs to deblocking filter would consistently have the same resolution. The contributor also suggested that this should improve motion vector prediction. Experiment results report average BD rate savings of up to 0.2% from the proposed method across all testing configurations and classes. The overall average impact was not reported in the contribution's summary abstract, but from a quick glance at the contribution, it appears to be about 0.0%.

See notes for U0081.

JCTVC-U0111 Non-CE: MV resolution unification for MV derivation in Intra-Block Copy search [J.-S. Tu, C.-C. Lin, C.-L. Lin, Y.-J. Chang (ITRI)]

This contribution proposes a modified MV resolution handling scheme, in which, the MV resolution of IBC would be set in integer-sample precision without considering the “use_integer_mv_flag”. The proposed scheme reportedly removes extra operations and is asserted to further unify IBC and Inter mode. Experiment results reportedly show that the BD-rate changes of the proposed method range from −0.1% to 0.1% under lossy coding conditions for the 4:4:4-format.

See notes for U0081.

JCTVC-U0161 Cross-check of JCTVC-U0111 on Non-CE: MV resolution unification for MV derivation in Intra-Block Copy search [K. Rapaka (Qualcomm)] [late]

JCTVC-U0083 On indication of the use of current reference picture [C. Liu, S. Liu, X. Xu, S. Lei (MediaTek)]

In the current draft of High Efficiency Video Coding (HEVC) extension on Screen Content Coding (SCC), the use of current picture as a reference picture is signalled in the sequence parameter set (SPS). This contribution presents methods of signalling the use of the current reference picture in picture parameter sets (PPS) or slice headers.

During the discussion, the following remarks were made:

- The flag was put into the SPS on purpose, allowing the decoder to know at a very high level whether IBC will be used

- No evidence was brought that enabling/disabling IBC at the picture level gives benefit. If the characteristics of the content radically chang (e.g., due to a change between natural and screen content), an IDR picture will be used anyway, such that sending a new SPS is not an issue.

It was remarked that it is already possible with the current spec to exclude the current picture from the list of active reference pictures (at the slice level).

U0079, U0100, and U0181 are also related. See the notes for those contributions.

JCTVC-U0100 On operation of DPB in Screen Content Coding [C. Liu, S. Liu, X. Xu, S. Lei (MediaTek)]

This contribution presents a few modifications of operation of the decoded picture buffer (DPB) for High Efficiency Video Coding (HEVC) Screen Content Coding (SCC) (relative to draft 3) in regard to the current picture being used as a reference picture.

IBC may require approximately one more picture of memory storage, because the current picture has to be retained once in a non-deblocked version (for IBC) and once in deblocked form (for subsequent predictions). The proposal is to put the non-deblocked picture into the decoded picture buffer, and replace it by the deblocked version (which is stored in an intermediate memory) after the current picture has been fully decoded.

It was generally agreed that the approach is pointing into the right direction, in terms of handling the current non-deblocked picture as a reference picture, as well as considering the additional memory necessary to store it as part of the DPB, as well as removing it / replacing it by the deblocked version after the current picture has been fully processed. Basically this means that one less temporal reference is available when IBC is used, but the amount of memory is not increased.

However, the provided specification text did not seem to be complete, e.g., some changes also appear necessary in section 8.1.3. Offline consideration was performed with K. Rapaka, Y.-K. Wang, and this was further discussed Mon 06-22 at 18:00 (chaired by JRO).

It turned out that the intention of the current specification of SCC is to store the unfiltered picture in the DPB, but not to store the filtered picture in the DPB. In HEVC version 1, the filtered picture is stored in the DPB (not to be used as a reference for decoding the current picture, but so it can be used as a reference for decoding subsequent pictures). As the most consistent solution it was agreed, that in the case with curr_pic_as_ref_pic_flag equal to 1, both the filtered and unfiltered pictures are to be in the DPB, but when curr_pic_as_ref_pic_flag is equal to 0, only the filtered picture will be in the DPB.

Text was presented by Y.-K. Wang.

Another clause was added that removes the unfiltered current picture from DPB once it has been fully decoded. It was discussed whether another option would be to use conventional RPS-based DPB management for that purpose. For example, this might have the advantage that an encoder could signal that the current picture is unused for reference earlier than at the end of the decoding of the picture (when no further blocks would use IBC).

Text was provided in a new input document JCTVC-U0181. See additional notes for that document.

JCTVC-U0181 On storage of filtered and unfiltered current decoded pictures [C. Liu, S. Liu, X. Xu, S. Lei (MediaTek), Y.-K. Wang, K. Rapaka, V. Seregin (Qualcomm)] [late]

(Consideration of this topic was chaired by GJS on Thursday 06-25 at 16:45.)

This contribution is a follow-up of JCTVC-U0100 after its review and further prior discussion at this meeting, for further consideration of the topic of the storage of filtered and unfiltered versions of the current decoded picture.

As proposed, the possible depth of hierarchical referencing would need to be smaller when IBC is used and in-loop filtering is enabled than otherwise.

As presented, the draft did not consider the possibility of disabling the ILFs, but it was said that this would not be difficult. It was agreed that ILF on/off should be treated differently.

It was said that it would be editorially difficult to consider the unfiltered picture to be outside the DPB.

It was suggested that the memory capacity requirement be increased in the SCC profiles to prevent these profiles from lacking the same degree of hierarchical referencing ability as in prior profiles when IBC is used (i.e., suggesting to prescribe maxDpbPicBuf equal to 7 for profiles supporting IBC).

Decision: Adopt (with maxDpbPicBuf equal to 7 for profiles supporting IBC, and with ILF on/off treated differently).

Regarding the syntax for determining ILF on/off and IBC on/off, currently we have SPS-level flags for IBC and SAO and PPS-level flags for DBF luma and chroma. It was suggested to change the IBC signalling so it has an enabling flag at the SPS level, and a PPS level flag to turn it off on a picture basis, and the DPB capacity calculation would use the picture-level flag (along with the ILF and SAO flags). Decision: Agreed.

(This agreement changed prior intent not to act on this aspect in discussion of U0079 and U0083.)

JCTVC-U0102 On constrained intra prediction for the unification framework of intra block copy [X. Xiu, Y. Ye, Y. He (InterDigital)]

A unification framework of intra block copy (IBC) and inter signalling had previously been adopted. However, it is asserted that, when constrained intra prediction (CIP) is enabled, the specification text in HEVC screen content coding draft 3 is ambiguous, and that the reference software SCM-4.0 was broken for this. In this contribution, it was proposed to disallow the samples of inter-predicted CUs (either predicted from temporal reference pictures or from the current picture itself), to be used as references for intra prediction when CIP is enabled.

The issue was previously presented in JCTVC-O0155 in the context of IBC consideration for RExt.

Now, the situation is different in the unified framework, where, in principle, bi-prediction for inter/IBC is also possible, and more than two CTBs of current-picture reference memory are used, and interpretation as inter/inter mode is different.

It was agreed that an issue exists, but that more consideration was necessary to decide whether any of the suggested solutions is appropriate. Side activity (coordinated by X. Xiu) was requested to further study the proposal and discuss whether an immediate solution can be found. (See the notes of further discussion for the BoG report U0178.)

Eventually, further study may be necessary also in the context of error resilience, because there could be a trade-off between compression efficiency and error propagation.

Further study was suggested (as recommended by the BoG).

JCTVC-U0123 Crosscheck report of JCTVC-U0102 [K. Misra, S.-H. Kim (Sharp)] [late]

JCTVC-U0178 BoG report on constrained intra prediction for IBC unification [X. Xiu]

This BoG report was presented and discussed Monday 06-22 at 18:00 (chaired by JRO).

A BoG discussion was held on Saturday June 20, 2015 from 17:00 to 17:30 to study in detail the suggested approaches related to constrained intra prediction (CIP) under the current IBC/Inter unification framework. The mandates include:

- Study in detail the CIP handling in the current HEVC SCC draft 3 and the one proposed in JCTVC-U0102.

- Evaluate different methods in the context of compression efficiency and error resilience.

When CIP is enabled, allowing intra and IBC prediction can use intra-coded and IBC-coded samples as reference data. Temporal motion vector prediction (TMVP) is disabled for IBC PUs to suppress error propagation through BV prediction.

It was found that both the current HEVC SCC draft 3 and SCM4.0 software are problematic when CIP is enabled. One ticket 1401 had already been filed to report the problem.

About JCTVC-U0102:

When CIP is enabled, it is proposed in JCTVC-U0102 to only allow the samples of intra-coded CUs to be used as reference samples for intra prediction. TMVP is enabled for IBC PUs.

This method is found to be similar to the second approach as proposed in JCTVC-O0155.

BoG recommendations:

- It was agreed that the bugs in both the current spec and reference software should be fixed (the corresponding software patch was already provided with the ticket 1401 and was evaluated by software coordinators and other experts.)

- It was suggested that the proposed method in JCTVC-U0102 should be tested against SCM4.0 CIP anchor based on the following testing cases for the next meeting circle:

- Common test conditions (CTC) with the CIP functionality being enabled.

- Two gradual intra refreshing (IR) cases will be tested together with LB configuration:

- IR case 1 (IBC RExt): pick one CTU column in one picture and force to code them using intra mode. Additionally, the intra CTU column moves from left to right along the picture decoding/display order.

- IR case 2: slice based intra refresh will be tested. Specifically, pictures are divided evenly into N slices and each time one slice is refreshed using intra mode on a cyclic basis.

From the discussion:

- Currently, only a ticket for a software bug has been issued.

- A ticket should be filed (responsible: X. Xiu) to point out a specific problem in the draft text. Further consideration of that problem should be up to editors.

JCTVC-U0104 On unification framework of intra block copy [X. Xiu, Y. Ye, Y. He (InterDigital)]

A unification framework of intra block copy (IBC) and inter signalling had been previously adopted. This contribution proposes several design modifications to that current IBC unification framework. Firstly, it was proposed to add the current reference picture that contains already-coded samples of the current picture prior to in-loop filtering to both initial reference picture lists L0 and L1 when IBC is enabled. Secondly, it was proposed to signal the IBC referenceable range in the SPS. Thirdly, it was proposed to disable weighted prediction signalling for the current picture.

For aspect 1: It was generally agreed that enabling the use of the current picture in both L0 and L1 is desirable; it had been disabled because there was a bug in the software implementation of the last meeting. The contribution is bringing a software patch which is claimed to fix that bug (to be confirmed by the software coordinator).

Decision: Adopt aspect 1 (allow the current picture in L0/L1). U0079 also proposes this aspect.

For aspect 2 (signalling of the referenceable range): This does not have an impact on the decoding process and therefore should not be placed in the SPS. The benefit of having such an indication was not seen to be apparent to other experts. In the VUI, a restriction of the inter MV range already exists, such that it would seem to contradict the unification idea to express it differently for IBC. See also the notes for U0142 for this aspect.

For aspect 3 (weighted prediction): The suggested approach is to set the weight parameters to the defaults (offset 0, weight 1) when the current picture is the reference picture (as a modification in the derivation of the weight table at the slice level). Currently, we don't have any evidence that weighted prediction gives a benefit in the case of IBC. This aspect could save some complexity in implementations that use dedicated building blocks for the current picture, but it does still allow re-using existing building blocks of MC.

Decision: Adopt aspect 3 (disabling WP with IBC).

It was also mentioned during the discussion that if evidence about benefit is brought in the future, this decision about aspect 3 could be reverted.

JCTVC-U0113 On reference picture list construction for intra block copy [X. Xu, S. Liu, S. Lei (MediaTek)]

In the current HEVC SCC draft text, the current picture is put in the last position of the reference picture list 0 during the initialization of this list (RefPicListTemp0) and before the possible reference picture list modification (RPLM) operations. As a consequence, when the number of reference pictures of list 0 for the current slice (num_ref_idx_l0_active_minus1 + 1) is smaller than the number of total reference pictures (NumRpsCurrTempList0) in the list, the current picture will not be included in the list (RefPicList0) as an active reference picture. This is different from the reportedly intended behaviour that the current picture is always used as a reference picture for intra block copy when the SPS flag “curr_pic_as_ref_enabled_flag” is enabled. In this contribution, it was proposed to modify the reference picture list construction procedure such that the current picture is by default put in the last position of RefPicList0, which is indicated by num_ref_idx_l0_active_minus1. Simulation results reportedly show that there is no change in the coding performance as compared to SCM-4.0 under CTC conditions.

Two methods were proposed which both put the current picture in the last position, where the first method allows modifying the position via RPLM afterwards and the second method does not allow this.

In method 1, if the number of active pictures is equal to 1 and the picture is not an I slice, the current picture is not included unless using RPLM.

No difference in compression was shown, since RPLM mechanisms are not used in the current implementation.

It was pointed out that the software bug that is mentioned in the proposal was fixed in SCM4.0.

With the current draft design, cases may happen where it is necessary to use RPLM in order to include the current picture into the list of active reference pictures. The proposal method 1 would revert this situation such that it is always included in the list of active pictures, but RPLM can be used to move the current picture out of the active range.

There was no evidence that the current spec (or the software) has a real problem, and therefore no urgent need to change it was apparent. See further notes regarding U0118.

JCTVC-U0120 Cross-check of JCTVC-U0113, On reference picture list construction for intra block copy [R. Cohen (MERL)]

JCTVC-U0142 Inter/Intra Block Copy Unification: Comments and Observations [A. M. Tourapis, Y. Su, D. Singer (Apple)] [late]

(Consideration of this topic was chaired by GJS & JRO on Tuesday 06-23, 12:20-12:30.)

At the 20th JCT-VC meeting it was decided to unify the inter prediction and intra block copy processes. This was done, given the strong similarities of the two processes, in an attempt to simplify the specification and the implementation of the intra block copy method, and to allow the use of additional prediction mechanisms that are already supported for inter prediction with the intra block copy framework. This reportedly included, for example, bi-prediction and weighted prediction. This contribution makes some additional observations with regards to this unification framework and provides some suggestions on how to unify the two processes. Intra Block Copy displacement vector limits are also discussed.

The general recommendation of the contribution was to avoid establishing constraints that may appear to help implementation, but which might have unforeseen negative impact on the capability of the standard.

No test results were provided in the contribution to try to quantify potential benefits of releasing current or proposed restrictions. Further study would be needed to determine the impact of some of the suggestions.

Several experts raised concerns about the possible complexity impact of the suggestions, in particular in implementations that would use different logic for IBC and inter MC.

Further study of these issues was encouraged.

The contribution suggested adding VUI parameters log2_max_ibc_dv_length_vertical and log2_max_ibc_dv_length_horizontal to describe the range of values of the IBC displacement vectors, and to constrain these as follows:

log2_max_ibc_dv_length_vertical <= log2_max_mv_length_vertical

log2_max_ibc_dv_length_horizontal <= log2_max_mv_length_horizontal

See also the notes for U0104 for this aspect.

Further study was encouraged – no action was taken on this.

JCTVC-U0172 Performance gain of IBC FF and loss of IBC FF vs 4CTU [X. Chen, T. Lin (Tongji)] [late]

(Consideration of this topic was chaired by GJS & JRO on Tuesday 06-23, 12:00-12:15.)

This document reports performance gain of IBC referencing the full frame (FF) and the loss of IBC FF versus using only 4 CTUs as the referencing region. The contribution asserts that full frame search would be difficult to implement at the encoder. The BD rate increases for TGM classes when restricting to 4 CTU at encoder were reported as approximately 22% for AI, 12% for RA, and 6% for LDB.

Experiment results were reported as follows.

| Reference: | Anchor (SCM-4.0) w/o IBC | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tested: | Full frame IBC search | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
|  | RGB, TGM, 1080p & 720p | −32.3% | −31.6% | −31.6% | −20.9% | −20.7% | −20.5% | −12.8% | −12.7% | −12.6% |
|  | RGB, MC, 1440p & 1080p | −28.7% | −27.5% | −27.4% | −18.3% | −17.3% | −17.2% | −8.4% | −7.1% | −6.9% |
|  | RGB, Ani, 720p | −1.1% | −1.1% | −1.0% | −0.5% | −0.5% | −0.4% | 0.1% | −0.2% | −0.1% |
|  | RGB, CC, 1080p | −0.1% | −0.1% | −0.1% | −0.2% | −0.1% | −0.2% | 0.1% | 0.1% | 0.1% |
|  | YUV, TGM, 1080p & 720p | −34.0% | −33.0% | −32.8% | −21.7% | −20.9% | −20.4% | −12.3% | −11.6% | −11.7% |
|  | YUV, MC, 1440p & 1080p | −30.2% | −26.8% | −27.3% | −20.2% | −16.6% | −16.9% | −9.4% | −5.2% | −5.1% |
|  | YUV, Ani, 720p | −1.6% | −1.8% | −1.7% | −0.6% | −1.1% | −1.0% | 0.0% | 0.0% | −0.6% |
|  | YUV, CC, 1080p | −0.1% | −0.2% | −0.2% | −0.2% | −0.2% | −0.1% | 0.1% | 0.0% | 0.0% |

| Reference: | Full frame IBC search | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tested: | Constrained IBC Search 4 CTUs | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
|  | RGB, TGM, 1080p & 720p | 22.1% | 22.0% | 22.0% | 12.6% | 12.6% | 12.5% | 6.6% | 6.7% | 6.6% |
|  | RGB, MC, 1440p & 1080p | 15.6% | 15.1% | 15.3% | 9.0% | 8.8% | 8.8% | 4.5% | 4.2% | 4.2% |
|  | RGB, Ani, 720p | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
|  | RGB, CC, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | YUV, TGM, 1080p & 720p | 23.3% | 23.1% | 23.2% | 12.9% | 12.9% | 12.6% | 6.1% | 6.0% | 6.3% |
|  | YUV, MC, 1440p & 1080p | 15.4% | 14.5% | 14.8% | 9.6% | 8.7% | 9.0% | 4.8% | 4.5% | 4.3% |
|  | YUV, Ani, 720p | 0.2% | 0.2% | 0.2% | 0.0% | −0.1% | 0.1% | 0.0% | 0.3% | 0.1% |
|  | YUV, CC, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | −0.1% | −0.1% | 0.0% | 0.0% |

The contributor said it is difficult for some encoders to search large regions, and that this will affect the coding efficiency experienced from such encoders. This may indeed be the case. No action was taken on this topic.

### Other IBC related aspects (5)

(Consideration of this topic was chaired by JRO on Saturday 06-20, 12:00-13:30.)

JCTVC-U0054 Non-CE2: On interpretation of the IBC vectors [C. Gisquet, G. Laroche, P. Onno (Canon)]

At the 20th meeting held in Geneva (CH) in February 2015, an alignment of Intra Block Copy on Inter signalling was adopted (see JCTVC-T0227). Various normative changes were thus introduced; however, it was decided that the motion prediction mechanisms of HEVC v1 would be kept unmodified, although there were some proposals reporting gains by changing these mechanisms. It was asserted that some aspects of JCTVC-T0227 can be changed to obtain some of those gains. It was reported that the proposed changes provide a BDR gain of 0.4% to 0.7% over SCM4.0 for Screen Content classes in the lossy AI scenario, and 0.2% to 0.7% for the lossy RA scenario.

The normative changes suggested in the contribution are partially touching the unification of integer MV and BV coding/compensation as per adoption of U0081. Furthermore, some constraints are released on the usage of the zero merge candidate (but still not allowing references outside of the picture boundary). This requires additional clipping at the decoder.

Some of the gain is likely due to further encoder optimization.

One problem with the proposal could be that the encoder may only be able to check the validity of a BV after performing the compensation.

U0118 has related aspect 4.

JCTVC-U0135 Crosscheck report of JCTVC-U0054 [J. Ye, S. Liu (MediaTek)] [late]

JCTVC-U0079 HLS: On Intra block copy signalling control [K. Rapaka, V. Seregin, R. Joshi, M. Karczewicz, Y.-K. Wang (Qualcomm), A. M. Tourapis, Y. Su, D. Singer (Apple)]

During the 20th JCT-VC meeting held in Geneva, an alignment of intra block copy (IBC) with inter signalling was adopted (see JCTVC-T0227). In the current draft text, I-slices are disallowed when IBC is enabled (i.e., when curr_pic_as_ref_enabled_flag is equal to 1) and support of the IBC mode in B-slices seems to be compromised, and it was suggested that this may potentially be problematic, since the current picture is only used as a reference in List 0 rather than being supported in both lists. This contribution proposes to allow I-slices when the IBC mode is enabled, to enable the use of IBC in both lists in B slices, and to provide some changes to teh control of IBC at the picture and/or slice level.

Aspect 1: Enable I slices when IBC is enabled at SPS. (This does not mean that IBC is used in I slices.)

Decision: Adopt aspect 1.

Aspect 2: PPS flag to enable/disable IBC

Aspect 3: slice flag to enable/disable IBC

For aspects 2 and 3, see the notes relating to U0083, U0100, and U0181.

The contribution also recommended enabling IBC to be used in both L0 and L1 for B slices, which had already been adopted at the meeting in response to U0104; see the notes about that contribution.

JCTVC-U0118 On intra block copy bitstream constraints [V. Seregin, K. Rapaka, R. Joshi, M. Karczewicz, Y.-K. Wang (Qualcomm)]

At the previous meeting, the intra block copy (IBC) design was changed to be aligned with inter mode, with several bitstream constraints added to control the intended intra block behaviour. It was asserted, however, the some of the constraints are not well aligned with inter mode. In this contribution, IBC bitstream constraints were proposed to be converted into signalling and decoder-side restrictions for robustness. It was proposed to modify the collocated_ref_idx signalling such that it cannot refer to the current picture, to allow the IBC motion vector to point outside of the picture as in inter mode, to infer the current picture as a reference when IBC mode is enabled and the number of active pictures in a slice is equal to 1, and to convert a not-valid motion vector into a valid reference.

Aspect 3 (releasing the constraint of the BV pointing within the picture) was discussed in the context of the 4:2:0 chroma interpolation proposals (U0077, etc.). This proposal is going further in also allowing outside-the-picture vectors for luma.

Generally, the benefit of unification was not seen to be too obvious here, because the memory for the current picture is handled differently from the other reference pictures, and the bitstream constraint does not seem to be a critical issue. No coding benefits were shown with the contribution, though it was verbally claimed that some benefits could eventually be achieved.

Further study of this was requested.

Other aspects of this contribution were reviewed later on Saturday 06-20 (chaired by JRO).

Aspect 4 (search range constraints), is a similar approach in spirit as in U0054. The idea is releasing the bitstream constraint and performing clipping of the vector at the decoder, such that it does not point to an area outside the current slice, follows WPP boundaries, etc. If no valid prediction area is available, a constant value is proposed to be used for prediction.

It was claimed that this would simplify encoder implementation, since encoders would not need to check bitstream constraints. However, this modifies the motion vector derivation process, and therefore deviates from the unification. No action was thus taken on aspect 4.

Aspect 1: Collocated picture constraint (current picture shall not be used as collocated picture)

The proposal is to add an additional check at the decoder (not sending the collocated picture reference index when it is the current picture) instead of formulating this as a bitstream constraint. It was claimed that this simplifies encoder implementation. However, this is not a complicated check that would be burdensome at the encoder. No action was thus taken on this aspect.

Aspect 2: This provides a mechanism such that the current picture would be in the reference picture list by default (without using RPLM). This is related to JCTVC-U0113.

It was agreed that there is a certain benefit to having a default where the current picture is in the reference picture list when IBC is enabled, and thus no need to use RPLM in certain cases.

Further consideration of this was requested in a side activity on DPB issues (Y.-K. Wang, et al.) to come up with a proposal for a modification. The solution should still allow the encoder to remove the current picture from the list of active reference pictures (as per U0113 method 1 and U0118 aspect 2).

This was further discussed Mon 06-22 at 18:00 (chaired by JRO). A new contribution U0180 was presented.

JCTVC-U0180 Combination of JCTVC-U0118 method 2.2 and JCTVC-U0113 method 1 [K. Rapaka, V. Seregin, R. Joshi, M. Karczewicz, Y.-K. Wang (Qualcomm), X. Xu, S. Liu, S. Lei (MediaTek)] [late]

This contribution presents a combination of two methods proposed in JCTVC-U0118 method 2.2 and JCTVC-U0113 method 1. The proposed method infers the current picture to be included into the reference picture list when RPLM is not present.

The contribution suggests to put the current picture into the last position of the active list for L0 as default. If it is desirable to remove it from the active list, RPLM can be used. It was discussed whether this default should also be applied to L1, but it was concluded that this is not desirable.

The initial upload of the specification text was a 0 byte file, so a fixed upload needed to be provided.

More study of text is requested by several experts before a decision could be made. This should be further clarified by offline discussion between proponents and those who have concerns.

The issue was briefly further discussed Thu morning in JCT-VC plenary (chaired by GJS & JRO). Concerns were raised because offline discussion and clarification had not happened yet.

(Further consideration Thursday 17:00-17:10 GJS)

A concern had been identified and a modified proposal was provided after offline discussion. The difference in the modified proposal was just editorial (avoiding to perform operations that will be discarded).

Decision: Adopt. (Editorial aspects may be refined by the editors.)

### SCC tool complexity (AHG9) (0)

No contributions specific to this topic were noted, although various contributions included consideration of complexity issues.

### SCC Other (4)

JCTVC-U0106 On signalling adaptive colour transform at the TU level [X. Xiu, Y. He, Y. Ye (InterDigital)]

(Consideration of this topic was chaired by GJS on Sunday 06-21, 10:15-10:45.)

In HEVC screen content coding draft 3, the adaptive colour transform is enabled/disabled at the CU level such that all the residuals of the CU are coded in the same colour space. This contribution proposes to improve the performance of adaptive colour transform by moving the adaptive colour transform flag to the TU level.

Compared to the SCM-4.0 anchor using full-frame IBC search, the proposed solution reportedly provides average {G, B, R} BD-rate reductions of {0.2%, 0.5%, 0.4%}, {0.1%, 0.6%, 0.5%} and {0.3%, 0.4%, 0.2%} for AI, RA and LD, respectively, in RGB coding, and average {Y, Cb, Cr} BD-rate reductions of {0.6%, 2.0%, 1.9%}, {0.3%, 1.4%, 1.4%} and {0.1%, 0.3%, 0.3%} for AI, RA and LD, respectively, in YCbCr coding. For lossless coding, the corresponding bit-rate reductions are reportedly 0.4%, 1.1% and 1.0% for AI, RA and LD in RGB coding, and 0.0%, 0.0% and 0.0% for AI, RA and LD in YCbCr coding.

CCP is currently controlled at the TU level, whereas ACT is controlled at the CU level (which seems potentially inconsistent).

Some encoder speed-up techniques were also included in the contribution.

It was remarked that some of the gain for YUV intra cases may be associated with performing a joint optimization of intra prediction modes for three colour components together rather than performing them separately, which is something that could be done without the normative change.

Approximately a 5% run-time increase was reported for all-intra coding. The cross-checker reported a larger run-time increase, but the cross-checker said that the run-times of the cross-check were likely not to reliable due to heterogeneous computing cluster issues.

The motivation for the proposal is partly harmonization with the existing CCP operation, not just coding efficiency improvement.

Decision (cleanup): Adopt (editorially, it was suggested to consider naming the flag to tu_residual_act_flag).

JCTVC-U0165 Cross-check of on signalling adaptive colour transform at TU level (JCTVC-U0106) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-U0119 On slice segment freeze signalling for screen content coding [T. Laude (Leibniz Universität Hannover)]

(Consideration of this topic was chaired by GJS on Sunday 06-21, 15:40-14:10.)

This document proposes a segment freeze process using high-level syntax elements. It was stated that the principle of the proposed freeze process consists of the freezing of a slice segment in the current picture for subsequently coded pictures. It was proposed that no syntax be coded in these subsequent pictures to reconstruct the samples of the part of the picture corresponding to the frozen slice segment. Instead, it was proposed that the frozen slice segment be copied to the corresponding region of these subsequent pictures. Additionally, a termination method for the freeze process was proposed to end the freezing of a slice segment.

For "frozen" slices, there would be a picture containing a coding of the slice area and a flag indicating that the area will then be "frozen". In subsequent pictures, missing areas will be filled in by the decoder.

It was asked what picture would be used to fill in the missing regions – e.g., a particular picture in the DPB or just the previous picture in decoding order, and whether extra memory would be required to retain a copy of this data.

Some somewhat similar proposals had previously been submitted.

It was commented that using low-level syntax instead of high-level syntax to accomplish the goal would not really have much overhead.

It was asked what happens if the slice that was intended to "unfreeze" a region is lost due to packet loss. The loss of the "unfreeze" might not be detectable.

It was commented that, in some sense, the functionality could be accomplished without the proposed flag. The flag would be unnecessary if there was a "normative error concealment" specified for how to fill in missing areas of a picture using some reference picture. The proposed flag is just a warning that this is happening intentionally.

No action was taken on this.

JCTVC-U0138 Chroma Deblocking for Screen Content Coding [A. M. Tourapis, Y. Su, D. Singer (Apple), W. S. Kim, W. Pu, J. Sole, M. Karczewicz (Qualcomm)] [late]

(Consideration of this topic was chaired by GJS on Sunday 06-21, 16:10-16:45.)

This contribution proposes extending the deblocking process for any new screen content coding profile with two additional deblocking modes. In the first additional proposed mode, the deblocking of chroma edges is extended to also cover edges with a boundary strength equal to 1. In the second proposed mode, which only applies to 4:4:4 encoding, chroma deblocking is performed using the luma deblocking filter process. These extensions were described as being motivated primarily from additional observations on the quality of the chroma components after HEVC decoding, as well as due to the unification of the intra block copy mode with inter prediction.

The selection of deblocking operation was proposed to be indicated at the PPS level.

It was suggested that the issue this is trying to address is not specific to SCC (other than the interaction with IBC), but is rather intended to be useful for all content.

It was remarked that similar proposals had previously been considered in earlier work on version 1 and RExt, but that efforts to prove there was a significant benefit to changing the DBF had not succeeded previously.

The contributor reported that recent work on HDR coding had shown a benefit for modified deblocking (as a post-filter, which would likely have performed better as a loop filter), both in objective and subjective terms.

Tests on SCC content (e.g., SCC CTC content), had not been performed.

Text was not provided, but probably would not be very substantial. Text for something similar had been provided previously in JCTVC-O0089.

It was suggested to perform a CE to study this.

## HL syntax (0)

No contributions on general matters of high-level syntax were specifically noted, although various contributions include consideration of high-level syntax issues relating to specific features.

## SEI and VUI (7)

(Consideration of this topic was chaired by XXX on XXday 06-XX, XX:00-XX:00.)

JCTVC-U0032 Proposed addition of transfer characteristics in VUI [Y. Nishida, T. Yamashita, A. Ichigaya (NHK), T. Shimizu (ARIB)]

(Consideration of this topic was chaired by GJS on Thursday 06-25 in a session that began at 15:45.)

This document proposes a new entry for the transfer characteristics syntax element video usability information (VUI) in Annex E of the HEVC specification to support a new opto-electronic transfer function (OETF) for extended image dynamic range television (EIDRTV) on the basis of a new ARIB standard STD-B67.

The proposal was HDR related. In joint discussion at the parent-body level (see section 6.2), the JCT-VC was authorized to proceed with consideration of this contribution.

See also the related contribution U0033.

Decision: Adopt.

JCTVC-U0033 High dynamic range compatibility information SEI message [M. Naccari, A. Cotton, S. Schwarz, M. Pindoria, M. Mrak, T. Borer (BBC)]

(Consideration of this topic was chaired by GJS on Thursday 06-25 in a session that began at 15:45.)

The proposal was HDR related. In joint discussion at the parent-body level (see section 6.2), the JCT-VC was authorized to proceed with consideration of this contribution.

See also the related contribution U0032.

The BBC and NHK jointly proposed a new opto-electronic transfer function (OETF) to ITU-R WP 6C for high dynamic range (HDR) television production and distribution. The same OETF also forms the basis of a new ARIB standard (STD-B67). This HDR OETF has been designed to also deliver compatible images to legacy standard dynamic range displays. To exploit the backwards compatibility in distribution applications, maintaining compatibility with legacy receivers using HEVC Main 10 profile, a new SEI message was proposed to indicate the type of HDR material.

In the discussion, it was agreed that a persistence flag is not needed, as the scope of the SEI message should apply to the whole CLVS.

In discussion, two alternatives to the initially proposed syntax were discussed:

- Using the code value in the SEI message that is defined in the Annex E table (e.g., Annex E says it's BT.709, and this SEI says the ARIB standard). In this case, any value included in Annex E (now or later) would automatically be possible to indicate in the SEI message.

- Sending no syntax, restricting the use to when BT.709 is indicated in the Annex E VUI, and having the ARIB spec be implicitly identified by the presence of the SEI message.

The SEI message would indicate that the alternative interpretation is preferred.

A revised version of the contribution was provided to reflect some of this feedback.

Decision: Adopt (with the above alternative #1 approach). The editor was asked to check/refine the editorial quality of the text.

JCTVC-U0048 Output code map SEI for fractional bit-depth increase [C. Fogg (MovieLabs), B. Mandel (Universal), A. Tourapis, Y. Su, D. Singer (Apple)]

This proposal was HDR related and was not considered in detail by the JCT-VC. It is available for study.

JCTVC-U0098 Dynamic Range Adjustment SEI [D. Bugdayci Sansli, A. Ramasubramonian, D. Rusanovskyy, S. Lee, J. Sole, M. Karczewicz (Qualcomm)]

This proposal was HDR related and was not considered in detail by the JCT-VC. It is available for study.

JCTVC-U0112 Ambient viewing environment SEI message [G. J. Sullivan (Microsoft)]

(Consideration of this topic was chaired by Y.K. Wang on Saturday 06-20, 16:10-17:00.)

This contribution proposes an SEI message to describe the ambient viewing environment assumed when mastering the associated video content. The contribution asserts that the ambient viewing environment in which video content is experienced by the viewer can have a substantial effect on perceptual quality and that this is widely recognized (e.g., in ITU-R BT.2035 and ITU-R BT.500). For example, viewing an image in the varying environments of a darkened theater, a typical home (at night or during the daytime), or outdoors can have a major effect on the viewing experience. It is reported that adaptive brightness and light sensor technology are becoming common in devices such as mobile phones, tablets, and laptop computers. It is further asserted that conveying a representation of the conditions of the nominal viewing environment that was assumed when mastering the video content can enable receiving systems to adapt their local display (e.g., based on a user pre-setting or light sensor input) for optimal display. The proposed message is capable of conveying either of the types of viewing conditions specified in ITU-R BT.2035, and other conditions that can be specified using similar parameters.

It was asked whether VUI or a SEI message is more appropriate for this. It could be VUI, but it is generally easier to add SEI messages than extending VUI.

It was asked whether the scope of the message should be whole-CVS or more adaptive. In theory it could be more adaptive than a CVS scope.

It was asked how receivers would use this information. The nominal ambient viewing environment is the expected environment for viewing the content. For the best user experience, the viewing environment should have been built towards the nominal ambient viewing environment.

It was asked what should be done if there is a difference between the actual viewing environment and the nominal viewing environment. It was remarked that it would be nice if there is some guidance for this. However, that seems not straightforward. If the environment itself is not easy to adjust, at the minimum, the display should be adjusted to compensate. But how this should be done seems unknown.

It should be good to have some clarification on what is "the nominal ambient viewing environment".

It was generally agreed that the information is useful.

Two options to proceed were discussed: 1) adopt the proposal and suggest further study on whether some guidelines can be made when the actual viewing environment differs from the nominal viewing environment; 2) further study on whether some guidelines can be made when the actual viewing environment differs from the nominal viewing environment, and make a decision after that.

It was agreed to discuss this more when more experts on display and related issues were available.

(Further consideration of this topic was chaired by JRO in plenary on Sunday 06-21, 11:30-11:45.)

The SEI message describes the ambient lighting with appropriate parameters. The intent is clear.

Decision: Adopt (into SCC draft text).

Further study was also encouraged, e.g., for usage guidelines.

JCTVC-U0115 Video Bitstream Transition SEI mesage [W. Wan (Broadcom)]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 17:50-18:30.)

This contribution proposes that SEI messages be defined to facilitate the signalling of an upcoming transition from a bitstream encoded with one compression standard to another bitstream encoded with a different compression standard. It was suggested in the contribution that this type of signalling is needed in the video layer to facilitate effective support of on-the-fly codec switching. It is proposed that this functionality be defined to support switching between the ITU-T Rec. H.262 | ISO/IEC 13818-2, ITU-T Rec. H.264 | ISO/IEC 14496-10 and ITU-T Rec. H.265 | ISO/IEC 23008-2 compression standards.

The desirability of having this within the video layer vs. systems layer was discussed.

The desirability of the message would depend on the system environment:

- In some environments, no switching would occur;

- In some environments, the service would operate by just switching between fixed sets of bits, and inserting the SEI message may not be so feasible.

The contribution proposed to have the SEI message on the last picture in decoding order before a transition (or two pictures before, or some other amount of decoding order or period of time).

With a pre-encoded bitstream, it might be necessary to remove and/or replace the SEI message when adapting or splicing content.

The scope is somewhat crossing the CVS boundary.

Properties of the upcoming bitstream were proposed to be defined as profile/level information.

It was commented that a NAL unit type could be used instead of an SEI message.

It was remarked that the applicability of the syntax to the coding format used for the next segment needs to be considered – how to refer to the properties of the next bitstream.

Exactly how the data would be used was discussed.

An asserted advantage is that the position of the transition could be more precisely identified. It was said that information carried at a system level is sometimes not precisely synchronized with the exact position where a transition will occur.

Broader discussion of the idea seemed desirable.

Input to determine whether this would be desired by various orgs (DVB, ATSC, SCTE, etc.) may be desirable to solicit.

It was agreed to discuss this more at a higher level and further study.

(Further consideration of this topic was chaired by GJS and JRO on Sunday 06-21, 11:45-12:00.)

A comment was that if application standards bodies would need methods to handle bitstream switching, they could define how to do it in their environment (or provide input on how it should be done). It was remarked that a harmonization of methods across applications could be desirable, if we can determine that multiple applications would need this. It was commented that we don't really need to send letters to solicit comment – rather, other organizations could provide input if they wish something to be done here.

Raising the topic for joint discussion seemed desirable, but no immediate need to act was identified.

JCTVC-U0128 An HEVC SEI Message for Green Metadata [S. Cheng (Nanjing Univ.), J. Wen (Tsinghua Univ.)] [late]

(Consideration of this topic was chaired by GJS on Sunday 06-21, 17:15-17:30.)

An HEVC SEI message was proposed for the carriage of "Green Metadata" (ISO/IEC 23001-11/PDAM 1). The metadata is designed to enable cross-segment decoding for quality recovery after low-power encoding and may be extended for other future metadata.

(Further consideration of this topic was chaired by GJS & JRO on Tuesday 06-23, 11:40-11:50.)

The proposed text used the number 133 for the SEI message, which conflicts with a prior use (for the scalable nesting SEI message). It was suggested that 56 is the appropriate choice based on the AVC PDAM 2 number used in WG 11 N 15124. The editors were given the discretion to double check what number should be used.

Decision: Adopt (with the editor given the discretion to select the appropriate message identification number).

## Non-normative: Encoder optimization, decoder speed improvement and cleanup, post filtering, loss concealment, rate control, other information (6)

JCTVC-U0040 HM reference software bug fixes and enhancements to address the HDR/WCG CfE [A. M. Tourapis, Y. Su, D. Singer (Apple), J. Sole, D. Rusanovskyy, S. Lee, D. Bugdayci, A. Ramasubramonian, M. Karczewicz (Qualcomm), C. Fogg (Movielabs), A. Duenas, F. Bossen (NGCodec)]

(Consideration of this report was chaired by GJS on Thursday 06-26, 10:30-10:45.)

This contribution presented several video encoder enhancements as well as bug fixes that were introduced into the HM reference software as part of the responses for the MPEG HDR/WCG CfE. The enhancements include a new, improved motion estimation technique, several new methods for weighted prediction, speedups to the distortion computation, and improvements to the rate distortion optimization, among others. Bugs that were impacting the performance and behaviour of the weighted prediction, among others, were also discovered and fixed. The contribution suggests that these enhancements should be valuable not only for the MPEG HDR/WCG activity but also for future video codec development activities that might be undertaken by the JCT-VC. It was therefore suggested that these enhancements are also adopted into the official reference software. Some recommendations, in terms of common coding conditions, were also provided.

Despite the title, the proposed modifications were not specific to HDR.

Includes:

- Motion estimation improvement (enhancement, additional mode, but actually faster)

- Biprediction ME iteration (enhancement)
Recommends not using FEN=1 (which is in the current CTC); changing to FEN=0 proposed.

- Weighted prediction improvements and bug fixes

- R-D optimization behaviour improvements and bug fixes

The SCM was not checked.

Substantial compression improvement was reported on some HDR test sequences.

When weighted prediction is not used, the software is reported to be faster with the changes as well as having better compression capability.

The algorithmic (non-bug-fix) changes were controlled by config parameters and disabled by default.

Decision (SW): Adopt the modifications into the software; disabled by default.

Further study was encouraged, with extra features to be explored toward refining the CTC settings.

Testing on CTC (SCC and camera content) sequences was not performed; analysis of this is encouraged.

Further suggestions from the contributor:

- The presenter suggested to develop a hierarchical motion estimation capability for the reference software.

- Using chroma for ME was also suggested to be developed.

- The contributor also suggested to increase the number of references used for prediction, whereas some of our CTC conditions use only 2 for most pictures.

JCTVC-U0095 Non-CE: Encoder modification for intra prediction mode using adaptive colour transform [Y.-J. Chang, C.-L. Lin, J.-S. Tu, C.-C. Lin (ITRI)]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 17:00-17:40.)

In SCM4.0, the intra prediction mode can enable the adaptive colour transform (ACT) to perform an in-loop colour space conversion in the prediction residual domain. Because the ACT is adaptively enabled/disabled by a CU-level flag, increased encoder complexity is needed to evaluate whether to enable the ACT. This contribution proposes to reduce the encoder complexity by disallowing the ACT-based intra prediction mode in a 64x64 CU when the video format uses a YUV colour space. It also further proposes to disallow TU splitting for ACT-based intra prediction in large CUs. Compared to the SCM4.0 anchor, it is reported that the proposed modification can save 5-8% encoding time in AI conditions with a minor impact on coding efficiency.

Two speed-up methods were proposed:

- Skipping evaluation of ACT when video is YUV and CU size is 64x64

- Skipping checking of small TU sizes with ACT enabled when CU size is large

A 5-8% savings in encoding time was reported, with basically no coding efficiency impact.

A prior related contribution was T0080, which proposed to change the CTC to disable ACT for YUV video. However, that proposal sometimes had a significant coding efficiency loss.

It was noted that to run the encoder using the first modification, it would become necessary to know whether the video is YUV or not.

Suggestion: Adopt method 2 only (with a macro to control it, but using it in the CTC).

It was asked whether, for method 2, it really makes a difference whether ACT is off or on. The contributor said they had only tested it as reported.

Testing whether it really matters whether ACT is off or on was requested, to be followed by further discussion.

(Further consideration of this topic was chaired by Rajan Joshi on Thursday 06-25, 19:00.)

Results for a method 3 were presented. Method 3 is identical to method 2 except that it is applied irrespective of whether ACT is on or off for a CU. The method is reportedly applied to intra blocks in all configurations.

One participant commented that method 2 applied to 32×32 and higher CU sizes was preferable since there was no loss with that scheme, whereas method 3 had a loss of 0.1% for YUV camera-captured content. Another participant commented that the results showed that the speed-up was possible independent of whether ACT was being applied or not.

For method 3, only full-frame AI results were available. One participant commented that for method 2, it would be desirable to see the results in light of adoption of JCTVC-U0106 (TU based ACT). Also, for method 2, 0.1%-0.2% losses were reported in two classes for the LB configuration without any encoding time speedup.

Decision (SW): Software adoption of method 2 (with a macro such that by default, the skipping of the checking would be turned off for the CTC).

JCTVC-U0164 Cross-check of encoder modification for intra prediction mode using adaptive colour transform (JCTVC-U0095) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-U0132 Target bits saturation to avoid CPB overflow and underflow under the constraint of HRD [Y.-J. Ahn, X. Wu, W. Lim (KWU), D. Sim (Digital Insights)] [late]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 18:30-17:00.)

In this contribution, a scheme to saturate a target bit cost was presented which is asserted to avoid overflow and underflow of the coded picture buffer (CPB) in rate control of HEVC reference model (HM). Rate control based on R-lambda model is employed for the HM. Even though a hypothetical reference decoder (HRD) has been defined in HEVC for prevention of CPB overflow and underflow, it was reported that the reference software does not include any consideration for the HRD constraint. This contribution proposes a target bit cost saturation method to avoid CPB overflow and underflow under the HRD constraint.

The focus of the contribution was CBR mode.

It was reported that our current rate control does not consider overflow/underflow.

HRD is a tough subject, and not considered substantially in the HM.

The direction of work is encouraged.

The amount of code was estimated at 100 lines.

It was asked how some of the values of the controlling constants were derived.

Decision (N-N SW): Adopt, with a controlling macro, disabled by default (pending further study to confirm behavior).

JCTVC-U0152 R-lambda model based rate control with pre-encoding process [M. Fang, M. Tang, J. Wen, Z. Wen (Tsinghua Univ.)] [late]

(Consideration of this topic was chaired by GJS on Sunday 06-21, 09:15-09:45.)

This contribution proposes an R-lambda model based rate control with a pre-encoding process.

It was reported that the R-lambda model, which was proposed in JCTVC-K0103, failed to achieve expected performance in some cases, and two different modifications were proposed in JCTVC-T0216. At the previous meeting, it was agreed that the non-normative modification proposed in T0216 could be studied and evaluated in coordination with the software coordinators, and incorporated into the software at their discretion if they consider it appropriate.

However, further study indicated some problem with that.

The amount of code for the method is now a few hundred lines rather than the 50-100 lines previously reported for T0216. It was commented that it is desirable not only to minimize the number of lines of code, but also to avoid touching too many different parts of the software.

Cross-check work was reported to be in progress, but not yet completed.

It was commented that if we put this into the HM software, we should also include a good description of it into the HM algorithm documentation, and the proponent agreed to do this.

It was suggested to discuss this further after waiting for the outcome of a cross-check.

(Further consideration of this topic was chaired by Rajan Joshi on Thursday 06-25, 21.45-20.00.)

The crosscheck report was provided as JCTVC-U0187. The cross-checker commented that most of the changes were restricted to a single file.

One participant commented that changes in this area should not be discouraged. There is not much documentation in this area. One participant commented that even the new bit rates were far away from the target bit rate. This was especially true for the target bit rate of 100 kbits/sec. The proponent commented that they had discovered a bug recently and were re-running the tests. Another participant commented that there were a number of bug tracker tickets open in the area of rate control and slice-level QPs. It would be desirable for people with knowledge of the HM rate-control method to take a look at these. Another participant suggested comparing BD-rates with the fixed QP case and also to look at the standard deviation of PSNRs.

The proponent was encouraged to share the software for further study.

Further study of this was recommended.

JCTVC-U0187 Cross-check report of JCTVC-U0152: R-lambda model based rate control with pre-encoding process [X. Xu, S. Liu (MediaTek)] [late]

# Plenary Discussions, Joint meetings, BoG Reports, and Summary of Actions Taken

## General

Topics for general discussion at the plenary level:

- SCC PDAM ballot comments (WG 11 m36255):

(Preliminary consideration of this topic was chaired by GJS on Sunday 06-21, 17:00-17:15.)

  - Editorial review should be conducted to ensure that the specification is written as well as possible and follows consistent editorial conventions. The text should be thoroughly checked to ensure that the specification is complete, correct, technically mature, and harmonized with prior extensions of the syntax and semantics.

  - Editorial review should be conducted to ensure that the specification is written as well as possible and follows consistent editorial conventions.

  - The text should be thoroughly checked to ensure that the specification is complete, correct, technically mature, and harmonized with prior extensions and corrections of the syntax and semantics of the basis text.

  - Two SEI messages were intended to be included in the specification that were apparently inadvertently missed during the editing process. Specifically, these are the JCTVC-S0095 dependent RAP picture SEI message, and the JCTVC-S0148 / JCTVC-T0047 coded region completion SEI message (with a planned restriction about the layers not present SEI preceding the coded region completion SEI message). Add the specification of these SEI messages. Decision (Ed.): Agreed – fix.

  - No profile(s) are specified for the Screen Content Coding extension technology. Specify a 4:4:4 8 bit Screen Content Coding profile and a (nested) 4:2:0 8 bit Screen Content Coding profile. See notes on profile specifications in the next section (section 6.2).

  - Any mature errata corrections identified for the second edition of ISO/IEC 23008-2 should be considered for correction in the text of the amendment. This may include addressing CPB underflow for whole-picture decoding units. Include errata corrections as appropriate and identified in the second edition of ISO/IEC 23008-2 (when published). This essentially-editorial suggestion was consistent with our existing plans and is to be addressed by the editors.

  - Note that ISO/IEC 23001-8 is being modified to add the specification of additional colour primaries, transfer functions, and matrix coefficients. This includes some colour primaries that could be useful for HEVC but are not yet supported in HEVC – specifically, DCI P3 (the value 11), D65 P3 (the value 12), and P22 (the value 22). Add support for the additional colour-related enumeration codes, either by reference to ISO/IEC 23001-8 or direct specification within the text. Decision: Agreed.

  - In order to avoid confusion on the palette QP related new syntax elements coding, SCM 4 software and text should be aligned for 7.3.8.8 Palette syntax versus 7.3.8.11 Transform unit syntax. Decision (Ed.): Agreed.

  - ACT QP offset was adopted in the last meeting. PDAM text contains text integration errors. Text should be modified to be aligned with SCM 4 software and JCTVC-T0140 in 8.6.1 and 8.6.2. Decision (Ed.): Agreed.

## Project development

Joint meetings are discussed in this section of this report.

A joint meeting with the parent bodies (VCEG and MPEG Requirements and Video) was held on Monday 22 June during 16:30-17:00 on SCC, then 17:00-18:00 on HDR

- SCC profiles/levels and limits (coding tools, IBC referencing range, chroma format, bit depth, …)
Decision:

  - Palette size <= 64, palette predictor size <= 128

  - Intra block copy referencing range – not restricted

  - Suggestion to specify a 4:4:4 8 bit profile and a (nested) 4:2:0 8 bit profile.

  - Four profiles: 8 & 10 bit × 4:4:4 & 4:2:0

    - The 4:4:4 profile will support 4:2:0 video (in a nested fashion)

    - 4:2:2 support not to be required

  - RExt coding tools supported

  - INBLD supported

  - Friday closing plenary 12:30: Aspects not mentioned, such as bit rate and CPB size are to be set as in previous comparable profiles – editors have some discretion if aspects are not noted. A participant commented that for 4:4:4 10, they would like support of the CABAC alignment of the high throughput – this aspect not included for now; for further study.

- It was noted that MPEG will ballot a DIS, rather than a DAM, for the next stage of balloting for SCC, and the editors were asked to prepare the necessary output text accordingly.

- The proposed palette with pixel copy prediction mode (U0116 and U0171) was discussed – no action.

- Proposed SEI messages were discussed

  - Some were noted to be HDR related

  - Ambient viewing environment SEI message (U0112)

    - Suggestion: Study ST 2094 and MPEG-V (Marius)

  - Video bitstream transition SEI message (U0115) – to be further discussed (joint meeting of JCT-VC and MPEG systems and video on Tuesday 17:30-18:00)

- HDR video coding: Upon evaluation of the responses to the MPEG Call for Evidence (CfE) on HDR/WCG video, it was concluded that a substantial commonality existed between the proposed technical approaches and that the rapid standardization of an approach for improved HDR video coding might be feasible to develop without making any changes to the central HEVC decoding process and low-level syntax. Due to the desire to act quickly and the limited scope anticipated for that work, no plan to follow the CfE with a formal Call for Proposals seemed necessary. General support was expressed for allocating to the JCT-VC the technical work to develop the short-term specification, and management-level discussions were to take place toward that action. Additional longer-term work on HDR video coding may also occur in another phase - e.g., as part of a next-generation video coding standardization project.

Tuesday 23 June 17:30-18:00 Joint meeting with MPEG Systems and Video

- WG 11 input M36167 from M. M. Hannuksela et al. on dynamic resolution conversion (DRC/RPR) for video streaming had been reviewed by MPEG Systems prior to the joint meeting, and was shortly summarized again (this is a follow-up of docs M35766 and M36037 of the 111th WG 11 meeting) was discussed, no specific action was planned, other than encouraging further study for future video coding developments. Currently with HEVC, switching between streams (and potentially at the same time changing spatial resolution) is possible at IDR/IRAP pictures, which is hardly used in practice (due to overhead with closed GOP IDR pictures). A desire was expressed to allow switching at CRA positions as well (not just at IDR pictures) seamlessly, which is currenty supported only when no resolution change is done. Here it is requested to do this with reference picture resampling (to be specified normatively). The proponents of such technology were asked whether drift occurs, but this was said to be subjectively unnoticeable.

- It was also mentioned that such an approach could be implemented by using SHVC (even without drift), where, however, there is some bit rate overhead and the necessity to implement two loops would be undesirable. Some provision for this usage was discussed previously for enabling an indication in high-level syntax that only one resolution layer will need to be decoded in each picture interval (to avoid the extra complexity of two-loop decoding), and this may already be supported in high-level syntax. The proposed solution could also be interpreted as lightweight SHVC, and even could be using the same filter design as for HEVC upsampling, however these would need to be run only at certain picture positions, similar to switching pictures in AVC. Also, although SHVC enables switching up in picture resolution in a predictive manner, it does not enable switching down in resolution predictively.

- Conformant decoding would require defining a new profile to support this. Some concern was raised whether this would be justified for this specific case, where it would be questionable if that would ever be widely used. Even without a profile, it was suggested that decoders could be implemented running such a method in a non-conforming way, although this would presumably violate the current standard and cause interoperability problems with decoders not designed for it.

- Further study of this is necessary. Several experts mentioned that such a functionality might be desirable in next-generation video coding.

Thursday 25 June 14:30-15:15 Joint meeting with VCEG and MPEG Requirements and Video

- For new work on HDR video coding, it was reported that further management-level discussions at the parent-body level needed to take place before the JCT-VC would be given the mandate to work on this topic. Thus, the work toward standardization action on HDR (except as otherwise noted) was to remain at the parent-body level for the time being.

- Scalable lossless coding was discussed. A contribution m36373 had been submitted to MPEG by Fujitsu that reported a potential gain of 1–6% over JPEG 2000 for a scalable lossless coding scheme based on HEVC all-intra coding with a lossless enhancement. Further study of this was encouraged.

- The ARIB EOTF/OETF U0032 and U0033 were approved to be considered by JCT-VC – see notes on those contributions.

- On 4:2:0 chroma positioning for ITU-R BT.2020 video, a planned liaison communication with ITU-R WP6C was discussed (see notes for U0156)

## BoGs

JCTVC-U0175 BoG report on worst case memory bandwidth assessment [K. Rapaka]

See section 4.2.1.

JCTVC-U0178 BoG report on constrained intra prediction for IBC unification [X. Xiu]

See section 5.1.3.

## Summary of normative decisions

The following is a summary of the normative decisions (not including purely editorial decisions) made at the meeting for the HEVC specification text:

- Bug fix to technical content of prior edition:

  - Change the text to match the software use of CABAC alignment in the High Throughput profile (ticket #1391, U0031).

- Intra block copy:

  - Prohibit the combination of 8x8 bipred with IBC if use_integer_mv_flag is 0 (U0078 solution 5.3).

  - Allow IBC chroma displacement vectors that are not integers (U0077 A2 / U0080 / U0103, with combined text in U0175).

  - Allow the current picture to appear in both list 0 and list 1 (U0104 aspect #1).

  - Disable weighted prediction with IBC (U0104 aspect #3).

  - Allow I slices to be present when IBC is enabled in the SPS (U0079 aspect #1).

  - Place the current picture at the last position in the reference picture list when RPLM is not present and the list is long enough to contain it (U0180).

  - Store MVs in quarter-pel resolution always (i.e., for ordinary MVs, integer-only MVs, and IBC MVs) and derive integer-valued MVs by first computing a quarter-pel MV prediction, then right shifting it (with a rounding offset) to convert it to integer precision, adding integer-valued MV delta, then left-shifting it so that it is scaled for storage as a value with quarter-pel-resolution, and remove clipping of the value of MVs (U0081 method #1).

  - When ILF and IBC are both enabled, consider the unfiltered current picture as occupying a picture storage space in the DPB (in addition to the spaced used by the current picture) and increase maxDpbPicBuf to 7 (instead of 6) to prevent this from reducing the maximum depth of referencing hierarchy (based on U0181).

  - Change the IBC signalling so it has an enabling flag at the SPS level, and a PPS level flag to turn it off on a picture basis, and the DPB capacity calculation would use the picture-level flag along with the ILF and SAO flags (from discussion of U0181).

- Adaptive motion vector resolution:

  - See above for displacement value calculation and storage modification that affects both IBC and AMVR.

- Palette coding:

  - Swap the nesting of the the loops for sending the escape-coded entries so that they are sent in component-grouped order (U0087 approach #2).

  - Change the derivation of the number of palette entries to avoid excess condition checking in the derivation process (U0086 method #1).

  - For last_palette_run_type_flag, use the same CABAC context that is used for palette_run_type_flag (U0090 aspect #1).

  - Place the palette_transpose_flag after last_palette_run_type_flag and signal the syntax elements related to palette delta QP and palette chroma QP after the palette_transpose_flag, so that all the bins up to last_palette_run_type_flag are bypass-coded and grouped together (U0133).

  - Fix a mismatch with the software for derivation of maxPaletteRun (U0091).

  - Avoid QP dependence for coding the value of escape pixels, by using fixed-length coding for transform-quantization-bypass blocks and third-order Exp-Golomb binarization for other cases (U0052 method 2).

  - Allow signalling a palette predictor initialization in the SPS (U0084)

  - Simplify the Rice parameter derivation of numIndices using the formula c=3+(Idx>>3) rather than c=2+Idx/6 (U0176).

  - To enforce sensibility, require palette_max_predictor_size to be equal to 0 when palette_max_size is 0, and require palette_predictor_initializer_present_flag to be equal to 0 when palette_max_size or palette_mode_enabled_flag is 0 (U0036).

- Adaptive colour transform:

  - Control ACT at the TU level rather than at the CU level, as is done for CCP (U0106).

- SCC profiles:

  - Specify four SCC profiles: 8 bit and 10 bit, 4:4:4 and nested 4:2:0 (without support for 4:2:2), with palette size <= 64, palette predictor size <= 128, unrestricted IBC MV range, RExt coding tools and INBLD supported, otherwise as in previous comparable profiles.

- SEI and VUI:

  - Add dependent random-access SEI message (previously omitted by accident, see U0038).

  - Add coded region completion SEI message (previously omitted by accident, see U0038).

  - Add green metadata SEI message (U0128).

  - Add ambient viewing environment SEI message (U0112).

  - Add VUI indicators of colour_primaries standardized in the CICP standard ISO/IEC 23001-8 (ballot comment).

  - Add VUI value for transfer_characteristics standardized in ARIB (U0032), and an associated SEI message to indicate a preferred alternative transfer_characteristics interpretation (U0033).

For further detail, see the notes in other sections. The above list is only provided as a summary.

# Project planning

## WD drafting and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- HM text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be the Monday of the week preceding the meeting (5 October 2015).

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name. Also, core experiment responsibility descriptions should name individuals, not companies. AHG reports and CE descriptions/summaries are considered to be the contributions of individuals, not companies.

## General issues for CEs and TEs

Group coordinated experiments have been planned. These may generally fall into one of two categories:

- "Core experiments" (CEs) are the experiments for which there is a draft design and associated test model software that have been established.

- "Tool experiments" (TEs) are the coordinated experiments on coding tools at a more preliminary stage of work than those of "core experiments".

A preliminary description of each experiment is to be approved at the meeting at which the experiment plan is established.

It is possible to define sub-experiments within particular CEs and TEs, for example designated as CEX.a, CEX.b, etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the HM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments were as described in the prior output document JCTVC-L1100.

The general timeline agreed for CEs of this meeting was expected to be as follows: 3 weeks to obtain the software to be used as the basis of experimental feature integration, 1 more week to finalize the description and participation, 2 more weeks to finalize the software.

A deadline of four weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-VC reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the relevant software basis (e.g. the SCM). Exceptions must be justified, discussed on the general JCT-VC reflector, and recorded in the abstract of the summary report.

Final CE descriptions shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

Experiment descriptions should be written in a way such that it is understood as a JCT-VC output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-VC document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

A non-final CE plan document was reviewed and given tentative approval during the meeting (with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE is described in an associated output document JCTVC-S11xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-VC is not obligated to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-VC members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

- If combinations of proposals are intended to be tested in a CE, the precise description shall be available with the final CE description; otherwise it cannot be claimed to be part of the CE.

## Alternative procedure for handling complicated feature adoptions

The following alternative procedure had been approved at a preceding meeting as a method to be applied for more complicated feature adoptions:

- Run CE + provide software + text, then, if successful,

- Adopt into HM, including refinements of software and text (both normative & non-normative); then, if successful,

- Adopt into WD and common conditions.

Of course, we have the freedom (e.g. for simple things) to skip step 2.

## Common Conditions for HEVC Coding Experiments

Other than the addition of a Chinese text editing sequence to the TGM category of the CTC (see the notes for JCTVC-U0150 (later uploaded as JCTVC-U0188) and the AHG report JCTVC-U0010), no particular changes were noted w.r.t. the prior CTC.

## Software development

Software coordinators were asked to work out the detailed schedule with the proponents of adopted changes.

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

The planned timeline for software releases was established as follows:

- HM 16.6 available prior to the meeting.

- SCM 5.0 (based on HM 16.6 or newer) should be available within 3 weeks after the meeting.

- SHM 10.x U1013 (DAM, based on HM 16.2 or newer) should be available within 5 weeks after the meeting.

At a previous meeting (Sapporo, July 2014), it was noted that it should be relatively easy to add MV-HEVC capability to the SHVC software, and it was strongly suggested that this should be done. This remains desirable.

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-VC reflector (jct-vc@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-VC project management (AHG1) (jct-vc@lists.rwth-aachen.de) - Coordinate overall JCT-VC interim efforts. - Report on project status to JCT-VC reflector. - Provide a report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| HEVC test model editing and errata reporting (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-U1002 HEVC Test Model 16 (HM 16) Update 3 of Encoder Description - Collect reports of errata for HEVC - Gather and address comments for refinement of these documents. - Coordinate with AHG3 on software development and HM software technical evaluation to address issues relating to mismatches between software and text. | B. Bross, C. Rosewarne (co-chairs), M. Naccari, J.-R. Ohm, K. Sharman, G. J. Sullivan, Y.-K. Wang (vice-chairs) | N |
| HEVC HM software development and software technical evaluation (AHG3) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM software and its distribution. - Produce documentation of software usage for distribution with the software. - Prepare and deliver HM 16.x software versions and the reference configuration encodings according to JCTVC-L1100 and JCTVC-P1006 common conditions. - Suggest configuration files for additional testing of tools. - Coordinate with AHG2 on HEVC test model editing and errata reporting to identify any mismatches between software and text. | K. Sühring (chair), K. Sharman (vice-chair) | N |
| HEVC conformance test development (AHG4) (jct-vc@lists.rwth-aachen.de) - Study the requirements of HEVC conformance testing to ensure interoperability. - Prepare and deliver the JCTVC-U1008 SHVC conformance draft 3, and JCTVC-U1012 Version 1 and RExt conformance draft 5 specifications. - Discuss work plans and testing methodology to develop and improve HEVC v.1, RExt and SHVC conformance testing. - Establish and coordinate bitstream exchange activities for HEVC. - Identify needs for HEVC conformance bitstreams with particular characteristics. - Collect, distribute, and maintain bitstream exchange database and draft HEVC conformance bitstream test set. | T. Suzuki (chair), J. Boyce, K. Kazui, A. K. Ramasubramonian, W. Wan, Y. Ye (vice-chairs) | N |
| HEVC range extensions and interlaced video verification test report preparation (AHG5) (jct-vc@lists.rwth-aachen.de) - Finalize the report JCTVC-U1003 of the verification testing of HEVC for interlaced video content and format range extensions. | V. Baroncini (chair), G. Barroux, M. Karczewicz, M. Naccari, N. Ramzan, C. Rosewarne, G. J. Sullivan, T. K. Tan, J.-M. Thiesse, W. Wan (vice-chairs) | N |
| SCC coding performance analysis (AHG6) (jct-vc@lists.rwth-aachen.de) - Study test conditions and coding performance analysis methods for SCC coding performance. - Analyze coding performance of draft and proposed SCC coding features | H. Yu (chair), R. Cohen, A. Duenas, P. Lai, K. Rapaka, J. Xu (vice-chairs) | N |
| SCC extensions text editing (AHG7) (jct-vc@lists.rwth-aachen.de) - Produce and finalize HEVC screen content coding extensions draft 4 and test model 5 text. - Gather and address comments for refinement of the test model. - Coordinate with AHG8 to address issues relating to mismatches between software and text. | J. Xu, R. Joshi (co-chairs), R. Cohen, S. Liu, G. Sullivan, Y. Ye (vice-chairs) | N |
| SCC extensions software development (AHG8) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the SCM software and its distribution. - Prepare and deliver HM 16.x-SCM-5.0 software version and the reference configuration encodings according to JCTVC-U1015. - Prepare and deliver additional "dot" version software releases and software branches as appropriate. - Perform analysis and reconfirmation checks of the behaviour of the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with AHG7 to address any identified issues regarding text and software relationship. | B. Li, K. Rapaka (chairs), R. Cohen, P. Chuang, X. Xiu, M. Xu (vice-chairs) | N |
| Complexity of SCC extensions (AHG9) (jct-vc@lists.rwth-aachen.de) - Analyze complexity characteristics of current and proposed SCC coding methods with regards to throughput, amount of memory, memory bandwidth, parsing dependencies, parallelism, pixel processing, chroma position interpolation, and other aspects of complexity as appropriate. - Quantify and compare the average and worse case throughput (context-coded as well as bypass bins) for SCC coding methods. - Study latency and parallelism implications of SCC coding techniques, considering multicore and single-core architectures. - Identify criteria to determine the hardware implementability of the key hardware modules. - Identify bottlenecks in the current design with regard to implementation complexity. | A. Duenas (chair), M. Budagavi, R. Joshi, S.-H. Kim, P. Lai, W. Wang, X. Xiu (vice-chairs) | N |
| Test sequence material (AHG10) (jct-vc@lists.rwth-aachen.de) - Maintain the video sequence test material database for development of HEVC and its RExt, SHVC and SCC extensions. - Identify, collect, and make available a variety of video sequence test material. - Study coding performance and characteristics in relation to video test materials. - Identify and recommend appropriate test materials and corresponding test conditions for use in development of HEVC and its extensions. - Coordinate with the activities in AHG5 regarding interlaced video and range extensions development, and AHG6 regarding screen content coding. | T. Suzuki, V. Baroncini, R. Cohen (co-chairs), T. K. Tan, S. Wenger, H. Yu (vice-chairs) | N |
| SHVC test model editing (AHG11) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-U1007 SHVC Test Model 10 (SHM 10) text. - Coordinate with AHG12 on SHVC software development to address issues relating to mismatches between software and text. | J. Chen (chair), J. Boyce, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| SHVC software development (AHG12) (jct-vc@lists.rwth-aachen.de) - Prepare SHM 10.0 software (based on HM 16.x) for experimentation. - Generate anchors and templates based on common test conditions. - Discuss and identify additional issues related to SHVC software. | V. Seregin, Y. He, (co-chairs) | N |
| SHVC verification testing (AHG13) (jct-vc@lists.rwth-aachen.de) - Identify test sequences and test conditions for SHVC verification testing. - Prepare and propose a finalized SHVC verification test plan. | V. Baroncini, Y.-K. Wang, Y. Ye (co-chairs) | N |

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production.

JCTVC-U1000 Meeting Report of the 21st JCT-VC Meeting [G. J. Sullivan, J.-R. Ohm (chairs)] [2015-09-30] (near next meeting)

Remains valid – not re-issued: JCTVC-H1001 HEVC software guidelines [K. Sühring, D. Flynn, F. Bossen (software coordinators)]

(Remains valid, although from a prior meeting.)

JCTVC-U1002 High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Update 3 of Encoder Description [C. Rosewarne (primary editor), B. Bross, M. Naccari, K. Sharman, G. J. Sullivan (co-editors)] (WG 11 N 15436) [2015-09-30] (near next meeting)

JCTVC-U1003 Verification test report for format range extensions and interlaced video [C. Rosewarne, V. Baroncini, G. Barroux, G. J. Sullivan, A. M. Tourapis, M. Naccari (editors)] (WG 11 N 15437) [2015-07-24] (4 weeks)

Thanks were expressed to those involved in preparing for this verification test and conducting it successfully (see section 10).

JCTVC-U1004 SHVC draft verification test plan [Y. Ye, V. Baroncini, Y.-K. Wang (editors)] (WG 11 N 15438) [2015-09-30] (near next meeting)

JCTVC-U1005 HEVC Screen Content Coding Draft Text 4 [R. Joshi, S. Liu, G. J. Sullivan, J. Xu, Y. Ye (editors)] (WG 11 N 15435 ISO/IEC DIS 23008-2:201X 3rd Edition) [2015-07-31] (5 weeks)

Basic elements:

- IBC

- Adaptive colour transform

- Palette mode

- Adaptive MV resolution

- Intra boundary filtering disabling

Remains valid – not reissued: JCTVC-P1006 Common test conditions and software reference configurations for HEVC range extensions [D. Flynn, C. Rosewarne, K. Sharman (editors)]

JCTVC-U1007 SHVC Test Model 10 (SHM 10) Introduction and Encoder Description [J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela (editors)] (WG 11 N 15439) [2015-09-30] (near next meeting)

JCTVC-U1008 SHVC Conformance Testing Draft 3 [J. Boyce, A. K. Ramasubramonian (editors)] (WG 11 N 15449 ISO/IEC 23008-8/DAM 2) [2015-07-31] (5 weeks)

Remains valid – not updated JCTVC-Q1009 Common SHM Test Conditions and Software Reference Configurations [V. Seregin, Y. He (editors)]

Remains valid – not updated JCTVC-O1010 Guidelines for Conformance Testing Bitstream Preparation [T. Suzuki, W. Wan (editors)]

JCTVC-U1011 Reference Software for HEVC Format Range Extensions Draft 3 [K. Sharman, D. Flynn, K. Sühring, T. Suzuki (editors)] (WG 11 N 15442 Study Text of ISO/IEC 23008-5/DAM1) [2015-08-21] (8 weeks)

JCTVC-U1012 Conformance Testing for Improved HEVC Version 1 Testing and Format Range Extensions Profiles Draft 5 (WG 11 N 15449 ISO/IEC 23008-8/DAM2) [T. Suzuki, K. Kazui (editors)] [2015-07-31] (5 weeks)

(Includes enhancement & corrections of version 1 conformance testing.)

JCTVC-U1013 Reference software for Scalable HEVC (SHVC) Extensions Draft 2 (WG 11 N 15444 ISO/IEC 23008-5/DAM3) [Y. He, V. Seregin (editors)] [2015-07-31] (5 weeks)

JCTVC-U1014 Screen Content Coding Test Model 5 Encoder Description (SCM 5) [R. Joshi, J. Xu, R. Cohen, S. Liu, Y. Ye (editors)] (WG 11 N 15440) [2015-09-30] (near next meeting)

JCTVC-U1015 Common Test Conditions for Screen Content Coding [H. Yu, R. Cohen, K. Rapaka, J. Xu (editors)] [2015-07-17] (3 weeks)

(Includes additional TGM sequence.)

Remains valid – not re-issued: JCTVC-L1100 Common Test Conditions and Software Reference Configurations for HM [F. Bossen (editor)]

(Remains valid, although from a prior meeting.)

Note that regardless of preliminary CE plans established earlier in the meeting, such plans were not considered binding on final CE plans as reviewed in the closing plenary.

JCTVC-U1101 Description of Core Experiment 1 (CE1): Chroma deblocking filtering [A. Tourapis, K. Rapaka, X. Xiu (CE coordinators)] [2015-07-24] (4 weeks)

This was reviewed in preliminary form Friday 10:35-10:40.

Finalization of plans for subjective evaluation needed to be conducted; the review at the next meeting will include considering how well this was sorted out. Preparation for informal or formal subjective evaluation at the next meeting would be desirable.

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Thursday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting – a total of 6–6.5 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Friday prior to such meetings and closing it on the last day of the WG 11 meeting – a total of 7.5 meeting days).

Some specific future meeting plans (to be confirmed) were established as follows:

- Thu. 15 – Wed. 21 Oct. 2015, 22nd meeting under ITU-T auspices in Geneva, CH.

- Fri. 19 – Fri. 26 Feb. 2016, 23rd meeting under WG 11 auspices in San Diego, US.

- Thu. 26 May – Wed. 1 June 2016, 24th meeting under ITU-T auspices in Geneva, CH.

- Fri. 14 – Fri. 21 Oct. 2016, 25th meeting under WG 11 auspices in Chengdu, CN.

The agreed document deadline for the 22nd JCT-VC meeting is 5 October 2015. Plans for scheduling of agenda items within that meeting remained TBA.

Editors of output documents were reminded that final output documents (if also registered under a WG11 output doc number) have to be uploaded separately with a WG11 header. To do this in a timely fashion is particularly important for standards submitted for ballot, which should be sent to the chairs by the due date.

Contributors were reminded that any slide presentation decks used when presenting contributions should be made available as part of the uploaded archive for the contribution.

Poznan University of Technology and Kenzler Conference Management were thanked for the excellent hosting and organization of logistics for the 21st meeting of the JCT-VC.

Huawei was thanked for providing a new test sequence that could be used in standards development for screen content coding.

Vittorio Baroncini was thanked for conducting the verification tests reported in JCTVC-U1003, and the following companies were thanked for financially sponsoring the tests: CBS Corp., FhG-HHI, Microsoft Corp., Sony Corp., CableLabs, and Fujitsu Laboratories Ltd.

The JCT-VC meeting was closed at approximately 12:40 hours on Fri. 26 June 2015.

# Annex A to JCT-VC report:
List of documents

| JCT-VC number | MPEG number | Created | First upload | Last upload | Title | Authors |
| --- | --- | --- | --- | --- | --- | --- |
| JCTVC-U0001 | m36690 | 2015-06-18 21:27:27 | 2015-06-19 05:32:14 | 2015-06-19 05:32:14 | JCT-VC AHG report: Project management (AHG1) | G. J. Sullivan, J.-R. Ohm (AHG chairs) |
| JCTVC-U0002 | m36716 | 2015-06-19 06:34:00 | 2015-06-19 06:34:30 | 2015-06-19 06:34:30 | JCT-VC AHG report: HEVC test model editing and errata reporting (AHG2) | B. Bross, C. Rosewarne, M. Naccari, J.-R. Ohm, K. Sharman, G. J. Sullivan, Y.-K. Wang |
| JCTVC-U0003 | m36668 | 2015-06-17 17:16:44 | 2015-06-19 11:16:03 | 2015-06-19 14:21:57 | JCT-VC AHG report: HEVC HM software development and software technical evaluation (AHG3) | K. Suehring, K. Sharman, D. Flynn (AHG chairs) |
| JCTVC-U0004 | m36675 | 2015-06-18 10:34:19 | 2015-06-18 10:35:06 | 2015-06-18 10:35:06 | JCT-VC AHG report: HEVC conformance test development (AHG4) | T. Suzuki, J. Boyce, K. Kazui, A. K. Ramasubramonian, Y. Ye (AHG chairs) |
| JCTVC-U0005 | m36670 | 2015-06-17 19:13:18 | 2015-06-24 10:02:38 | 2015-06-24 10:02:38 | JCT-VC AHG report: HEVC and range extensions verification test preparation (AHG5) | V. Baroncini, M. Karczewicz, M. Naccari, N. Ramzan, C. Rosewarne, T. K. Tan, J.-M. Thiesse, W. Wan (AHG chairs) |
| JCTVC-U0006 | m36687 | 2015-06-18 19:24:07 | 2015-06-19 09:39:02 | 2015-06-19 09:39:02 | JCT-VC AHG report: SCC coding performance analysis (AHG6) | H. Yu (AHG chair), R. Cohen, A. Duenas, P. Lai, K. Rapaka, J. Xu (AHG chairs) |
| JCTVC-U0007 | m36692 | 2015-06-18 22:20:50 | 2015-06-19 10:14:49 | 2015-06-19 10:14:49 | JCT-VC AHG report: SCC extensions text editing (AHG7) | R. Joshi, J. Xu (AHG co-chairs), S. Liu, Y. Ye, R. Cohen, G. Sullivan, Z. Ma (AHG vice-chairs) |
| JCTVC-U0008 | m36708 | 2015-06-19 04:07:53 | 2015-06-19 04:10:26 | 2015-06-19 10:02:08 | JCT-VC AHG report: SCC extensions software development (AHG8) | K. Rapaka, B.Li (AHG co-chairs), R. Cohen, T.-D. Chuang, X. Xiu, M. Xu (AHG vice-chairs) |
| JCTVC-U0009 | m36725 | 2015-06-19 09:46:52 | 2015-06-22 15:43:32 | 2015-06-22 15:43:32 | JCT-VC AHG report: Complexity of SCC extensions (AHG9) | A. Duenas (chair), M. Budagavi, R. Joshi, S.-H. Kim, P. Lai, W. Wang, X. Xiu (vice-chairs) |
| JCTVC-U0010 | m36676 | 2015-06-18 10:37:34 | 2015-06-20 10:47:42 | 2015-06-20 10:47:42 | JCT-VC AHG report: Test sequence material (AHG10) | T. Suzuki, V. Baroncini, R. Cohen, T. K. Tan, S. Wenger, H. Yu (AHG chairs) |
| JCTVC-U0011 | m36723 | 2015-06-19 07:53:15 | 2015-06-19 07:55:53 | 2015-06-19 07:55:53 | JCT-VC AHG report: SHVC text editing (AHG11) | J. Chen, J. Boyce, Y. Ye, M. Hannuksela, G. J. Sullivan, Y.-K. Wang (AHG chairs) |
| JCTVC-U0012 | m36673 | 2015-06-17 20:06:40 | 2015-06-17 20:11:39 | 2015-06-19 10:51:30 | JCT-VC AHG report: SHVC software development (AHG12) | V. Seregin, Y. He (AHG chairs) |
| JCTVC-U0013 | m36674 | 2015-06-17 21:39:18 | 2015-06-18 10:40:57 | 2015-06-18 10:40:57 | JCT-VC AHG report: SHVC verification testing (AHG13) | V. Baroncini, Y.-K. Wang, Y. Ye (AHG chairs) |
| JCTVC-U0021 | m36680 | 2015-06-18 12:15:15 | 2015-06-18 12:17:57 | 2015-06-19 14:13:10 | CE1: Summary report of CE on palette mode improvement | P. Lai, V. Seregin, X. Xiu, C. Gisquet (CE coordinators) |
| JCTVC-U0022 | m36709 | 2015-06-19 04:11:51 | 2015-06-19 04:14:43 | 2015-06-19 12:52:13 | CE2: Summary report for Core Experiment 2 on Intra block copy memory access | K. Rapaka, C. Gisquet, J. Lainema, X. Xu (CE coordinators) |
| JCTVC-U0031 | m36248 | 2015-06-05 15:46:42 | 2015-06-05 15:49:27 | 2015-06-05 15:49:27 | Discrepancy between Version 2 text and software for High Throughput Profile | K. Sharman, N. Saunders (Sony) |
| JCTVC-U0032 | m36250 | 2015-06-07 07:17:43 | 2015-06-09 15:06:12 | 2015-06-15 15:49:16 | Proposed addition of transfer characteristics in VUI | Y. Nishida, T. Yamashita, A. Ichigaya (NHK), T. Shimizu (ARIB) |
| JCTVC-U0033 | m36258 | 2015-06-08 11:00:50 | 2015-06-09 22:47:19 | 2015-06-29 13:17:55 | High dynamic range compatibility information SEI message | M. Naccari, A. Cotton, S. Schwarz, M. Pindoria, M. Mrak, T. Borer (BBC) |
| JCTVC-U0034 | m36259 | 2015-06-08 12:21:24 | 2015-06-08 14:51:16 | 2015-06-08 14:51:16 | CE2: Test 3 on intra block copy constraints on filtering (single-pass encoding decisions) | J. Lainema, M. M. Hannuksela (Nokia) |
| JCTVC-U0035 | m36260 | 2015-06-08 12:21:55 | 2015-06-11 09:23:26 | 2015-06-11 09:23:26 | CE2: Crosscheck of Test 2 on intra block copy constraints on filtering (frame level encoding decisions) | J. Lainema (Nokia) |
| JCTVC-U0036 | m36265 | 2015-06-08 22:41:46 | 2015-06-08 22:45:09 | 2015-06-08 22:45:09 | On Zero Maximum Palette Size | K. Misra, S.-H. Kim (Sharp) |
| JCTVC-U0037 | m36267 | 2015-06-09 00:16:21 | 2015-06-09 20:31:19 | 2015-06-23 17:38:16 | SHVC Verification Test Draft Plan | Y. He, Y. Ye (InterDigital), Hendry, Y.-K. Wang (Qualcomm) |
| JCTVC-U0038 | m36268 | 2015-06-09 01:14:01 | 2015-06-19 05:04:17 | 2015-06-19 05:04:17 | Proposed editorial improvements to HEVC Screen Context Coding Draft Text 3 | R. Joshi (Qualcomm), J. Xu (Microsoft), S. Liu (MediaTek), Y. Ye (InterDigital), G. Sullivan (Microsoft) |
| JCTVC-U0039 | m36269 | 2015-06-09 07:00:03 | 2015-06-09 10:49:42 | 2015-06-09 10:49:42 | Overview of the High Efficiency Image File Format | M. M. Hannuksela, V. K. Malamal Vadakital, J. Lainema (Nokia) |
| JCTVC-U0040 | m36270 | 2015-06-09 07:43:34 | 2015-06-09 08:01:01 | 2015-06-26 12:15:41 | HM reference software bug fixes and enhancements to address the HDR/WCG CfE | A.M. Tourapis, Y. Su, D. Singer (Apple), J. Sole, D. Rusanovskyy, S. Lee, D. Bugdayci, A. Ramasubramonian, M. Karczewicz (Qualcomm), C. Fogg (Movielabs), A. Duenas, F. Bossen (NGCodec) |
| JCTVC-U0041 | m36271 | 2015-06-09 07:48:20 | 2015-06-13 00:13:34 | 2015-06-16 03:40:33 | On 4:4:4 to 4:2:0 conversion - Performance of downsampling and upsampling filters, and MinMax and Closed Loop Filtering. | A.M. Tourapis, Y. Su, D. Singer (Apple), J. Sole, D. Rusanovskyy, S. Lee, D. Rusanovskyy, A. Ramasubramonian, M. Karczewicz (Qualcomm), C. Fogg (Movielabs), A. Duenas, F. Bossen (NGCodec) |
| JCTVC-U0042 | m36272 | 2015-06-09 07:54:42 | 2015-06-10 03:26:55 | 2015-06-16 03:40:56 | Clipping during 4:2:0 to 4:4:4 conversions | A.M. Tourapis, Y. Su, D. Singer (Apple), J. Sole, D. Rusanovskyy, M. Karczewicz (Qualcomm), C. Fogg (Movielabs), A. Duenas, F. Bossen (NGCodec) |
| JCTVC-U0043 | m36273 | 2015-06-09 07:55:32 | 2015-06-16 01:41:00 | 2015-06-16 03:41:21 | Deblocking in HEVC: Some observations from the HDR/WCG CfE | A.M. Tourapis, Y. Su, D. Singer (Apple), J. Sole, D. Rusanovskyy, M. Karczewicz (Qualcomm), C. Fogg (Movielabs), A. Duenas, F. Bossen (NGCodec) |
| JCTVC-U0044 | m36274 | 2015-06-09 07:59:47 | 2015-06-10 08:48:52 | 2015-07-29 17:59:55 | AVC/HEVC coding results of noise-reduced 4:2:0 and 4:4:4 HDR signals. | K. Slavin (isovideo), C. Fogg (MovieLabs) |
| JCTVC-U0045 | m36275 | 2015-06-09 08:00:02 | 2015-06-10 08:49:49 | 2015-06-22 06:56:33 | High Dynamic Range with HEVC Main10 | D. Le Gall (Ambarella), A. Tourapis (Apple), M. Raulet (Ateme), W. Wan (Broadcom), D. Malas (Cablelabs), Y. Syed, J. Parkins (Comcast), J. Boyce (Intel), C. Fogg, J. Helman (MovieLabs), B. Mandel, G. Reitmeier (NBCUniversal), A. Duenas (NG Codec), D. Rusanovskyy (Qualcomm), E. Alshina, M. Budagavi (Samsung), T. Suzuki (Sony) |
| JCTVC-U0046 | m36276 | 2015-06-09 08:00:12 | 2015-06-10 08:50:38 | 2015-06-22 05:04:54 | Studio requirements for next-generation video codecs (and HExt: HEVC HDR Extensions) | C. Fogg, J. Helman (MovieLabs), M. DeValue (Disney), H. Basse (Fox), S. Stephens (Sony Pictures), B. Mandel (Universal Pictures), G. Gewickey (Warner Bros) |
| JCTVC-U0047 | m36277 | 2015-06-09 08:00:22 | 2015-06-25 10:19:36 | 2015-06-25 10:19:36 | HDR Workflow precision and steps. | C. Fogg (MovieLabs) |
| JCTVC-U0048 | m36278 | 2015-06-09 08:00:31 | 2015-06-10 08:51:26 | 2015-06-10 08:51:26 | Output code map SEI for fractional internal bit-depth increase | C. Fogg (MovieLabs), B. Mandel (Universal), A. Tourapis, Y. Su, D. Singer (Apple) |
| JCTVC-U0049 | m36279 | 2015-06-09 08:01:02 | 2015-06-10 10:01:14 | 2015-06-25 10:19:58 | Wiener filter for noise reduction pre-processing of HDR video signals | A. Tourapis (Apple), C. Fogg (MovieLabs) |
| JCTVC-U0050 | m36281 | 2015-06-09 09:48:50 | 2015-06-10 01:17:19 | 2015-06-18 22:51:55 | CE1 Test B : Simplification for index map coding in palette mode | J. Kim, J. Ye, P. Lai, S. Liu, S. Lei (MediaTek) |
| JCTVC-U0051 | m36283 | 2015-06-09 13:25:06 | 2015-06-09 13:29:04 | 2015-06-25 18:56:08 | Comparison of Compression Performance of HEVC Test Model 16.4 and HEVC Screen Content Coding Extensions Test Model 4 with AVC High 4:4:4 Predictive profile | B. Li, J. Xu, G. J. Sullivan (Microsoft) |
| JCTVC-U0052 | m36284 | 2015-06-09 13:25:06 | 2015-06-09 13:29:31 | 2015-06-09 13:29:31 | On the palette escape pixel coding | B. Li, J. Xu (Microsoft) |
| JCTVC-U0053 | m36285 | 2015-06-09 16:18:00 | 2015-06-09 17:36:23 | 2015-06-15 10:58:23 | CE2 Test 2: Intra block copy constraints on filtering | G. Laroche, G. Malard, C. Gisquet, P. Onno (Canon) |
| JCTVC-U0054 | m36286 | 2015-06-09 16:18:07 | 2015-06-09 17:24:39 | 2015-06-19 18:25:05 | Non-CE2: On interpretation of the IBC vectors | C. Gisquet, G. Laroche, P. Onno (Canon) |
| JCTVC-U0055 | m36287 | 2015-06-09 16:18:17 | 2015-06-09 17:27:23 | 2015-06-09 17:27:23 | High-level syntax refinement in monochrome signalling | C. Gisquet, G. Laroche, P. Onno (Canon) |
| JCTVC-U0056 | m36288 | 2015-06-09 16:18:26 | 2015-06-09 17:37:42 | 2015-06-19 16:40:54 | Unification of the Adaptive Motion Resolution signalling for both IBC and Inter modes | G. Laroche, G. Malard, C. Gisquet, P. Onno (Canon) |
| JCTVC-U0057 | m36289 | 2015-06-09 16:18:38 | 2015-06-09 17:38:00 | 2015-06-23 13:01:44 | On Inter modes constraint for slices | G. Laroche, G. Malard, P. Onno (Canon) |
| JCTVC-U0058 | m36290 | 2015-06-09 16:18:53 | 2015-06-17 17:55:50 | 2015-06-17 17:55:50 | Compression Performance of HEVC Screen Content Coding Extensions Test Model 4.x with slices | C. Gisquet, G. Laroche, P. Onno (Canon) |
| JCTVC-U0059 | m36291 | 2015-06-09 16:19:11 | 2015-06-17 15:30:42 | 2015-06-17 15:30:42 | CE1: Cross-check of Test C.1 from Interdigital/Qualcomm | C. Gisquet (Canon) |
| JCTVC-U0060 | m36292 | 2015-06-09 16:19:21 | 2015-06-19 14:47:55 | 2015-06-19 14:47:55 | CE2: Cross check of Test 5 on Intra block copy constraints on Bi-prediction samples with use of local cache | P. Onno, G. Malard (Canon) |
| JCTVC-U0061 | m36293 | 2015-06-09 16:25:05 | 2015-06-10 07:39:47 | 2015-06-12 00:58:48 | CE1 : Test A.1 : Extended copy above mode to the first line with index adjustment bits | Y.-C. Sun, J. Kim, T.-D. Chuang, S. Liu, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek), V. Seregin, F. Zou, W. Pu, R. Joshi, M. Karczewicz (Qualcomm) |
| JCTVC-U0062 | m36294 | 2015-06-09 16:25:25 | 2015-06-10 07:40:12 | 2015-06-21 09:27:18 | CE1 : Test A.2 : Extended copy above mode to the first line with grouping the palette_run_type_flag in front | Y.-C. Sun, J. Kim, T.-D. Chuang, P. Lai, S. Liu, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-U0063 | m36295 | 2015-06-09 16:26:06 | 2015-06-10 07:40:37 | 2015-06-19 14:55:49 | CE1-related: Colour-plane-based escape pixel coding | T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-U0064 | m36296 | 2015-06-09 16:26:30 | 2015-06-10 07:41:12 | 2015-06-19 12:00:18 | CE1-related: Palette coding with inter-prediction | W. Zhu, K. Zhang, X. Zhang, S. Lei (MediaTek) |
| JCTVC-U0065 | m36297 | 2015-06-09 16:26:49 | 2015-06-10 07:41:34 | 2015-06-19 16:49:33 | CE2-related: Intra block copy searching constraints for reducing worst case bandwidth | T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-U0066 | m36298 | 2015-06-09 16:27:15 | 2015-06-10 07:42:10 | 2015-06-19 09:14:21 | CE1-related: Row-based copy pixel from neighbouring CU | T.-D. Chuang, Y.-C. Sun, J. Kim, P. Lai, Y.-W. Chen, S. Liu, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-U0067 | m36299 | 2015-06-09 19:02:37 |  |  | Withdrawn |  |
| JCTVC-U0068 | m36300 | 2015-06-09 19:02:59 |  |  | Withdrawn |  |
| JCTVC-U0069 | m36301 | 2015-06-09 19:03:19 |  |  | Withdrawn |  |
| JCTVC-U0070 | m36302 | 2015-06-09 20:16:15 | 2015-06-10 06:54:53 | 2015-06-10 06:54:53 | CE1: Cross check of Test B on Simplification for index map coding in palette mode | J. Zhao, S.-H. Kim (Sharp) |
| JCTVC-U0071 | m36303 | 2015-06-09 21:11:39 |  |  | Withdrawn |  |
| JCTVC-U0072 | m36304 | 2015-06-09 21:11:50 |  |  | Withdrawn |  |
| JCTVC-U0073 | m36305 | 2015-06-09 21:12:01 |  |  | Withdrawn |  |
| JCTVC-U0074 | m36306 | 2015-06-09 21:12:12 |  |  | Withdrawn |  |
| JCTVC-U0075 | m36307 | 2015-06-09 21:12:28 |  |  | Withdrawn |  |
| JCTVC-U0076 | m36308 | 2015-06-09 21:17:42 | 2015-06-10 08:52:22 | 2015-06-19 17:18:15 | On unification of motion vector resolution for screen content coding | X. Xu, S. Liu, S. Lei (MediaTek) |
| JCTVC-U0077 | m36309 | 2015-06-09 21:19:59 | 2015-06-10 08:51:04 | 2015-06-19 01:53:27 | On chroma motion vector derivation for intra block copy | X. Xu, S. Liu, S. Lei (MediaTek) |
| JCTVC-U0078 | m36310 | 2015-06-09 22:12:27 | 2015-06-10 13:53:31 | 2015-06-24 10:24:04 | CE2 : Test 5 on intra block copy constraints on prediction | K. Rapaka, V. Seregin, R. Joshi, T. Hsieh, M. Karczewicz (Qualcomm) |
| JCTVC-U0079 | m36311 | 2015-06-09 22:13:34 | 2015-06-10 10:33:30 | 2015-06-15 23:02:53 | HLS : On Intra block copy signaling control | K. Rapaka, V. Seregin, R. Joshi, M. Karczewicz, Y.-K. Wang (Qualcomm), A. Tourapis, Y. Su, D. Singer (Apple), |
| JCTVC-U0080 | m36312 | 2015-06-09 22:14:28 | 2015-06-10 12:49:54 | 2015-06-15 19:38:16 | Intra block copy chroma interpolation for Non-444 | K. Rapaka, V. Seregin, R. Joshi, M. Karczewicz (Qualcomm) |
| JCTVC-U0081 | m36313 | 2015-06-09 22:15:05 | 2015-06-10 11:48:37 | 2015-06-26 12:33:28 | On unification of adapative motion vector resolution | K. Rapaka, V. Seregin, C. Pang, M. Karczewicz, R. Joshi (Qualcomm), |
| JCTVC-U0082 | m36314 | 2015-06-09 22:18:02 | 2015-06-24 04:50:40 | 2015-06-24 04:50:40 | CE2: cross check of CE2 Test 2: Intra block copy constraints on filtering | K. Rapaka (Qualcomm) |
| JCTVC-U0083 | m36315 | 2015-06-09 22:39:38 | 2015-06-10 05:31:33 | 2015-06-10 05:31:33 | On indication of the use of current reference picture | C. Liu, S. Liu, X. Xu, S. Lei (MediaTek) |
| JCTVC-U0084 | m36316 | 2015-06-09 23:14:20 | 2015-06-10 08:42:48 | 2015-06-20 01:10:11 | On palette predictor initialization for Screen Content Coding | J. Ye, S. Liu, S. Lei (MediaTek) |
| JCTVC-U0085 | m36317 | 2015-06-09 23:20:48 | 2015-06-09 23:25:44 | 2015-06-15 18:57:26 | Modulation-based HDR video coding with SDR backward compatibility | S. Lasserre, F. Le Leannec, E. Franois, T. Poirier (Technicolor) |
| JCTVC-U0086 | m36318 | 2015-06-09 23:21:06 | 2015-06-10 09:02:11 | 2015-06-19 02:16:23 | CE1-related: Simplification on coding NumPaletteIndices | J. Ye, J. Kim, S. Liu, P. Lai, W. Zhu, K. Zhang, T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-U0087 | m36319 | 2015-06-09 23:34:03 | 2015-06-09 23:37:09 | 2015-06-11 19:56:59 | Syntax Cleanup for Palette Mode | W. Pu, R. Joshi, T. Hsieh, M. Karczewicz, F. Zou, V. Seregin (Qualcomm) |
| JCTVC-U0088 | m36320 | 2015-06-09 23:34:30 | 2015-06-10 08:22:24 | 2015-06-16 20:50:35 | CE1 Test C.2: Palette run-length coding with rotated index scans (scan from bottom-right) | P. Lai, J. Kim, S. Liu, S. Lei (MediaTek) |
| JCTVC-U0089 | m36321 | 2015-06-09 23:35:04 | 2015-06-10 08:49:05 | 2015-06-20 10:43:28 | Constraints on palette syntax elements | P. Lai, J. Kim, S. Liu, J. Ye, T.-D. Chuang, S. Lei (MediaTek) |
| JCTVC-U0090 | m36322 | 2015-06-09 23:37:37 | 2015-06-10 08:59:05 | 2015-06-19 02:12:16 | CE1-related: Palette Mode Context and Codeword Simplification | J. Ye, J. Kim, S. Liu, P. Lai, W. Zhu, K. Zhang, T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-U0091 | m36323 | 2015-06-09 23:38:25 | 2015-06-10 08:45:13 | 2015-06-19 02:05:18 | CE1-related: On the mismatch of SCM4.0 and HEVC SCC working draft | J. Ye, S. Liu, P. Lai, S. Lei (MediaTek) |
| JCTVC-U0092 | m36324 | 2015-06-09 23:41:15 | 2015-06-10 08:59:40 | 2015-06-23 11:24:54 | Draft Text modifications on palette | P. Lai, J. Ye, J. Kim, X. Xu, S. Liu, S. Lei (MediaTek) |
| JCTVC-U0093 | m36325 | 2015-06-10 00:29:59 | 2015-06-10 01:54:05 | 2015-06-10 01:54:05 | Non-CE1: On Number of Palette Indices coding | S.-H. Kim, K. Misra, J. Zhao, A. Segall (Sharp) |
| JCTVC-U0094 | m36326 | 2015-06-10 00:33:18 | 2015-06-10 09:20:27 | 2015-06-23 19:11:47 | Non-CE1: Palette Run Hiding | M. Karczewicz, W. Pu, R. Joshi, F. Zou, V. Seregin (Qualcomm) |
| JCTVC-U0095 | m36327 | 2015-06-10 01:08:38 | 2015-06-10 04:45:24 | 2015-06-24 11:17:50 | Non-CE: Encoder modification for intra prediction mode using adaptive colour transform | Y.-J. Chang, C.-L. Lin, J.-S. Tu, C.-C. Lin (ITRI) |
| JCTVC-U0096 | m36328 | 2015-06-10 01:09:36 | 2015-06-10 11:07:12 | 2015-06-19 15:31:58 | CE1 Related: Improved palette encoder | M. Karczewicz, F. Zou, V. Seregin, W. Pu, R. Joshi (Qualcomm) |
| JCTVC-U0097 | m36329 | 2015-06-10 01:10:00 | 2015-06-10 06:03:01 | 2015-06-19 15:39:39 | Non-CE1: On maximum palette predictor size | Y.-J. Chang, C.-C. Lin, C.-L. Lin, J.-S. Tu (ITRI) |
| JCTVC-U0098 | m36330 | 2015-06-10 02:00:00 | 2015-06-10 09:13:03 | 2015-06-25 10:10:57 | Dynamic Range Adjustment SEI | D. Bugdayci Sansli, A. Ramasubramonian, D. Rusanovskyy, S. Lee, J. Sole, M. Karczewicz (Qualcomm) |
| JCTVC-U0099 | m36331 | 2015-06-10 02:29:20 | 2015-06-10 09:03:29 | 2015-06-10 09:03:29 | Modified method for sending number of palette indices | R. Joshi, W. Pu, M. Karczewicz, F. Zou, V. Seregin (Qualcomm) |
| JCTVC-U0100 | m36332 | 2015-06-10 02:43:18 | 2015-06-10 05:17:46 | 2015-06-10 05:17:46 | On operation of DPB in Screen Content Coding | C. Liu, S. Liu, X. Xu, S. Lei (MediaTek) |
| JCTVC-U0101 | m36333 | 2015-06-10 02:44:09 | 2015-06-10 08:53:20 | 2015-06-18 23:39:36 | CE1 Test C.1: Improved palette run-length coding with palette reverse scan order | X. Xiu, Y. Ye, Y. He (InterDigital), W. Pu, R. Joshi, M. Karczewicz, F. Zou, V. Seregin (Qualcomm) |
| JCTVC-U0102 | m36334 | 2015-06-10 02:44:39 | 2015-06-10 08:52:40 | 2015-06-21 04:46:33 | On constrained intra prediction for the unification framework of intra block copy | X. Xiu, Y. Ye, Y. He (InterDigital) |
| JCTVC-U0103 | m36335 | 2015-06-10 02:45:10 | 2015-06-10 08:51:57 | 2015-06-20 02:32:45 | On chroma derivation of intra block copy for non-444 video | X. Xiu, Y. Ye, Y. He (InterDigital) |
| JCTVC-U0104 | m36336 | 2015-06-10 02:45:39 | 2015-06-10 08:51:22 | 2015-06-18 23:27:49 | On unification framework of intra block copy | X. Xiu, Y. Ye, Y. He (InterDigital) |
| JCTVC-U0105 | m36337 | 2015-06-10 02:46:05 | 2015-06-10 08:50:50 | 2015-06-18 23:25:36 | Non-CE1: Simplification on deriving NumPaletteIndices for palette mode | X. Xiu, Y. He, Y. Ye (InterDigital) |
| JCTVC-U0106 | m36338 | 2015-06-10 02:46:23 | 2015-06-10 08:50:07 | 2015-06-18 23:22:59 | On signaling adaptive color transform at TU level | X. Xiu, Y. He, Y. Ye (InterDigital) |
| JCTVC-U0107 | m36339 | 2015-06-10 03:12:07 | 2015-06-10 08:48:12 | 2015-06-19 02:05:58 | On stored decoded motion vector resolution | X. Xu, S. Liu, S. Lei (MediaTek) |
| JCTVC-U0108 | m36340 | 2015-06-10 03:30:14 | 2015-06-18 23:18:52 | 2015-06-18 23:18:52 | Cross-verification of CE1 Test C.2: Palette run-length coding with rotated index scans | X. Xiu, Y. He, Y. Ye (InterDigital) |
| JCTVC-U0109 | m36341 | 2015-06-10 03:35:11 | 2015-06-10 05:42:38 | 2015-06-20 15:29:38 | Non-CE1: Simplification for parsing the index of the most significant bit in the paletteRun binarization | Y.-J. Chang, C.-L. Lin, C.-C. Lin, J.-S. Tu, W. Digonnet (ITRI) |
| JCTVC-U0110 | m36342 | 2015-06-10 03:44:15 | 2015-06-10 04:53:31 | 2015-06-18 16:44:06 | Bugfixes and cleanup for the text of palette mode in HEVC SCC draft | Y.-J. Chang, J.-S. Tu, C.-C. Lin, C.-L. Lin (ITRI) |
| JCTVC-U0111 | m36343 | 2015-06-10 05:35:55 | 2015-06-10 05:54:24 | 2015-06-17 08:39:41 | Non-CE: MV resolution unification for MV derivation in Intra-Block Copy search | J.-S. Tu, C.-C. Lin, C.-L. Lin, Y.-J. Chang (ITRI) |
| JCTVC-U0112 | m36344 | 2015-06-10 06:08:46 | 2015-06-10 06:10:40 | 2015-06-10 06:10:40 | Ambient viewing environment SEI message | G. J. Sullivan (Microsoft) |
| JCTVC-U0113 | m36345 | 2015-06-10 06:27:44 | 2015-06-10 08:46:19 | 2015-06-19 02:22:35 | On reference picture list construction for intra block copy | X. Xu, S. Liu, S. Lei (MediaTek) |
| JCTVC-U0114 | m36346 | 2015-06-10 07:07:13 | 2015-06-10 09:03:49 | 2015-06-19 18:49:35 | Generate the palette predictor initializers with low delay | J. Ye, T.-D. Chuang, S. Liu, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-U0115 | m36347 | 2015-06-10 07:47:38 | 2015-06-10 07:55:41 | 2015-06-10 07:55:41 | Video Bitstream Transition SEI | W. Wan (Broadcom) |
| JCTVC-U0116 | m36348 | 2015-06-10 08:24:44 | 2015-06-10 16:58:45 | 2015-06-20 17:26:56 | Non-CE1: Enhancement to Palette Coding by Palette with Pixel Copy (PPC) Coding | T. Lin, K. Zhou, L. Zhao, X. Chen (Tongji Univ.) |
| JCTVC-U0117 | m36349 | 2015-06-10 09:03:53 | 2015-06-18 11:57:06 | 2015-06-18 11:57:06 | Cross-check of JCTVC-U0122 CE1 Test D: Modification of Palette Run Coding | P. Lai, J. Ye, J. Kim (MediaTek) |
| JCTVC-U0118 | m36350 | 2015-06-10 09:30:40 | 2015-06-10 12:00:34 | 2015-06-19 18:25:43 | On intra block copy bitstream constraints | V. Seregin, K. Rapaka, R. Joshi, M. Karczewicz, Y.-K. Wang (Qualcomm) |
| JCTVC-U0119 | m36351 | 2015-06-10 09:39:18 | 2015-06-10 09:43:54 | 2015-06-20 11:47:24 | On slice segment freeze signalling for screen content coding | T. Laude (Leibniz Univ. Hannover) |
| JCTVC-U0120 | m36355 | 2015-06-10 19:51:51 | 2015-06-15 20:37:46 | 2015-06-15 20:37:46 | Cross-check of JCTVC-U0113, On reference picture list construction for intra block copy | R. Cohen (MERL) |
| JCTVC-U0121 | m36359 | 2015-06-11 04:01:05 | 2015-06-11 05:03:46 | 2015-06-11 05:03:46 | CE1: Crosscheck of CE1 Test A.1 (JCTVC-U0061) | W. Digonnet, C.-C. Lin, J.-S. Tu, Y.-J. Chang, C.-L. Lin (ITRI) |
| JCTVC-U0122 | m36369 | 2015-06-11 23:40:31 | 2015-06-12 21:07:15 | 2015-06-12 21:07:15 | CE1 Test D: Modification of Palette Run Coding | M. Karczewicz, W. Pu, R. Joshi, F. Zou, V. Seregin (Qualcomm) |
| JCTVC-U0123 | m36370 | 2015-06-11 23:43:51 | 2015-06-16 20:01:30 | 2015-06-16 20:01:30 | Crosscheck report of JCTVC-U0102 | K. Misra, S.-H. Kim (Sharp) |
| JCTVC-U0124 | m36371 | 2015-06-12 00:32:31 | 2015-06-15 22:13:43 | 2015-06-15 22:13:43 | Crosscheck of JCTVC-U0091, CE1-related: On the mismatch of SCM4.0 and HEVC SCC working draft | R. Cohen (MERL) |
| JCTVC-U0125 | m36406 | 2015-06-12 20:19:33 | 2015-06-16 01:09:17 | 2015-06-16 01:09:17 | Cross-verfication of JCTVC-U0093: Non-CE1: On Number of Palette Indices Coding | X. Xiu (InterDigital) |
| JCTVC-U0126 | m36408 | 2015-06-13 00:03:18 | 2015-06-17 05:15:19 | 2015-06-17 05:15:19 | Crosscheck report of JCTVC-U0090 | S.-H. Kim (Sharp) |
| JCTVC-U0127 | m36409 | 2015-06-13 02:05:40 | 2015-06-13 02:22:07 | 2015-06-23 12:24:19 | CE1: Additional results for CE1 subtest C | R. Joshi, M. Karczewicz, V. Seregin, W. Pu, F. Zou (Qualcomm) |
| JCTVC-U0128 | m36410 | 2015-06-13 04:03:16 | 2015-06-16 07:34:38 | 2015-06-22 11:38:28 | An HEVC SEI Message for Green Metadata | S. Cheng (Nanjing Yunyan Tech.), J. Wen (Tsinghua Univ.) |
| JCTVC-U0129 | m36411 | 2015-06-13 04:31:24 | 2015-06-19 10:07:34 | 2015-06-19 10:07:34 | Crosscheck for unification of motion vector resolution for screen content coding (JCTVC-U0076) | W. Zhang, L. Xu, Y. Chiu (Intel) |
| JCTVC-U0130 | m36412 | 2015-06-13 04:31:39 | 2015-06-19 10:30:43 | 2015-06-19 10:30:43 | Crosscheck for palette predictor initialization for Screen Content Coding (JCTVC-U0084) | W. Zhang, L. Xu, Y. Chiu (Intel) |
| JCTVC-U0131 | m36413 | 2015-06-13 04:31:51 | 2015-06-19 11:06:31 | 2015-06-19 11:06:31 | Crosscheck for simplification on coding numPaletteIndices (JCTVC-U0086) | W. Zhang, L. Xu, Y. Chiu (Intel) |
| JCTVC-U0132 | m36415 | 2015-06-13 19:03:32 | 2015-06-17 00:21:05 | 2015-06-20 09:28:59 | Target bits saturation to avoid CPB overflow and underflow under the constraint of HRD | Y.-J. Ahn, X. Wu, W. Lim (KWU), D. Sim (Digital Insights Co.) |
| JCTVC-U0133 | m36427 | 2015-06-15 07:27:42 | 2015-06-15 07:38:09 | 2015-06-15 07:38:09 | Comment on signalling the palette_transpose_flag after last_palette_run_type_flag (JCTVC-U0090) | R. Joshi, V. Seregin, M. Karczewicz, W. Pu, F. Zou (Qualcomm) |
| JCTVC-U0134 | m36434 | 2015-06-15 09:58:59 | 2015-06-15 10:09:55 | 2015-06-16 07:46:18 | Upsampling and downsampling filters for HDR content | A. K. Ramasubramonian, D. Rusanovskyy, J. Sole, M. Karczewicz, S. Lee, D. Bugdayci Sansli, J. Chen (Qualcomm) |
| JCTVC-U0135 | m36543 | 2015-06-15 18:41:11 | 2015-06-19 18:28:37 | 2015-06-19 18:28:37 | Crosscheck report of JCTVC-U0054 | J. Ye, S. Liu (MediaTek) |
| JCTVC-U0136 | m36546 | 2015-06-15 19:45:17 | 2015-06-17 07:40:02 | 2015-06-17 07:40:02 | Cross-checking of JCTVC-U0109: Non-CE1: Simplification for parsing the index of the most significant bit in the paletteRun binarization | Y. He, X. Xiu, Y. Ye (InterDigital) |
| JCTVC-U0137 | m36550 | 2015-06-15 20:06:14 | 2015-06-15 23:28:28 | 2015-06-15 23:28:28 | On the use of AVC and HEVC "unspecified" NAL unit types in file formats and systems | Y.-K. Wang (Qualcomm), D. Singer (Apple), G. J. Sullivan (Microsoft) |
| JCTVC-U0138 | m36622 | 2015-06-16 02:15:51 | 2015-06-16 04:19:12 | 2015-06-16 04:19:12 | Chroma Deblocking for Screen Content Coding | A. Tourapis, Y. Su, D. Singer (Apple), W. S. Kim, W. Pu, J. Sole, M. Karczewicz (Qualcomm) |
| JCTVC-U0139 | m36624 | 2015-06-16 06:25:16 | 2015-06-25 12:58:03 | 2015-06-25 12:58:03 | Cross check report of JCTVC-U0080: Intra block copy chroma interpolation for Non-444 | X. Xu, S. Liu (MediaTek) |
| JCTVC-U0140 | m36625 | 2015-06-16 06:27:14 | 2015-06-25 12:58:51 | 2015-06-25 12:58:51 | Cross check report of JCTVC-U0081: On unification of adapative motion vector resolution | X. Xu, S. Liu (MediaTek) |
| JCTVC-U0141 | m36638 | 2015-06-16 12:15:56 | 2015-06-20 13:13:25 | 2015-06-20 13:13:25 | Crosscheck of Generate the palette predictor initializers with low delay (JCTVC-U0114) | C. Gisquet (Canon) |
| JCTVC-U0142 | m36648 | 2015-06-16 19:31:30 | 2015-06-17 00:32:35 | 2015-06-17 00:32:35 | Inter/Intra Block Copy Unification: Comments and Observations | A.M. Tourapis, Y. Su, D. Singer (Apple) |
| JCTVC-U0143 | m36649 | 2015-06-16 19:57:11 | 2015-06-16 20:33:33 | 2015-06-16 20:33:33 | Crosscheck report of JCTVC-U0097 | S.-H. Kim (Sharp) |
| JCTVC-U0144 | m36654 | 2015-06-16 23:25:00 |  |  | Withdrawn |  |
| JCTVC-U0145 | m36655 | 2015-06-17 00:33:37 | 2015-06-21 14:38:27 | 2015-06-21 14:38:27 | Cross-verification of JCTVC-U0096: Improved palette encoder | X. Xiu (InterDigital) |
| JCTVC-U0146 | m36656 | 2015-06-17 01:31:44 | 2015-06-17 06:47:37 | 2015-06-19 10:24:11 | CE1 Related: Combination of JCTVC-U0101 reverse scan and JCTVC-U0096 palette encoder improvement | V. Seregin, F. Zou, M. Karczewicz, W. Pu, R. Joshi (Qualcomm), X. Xiu, Y. Ye, Y. He (InterDigital) |
| JCTVC-U0147 | m36657 | 2015-06-17 02:13:04 | 2015-06-20 21:15:22 | 2015-06-22 19:24:33 | Cross check CE1 Test A.2: Extended copy above mode to the first line with grouping the palette_run_type_flag in front (U0062) | W. Pu (Qualcomm) |
| JCTVC-U0148 | m36658 | 2015-06-17 03:29:06 | 2015-06-17 03:53:03 | 2015-06-17 03:53:03 | Non-CE1: Context model unification for palette run type flags in HEVC SCC | M. Xu, W. Wang, F. Duanmu, H. Yu (Huawei) |
| JCTVC-U0149 | m36659 | 2015-06-17 03:31:52 | 2015-06-17 03:53:34 | 2015-06-18 19:14:48 | Non-CE1: Index Map Splitting (IMS) Mode for Palette Coding in HEVC SCC | M. Xu, F. Duanmu, W. Wang, S. Minaee, H. Yu (Huawei) |
| JCTVC-U0150 | m36660 | 2015-06-17 03:33:13 | 2015-06-17 03:54:16 | 2015-06-17 03:54:16 | Withdrawn |  |
| JCTVC-U0151 | m36661 | 2015-06-17 03:35:01 | 2015-06-21 09:51:57 | 2015-06-21 09:51:57 | Withdrawn |  |
| JCTVC-U0152 | m36677 | 2015-06-18 10:45:12 | 2015-06-18 10:47:21 | 2015-06-25 06:52:13 | R-lambda model based rate control with pre-encoding process | M. Fang, M. Tang, J. Wen, Z. Wen (Tsinghua Univ.) |
| JCTVC-U0153 | m36688 | 2015-06-18 20:11:44 | 2015-06-20 20:50:07 | 2015-06-23 19:10:23 | Cross check: On the palette escape pixel coding (U0052) | W. Pu (Qualcomm) |
| JCTVC-U0154 | m36689 | 2015-06-18 20:54:22 | 2015-06-20 18:58:12 | 2015-06-23 18:55:43 | Cross check CE1-related: Colour-plane-based escape pixel coding (U0063) | W. Pu (Qualcomm) |
| JCTVC-U0155 | m36694 | 2015-06-19 00:24:00 | 2015-06-19 04:38:02 | 2015-06-19 04:38:02 | Crosscheck of JCTVC-U0105: Non-CE1: Simplification on deriving NumPaletteIndices for palette mode | W. Digonnet, C.-C. Lin, J.-S. Tu, Y.-J. Chang, C.-L. Lin (ITRI) |
| JCTVC-U0156 | m36698 | 2015-06-19 02:29:08 | 2015-06-19 02:46:24 | 2015-06-19 02:46:24 | On 4:2:0 chroma positioning for Rec. ITU-R BT.2020 | C. Fogg (MovieLabs), G. J. Sullivan (Microsoft), A. M. Tourapis (Apple) |
| JCTVC-U0157 | m36700 | 2015-06-19 03:15:08 |  |  | Withdrawn |  |
| JCTVC-U0158 | m36711 | 2015-06-19 04:23:22 | 2015-06-24 05:14:22 | 2015-06-24 05:14:22 | Cross-check of JCTVC-U0103 on chroma derivation of intra block copy for non-444 video | K. Rapaka (Qualcomm) |
| JCTVC-U0159 | m36712 | 2015-06-19 04:25:21 | 2015-06-24 06:43:56 | 2015-06-24 06:43:56 | Cross-check of JCTVC-U0056 on unification of the Adaptive Motion Resolution signalling for both IBC and Inter modes | K. Rapaka (Qualcomm) |
| JCTVC-U0160 | m36713 | 2015-06-19 04:25:43 | 2015-06-25 16:40:58 | 2015-06-25 16:40:58 | Cross-check of JCTVC-U0057 on Inter modes constraint for slices | K. Rapaka (Qualcomm) |
| JCTVC-U0161 | m36714 | 2015-06-19 04:27:22 | 2015-06-25 15:45:54 | 2015-06-25 15:45:54 | Cross-check of JCTVC-U0111 on Non-CE: MV resolution unification for MV derivation in Intra-Block Copy search | K. Rapaka (Qualcomm) |
| JCTVC-U0162 | m36717 | 2015-06-19 06:58:12 | 2015-06-19 07:59:03 | 2015-06-19 07:59:03 | Cross-check of CE1-related: Palette coding with inter-prediction (JCTVC-U0064) | B. Li, J. Xu (Microsoft) |
| JCTVC-U0163 | m36718 | 2015-06-19 06:58:25 | 2015-06-24 03:56:11 | 2015-06-24 03:56:11 | Cross-check of Non-CE1: Palette Run Hiding (JCTVC-U0094) | B. Li, J. Xu (Microsoft) |
| JCTVC-U0164 | m36719 | 2015-06-19 06:58:32 | 2015-06-23 07:50:05 | 2015-06-23 09:31:56 | Cross-check of encoder modification for intra prediction mode using adaptive colour transform (JCTVC-U0095) | B. Li, J. Xu (Microsoft) |
| JCTVC-U0165 | m36720 | 2015-06-19 06:58:40 | 2015-06-19 08:00:17 | 2015-06-19 08:00:17 | Cross-check of on signaling adaptive color transform at TU level (JCTVC-U0106) | B. Li, J. Xu (Microsoft) |
| JCTVC-U0166 | m36721 | 2015-06-19 06:58:47 | 2015-06-19 08:17:10 | 2015-06-19 08:17:10 | Cross-check of Non-CE1: Context model unification for palette run type flags in HEVC SCC (JCTVC-U0148) | B. Li, J. Xu (Microsoft) |
| JCTVC-U0167 | m36722 | 2015-06-19 06:58:58 | 2015-06-23 03:30:25 | 2015-06-23 09:33:01 | Cross-check of Non-CE1: Index Map Splitting (IMS) Mode for Palette Coding in HEVC SCC (JCTVC-U0149) | B. Li, J. Xu (Microsoft) |
| JCTVC-U0168 | m36727 | 2015-06-19 10:33:16 | 2015-06-20 14:42:19 | 2015-06-20 14:42:19 | Crosscheck of JCTVC-U0066: CE1-related: Row-based copy pixel from neighbouring CU | F. Zou (Qualcomm) |
| JCTVC-U0169 | m36732 | 2015-06-19 13:34:32 | 2015-06-19 14:18:52 | 2015-06-19 14:18:52 | CE1 Related: Simplification of num_palette_indices_idc coding parameter derivation | F. Zou, V. Seregin, M. Karczewicz, W. Pu, R. Joshi (Qualcomm) |
| JCTVC-U0170 | m36734 | 2015-06-19 14:27:21 |  |  | Withdrawn |  |
| JCTVC-U0171 | m36751 | 2015-06-20 08:15:29 | 2015-06-20 08:23:01 | 2015-06-20 08:23:01 | Non-CE1: Palette with Pixel Copy (PPC) decoder IC reference design | Kailun Zhou, Tao Lin (Tongji) |
| JCTVC-U0172 | m36752 | 2015-06-20 08:17:33 | 2015-06-20 08:23:25 | 2015-06-20 08:23:25 | Performance gain of IBC FF and loss of IBC FF vs 4CTU | Xianyi Chen, Tao Lin (Tongji) |
| JCTVC-U0173 | m36756 | 2015-06-20 11:09:31 | 2015-06-20 19:07:11 | 2015-06-20 19:07:11 | Crosscheck of Non-CE1: Enhancement to Palette Coding by Palette with Pixel Copy (PPC) Coding (JCTVC-U0116) | R.-L. Liao, C.-C. Chen, W.H. Peng, H.-M. Hang (NCTU/ITRI) |
| JCTVC-U0174 | m36761 | 2015-06-20 21:22:08 | 2015-06-24 18:46:27 | 2015-06-24 18:46:27 | Withdrawn |  |
| JCTVC-U0175 | m36780 | 2015-06-21 17:34:33 | 2015-06-21 19:30:18 | 2015-06-24 12:07:48 | BoG report on worst case memory bandwidth assessment | K. Rapaka |
| JCTVC-U0176 | m36785 | 2015-06-21 20:55:47 | 2015-06-21 21:46:08 | 2015-06-24 17:30:42 | Non-CE1: Joint proposal of JCTVC-U0169 and JCTVC-U0090 Item 3 | F. Zou, M. Karczewicz, V. Seregin, W. Pu, R. Joshi (Qualcomm), J. Ye, J. Kim, S. Liu, P. Lai, W. Zhu, K. Zhang, T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-U0177 | m36820 | 2015-06-22 09:53:44 | 2015-06-25 11:01:58 | 2015-06-25 11:01:58 | Crosscheck for the Joint proposal of JCTVC-U0169 and JCTVC-U0090 Item 3 (JCTVC-U0176) | W. Zhang (Intel) |
| JCTVC-U0178 | m36827 | 2015-06-22 18:05:09 | 2015-06-22 18:08:34 | 2015-06-22 18:08:34 | BoG report on constrained intra prediction for IBC unification | X. Xiu |
| JCTVC-U0179 | m36828 | 2015-06-22 18:05:42 |  |  | Withdrawn |  |
| JCTVC-U0180 | m36829 | 2015-06-22 20:34:03 | 2015-06-22 20:38:47 | 2015-06-25 15:35:45 | Combination of JCTVC-U0118 method 2.2 and JCTVC-U0113 method 1 | K. Rapaka, V. Seregin, R. Joshi, M. Karczewicz, Y. -K. Wang(Qualcomm), X. Xu, S. Liu, S. Lei(MediaTek) |
| JCTVC-U0181 | m36830 | 2015-06-22 20:36:41 | 2015-06-23 13:27:29 | 2015-06-23 13:27:29 | On storage of filtered and unfiltered current decoded pictures | X. Xu, S. Liu, C. Liu, S. Lei (MediaTek), Y.-K. Wang, K. Rapaka, V. Seregin (Qualcomm) |
| JCTVC-U0182 | m36832 | 2015-06-23 07:19:00 | 2015-06-23 12:07:01 | 2015-06-23 12:07:01 | Non-CE1: Further harmonization of PPC with no-pixel-copy palette coding | Tao Lin, Kailun Zhou, Liping Zhao, Xianyi Chen (Tongji) |
| JCTVC-U0183 | m36839 | 2015-06-23 15:10:03 | 2015-06-23 16:21:59 | 2015-06-23 16:21:59 | Visualisation of BD-rate interpolation curves for MOS and PSNR results | A. S. Dias, M. Mrak (BBC) |
| JCTVC-U0184 | m36840 | 2015-06-23 17:47:48 | 2015-06-23 18:30:11 | 2015-06-24 15:23:47 | Interlaced video and format range extensions verification test results | Guillaume Barroux, Chris Rosewarne, Vittorio Baroncini |
| JCTVC-U0185 | m36844 | 2015-06-24 03:00:23 | 2015-06-24 10:48:04 | 2015-06-24 10:48:04 | Comments on intra block copy with interpolation filters | M. Zhou, T. Hellman (Broadcom) |
| JCTVC-U0186 | m36845 | 2015-06-24 11:36:30 | 2015-06-24 11:41:53 | 2015-06-25 08:47:11 | Flexible coding tools to significantly improve SCC performance in cloud and mobile computing | Liping Zhao, Wenting Cai, Jing Guo, Tao Lin (Tongji) |
| JCTVC-U0187 | m36850 | 2015-06-25 06:14:40 | 2015-06-25 09:19:45 | 2015-06-25 12:30:18 | Cross-check report of JCTVC-U0152: R-lambda model based rate control with pre-encoding process | X. Xu, S. Liu (MediaTek) |
| JCTVC-U0188 | m36852 | 2015-06-25 15:11:56 | 2015-06-25 15:18:14 | 2015-06-25 15:18:14 | New SCC test sequence for consideration | W. Wang, M. Xu, F. Duanmu, H. Yu (Huawei) |
| JCTVC-U0189 | m36853 | 2015-06-25 15:13:58 | 2015-06-25 15:23:20 | 2015-06-25 15:23:20 | Cross-check report of U0116: Enhancement to Palette Coding by Palette with Pixel Copy (PPC) Coding | W. Wang, M. Xu (Huawei) |
| JCTVC-U0190 | m37033 | 2015-10-07 10:55:43 |  |  | Overview of the High Efficiency Image File Format | M. M. Hannuksela, E. B. Aksu, V. K. Malamal Vadakital, J. Lainema (Nokia) |
| JCTVC-U1000 | m36866 | 2015-06-29 10:45:59 |  |  | Meeting Report of the 21st JCT-VC Meeting | G. J. Sullivan, J.-R. Ohm (JCT-VC chairs) |
| JCTVC-U1002 | m36867 | 2015-06-29 10:58:27 |  |  | High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Update 3 of Encoder Description | C. Rosewarne (primary editor), B. Bross, M. Naccari, K. Sharman, G. J. Sullivan (co-editors) |
| JCTVC-U1003 | m36868 | 2015-06-29 11:03:16 | 2015-08-03 14:56:45 | 2015-08-04 04:47:40 | HEVC interlaced video and format range extensions verification test report | C. Rosewarne, V. Baroncini, G. Barroux, G. J. Sullivan, A. M. Tourapis, M. Naccari (editors) |
| JCTVC-U1004 | m36869 | 2015-06-29 11:09:00 |  |  | Draft verification test plan for scalable HEVC (SHVC) profiles | Y. Ye, V. Baroncini, Y.-K. Wang (editors) |
| JCTVC-U1005 | m36870 | 2015-06-29 11:12:34 | 2015-09-05 03:42:47 | 2015-09-05 03:43:30 | HEVC Screen Content Coding Draft Text 4 | R. Joshi, S. Liu, G. J. Sullivan, J. Xu, Y. Ye (editors) |
| JCTVC-U1007 | m36871 | 2015-06-29 12:32:35 |  |  | SHVC Test Model 10 (SHM 10) Introduction and Encoder Description | J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela (editors) |
| JCTVC-U1008 | m36872 | 2015-06-29 12:33:50 | 2015-09-25 22:50:07 | 2015-09-25 22:50:07 | SHVC Conformance Testing Draft 3 | J. Boyce, A. K. Ramasubramonian (editors) |
| JCTVC-U1011 | m36873 | 2015-06-29 12:38:47 | 2015-09-22 11:33:24 | 2015-09-22 11:33:24 | Reference Software for HEVC Format Range Extensions Draft 3 | K. Sharman, D. Flynn, K. SÃ¼hring, T. Suzuki (editors) |
| JCTVC-U1012 | m36874 | 2015-06-29 12:47:58 | 2015-09-08 04:40:35 | 2015-09-08 04:40:35 | Conformance Testing for Improved HEVC Version 1 Testing and Format Range Extensions Profiles Draft 5 | T. Suzuki, K. Kazui (editors) |
| JCTVC-U1013 | m36875 | 2015-06-29 12:51:41 | 2015-08-27 10:59:58 | 2015-08-27 10:59:58 | Reference software for Scalable HEVC (SHVC) Extensions Draft 2 | Y. He, V. Seregin (editors) |
| JCTVC-U1014 | m36876 | 2015-06-29 12:54:36 |  |  | Screen Content Coding Test Model 5 Encoder Description (SCM 5) | R. Joshi, J. Xu, R. Cohen, S. Liu, Y. Ye (editors) |
| JCTVC-U1015 | m36877 | 2015-06-29 12:57:04 | 2015-07-17 18:24:41 | 2015-09-18 17:03:16 | Common Test Conditions for Screen Content Coding | H. Yu, R. Cohen, K. Rapaka, J. Xu (editors) |
| JCTVC-U1101 | m36878 | 2015-06-29 12:58:55 | 2015-06-30 00:39:06 | 2015-06-30 00:39:06 | Description of Core Experiment 1 (CE1): Chroma deblocking filtering | A. Tourapis, K. Rapaka, X. Xiu (CE coordinators) |

# Annex B to JCT-VC report:
List of meeting participants

The participants of the twenty-first meeting of the JCT-VC, according to a sign-in sheet circulated during the meeting sessions (approximately 91 people in total), were as follows:

- Yongjo Ahn (Kwangwoon Univ.)

- Elena Alshina (Samsung Electronics)

- Peter Amon (Siemens AG)

- Cheung Auyeung (Sony)

- Vittorio Baroncini (Fondazione Ugo Bordoni)

- Guillaume Barroux (Fujitsu)

- Jill Boyce (Intel)

- Yao-Jen Chang (ITRI Intl.)

- Jianle Chen (Qualcomm)

- Peisong Chen (Broadcom)

- Keiichi Chono (NEC)

- Tzu-Der Chuang (MediaTek)

- Takeshi Chujoh (Toshiba)

- Marek Domanski (Poznań Univ. Tech.)

- Chad Fogg (MovieLabs)

- Christophe Gisquet (Canon Research France)

- Dan Grois (Fraunhofer HHI)

- Miska Hannuksela (Nokia)

- Ryoji Hashimoto (Renesas)

- Félix Henry (Orange Labs)

- Ted Hsieh (Qualcomm Tech.)

- Yu-Wen Huang (MediaTek)

- Atsuro Ichigaya (NHK (Japan Broadcasting Corp.))

- Tomohiro Ikai (Sharp)

- Sergey Ikonin (Huawei)

- Shunsuke Iwamura (NHK (Japan Broadcasting Corp.))

- Wonkap Jang (Vidyo)

- Marcin Jasinski (Imagination Tech.)

- Minyoung Jeon (Chips & Media)

- Rajan Joshi (Qualcomm)

- Dongsan Jun (ETRI)

- Joël Jung (Orange Labs)

- Jung Won Kang (ETRI (Electronics and Telecommunications Research Institute))

- Marta Karczewicz (Qualcomm Tech.)

- Kei Kawamura (KDDI)

- Kimihiko Kazui (Fujitsu Labs)

- Seung Hwan Kim (Sharp)

- Phillipe Laffont (ST Microelectronics)

- PoLin (Wang) Lai (Mediatek USA)

- Jani Lainema (Nokia)

- Thorsten Laude (Leibniz Univ. Hannover)

- Bae-Keun Lee (KT)

- Jinho Lee (ETRI (Electronics and Telecommunications Research Institute))

- Sungwon Lee (Qualcomm)

- Shawmin Lei (MediaTek)

- Chong Soon Lim (Panasonic)

- Jeong Yun Lim (KWU)

- Jiunn Bin Lim (Huawei)

- Woong Lim (KWU (Kwangwoon Univ.)

- Ching-Chieh Lin (ITRI Intl.)

- Tao Lin (Tongji Univ.)

- Shan Liu (MediaTek USA)

- Akira Minezawa (Mitsubishi Electric)

- Ohji Nakagami (Sony)

- Yukihiro Nishida (NHK (Japan broadacsting Corp.))

- Jens-Rainer Ohm (RWTH Aachen Univ.)

- Manindra Parhy (Nvidia)

- Dong-Jin Park (Chips & Media)

- Seanae Park (KWU)

- Krishnakanth Rapaka (Qualcomm Tech.)

- Justin Ridge (Nokia)

- Christopher Rosewarne (CiSRA / Canon Inc)

- Jesus Sampedro (Polycom)

- Nicholas Saunders (Sony Europe Broad. & Prof. Research Labs)

- Vadim Seregin (Qualcomm)

- Karl Sharman (Sony Europe Broad. & Prof. Research Labs)

- Masato Shima (Canon)

- Gary Sullivan (Microsoft)

- Teruhiko Suzuki (Sony)

- Maxim Sychev (Huawei Tech.)

- Seishi Takamura (NTT)

- Kenshiro Takenchi (Fujitsu Labs)

- Han Boon Teo (Panasonic)

- Alexandros Tourapis (Apple)

- Yi-Shin Tung (ITRI Intl./ MStar Semi.)

- Wade Wan (Broadcom)

- Wei Wang (Huawei)

- Ye-Kui Wang (Qualcomm)

- Ziyu Wen (Tsinghua Univ.)

- Stephan Wenger (Vidyo)

- Dominik Wojt (Imagination Tech.)

- Ping Wu (ZTE UK)

- Xiaoyu Xiu (InterDigital Commun.)

- Jizheng Xu (Microsoft)

- Xiaozhong Xu (MediaTek)

- Jar-Ferr (Kevin) Yang (ITRI Intl.)

- Yan Ye (InterDigital Commun.)

- Haoping Yu (Futurewei (Huawei R&D USA))

- Minhua Zhou (Broadcom)

- Jian Qing Zhu (Fujitsu R&D Center)

- Feng Zou (Qualcomm Tech.)

The definitions of PB and PU are tricky for a 64x64 intra luma CB when the prediction control information is sent at the 64x64 level but the prediction operation is performed on 32x32 blocks. The PB, PU, TB and TU definitions are also tricky in relation to chroma for the smallest block sizes with the 4:2:0 and 4:2:2 chroma formats. Double-checking of these definitions is encouraged.