| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U1008 supplemental notes |
| --- | --- |

| Title: | SHVC Conformance Testing Draft 3 Supplemental Notes |  |  |
| --- | --- | --- | --- |
| Status: | Output Document from JCT-VC |  |  |
| Purpose: | Supplemental information to accompany draft text |  |  |
| Author(s) or Contact(s): | Jill Boyce Adarsh K. Ramasubramonian | Tel: Email: | jill.boyce@intel.com aramasub@qti.qualcomm.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document contains supplemental information to accompany the proposed draft 3 text for SHVC conformance testing. It duplicates some of the content of the accompanying draft text, but also provides some additional information, such as an identification of the SHM version used to produce various bitstreams and revision marks identifying some recent changes. In case of any conflict with the accompanying draft text, the other document should be considered the official output draft and this document should be considered only as supplemental and relatively informal. Please note that the parts of this document that appear to match the content of the other document are not necessarily an exact match, as the other document contains some intentional refinements.

# Conformance testing for SHVC

In an SHVC bitstream, conformance is expressed for each output layer set, through the signalling of a profile_tier_level( ) syntax structure associated with each necessary layer in the output layer set.  A decoder that conforms to a list of profile, tier, level, and INBLD capabilities adequate for each necessary layer of the output layer set (as described in subclause F.11.2 Decoder capabilities) must correctly decode all pictures of the necessary layers of the output layer set in order to correctly output all output pictures of target output layers of the output layer set.

To test correct decoding, per layer MD5 sum files are provided with each conformance bitstream, regardless of whether or not a layer is included within an output layer set. However, decoder conformance is only required for layers included within an output layer set, for which an associated profile_tier_level( ) syntax structure is present in the bitstream.

Correct output of individual pictures from only the target output layers in an output layer set, in the correct output order, is also a requirement of conformance. The output picture log files provided for each output layer set can be used to verify the presence and order of the appropriate output pictures of an output layer set.

The conformance bitstreams are publicly available at the following location:

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/draft_SHVC_conformance/

## Conformance bitstream summary table

The table below lists the bitstream features, the individual and company who have volunteered to provide bitstreams exercising the feature, availability status, and the # of bitstreams provided.

|  |  |  | Available | # | LAYER DEPENDENCIES |
| --- | --- | --- | --- | --- | --- |
| Nokia | Antti Hallapuro | LAYERID | Y | 1 | gaps in layer id |
| InterDigital | Yan Ye | MVD | Y | 1 | no MV dependency |
| Nokia | Antti Hallapuro | MVD | Y | 1 | varying sample, MV dependencies |
| ETRI | Hahyun Lee | MAXTID | Y | 3 | max_tid |
| Qualcomm | Adarsh Ramasubramonian | INACTIVE | Y | 1 | inactive ref layers |
| Vidyo | Jill Boyce | REFLAYER | Y | 4 | multiple active ref layers with various inter layer predictors and dependencies |
|  |  |  |  |  | VPS SYNTAX |
| HHI | Robert Skupin | SPLITFLAG | Y | 1 | splitting_flag |
| Qualcomm | Hendry | VUI | Y | 3 | VUI present |
| Qualcomm | Hendry | NONVUI | Y | 3 | non VUI extension |
|  |  |  |  |  | DPB |
| NEEDED | NEEDED | DPB | N |  | sub layer info |
| Vidyo | Won Kap Jang | DPB | Y |  | sub layer dpb info |
|  |  |  |  |  | RESOLUTIONS |
| Samsung | Elena Alshina | SRATIOS | Y | 2 | 2 spatial layers w/ unusual ratios |
| InterDigital | Yan Ye | SNR | Y | 3 | 2, 3 SNR layers |
| Vidyo | Jill Boyce | REPFMT | Y | 3 | VPS rep formats |
| Vidyo | Jill Boyce | RESCHANGE | Y | 1 | resolution change in SPS |
| Ericsson | Rickard Sjoberg | ADAPTRES | Y | 1 | adaptive resolution |
| Sony | Ohji Nakagami | SPSREPFMT | Y | 1 | SPS rep format idx |
| Vidyo | Jill Boyce | CONFCROP | Y | 3 | conformance cropping window in VPS |
|  |  |  |  |  | OFFSETS AND PHASE ADJUSTMENTS |
| Qualcomm | Adarsh Ramasubramonian | SCREFOFF | Y | 1 | scaled ref layer offsets |
| Sharp | Tomoyuki Yamamoto | REFREGOFF | Y | 1 | ref region offsets |
| Samsung | Elena Alshina | RESPHASE | Y | 1 | resample phase |
|  |  |  |  |  | OUTPUT LAYERS, PICS |
| Nokia | Antti Hallapuro | OLS | Y | 1 | add output layer sets, non-default target output layer sets, unnecessary layers |
| Qualcomm | Hendry | DISFLAG | Y | 1 | discardable flag |
|  |  |  |  |  | SCALING LIST |
| Sony | Ohji Nakagami | PPSSLIST | Y | 1 | SPS scaling list infer |
| Sony | Ohji Nakagami | SPSSLIST | Y | 1 | PPS scaling list infer |
|  |  |  |  |  | COLOR GAMUT SCALABILITY |
| Technicolor | Franck Hiron | CGS | Y | 9 | Colour gamut scalability 8-to-10 bits, 10-to-10 bits, spatial ratio change with CGS |
|  |  |  |  |  | ADDITIONAL EXTENSIBILITY |
| Vidyo | Jill Boyce | PSEXT | Y | 1 | SPS, PPS additional extension |
| HHI | Karsten Suehring | LAYERID63 | Y | 1 | NUHs with layer_id of 63 |
|  |  |  |  |  | POC |
| Ericsson | Rickard Sjoberg | POC | N |  | unaligned POC |
|  |  |  |  |  | BASE LAYER TYPE |
| Qualcomm | Hendry | HYBRID | N |  | hybrid scalability |
| Nokia | Antti Hallapuro | INBLD | Y | 1 | base layer not available (INBLD) |
| InterDigital | Yan Ye | SIM | Y | 2 | simulcast |
|  |  |  |  |  | LEVEL SIGNALING |
| Vidyo | Jill Boyce | SLLEV | Y | 1 | sub_layer level signaling |
|  |  |  |  |  | AUX PICS |
| BBC | Matteo Naccari | ALPHA | Y | 1 | Alpha |
| Nokia | Antti Hallapuro | DEPTH | Y | 1 | Depth |
| Disney | Nikolce Stefanoski | OVERLAY | N |  | Overlay |
|  |  |  |  |  | LAYERS |
| NEEDED | NEEDED | LAYERS | N |  | 8 layers |

## Specification of the test bitstreams

### Test bitstreams – LAYERID

#### Test bitstreams #LAYERID_A_NOKIA

Bitstream file name: LAYERID_A_NOKIA_1.bit

Bitstream feature name: Gaps in layer ID

Bitstream feature description:

The bitstream has two layers with IDs 0 and 2. Layer 2 is coded as an auxiliary layer (AuxId = 2). Information related to layer 1 is not coded in VPS.

Coding structure: Hierarchical B-frames with GOP size of 8

Number of Frames: 9

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 2, PTL Idx: 2 (Main 4)

Number of layers: 2

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: MAIN [Auxiliary layer PTL]

Level: 4

Each layer resolution: 1280x720

Frame rate: 24 fps

Software used:

- HTM encoder 13.0 with a bug fix to layer ID mapping and pps_scaling_list_ref_layer_id coding

- SHM-dev decoder r1015 with bug fixes to layer ID mapping and baseLayerPSCompatibilityFlag default values

### Test bitstreams – MVD

#### Test bitstreams #MVD_A_IDCC

Bitstream file name: MVD_A_IDCC_1.bit

Bitstream feature name: No MV dependency

Bitstream feature description:

Inter-layer motion prediction is disabled when coding the enhancement layer motion vectors

Number of layers: 2

- Layer 1	: 	NumSamplePredRefLayers1 = 1

- Layer 1	: 	SamplePredRefLayerIds1 = 0

- Layer 1	: 	NumMotionPredRefLayers1 = 0

- Layer 1	: 	MotionPredRefLayerIds1 = 0

- Layer 1	: 	NumActiveRefLayers1 = 1

- Layer 1	: 	PredLayerIds1 = 0

Coding structure: Random Access with 4 temporal sub-layers

Number of Frames: 8

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

- DefaultTargetOutputLayerIdc   : 1

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution:

- Layer 0 resolution - Output: 960x540

Coded:  960x544

- Layer 1 resolution - Output: 1920x1080

Coded:  1920x1280

Frame rate: 24 fps for both layers

SHM Version: SHM Dev branch, rev 1021

#### Test bitstreams #MVD_A_NOKIA

Bitstream file name: MVD_A_NOKIA_1.bit

Bitstream feature name: Varying sample, MV dependencies

Bitstream feature description:

Sample prediction and motion vector prediction are performed from different layers.

- Layer 1 - predict both samples and motion vectors from layer 0

- Layer 2 - predict samples from layer 1 and motion vectors from layer 0

Coding structure: Low Delay P

Number of Frames: 4

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

- OLS_2 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

Layer: 2, PTL Idx: 2 (Scalable Main 4)

Number of layers: 3

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution: 1280x720

Frame rate: 24 fps

SHM Version: SHM Dev branch, rev 1015

### Test bitstreams – MAXTID

#### Test bitstreams #MAXTID_A_ETRI

Bitstream file name: MAXTID_A_ETRI_1.bit

Bitstream feature name: max_tid_il_ref_pics_plus1 in VPS extension

Bitstream feature description:

The max_tid_il_ref_pics_plus1 in VPS extension are used to support controlling the use of inter-layer prediction based on temporal sub-layer

- max_tid_ref_presenet_flag = 1

- Layer 0 : max_tid_il_ref_pics_plus1[0][1] = 3

- Layer 1 : max_tid_il_ref_pics_plus1[1][2] = 2

Coding structure: Random Acess with 4 temporal sub-layers

Number of Frames: 9

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

- OLS_2 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

Layer: 2, PTL Idx: 2 (Scalable Main 4)

Number of layers: 3

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution:

- Layer 0 resolution - Output:  960x540

Coded:  960x544

- Layer 1 resolution - Coded:  1920x1080

- Layer 2 resolution - Coded:  1920x1080

Frame rate: 24 fps for all layers

SHM Version: SHM Dev branch, rev 1025

#### Test bitstreams #MAXTID_B_ETRI

Bitstream file name: MAXTID_B_ETRI_1.bit

Bitstream feature name: max_tid_il_ref_pics_plus1 in VPS extension

Bitstream feature description:

The max_tid_il_ref_pics_plus1 in VPS extension are used to support controlling the use of inter-layer prediction based on temporal sub-layer

- max_tid_ref_presenet_flag = 0

Coding structure: Random Acess with 4 temporal sub-layers

Number of Frames: 9

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

- OLS_2 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

Layer: 2, PTL Idx: 2 (Scalable Main 4)

Number of layers: 3

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution:

- Layer 0 resolution - Output: 960x540

Coded:  960x544

- Layer 1 resolution - Coded:  1920x1080

- Layer 2 resolution - Coded:  1920x1080

Frame rate: 24 fps for all layers

SHM Version: SHM Dev branch, rev 1025

#### Bitstream MAXTID_C_ETRI

Bitstream file name: MAXTID_C_ETRI_1.bit

Bitstream feature name: max_tid_il_ref_pics_plus1 in VPS extension

Bitstream feature description:

The max_tid_il_ref_pics_plus1 in VPS extension are used to support controlling the use of inter-layer prediction based on temporal sub-layer

- max_tid_ref_presenet_flag = 1

- Layer 0 : max_tid_il_ref_pics_plus1[0][1] = 0

- Layer 1 : max_tid_il_ref_pics_plus1[1][2] = 0

Coding structure: Random Acess with 4 temporal sub-layers

Number of Frames: 9

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

- OLS_2 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

Layer: 2, PTL Idx: 2 (Scalable Main 4)

Number of layers: 3

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution:

- Layer 0 resolution - Output: 960x540

Coded:  960x544

- Layer 1 resolution - Coded:  1920x1080

- Layer 2 resolution - Coded:  1920x1080

Frame rate: 24 fps for all layers

SHM Version: SHM Dev branch, rev 1025

### Test bitstreams - INACTIVE

#### Test bitstreams #INACTIVE_A_QCOM

Bitstream file name:

INACTIVE_A_QCOM_1.txt

Bitstream feature name:

Setting default_ref_layers_active_flag to zero

Explanation of bitstream features:

This bitstream is coded using the RA configuration, with three layers; layer 0 is a direct reference layer of layer 1, and layers 0 and 1 are direct reference layers of layer 2. The value of default_ref_layers_active_flag is set equal to 0 in the VPS, and hence the syntax elements inter_layer_pred_enabled_flag, num_inter_layer_ref_pics_minus1, and inter_layer_pred_layer_idc[ i ] are signalled in the slice segment header. The layers are SNR scalable layers, and all layers have the same resolution.

Number of output layer sets in the bitstream:

3

Number of layers present in the bitstream:

3

Profile, tier, and level (P,T,L) for each layer in each output layer set in the bitstream:

OLS 0 - L0: (main, main, 2.1)

OLS 1 - L0: (main, main, 2.1)

L1: (scalable-main, main, 2.1)

OLS 2 - L0: (main, main, 2.1)

L1: (scalable-main, main, 2.1)

L2: (scalable-main, main, 2.1)

Picture size of each layer:

L0: 416 x 240

L1: 416 x 240

L2: 416 x 240

Frame rate of each layer:

L0: 50 fps

L1: 50 fps

L2: 50 fps

SHM version number:

SHM-dev branch, r1025

Number of frames:

100

### Test bitstreams – REFLAYER

#### Test bitstreams #REFLAYER_A_VIDYO

Bitstream file name: REFLAYER_A_VIDYO_1.bit

Bitstream feature name: Multiple active ref layers with various inter layer predictors

Bitstream feature description:

In this bitstream, NumDirectRefLayers=2 for Layer 2. Various frames may use either Layer 0 or Layer 1 for sample prediction. All frames always may only use Layer 1 for motion prediction.

Some important params in the VPS Extension are as below:

- Layer 1 (i = 1):

direct_dependency_flag[ i ][ j ]: 1, i = 1, j = 0

NumDirectRefLayers = 1

- Layer 2 (i = 2):

direct_dependency_flag[ i ][ j ]: 1, i = 2, j = 0

direct_dependency_flag[ i ][ j ]: 1, i = 2, j = 1

NumDirectRefLayers = 2

- direct_dependency_all_layers_flag: 0

- Layer 1 (i = 1)

direct_dependency_type[ i ][ j ]: 2, i = 1, j = 0 (inter-layer sample prediction & inter-layer motion prediction)

- Layer 2 (i = 2)

direct_dependency_type[ i ][ j ]: 0, i = 2, j = 0 (inter-layer sample prediction only)

direct_dependency_type[ i ][ j ]: 2, i = 2, j = 1 (inter-layer sample prediction & inter-layer motion prediction)

Some important params in the Slice segment header are as below:

Frame 0 (POC id = 0), Frame 1 (POC id = 1), Frame 3 (POC id = 3), Frame 5 (POC id = 5), Frame 7 (POC id = 7)

- Layer 1: NumDirectRefLayers = 1

- Layer 2: NumDirectRefLayers = 2

num_inter_layer_ref_pics_minus1: 0 (+1 = 1)

inter_layer_pred_layer_idc[ i ]: 1, i = 0 (Layer 1 is used for prediction of Layer 2)

Frame 2 (POC id = 2), Frame 4 (POC id = 4), Frame 6 (POC id = 6), Frame 8 (POC id = 8)

- Layer 1: NumDirectRefLayers = 1

- Layer 2: NumDirectRefLayers = 2

num_inter_layer_ref_pics_minus1: 0 (+1 = 1)

inter_layer_pred_layer_idc[ i ]: 0, i = 0 (Layer 0 is used for prediction of Layer 2)

Number of Frames: 9

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3)

Layer: 1, PTL Idx: 2 (Scalable Main 3.1)

- OLS_2 -   Layer: 0, PTL Idx: 1 (Main 3)

Layer: 1, PTL Idx: 2 (Scalable Main 3.1)

Layer: 2, PTL Idx: 3 (Scalable Main 4)

Number of layers: 3

Profile, Tier, Level information: Num PTL = 4

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 3.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 3.1

- PTL_Idx 3 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution:

- Layer 0 resolution - Output: 960x540

Coded:  960x544

- Layer 1 resolution - Output: 960X540

Coded:  960x544

- Layer 2 resolution - Output: 1920x1080

Coded:  1920x1080

Frame rate: 24 fps for both layers

SHM Version: This bitstream is generated using SHM rev 1084

#### Test bitstreams #REFLAYER_B_VIDYO

Bitstream file name: REFLAYER_B_VIDYO_1.bit

Bitstream feature name: Multiple active ref layers with various inter layer predictors

Bitstream feature description:

In this bitstream, NumDirectRefLayers=2 for Layer 2. Various frames may use either Layer 0 or Layer 1 for sample prediction. All frames always may only use Layer 1 for motion prediction.

Some important params in the VPS Extension are as below:

- Layer 1 (i = 1):

direct_dependency_flag[ i ][ j ]: 1, i = 1, j = 0

NumDirectRefLayers = 1

- Layer 2 (i = 2):

direct_dependency_flag[ i ][ j ]: 1, i = 2, j = 0

direct_dependency_flag[ i ][ j ]: 1, i = 2, j = 1

NumDirectRefLayers = 2

- direct_dependency_all_layers_flag: 0

- Layer 1 (i = 1)

direct_dependency_type[ i ][ j ]: 2, i = 1, j = 0 (inter-layer sample prediction & inter-layer motion prediction)

- Layer 2 (i = 2)

direct_dependency_type[ i ][ j ]: 0, i = 2, j = 0 (inter-layer sample prediction only)

direct_dependency_type[ i ][ j ]: 2, i = 2, j = 1 (inter-layer sample prediction & inter-layer motion prediction)

Some important params in the Slice segment header are as below:

For all frames in the bitstream

- Layer 1: NumDirectRefLayers = 1

- Layer 2: NumDirectRefLayers = 2

num_inter_layer_ref_pics_minus1: 0 (+1 = 1)

inter_layer_pred_layer_idc[ i ]: 1, i = 0 (Layer 1 is used for prediction of Layer 2)

Coding structure: Random Access B

Number of Frames: 9

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3)

Layer: 1, PTL Idx: 2 (Scalable Main 3)

- OLS_2 -   Layer: 0, PTL Idx: 1 (Main 3)

Layer: 1, PTL Idx: 2 (Scalable Main 3)

Layer: 2, PTL Idx: 3 (Scalable Main 4)

Number of layers: 3

Profile, Tier, Level information: Num PTL = 4

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 3.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 3

- PTL_Idx 3 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution:

- Layer 0 resolution - Output: 960x540

Coded:  960x544

- Layer 1 resolution - Output: 960X540

Coded:  960x544

- Layer 2 resolution - Output: 1920x1080

Coded:  1920x1080

Frame rate: 24 fps for both layers

SHM Version: This bitstream is generated using SHM rev 1084

#### Test bitstreams #REFLAYER_C_VIDYO

Bitstream file name: REFLAYER_C_VIDYO_1.bit

Bitstream feature name: Multiple active ref layers with various inter layer predictors

Bitstream feature description:

In this bitstream, NumDirectRefLayers=2 for Layer 2. Various frames may use either Layer 0 or Layer 1 for sample prediction. All frames always may only use Layer 1 for motion prediction.

Some important params in the VPS Extension are as below:

- Layer 1 (i = 1):

direct_dependency_flag[ i ][ j ]: 1, i = 1, j = 0

NumDirectRefLayers = 1

- Layer 2 (i = 2):

direct_dependency_flag[ i ][ j ]: 1, i = 2, j = 0

direct_dependency_flag[ i ][ j ]: 1, i = 2, j = 1

NumDirectRefLayers = 2

- direct_dependency_all_layers_flag: 0

- Layer 1 (i = 1)

direct_dependency_type[ i ][ j ]: 2, i = 1, j = 0 (inter-layer sample prediction & inter-layer motion prediction)

- Layer 2 (i = 2)

direct_dependency_type[ i ][ j ]: 0, i = 2, j = 0 (inter-layer sample prediction & inter-layer motion prediction)

direct_dependency_type[ i ][ j ]: 2, i = 2, j = 1 (inter-layer sample prediction & inter-layer motion prediction)

Some important params in the Slice segment header are as below:

Frame 0 (POC id = 0), Frame 1 (POC id = 8), Frame 3 (POC id = 2), Frame 5 (POC id = 3), Frame 7 (POC id = 5)

- Layer 1: NumDirectRefLayers = 1

- Layer 2: NumDirectRefLayers = 2

num_inter_layer_ref_pics_minus1: 0 (+1 = 1)

inter_layer_pred_layer_idc[ i ]: 1, i = 0 (Layer 1 is used for prediction of Layer 2)

Frame 2 (POC id = 4), Frame 4 (POC id = 1), Frame 6 (POC id = 6), Frame 8 (POC id = 7)

- Layer 1: NumDirectRefLayers = 1

- Layer 2: NumDirectRefLayers = 2

num_inter_layer_ref_pics_minus1: 0 (+1 = 1)

inter_layer_pred_layer_idc[ i ]: 0, i = 0 (Layer 0 is used for prediction of Layer 2)

Coding structure: Random Access B

Number of Frames: 9

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3)

Layer: 1, PTL Idx: 2 (Scalable Main 3.1)

- OLS_2 -   Layer: 0, PTL Idx: 1 (Main 3)

Layer: 1, PTL Idx: 2 (Scalable Main 3.1)

Layer: 2, PTL Idx: 3 (Scalable Main 4)

Number of layers: 3

Profile, Tier, Level information: Num PTL = 4

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 3.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 3.1

- PTL_Idx 3 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution:

- Layer 0 resolution - Output: 960x540

Coded:  960x544

- Layer 1 resolution - Output: 1280X720

Coded:  1280x720

- Layer 2 resolution - Output: 1920x1080

Coded:  1920x1080

Frame rate: 24 fps for all 3 layers

SHM Version: This bitstream is generated using SHM rev 1084

#### Test bitstreams #REFLAYER_D_VIDYO

Bitstream file name: REFLAYER_D_VIDYO_2.bit

Bitstream feature name: Multiple active ref layers with various inter layer predictors

Bitstream feature description: In this bitstream, NumDirectRefLayers=2 for Layer 2. Various frames may use either Layer 0 or Layer 1 for sample prediction or motion prediction. This bitstream uses direct_dependency_all_layers_type in the VPS instead of signalling direct_dependency_type for each reference layer. Each spatial layer in the bitstream has 4 temporal layers. The max temporal layer () available for inter-layer prediction is temporal layer 2. The GOP size is 8. The 8th picture which is the first frame in encoding order in the GOP, only uses inter-layer prediction for ELs.

Temporal Layer structure for the bitstream

Frame 0 (POC id = 0), Frame 1 (POC id = 8): Temporal sub-layer id = 0

Frame 2 (POC id = 4) : Temporal sub-layer id = 1

Frame 3 (POC id = 2), Frame 6 (POC id = 6): Temporal sub-layer id = 2

Frame 4 (POC id = 1), Frame 5 (POC id = 3), Frame 7 (POC id = 5), Frame 8 (POC id = 7): Temporal sub-layer id = 3

Some important params in the VPS Extension are as below:

- Layer 1 (i = 1):

direct_dependency_flag[ i ][ j ]: 1, i = 1, j = 0

NumDirectRefLayers = 1

- Layer 2 (i = 2):

direct_dependency_flag[ i ][ j ]: 1, i = 2, j = 0

direct_dependency_flag[ i ][ j ]: 1, i = 2, j = 1

NumDirectRefLayers = 2

- max_tid_ref_present_flag: 1

- Layer 0 (i = 0)

max_tid_il_ref_pics_plus1[ i ][ j ]: 3, i = 0, j = 1

max_tid_il_ref_pics_plus1[ i ][ j ]: 3, i = 0, j = 2

- Layer 1 (i = 1)

max_tid_il_ref_pics_plus1[ i ][ j ]: 3, i = 1, j = 2

- max_one_active_ref_layer_flag: 0

- direct_dependency_all_layers_flag: 1

- direct_dependency_all_layers_type: 2 (inter-layer sample prediction & inter-layer motion prediction allowed for all reference layers)

Some important params in the Slice segment header are as below:

Frame 0 (POC id = 0), Frame 1 (POC id = 8), Frame 3 (POC id = 2)

- Layer 1: NumDirectRefLayers = 1

- Layer 2: NumDirectRefLayers = 2

num_inter_layer_ref_pics_minus1: 0 (+1 = 1)

inter_layer_pred_layer_idc[ i ]: 0, i = 0 (Layer 0 is used for prediction of Layer 2)

Frame 2 (POC id = 4), Frame 6 (POC id = 6)

- Layer 1: NumDirectRefLayers = 1

- Layer 2: NumDirectRefLayers = 2

num_inter_layer_ref_pics_minus1: 0 (+1 = 1)

inter_layer_pred_layer_idc[ i ]: 1, i = 0 (Layer 1 is used for prediction of Layer 2)

Frame 4 (POC id = 1), Frame 5 (POC id = 3), Frame 7 (POC id = 5), Frame 8 (POC id = 7)

- Layer 1 & Layer 2: inter_layer_pred_enabled_flag: false (Temporal Sub-layer id = 3, which is greater than max_tid_il_ref_pics_plus1)

Coding structure: Random Access B

Number of Frames: 9

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3)

Layer: 1, PTL Idx: 2 (Scalable Main 3.1)

- OLS_2 -   Layer: 0, PTL Idx: 1 (Main 3)

Layer: 1, PTL Idx: 2 (Scalable Main 3.1)

Layer: 2, PTL Idx: 3 (Scalable Main 4)

Number of layers: 3

Profile, Tier, Level information: Num PTL = 4

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 3.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 3.1

- PTL_Idx 3 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution:

- Layer 0 resolution - Output: 960x540

Coded:  960x544

- Layer 1 resolution - Output: 1280X720

Coded:  1280x720

- Layer 2 resolution - Output: 1920x1080

Coded:  1920x1080

Frame rate: 24 fps for all 3 layers

SHM Version: This bitstream is generated using SHM rev 1087f

    - Test bitstreams – SPLITFLAG

#### Test bitstreams #SPLITFLAG_A_HHI

Bitstream file name:

SPLITFLAG_A_HHI_1.bit

Explanation of bitstream features:

splitting_flag enabled, i.e. inferring of layer dimension identifiers from layer identifier instead of explicit signalling.

Number of output layer sets in the bitstream:

2

Number of layers present in the bitstream:

2

Profile, tier, and level for each layer in each output layer set in the bitstream:

OLS0 - L0: Main profile, Main tier, Level 3.1

OLS1 - L1: Scalable Main profile, Main tier, Level 4.0

Picture size of each layer:

L0: 960x540

L1: 1920x1080

Frame rate of each layer: 24 fps

fSHM version number used to generate the bitstream: SHM-8.0 r1026

### Test bitstreams – VUI

#### Test bitstreams #VUI_A_QUALCOMM

Bitstream file name: VUI_A_QUALCOMM_1.bit

Bitstream feature name: Conformance VUI in VPS Extension

Bitstream feature description:

Conformance SHVC in which VUI part within VPS extension is present (vps_vui_present_flag = 1). A c

onforming decoder is expected to be able to parse the VUI syntax elements.

- Value of vps_vui_present_flag in VPS Extension is set to 10

Coding structure: Random access main

Number of Frames: 5

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4.1)

Number of layers: 2

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Ehhancement layer PTL]

Level: 4.1

Each layer resolution:

- Layer 0 resolution - Output: 960x540

Coded:  960x640

- Layer 1 resolution - Output: 1920x1080

Coded:  1920x1280

Frame rate: 50 fps for both layers

SHM Version: SHM Dev branch, rev 1025

#### Test bitstreams #VUI_B_QUALCOMM

Bitstream file name: VUI_B_QUALCOMM_1.bit

Bitstream feature name: Conformance VUI in VPS Extension

Bitstream feature description:

Conformance SHVC in which VUI part within VPS extension is present (vps_vui_present_flag = 1). A c

onforming decoder is expected to be able to parse the VUI syntax elements.

- Value of vps_vui_present_flag in VPS Extension is set to 10

- Information about tiling for each layer is present

Coding structure: Random access main

Number of Frames: 5

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4.1)

Number of layers: 2

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Ehhancement layer PTL]

Level: 4.1

Each layer resolution:

- Layer 0 resolution - Output: 960x540

Coded:  960x640

- Layer 1 resolution - Output: 1920x1080

Coded:  1920x1280

Frame rate: 50 fps for both layers

SHM Version: SHM Dev branch, rev 1025

#### Test bitstreams #VUI_C_QUALCOMM

Bitstream file name: VUI_C_QUALCOMM_1.bit

Bitstream feature name: Conformance VUI in VPS Extension

Bitstream feature description:

Conformance SHVC in which VUI part within VPS extension is present (vps_vui_present_flag = 1). A c

onforming decoder is expected to be able to parse the VUI syntax elements.

- Value of vps_vui_present_flag in VPS Extension is set to 10

- Information about wavefront for each layer is present

Coding structure: Random access main

Number of Frames: 5

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4.1)

Number of layers: 2

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Ehhancement layer PTL]

Level: 4.1

Each layer resolution:

- Layer 0 resolution - Output: 960x540

Coded:  960x640

- Layer 1 resolution - Output: 1920x1080

Coded:  1920x1280

Frame rate: 50 fps for both layers

SHM Version: SHM Dev branch, rev 1025

### Test bitstreams – NONVUI

#### Test bitstreams #NONVUI_A_QUALCOMM

Bitstream file name: NONVUI_A_QUALCOMM_1.bit

Bitstream feature name: Conformance Non VUI in VPS Extension

Bitstream feature description:

The conformance Non VUI length in the VPS extension.

Conforming decoder is expected to be able to parse the length and the bytes for the non VUI and ignore them.

- Value of vps_non_vui_extension_length in VPS Extension is set to 10

- Bytes for Non VUI is set all to 1

Coding structure: Random access main

Number of Frames: 5

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4.1)

Number of layers: 2

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4.1

Each layer resolution:

- Layer 0 resolution - Output: 960x540

Coded:  960x640

- Layer 1 resolution - Output: 1920x1080

Coded:  1920x1280

Frame rate: 50 fps for both layers

SHM Version: SHM Dev branch, rev 1025

#### Test bitstreams #NONVUI_B_QUALCOMM

Bitstream file name: NONVUI_B_QUALCOMM_1.bit

Bitstream feature name: Conformance Non VUI in VPS Extension

Bitstream feature description:

The conformance Non VUI length in the VPS extension.

Conforming decoder is expected to be able to parse the length and the bytes for the non VUI and ignore them.

- Value of vps_non_vui_extension_length in VPS Extension is set to 10

- Bytes for Non VUI is set all to 0 (It is expected emulation prevention bytes will occur in this Non VUI part)

Coding structure: Random access main

Number of Frames: 5

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4.1)

Number of layers: 2

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4.1

Each layer resolution:

- Layer 0 resolution - Output: 960x540

Coded:  960x640

- Layer 1 resolution - Output: 1920x1080

Coded:  1920x1280

Frame rate: 50 fps for both layers

SHM Version: SHM Dev branch, rev 1025

#### Test bitstreams #NONVUI_C_QUALCOMM

Bitstream file name: NONVUI_C_QUALCOMM_1.bit

Bitstream feature name: Conformance Non VUI in VPS Extension

Bitstream feature description:

The conformance Non VUI length in the VPS extension.

Conforming decoder is expected to be able to parse the length and the bytes for the non VUI and ignore them.

- Value of vps_non_vui_extension_length in VPS Extension is set to 100

- Bytes for Non VUI is set randomly

Coding structure: Random access main

Number of Frames: 5

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4.1)

Number of layers: 2

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4.1

Each layer resolution:

- Layer 0 resolution - Output: 960x540

Coded:  960x640

- Layer 1 resolution - Output: 1920x1080

Coded:  1920x1280

Frame rate: 50 fps for both layers

SHM Version: SHM Dev branch, rev 1025

### Test bitstreams – DPB

#### Test bitstreams #DPB_A_VIDYO

Bitstream file name: DPB_A_VIDYO_1.bit

Bitstream feature name: Sublayer DPB info present

Bitstream feature description: This bitstream has 4 temporal layers and Max sublayer DPB size of 4 for all sublayers. Various parameters in the dpb_size() structure are as follows

- OLS 1 (not specified for default OLS 0) [max OLS = 2] (i = 1)

sub_layer_flag_info_present_flag[ i ]: 1, i = 1

Temporal sublayer 0 (j = 0)

* for sublayer 0, sub_layer_dpb_info_present_flag[ i ][ j ] is not signalled, always 1

max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ]: 4, i = 1, k = 0 to 1, j = 0 [num layers in OLS_1 = 2, k = 0, 1]

max_vps_num_reorder_pics[ i ][ j ]: 0, i = 1, j = 0

max_vps_latency_increase_plus1[ i ][ j ]: 0, i = 1, j = 0

Temporal sublayer 1 (j = 1)

sub_layer_dpb_info_present_flag[ i ][ j ]: 1, i = 1, j = 1

max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ]: 4, i = 1, k = 0 to 1, j = 1 [num layers in OLS_1 = 2, k = 0, 1]

max_vps_num_reorder_pics[ i ][ j ]: 0, i = 1, j = 1

max_vps_latency_increase_plus1[ i ][ j ]: 0, i = 1, j = 1

Temporal sublayer 2 (j = 2)

sub_layer_dpb_info_present_flag[ i ][ j ]: 1, i = 1, j = 2

max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ]: 4, i = 1, k = 0 to 1, j = 2 [num layers in OLS_1 = 2, k = 0, 1]

max_vps_num_reorder_pics[ i ][ j ]: 2, i = 1, j = 2

max_vps_latency_increase_plus1[ i ][ j ]: 0, i = 1, j = 2

Temporal sublayer 3 (j = 3)

sub_layer_dpb_info_present_flag[ i ][ j ]: 1, i = 1, j = 3

max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ]: 4, i = 1, k = 0 to 1, j = 3 [num layers in OLS_1 = 2, k = 0, 1]

max_vps_num_reorder_pics[ i ][ j ]: 3, i = 1, j = 3

max_vps_latency_increase_plus1[ i ][ j ]: 0, i = 1, j = 3

Coding structure: Random Access B

Number of Frames: 9

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3)

Layer: 1, PTL Idx: 2 (Scalable Main 3.1)

Number of layers: 2

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 3.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 3.1

Each layer resolution:

- Layer 0 resolution - Coded:  960x544

Output: 960x540

- Layer 1 resolution - Coded:  1280x720

Frame rate: 30 fps for both layers

SHM Version: SHM Dev branch, rev 1087

#### Test bitstreams #DPB_B_VIDYO

Bitstream file name: DPB_B_VIDYO_1.bit

Bitstream feature name: Sublayer DPB info present

Bitstream feature description: This bitstream has 4 temporal layers and Max sublayer DPB size varies for all sublayers. Various parameters in the dpb_size() structure are as follows

- OLS 1 (not specified for default OLS 0) [max OLS = 2] (i = 1)

sub_layer_flag_info_present_flag[ i ]: 1, i = 1

Temporal sublayer 0 (j = 0)

* for sublayer 0, sub_layer_dpb_info_present_flag[ i ][ j ] is not signalled, always 1

max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ]: 2, k = 0 to 1 [num layers in OLS_1 = 2, k = 0, 1]

max_vps_num_reorder_pics[ i ][ j ]: 0

max_vps_latency_increase_plus1[ i ][ j ]: 0

Temporal sublayer 1 (j = 1)

sub_layer_dpb_info_present_flag[ i ][ j ]: 1

max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ]: 3, k = 0 to 1 [num layers in OLS_1 = 2, k = 0, 1]

max_vps_num_reorder_pics[ i ][ j ]: 0

max_vps_latency_increase_plus1[ i ][ j ]: 0

Temporal sublayer 2 (j = 2)

sub_layer_dpb_info_present_flag[ i ][ j ]: 1

max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ]: 4, k = 0 to 1 [num layers in OLS_1 = 2, k = 0, 1]

max_vps_num_reorder_pics[ i ][ j ]: 0

max_vps_latency_increase_plus1[ i ][ j ]: 0

Temporal sublayer 3 (j = 3)

sub_layer_dpb_info_present_flag[ i ][ j ]: 0

Coding structure: Low Delay B

Number of Frames: 9

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3)

Layer: 1, PTL Idx: 2 (Scalable Main 3.1)

Number of layers: 2

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 3.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 3.1

Each layer resolution:

- Layer 0 resolution - Coded:  960x544

Output: 960x540

- Layer 1 resolution - Coded:  1280x720

Frame rate: 30 fps for both layers

SHM Version: SHM Dev branch, rev 1087

### Test bitstreams – SRATIOS

#### Test bitstreams #SRATIOS_A_SAMSUNG

Bitstream file name: SRATIOS_SAMSUNG_2.bit

Bitstream feature name: 2 spatial layers with unusual ratios

Bitstream feature description:

By setting non-trivial ScaledReferenceOffests and ReferenceLayerOffests spatial ratio used in re-sampling process is

936:627, which is approximately 1.492, horizontally and 516:347, which is approximately 1.487, vertically.

Coding structure: Random Access

Number of Frames: 60

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 5)

Number of layers: 2

Level0                        : 4.1         # Level of the whole bitstream

Level1                        : 3.1         # Level of the base layer

Level2                        : 4.1         # Level of the enhancement layer

Each layer resolution:

- Layer 0 resolution - Output: 1280x720

Coded:  1280x720

- Layer 1 resolution - Output: 1920x1080

Coded:  1920x1280

Frame rate: 60 fps for both layers

SHM Version: SHM Dev branch, rev 1008

    - Test bitstreams – SNR

#### Test bitstreams #SNR_A_IDCC

Bitstream file name: SNR_A_IDCC_1.bit

Bitstream feature name: 2 SNR layers

Bitstream feature description:

SNR scalability coded video at a single spatial resolution but at different qualities, the lower layer can be used to predict the high layer to reduce the bits.

Number of Layers: 2

- Layer 0 	: 	QP0 = 22

- Layer 1 	: 	QP1 = 20

- Layer 1	: 	NumSamplePredRefLayers1 = 1

- Layer 1	: 	SamplePredRefLayerIds1 = 0

- Layer 1	: 	NumMotionPredRefLayers1 = 1

- Layer 1	: 	MotionPredRefLayerIds1 = 0

- Layer 1	: 	NumActiveRefLayers1 = 1

- Layer 1	: 	PredLayerIds1 = 0

Coding structure: Low delay B

Number of Frames: 4

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 4)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 4)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

-

DefaultTargetOutputLayerIdc   : 1

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 4

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution:

- Layer 0 resolution - Output: 1920x1080

Coded:  1920x1080

- Layer 1 resolution - Output: 1920x1080

Coded:  1920x1280

Frame rate: 24 fps for both layers

SHM Version: SHM Dev branch, rev 1021

Test b

itstreams SNR_B_IDCC

Bitstream file name: SNR_B_IDCC_1.bit

Bitstream feature name: 3 SNR layers

Bitstream feature description:

SNR scalability coded video at a single spatial resolution but at different qualities, the lower layer can be used to predict the high layer to reduce the bits.

Number of Layers: 3

- Layer 0 	: 	QP0 = 22

- Layer 1 	: 	QP1 = 20

- Layer 1	: 	NumSamplePredRefLayers1 = 1

- Layer 1	: 	SamplePredRefLayerIds1 = 0

- Layer 1	: 	NumMotionPredRefLayers1 = 1

- Layer 1	: 	MotionPredRefLayerIds1 = 0

- Layer 1	: 	NumActiveRefLayers1 = 1

- Layer 1	: 	PredLayerIds1 = 0

- Layer 2 	: 	QP1 = 16

- Layer 2	: 	NumSamplePredRefLayers2 = 1

- Layer 2	: 	SamplePredRefLayerIds2 = 0

- Layer 2	: 	NumMotionPredRefLayers2 = 1

- Layer 2	: 	MotionPredRefLayerIds2 = 0

- Layer 2	: 	NumActiveRefLayers2 = 1

- Layer 2	: 	PredLayerIds2 = 0

Coding structure: Low delay B

Number of Frames: 4

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 4)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 4)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

- OLS_2 -   Layer: 0, PTL Idx: 1 (Main 4)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

Layer: 2, PTL Idx: 2 (Scalable Main 4)

- DefaultTargetOutputLayerIdc   : 1

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 4

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution:

- Layer 0 resolution - Output: 1920x1080

Coded:  1920x1080

- Layer 1 resolution - Output: 1920x1080

Coded:  1920x1280

- Layer 2 resolution - Output: 1920x1080

Coded:  1920x1280

Frame rate: 24 fps for both layers

SHM Version: SHM Dev branch, rev 1021

#### Test bitstreams #SNR_C_IDCC

Bitstream file name: SNR_C_IDCC_1.bit

Bitstream feature name: 3 SNR layers

Bitstream feature description:

SNR scalability coded video at a single spatial resolution but at different qualities, the lower layer can be used to predict the high layer to reduce the bits.

Number of Layers: 3

- Layer 0 	: 	QP0 = 22

- Layer 1 	: 	QP1 = 20

- Layer 1	: 	NumSamplePredRefLayers1 = 1

- Layer 1	: 	SamplePredRefLayerIds1 = 0

- Layer 1	: 	NumMotionPredRefLayers1 = 1

- Layer 1	: 	MotionPredRefLayerIds1 = 0

- Layer 1	: 	NumActiveRefLayers1 = 1

- Layer 1	: 	PredLayerIds1 = 0

- Layer 2 	: 	QP1 = 16

- Layer 2	: 	NumSamplePredRefLayers2 = 1

- Layer 2	: 	SamplePredRefLayerIds2 = 1

- Layer 2	: 	NumMotionPredRefLayers2 = 1

- Layer 2	: 	MotionPredRefLayerIds2 = 1

- Layer 2	: 	NumActiveRefLayers2 = 1

- Layer 2	: 	PredLayerIds2 = 0

Coding structure: Low delay B

Number of Frames: 4

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 4)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 4)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

- OLS_2 -   Layer: 0, PTL Idx: 1 (Main 4)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

Layer: 2, PTL Idx: 2 (Scalable Main 4)

- DefaultTargetOutputLayerIdc   : 1

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 4

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution:

- Layer 0 resolution - Output: 1920x1080

Coded:  1920x1080

- Layer 1 resolution - Output: 1920x1080

Coded:  1920x1280

- Layer 2 resolution - Output: 1920x1080

Coded:  1920x1280

Frame rate: 24 fps for both layers

SHM Version: SHM Dev branch, rev 1021

    - Test bitstreams – REPFMT

#### Test bitstreams #REPFMT_A_VIDYO

Bitstream file name: REPFMT_A_VIDYO_1.bit

Bitstream feature name: Rep format in VPS

Bitstream feature description:

rep_format( ) structure in VPS signals picture width & height, chroma format, bit depths and conformance crop window parameters.

The bitstream has 1 rep_format( ) structure signalled in VPS Extension and 2 Layers.

rep_format_idx_present_flag = 0 is inferred if vps_num_rep_formats_minus1 = 0.

vps_rep_format_idx[ i ] is inferred to be equal to Min( i, vps_num_rep_formats_minus1 ), where i = layer_id

- vps_num_rep_formats_minus1 = 0

- Rep Format 0 - pic_width_vps_in_luma_samples: 960

pic_height_vps_in_luma_samples: 544

chroma_and_bit_depth_vps_present_flag: 1

chroma_format_vps_idc: 1

bit_depth_vps_luma_minus8: 0

bit_depth_vps_chroma_minus8: 0

conformance_window_vps_flag: 1

conf_win_vps_bottom_offset: 2

- rep_format_idx_present_flag: 0

- Rep format Layer id map

Layer 0 - Rep format 0

Layer 1 - Rep format 0

- Conformance crop offsets

Layer 0 -   Top: 0

Bottom: 4 (signalled as 2)

Left: 0

Right: 0

Layer 1 -   Top: 0

Bottom: 4 (signalled as 2)

Left: 0

Right: 0

Coding structure: Low Delay P

Number of Frames: 4

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3)

Layer: 1, PTL Idx: 2 (Scalable Main 3.1)

Number of layers: 2

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 3.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 3.1

Each layer resolution:

- Layer 0 resolution - Output: 960x540

Coded:  960x544

- Layer 1 resolution - Output: 960x540

Coded:  960x544

Frame rate: 30 fps for both layers

SHM Version: SHM Dev branch, rev 1004

Test b

itstreams #REPFMT_B_VIDYO

Bitstream file name: REPFMT_B_VIDYO_1.bit

Bitstream feature name: Rep format in VPS

Bitstream feature description:

rep_format( ) structure in VPS signals picture width & height, chroma format, bit depths and conformance crop window parameters.

The bitstream has 2 rep_format( ) structures signalled in VPS Extension and 3 Layers.

vps_rep_format_idx[ i ] (i = layer_id) signalled in VPS Extension maps the right rep_format( ) to each layer.

- vps_num_rep_formats_minus1 = 1

- Rep Format 0 - pic_width_vps_in_luma_samples: 960

pic_height_vps_in_luma_samples: 544

chroma_and_bit_depth_vps_present_flag: 1

chroma_format_vps_idc: 1

bit_depth_vps_luma_minus8: 0

bit_depth_vps_chroma_minus8: 0

conformance_window_vps_flag: 1

conf_win_vps_bottom_offset: 2

- Rep Format 1 - pic_width_vps_in_luma_samples: 1920

pic_height_vps_in_luma_samples: 1080

chroma_and_bit_depth_vps_present_flag: 1

chroma_format_vps_idc: 1

bit_depth_vps_luma_minus8: 0

bit_depth_vps_chroma_minus8: 0

conformance_window_vps_flag: 0

- rep_format_idx_present_flag: 1

- vps_rep_format_idx[ 0 ] = 0 (default value not signalled)

vps_rep_format_idx[ 1 ] = 0

vps_rep_format_idx[ 2 ] = 1

- Conformance crop offsets

Layer 0 -   Top: 0

Bottom: 4 (signalled as 2)

Left: 0

Right: 0

Layer 1 -   Top: 0

Bottom: 4 (signalled as 2)

Left: 0

Right: 0

Coding structure: Low Delay P

Number of Frames: 4

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3)

Layer: 1, PTL Idx: 2 (Scalable Main 3.1)

- OLS_2 -   Layer: 0, PTL Idx: 1 (Main 3)

Layer: 1, PTL Idx: 2 (Scalable Main 3.1)

Layer: 2, PTL Idx: 3 (Scalable Main 4)

Number of layers: 3

Profile, Tier, Level information: Num PTL = 4

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 3.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 3.1

- PTL_Idx 3 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution:

- Layer 0 resolution - Output: 960x540

Coded:  960x544

- Layer 1 resolution - Output: 960x540

Coded:  960x544

- Layer 2 resolution - Coded:  1920x1080

Frame rate: 30 fps for all layers

SHM Version: SHM Dev branch, rev 1004

#### Test bitstreams #REPFMT_C_VIDYO

Bitstream file name: REPFMT_C_VIDYO_1.bit

Bitstream feature name: Rep format in VPS

Bitstream feature description:

rep_format( ) structure in VPS signals picture width & height, chroma format, bit depths and conformance crop window parameters.

The bitstream has 2 rep_format( ) structures signalled in VPS Extension and 3 Layers.

The rep_format_idx_present_flag = 0. vps_rep_format_idx[ i ] is inferred to be equal to Min( i, vps_num_rep_formats_minus1 ), where i=layer_id

- vps_num_rep_formats_minus1 = 1

- Rep Format 0 - pic_width_vps_in_luma_samples: 960

pic_height_vps_in_luma_samples: 544

chroma_and_bit_depth_vps_present_flag: 1

chroma_format_vps_idc: 1

bit_depth_vps_luma_minus8: 0

bit_depth_vps_chroma_minus8: 0

conformance_window_vps_flag: 1

conf_win_vps_bottom_offset: 2

- Rep Format 1 - pic_width_vps_in_luma_samples: 1920

pic_height_vps_in_luma_samples: 1080

chroma_and_bit_depth_vps_present_flag: 0

conformance_window_vps_flag: 0

- rep_format_idx_present_flag: 0

- Rep format Layer id map

Layer 0 - Rep format 0

Layer 1 - Rep format 1

Layer 2 - Rep format 1

- Conformance crop offsets

Layer 0 -   Top: 0

Bottom: 4 (signalled as 2)

Left: 0

Right: 0

Coding structure: Low Delay P

Number of Frames: 4

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

- OLS_2 -   Layer: 0, PTL Idx: 1 (Main 3)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

Layer: 2, PTL Idx: 2 (Scalable Main 4)

Number of layers: 3

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 3.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution:

- Layer 0 resolution - Output: 960x540

Coded:  960x544

- Layer 1 resolution - Coded:  1920x1080

- Layer 2 resolution - Coded:  1920x1080

Frame rate: 30 fps for all layers

SHM Version: SHM Dev branch, rev 1004

### Test bitstreams – RESCHANGE

#### Test bitstreams #RESCHANGE_A_VIDYO

Bitstream file name: RESCHANGE_B_VIDYO_1.bit

Bitstream feature name: Resolution change in SPS

Bitstream feature description:

rep_format( ) structure in VPS signals picture width & height, chroma format, bit depths and conformance crop window parameters.

For base layer (layer_id = 0) or independent non-base layer, SPS signals picture width & height, chroma format, bit depths and conformance crop window parameters.

These parameters can override values signalled by VPS rep_format( ).

The bitstream has 2 rep_format( ) structures signalled in VPS Extension. SPS for layer 0 signals new resolution.

- vps_num_rep_formats_minus1 = 1

- Rep Format 0 - pic_width_vps_in_luma_samples: 1280

pic_height_vps_in_luma_samples: 720

chroma_and_bit_depth_vps_present_flag: 1

chroma_format_vps_idc: 1

bit_depth_vps_luma_minus8: 0

bit_depth_vps_chroma_minus8: 0

conformance_window_vps_flag: 0

- Rep Format 1 - pic_width_vps_in_luma_samples: 1920

pic_height_vps_in_luma_samples: 1080

chroma_and_bit_depth_vps_present_flag: 1

chroma_format_vps_idc: 1

bit_depth_vps_luma_minus8: 0

bit_depth_vps_chroma_minus8: 0

conformance_window_vps_flag: 0

- rep_format_idx_present_flag: 1

- vps_rep_format_idx[ 0 ] = 0 (default value not signalled)

vps_rep_format_idx[ 1 ] = 1

- In SPS for Layer 0 - chroma_format_idc: 1

pic_width_in_luma_samples: 960

pic_height_in_luma_samples: 536

conformance_window_flag: 0

bit_depth_luma_minus8: 0

bit_depth_chroma_minus8: 0

- In SPS for Layer 1 - update_rep_format_flag: 0

Coding structure: Low Delay P

Number of Frames: 4

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

Number of layers: 2

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 3.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution:

- Layer 0 resolution - Coded:  960x536

- Layer 1 resolution - Coded:  1920x1080

Frame rate: 30 fps for both layers

SHM Version: SHM Dev branch, rev 1007

### Test bitstreams – ADAPTRES

#### Test bitstream #ADAPTRES_A_ERICSSON

Bitstream file name: ADAPTRES_A_ERICSSON_1.bit

Bitstream feature name: Adaptive resolution

Bitstream feature description:

The conformance bitstream tests the special case of skip pictures for adaptive resolution change. In adaptive resolution when switching up to higher layer, the

higher layer is coded as skip IRAP picture with P_SLICES using inter-layer picture as reference for this use case.

Coding structure: Low delay, one reference picture

Number of Frames: 30

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

- OLS_2 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

Layer: 2, PTL Idx: 2 (Scalable Main 4)

Number of layers: 3

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 3.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution:

- Layer 0 resolution - Output: 1280x720

Coded:  1280x720

- Layer 1 resolution - Output: 1920x1080

Coded:  1920x1080

- Layer 2 resolution - Output: 1920x1080

Coded:  1920x1080

Frame rate: 24 fps for all three layers

SHM Version: Independent SHVC encoder implementation used for encoding, SHM was not used. Correctly decodable by SHM-8.0.

### Test bitstreams – SPSREPFMT

#### Test bitstreams #SPSREPFMT_A_Sony

Bitstream file name: SPSREPFMT_A_Sony_1.bit

Specification: All slices are coded as I, P or B slices. Each picture contains one slice.

vps_max_layers_minus1 is equal to 1. vps_num_rep_formats_minus1 is equal to 3.

Functional stage: Test representation format update in SPS

Frame rate: 24 fps

Description:

The bitstream has two layers (See LX below for the coding width and height).

Four sets of representation format are signalled in VPS (See rep_formatX below for the signalled width and height).

In VPS, the default inference rule of the representation formant is applied to the layers.

In L1 SPS, the representation format is updated.

Number of layers: 2

Each layer resolution:

L0: 960x540

L1: 960x540

rep_format0: 960x540

rep_format1: 1280x720

rep_format2: 1920x1080

rep_format3: 3840x2160

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 5.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 5.1)

Layer: 1, PTL Idx: 2 (Scalable Main 5.1)

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 5.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 5.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 5.1

### Test bitstreams – CONFCROP

#### Test bitstreams #CONFCROP_A_VIDYO

Bitstream file name: CONFCROP_A_VIDYO_1.bit

Bitstream feature name: Conformance cropping window in VPS

Bitstream feature description:

The conformance cropping window parameters in the VPS (starting with conformance_window_vps_flag) are used to crop the output picture

- Conformance crop offsets

Layer 0 -   Top: 0

Bottom: 100 (signalled as 50)

Left: 0

Right: 0

Layer 1 -   Top: 0

Bottom: 200 (signalled as 100)

Left: 0

Right: 0

Coding structure: Low Delay P

Bitstream generated using - ConformanceMode = 2: user specified padding

Number of Frames:  8

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 5)

Number of layers: 2

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 3.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Ehhancement layer PTL]

Level: 5

Each layer resolution:

- Layer 0 resolution - Output: 960x540

Coded:  960x640

- Layer 1 resolution - Output: 1920x1080

Coded:  1920x1280

Frame rate: 30 fps for both layers

SHM Version: SHM Dev branch, rev 982

#### Test bitstreams #CONFCROP_B_VIDYO

Bitstream file name: CONFCROP_B_VIDYO_1.bit

Bitstream feature name: Conformance cropping window in VPS

Bitstream feature description:

The conformance cropping window parameters in the VPS (starting with conformance_window_vps_flag) are used to crop the output picture

- Conformance crop offsets

Layer 0 -   Top: 50 (signalled as 25)

Bottom: 6 (signalled as 3)

Left: 160 (signalled as 80)

Right: 80 (signalled as 40)

Layer 1 -   Top: 24 (signalled as 12)

Bottom: 148 (signalled as 74)

Left: 320 (signalled as 160)

Right: 0

Coding structure: Low Delay P

- Bitstream generated using - ConformanceMode = 3: conformance cropping

Number of Frames: 8

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

Number of layers: 2

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 3.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution:

- Layer 0 resolution - Output: 720x480

Coded:  960x536

- Layer 1 resolution - Output: 1600x900

Coded:  1920x1072

Frame rate: 30 fps for both layers

SHM Version: SHM Dev branch, rev 986

#### Test bitstreams #CONFCROP_C_VIDYO

Bitstream file name: CONFCROP_C_VIDYO_2.bit

Bitstream feature name: Conformance cropping window in VPS

Bitstream feature description: The conformance cropping window parameters in the VPS (starting with conformance_window_vps_flag) are used to crop the output picture

- Conformance crop offsets

Layer 0 -   Top: 0

Bottom: 4 (signalled as 2)

Left: 0

Right: 0

Layer 1 -   Top: 0

Bottom: 180 (signalled as 90)

Left: 0

Right: 320 (signalled as 160)

Layer 2 -   Top: 80 (signalled as 40)

Bottom: 100 (signalled as 50)

Left: 120 (signalled as 60)

Right: 40 (signalled as 20)

Coding structure: Low Delay P

- Bitstream generated using Layer 0: ConformanceMode = 1: automatic padding

Layer 1: ConformanceMode = 2: user specified padding

Layer 2: ConformanceMode = 3: conformance cropping

Number of Frames: 4

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3)

Layer: 1, PTL Idx: 2 (Scalable Main 3.1)

- OLS_2 -   Layer: 0, PTL Idx: 1 (Main 3)

Layer: 1, PTL Idx: 2 (Scalable Main 3.1)

Layer: 2, PTL Idx: 3 (Scalable Main 4)

Number of layers: 3

Profile, Tier, Level information: Num PTL = 4

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 3.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 3.1

- PTL_Idx 3 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution:

- Layer 0 resolution - Output: 960x540

Coded:  960x544

- Layer 1 resolution - Output: 960X540

Coded:  1280X720

- Layer 2 resolution - Output: 1760x900

Coded:  1920x1080

Frame rate: 30 fps for both layers

SHM Version: SHM Dev branch, rev 993

Test bitstreams – SCREFOFF

#### Test bitstreams #SCREFOFF_A_QCOM

Bitstream file name:

SCREFOFF_A_QCOM_1.bit

Bitstream feature name:

Setting scaled reference layer offset values to non-zero values

Explanation of bitstream features:

This bitstream is coded using the RA configuration, with two layers; layer 0 is a direct reference layer of layer 1. Although the resolutions of the two layers are different, the scaling factor is still 1. The scaled reference layer offsets are specified accordingly.

Number of output layer sets in the bitstream:

2

Number of layers present in the bitstream:

2

Profile, tier, and level (P,T,L) for each layer in each output layer set in the bitstream:

OLS 0: L0: (main, main, 3)

OLS 1: L0: (main, main, 3)

L1: (scalable-main, main, 3.1)

Picture size of each layer:

L0: 400 x 400

L1: 960 x 540

Frame rate of each layer:

50 fps

Number of frames: 20

SHM Version:

SHM-dev branch, r1079

    - Test bitstreams – REFREGOFF

#### Test bitstreams #REFREGOFF_A_SHARP

Bitstream file name: REFREGOFF_A_SHARP_1.bit

Bitstream feature name: Reference region offsets in PPS

Bitstream feature description:

The reference region offset parameters in the PPS (starting with ref_region_offset_present_flag) are used to indicate the region on the reference picture which corresponds to the scaled reference layer region defined by scaled reference layer offset parameters.

- Reference layer offsets

Layer 0 -   (NOT SIGNALLED)

Layer 1 -   Top: 100 (signalled as 50)

Bottom: 180 (signalled as 90)

Left: 540 (signalled as 270)

Right: 220 (signalled as 110)

Note: 1280x800 EL picture corresponds to 640x400 region of BL picture, thus ratio is 2x.

Coding structure: Random Access

Number of Frames: 9

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 4.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 4.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4.1)

Number of layers: 2

Profile, Tier, Level information: Num PTL = 2

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: SCALABLE MAIN [Ehhancement layer PTL]

Level: 4.1

Each layer resolution:

- Layer 0 resolution - Output: 1280x800

Coded:  1280x800

- Layer 1 resolution - Output: 1280x800

Coded:  1280x800

Frame rate: 30 fps for both layers

SHM Version: SHM-dev rev1033

### Test bitstreams – RESPHASE

#### Test bitstreams #RESPHASE_A_SAMSUNG

Bitstream file name: RESPHASE_A_SAMSUNG_2.bit

Bitstream feature name: Resample phase

Bitstream feature description:

Non-trivial resampling phases syntax and semantics implementation is exercised.

Coding structure: Random Access

Number of Frames: 60

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 5)

Number of layers: 2

Level information:

Level0                        : 4.1         # Level of the whole bitstream

Level1                        : 3.1         # Level of the base layer

Level2                        : 4.1         # Level of the enhancement layer

Each layer resolution:

- Layer 0 resolution - Output: 1280x720

Coded:  1280x720

- Layer 1 resolution - Output: 1920x1080

Coded:  1920x1280

Frame rate: 60 fps for both layers

SHM Version: SHM Dev branch, rev 1008

### Test bitstreams – OLS

#### Test bitstreams #OLS_A_NOKIA

Bitstream file name: OLS_A_NOKIA_1.bit

Bitstream feature name: Additional output layer sets

Bitstream feature description:

An additional output layer sets is defined (OLS_3)

Coding structure: Low Delay P

Number of Frames: 4

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

- OLS_2 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

Layer: 2, PTL Idx: 2 (Scalable Main 4)

- OLS_3 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

Layer: 2, PTL Idx: 2 (Scalable Main 4)

Number of layers: 3

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution: 1280x720

Frame rate: 24 fps

SHM Version: SHM Dev branch, rev 1015

#### Test bitstreams #OLS_B_NOKIA

Bitstream file name: OLS_b_NOKIA_1.bit

Bitstream feature name: Non-default output layer sets

Bitstream feature description:

default_target_output_layer_idc is set equal to 2 (i.e. layers in output layer sets are signaled explicitly)

Coding structure: Low Delay P

Number of Frames: 4

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

- OLS_2 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

Layer: 2, PTL Idx: 2 (Scalable Main 4)

Number of layers: 3

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution: 1280x720

Frame rate: 24 fps

SHM Version: SHM Dev branch, rev 1015

#### Test bitstreams OLS_C_NOKIA

Bitstream file name: OLS_C_NOKIA_1.bit

Bitstream feature name: Unnecessary layers

Bitstream feature description:

Layers with layer IDs 1 and 2 are not used for inter-layer prediction, thus making them "unnecessary" layers for prediction purposes.

Coding structure: Low Delay P

Number of Frames: 4

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

- OLS_2 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, Unnecessary layer, No PTL

Layer: 2, PTL Idx: 2 (Scalable Main 4)

- OLS_3 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, Unnecessary layer, No PTL

Layer: 2, Unnecessary layer, No PTL

Layer: 3, PTL Idx: 2 (Scalable Main 4)

Number of layers: 4

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution: 1280x720

Frame rate: 24 fps

SHM Version: SHM Dev branch, rev 1015

### Test bitstreams – DISFLAG

#### Test bitstreams #DISFLAG_A_QUALCOMM

Bitstream file name: DISFLAG_A_QUALCOMM_1.bit

Bitstream feature name: Discardable picture

Bitstream feature description:

Conformance SHVC in which one or more pictures is marked as discardable picture.

Conforming decoder is expected to be able to decode the bitstream.

Coding structure: Random access main

Number of Frames: 8

- One of the picture in the bitstream has value of discardable_flag equal to 1 in its slice header.

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4.1)

Number of layers: 2

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Ehhancement layer PTL]

Level: 4.1

Each layer resolution:

- Layer 0 resolution - Output: 960x540

- Layer 1 resolution - Output: 1920x1080

Frame rate: 50 fps for both layers

SHM Version: SHM8.0

### Test bitstreams - PPSLIST

#### Test bitstreams #PPSLIST_A_Sony

Bitstream file name: PPSSLIST_A_Sony_1.bit

Specification: All slices are coded as I, P or B slices. Each picture contains one slice.

vps_max_layers_minus1 is equal to 1. pps_infer_scaling_list_flag for the enhancement layer is equal to 1.

Functional stage: Test PPS scaling list inferring

Description:

The bitstream has two layers (See LX below for the coding width and height).

EL PPS scaling list is inferred from BL PPS scaling list.

Number of layers: 2

L0: 960x540

L1: 1280x720

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 4.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 4.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4.1)

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 4.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4.1

SHM Version: SHM Dev branch, rev 1015 (Trac ticket #59)

### Test bitstreams - SPSLIST

#### Test bitstreams #SPSLIST_A_Sony

Bitstream file name: SPSSLIST_A_Sony_1.bit

Specification: All slices are coded as I, P or B slices. Each picture contains one slice.

vps_max_layers_minus1 is equal to 1. sps_infer_scaling_list_flag for the enhancement layer is equal to 1.

Functional stage: Test SPS scaling list inferring

Frame rate: 24 fps

Description:

The bitstream has two layers (See LX below for the coding width and height).

EL SPS scaling list is inferred from BL SPS scaling list.

Number of layers: 2

Each layer resolution:

L0: 960x540

L1: 1280x720

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 4.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 4.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4.1)

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 4.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4.1

### Test bitstreams – CGS

#### Test bitstreams #CGS_A_TECHNICOLOR

Bitstream file name: CGS_A_TECHNICOLOR_1.bit

Bitstream feature name: Colour gamut scalability 8-to-10 bits

Bitstream feature description:

The bitstream has two layers. The base layer video format is 8-bit 1920x1080 BT.709 and the enhancement layer video format is 10-bit 1920x1080 BT.2020.

cm_octant_depth = 1

split_octant_flag = 1

Number of Frames: 1

Number of layers: 2

Each layer resolution:

- Layer 0 resolution : 1920X1080 8b  - Color Space : BT. 709

- Layer 1 resolution : 1920X1080 10b - Color Space : BT. 2020

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 4.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 4.1)

Layer: 1, PTL Idx: 2 (Scalable Main10 4.1)

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 4.1

- PTL_Idx 2 - Profile: SCALABLE MAIN10 [Enhancement layer PTL]

Level: 4.1

Frame rate: 50 fps for both layers

SHM Version: SHM Dev branch, rev 1021

#### Test bitstreams #CGS_B_TECHNICOLOR

Bitstream file name: CGS_B_TECHNICOLOR_1.bit

Bitstream feature name: Colour gamut scalability 8-to-10 bits

Bitstream feature description:

The bitstream has two layers. The base layer video format is 8-bit 1920x1080 BT.709 and the enhancement layer video format is 10-bit 1920x1080 BT.2020

cm_octant_depth = 0

Number of Frames: 1

Number of layers: 2

Each layer resolution:

- Layer 0 resolution : 1920X1080 8b  - Color Space : BT. 709

- Layer 1 resolution : 1920X1080 10b - Color Space : BT. 2020

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 4.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 4.1)

Layer: 1, PTL Idx: 2 (Scalable Main10 4.1)

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 4.1

- PTL_Idx 2 - Profile: SCALABLE MAIN10 [Enhancement layer PTL]

Level: 4.1

Frame rate: 50 fps for both layers

SHM Version: SHM Dev branch, rev 1021

#### Test bitstreams #CGS_C_TECHNICOLOR

Bitstream file name: CGS_C_TECHNICOLOR_1.bit

Bitstream feature name: Colour gamut scalability 10-to-10 bits

Bitstream feature description:

The bitstream has two layers. The base layer video format is 10-bit 1920x1080 BT.709 and the enhancement layer video format is 10-bit 1920x1080 BT.2020

cm_octant_depth = 1

split_octant_flag = 1

Number of Frames: 1

Number of layers: 2

Each layer resolution:

- Layer 0 resolution : 1920X1080 10b  - Color Space : BT. 709

- Layer 1 resolution : 1920X1080 10b - Color Space : BT. 2020

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main10 4.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main10 4.1)

Layer: 1, PTL Idx: 2 (Scalable Main10 4.1)

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN10 [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN10 [Base layer PTL]

Level: 4.1

- PTL_Idx 2 - Profile: SCALABLE MAIN10 [Enhancement layer PTL]

Level: 4.1

Frame rate: 50 fps for both layers

SHM Version: SHM Dev branch, rev 1021

#### Test bitstream #CGS_D_TECHNICOLOR

Bitstream file name: CGS_D_TECHNICOLOR_1.bit

Bitstream feature name: Colour gamut scalability 10-to-10 bits

Bitstream feature description:

The bitstream has two layers. The base layer video format is 10-bit 1920x1080 BT.709 and the enhancement layer video format is 10-bit 1920x1080 BT.2020

cm_octant_depth = 0

Number of Frames: 1

Number of layers: 2

Each layer resolution:

- Layer 0 resolution : 1920X1080 10b  - Color Space : BT. 709

- Layer 1 resolution : 1920X1080 10b - Color Space : BT. 2020

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main10 4.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main10 4.1)

Layer: 1, PTL Idx: 2 (Scalable Main10 4.1)

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN10 [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN10 [Base layer PTL]

Level: 4.1

- PTL_Idx 2 - Profile: SCALABLE MAIN10 [Enhancement layer PTL]

Level: 4.1

Frame rate: 50 fps for both layers

SHM Version: SHM Dev branch, rev 1021

#### Test bitstreams #CGS_E_TECHNICOLOR

Bitstream file name: CGS_E_TECHNICOLOR_1.bit

Bitstream feature name: Spatial ratio change with Colour Gamut Scalability

Bitstream feature description:

The bitstream has two layers. The base layer video format is 8-bit 960x540 BT.709 and the enhancement layer video format is 10-bit 1920x1080 BT.2020

cm_octant_depth = 1

split_octant_flag = 1

Number of Frames: 1

Number of layers: 2

Each layer resolution:

- Layer 0 resolution : 960x540 8b    - Color Space : BT. 709

- Layer 1 resolution : 1920X1080 10b - Color Space : BT. 2020

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 4.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 4.1)

Layer: 1, PTL Idx: 2 (Scalable Main10 4.1)

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 4.1

- PTL_Idx 2 - Profile: SCALABLE MAIN10 [Enhancement layer PTL]

Level: 4.1

Frame rate: 60 fps for both layers

SHM Version: SHM Dev branch, rev 1021

#### Test bitstreams CGS_F_TECHNICOLOR

Bitstream file name: CGS_F_TECHNICOLOR_1.bit

Bitstream feature name: Spatial ratio change with Colour Gamut Scalability

Bitstream feature description:

The bitstream has two layers. The base layer video format is 8-bit 960x540 BT.709 and the enhancement layer video format is 10-bit 1920x1080 BT.2020

cm_octant_depth = 0

Number of Frames: 1

Number of layers: 2

Each layer resolution:

- Layer 0 resolution : 960x540 8b    - Color Space : BT. 709

- Layer 1 resolution : 1920X1080 10b - Color Space : BT. 2020

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 4.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 4.1)

Layer: 1, PTL Idx: 2 (Scalable Main10 4.1)

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 4.1

- PTL_Idx 2 - Profile: SCALABLE MAIN10 [Enhancement layer PTL]

Level: 4.1

Frame rate: 60 fps for both layers

SHM Version: SHM Dev branch, rev 1021

#### Test bitstreams #CGS_G_TECHNICOLOR

Bitstream file name: CGS_G_TECHNICOLOR_1.bit

Bitstream feature name: Colour gamut scalability 8-to-10 bits

Bitstream feature description:

The bitstream has two layers. The base layer video format is 8-bit 1920X1080 BT.709 and the enhancement layer video format is 10-bit 1920X1080 BT.2020

cm_octant_depth = 1

split_octant_flag = 0

Number of Frames: 2

Number of layers: 2

Each layer resolution:

- Layer 0 resolution : 1920X1080 8b  - Color Space : BT. 709

- Layer 1 resolution : 1920X1080 10b - Color Space : BT. 2020

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 4.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 4.1)

Layer: 1, PTL Idx: 2 (Scalable Main10 4.1)

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 4.1

- PTL_Idx 2 - Profile: SCALABLE MAIN10 [Enhancement layer PTL]

Level: 4.1

Frame rate: 50 fps for both layers

SHM Version: SHM-8.0 tag (rev 1026)

Important note: the SHM-8.0 does not decode correctly pictures with different bit-depth in BL and EL (see ticket #65)

- to decode the Layer-0 reconstructed pictures, use "-olsidx 0" option

- to decode the Layer-1 reconstructed pictures, use "-olsidx 1" option, but the base layer pictures are wrong.

#### Test bitstreams CGS_H_TECHNICOLOR

Bitstream file name: CGS_H_TECHNICOLOR_1.bit

Bitstream feature name: Colour gamut scalability 10-to-10 bits

Bitstream feature description:

The bitstream has two layers. The base layer video format is 10-bit 1920x1080 BT.709 and the enhancement layer video format is 10-bit 1920x1080 BT.2020

cm_octant_depth = 1

split_octant_flag = 0

Number of Frames: 2

Number of layers: 2

Each layer resolution:

- Layer 0 resolution : 1920X1080 10b  - Color Space : BT. 709

- Layer 1 resolution : 1920X1080 10b - Color Space : BT. 2020

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main10 4.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main10 4.1)

Layer: 1, PTL Idx: 2 (Scalable Main10 4.1)

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN10 [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN10 [Base layer PTL]

Level: 4.1

- PTL_Idx 2 - Profile: SCALABLE MAIN10 [Enhancement layer PTL]

Level: 4.1

Frame rate: 50 fps for both layers

SHM Version: SHM-8.0 tag (rev 1026)

#### Test bitstreams #CGS_I_TECHNICOLOR

Bitstream file name: CGS_I_TECHNICOLOR_1.bit

Bitstream feature name: Spatial ratio change with Colour Gamut Scalability

Bitstream feature description:

The bitstream has two layers. The base layer video format is 8-bit 960x540 BT.709 and the enhancement layer video format is 10-bit 1920x1080 BT.2020

cm_octant_depth = 1

split_octant_flag = 0

Number of Frames: 2

Number of layers: 2

Each layer resolution:

- Layer 0 resolution : 960x540 8b    - Color Space : BT. 709

- Layer 1 resolution : 1920X1080 10b - Color Space : BT. 2020

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 4.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 4.1)

Layer: 1, PTL Idx: 2 (Scalable Main10 4.1)

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 4.1

- PTL_Idx 2 - Profile: SCALABLE MAIN10 [Enhancement layer PTL]

Level: 4.1

Frame rate: 60 fps for both layers

SHM Version: SHM-8.0 tag (rev 1026)

Important note: the SHM-8.0 does not decode correctly pictures with different bit-depth in BL and EL (see ticket #65)

- to decode the Layer-0 reconstructed pictures, use "-olsidx 0" option

- to decode the Layer-1 reconstructed pictures, use "-olsidx 1" option, but the base layer pictures are wrong.

### Test bitstreams – PSEXT

#### Test bitstreams #PSEXT_A_VIDYO

Bitstream file name: PSEXT_A_VIDYO_1.bit

Bitstream feature name: SPS, PPS additional extension

Bitstream feature description:

This bitstream has these syntax elements set as below.

- Base layer PPS - pps_extension_6bits: 0x5

pps_extension_data_flag: 0

pps_extension_data_flag: 0

pps_extension_data_flag: 1

- Enhancement layer PPS - pps_extension_6bits: 0x5

pps_extension_data_flag: 0

pps_extension_data_flag: 0

pps_extension_data_flag: 1

- Base layer SPS - sps_extension_6bits: 0

- Enhancement layer SPS - sps_extension_6bits: 0x63

sps_extension_data_flag: 0

sps_extension_data_flag: 1

Coding structure: Low Delay P

Number of Frames: 4

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 3.1)

Number of layers: 2

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 3.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 3.1

Each layer resolution:

- Layer 0 resolution - Coded:  1280x720

- Layer 1 resolution - Coded:  1280x720

Frame rate: 30 fps for both layers

SHM Version: SHM Dev branch, rev 1007

### Test bitstreams – LAYERID63

#### Test bitstreams #LAYERID63_A_HHI

Bitstream file name: LAYERID63_A_HHI_1.bit

Purpose: Add random data under nuh_layer_id equal to 63

Conformance: SHM-dev r1086

Number of frames: 10

    - Test bitstreams – POC (Missing)

### Test bitstreams – HYBRID (Missing)

### Test bitstreams – INBLD

#### Test bitstreams #INBLD_A_NOKIA

Bitstream file name: INBLD_A_NOKIA_1.bit

Bitstream feature name: Independent non-base layer decoding

Bitstream feature description:

The bitstream contains two layers having layer IDs 2 and 3. Layer with layer ID 2 is an independent non-base layer.

Base layer and layer with layer ID 1 are not present in the bitstream so only OLS_4 is decoded.

Coding structure: Low Delay P

Number of Frames: 4

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

- OLS_2 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

Layer: 2, PTL Idx: 3 (Main 4)

- OLS_3 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

Layer: 2, PTL Idx: 3 (Main 4)

Layer: 3, PTL Idx: 2 (Scalable Main 4)

- OLS_4 -   Layer: 2, PTL Idx: 3 (Main 4) 		(additional layer set)

Layer: 3, PTL Idx: 2 (Scalable Main 4)

Number of layers: 4

Profile, Tier, Level information: Num PTL = 4

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

- PTL_Idx 3 - Profile: MAIN [Independent non-base layer PTL]

Level: 4

Each layer resolution: 1280x720

Frame rate: 24 fps

SHM Version: SHM Dev branch, rev 1015

### Test bitstreams – SIM

#### Test bitstreams #SIM_A_IDCC

Bitstream file name: SIM_A_IDCC_1.bit

Bitstream feature name: Simulcast

Bitstream feature description:

Each enhancement layer video is coded independently without prediction from any lower layers

- Layer 1	: 	NumSamplePredRefLayers1 = 0

- Layer 1	: 	SamplePredRefLayerIds1 = 0

- Layer 1	: 	NumMotionPredRefLayers1 = 0

- Layer 1	: 	MotionPredRefLayerIds1 = 0

- Layer 1	: 	NumActiveRefLayers1 = 0

- Layer 1	: 	PredLayerIds1 = 0

Coding structure: Random Access with 4 temporal sub-layers

Number of layers: 2

Number of Frames: 8

Output Layer Sets:

OLS_0: 	Layer 0, PTL Idx: 1 (Main 3.1)

OLS_1: 	Layer 0, PTL Idx: 1 (Main 3.1)

Layer 1, PTL Idx: 2 (Scalable Main 4)

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

- DefaultTargetOutputLayerIdc   : 0

Each layer resolution:

- Layer 0 resolution - Output: 960x540

Coded:  960x544

- Layer 1 resolution - Output: 1920x1080

Coded:  1920x1280

Frame rate: 24 fps for both layers

SHM Version: SHM Dev branch, rev 1021

#### Test bitstreams #SIM_B_IDCC

Bitstream file name: SIM_B_IDCC_1.bit

Bitstream feature name: Simulcast

Bitstream feature description:

Each enhancement layer video is coded independently without prediction from any lower layers.

- Layer 1	: 	NumSamplePredRefLayers1 = 0

- Layer 1	: 	SamplePredRefLayerIds1 = 0

- Layer 1	: 	NumMotionPredRefLayers1 = 0

- Layer 1	: 	MotionPredRefLayerIds1 = 0

- Layer 1	: 	NumActiveRefLayers1 = 0

- Layer 1	: 	PredLayerIds1 = 0

- NumLayerSets                  : 2

- NumLayerInIdList1             : 2

- LayerSetLayerIdList1          : 0 1

- NumAddLayerSets               : 1

- NumHighestLayerIdx0           : 1

- HighestLayerIdx0              : 0

- NumOutputLayerSets            : 3

- NumOutputLayersInOutputLayerSet     : 2 1

- ListOfOutputLayers1	          : 0 1

- ListOfProfileTierLevelOls1    : 1 2

- ListOfOutputLayers2	          : 1

- ListOfProfileTierLevelOls2    : 1

Coding structure: Random Access with 4 temporal sub-layers

Number of layers: 2

Number of Frames: 8

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

- OLS_0 -   Layer: 1, PTL Idx  1 (Main 3.1)

-

DefaultTargetOutputLayerIdc   : 2

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution:

- Layer 0 resolution - Output: 960x540

Coded:  960x544

- Layer 1 resolution - Output: 1920x1080

Coded:  1920x1280

Frame rate: 24 fps for both layers

SHM Version: SHM Dev branch, rev 1021

### Test bitstreams – SLLEV

#### Test bitstreams #SLLEV_A_VIDYO

Bitstream file name: SLLEV_A_VIDYO_1.bit

Bitstream feature name: Sub-layer level signalling

Bitstream feature description:

In this bitstream, sub_layer_level_present_flag[i] is TRUE for PTL 1 & PTL 2. sub_layer_level_idc[ i ] value is as below.

- vps_max_sub_layers_minus1: 2

- profile_tier_level() structure 0

profilePresentFlag: 1, maxNumSubLayersMinus1: 2

general_profile_idc: 1

general_level_idc: 93

- profile_tier_level() structure 1

profilePresentFlag: 0, maxNumSubLayersMinus1: 2

general_level_idc: 93

sub_layer_level_present_flag[ 0 ]: 1

sub_layer_level_present_flag[ 1 ]: 1

sub_layer_level_idc[ 0 ]: 90

sub_layer_level_idc[ 1 ]: 90

- profile_tier_level() structure 2

profilePresentFlag: 1, maxNumSubLayersMinus1: 2

general_profile_idc: 7

general_level_idc: 123

sub_layer_level_present_flag[ 0 ]: 1

sub_layer_level_present_flag[ 1 ]: 1

sub_layer_level_idc[ 0 ]: 120

sub_layer_level_idc[ 1 ]: 120

Number of Frames: 8

Output Layer Sets:

- OLS_0 - Layer: 0, PTL Idx: 1 (Main 3.1, 3)

- OLS_1 - Layer: 0, PTL Idx: 1 (Main 3.1, 3)

Layer: 1, PTL Idx: 2 (Scalable Main 4.1, 4)

Number of layers: 2

Max number of Temporal sub-layers: 3

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 3.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

Sub-layer 0 level: 3

Sub-layer 1 level: 3

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4.1

Sub-layer 0 level: 4

Sub-layer 1 level: 4

Each layer resolution:

- Layer 0 resolution - Coded:  640x360

- Layer 1 resolution - Coded:  1280x720

Frame rate: 60 fps for both layers

SHM Version:

This bitstream is compatible with SHM rev 1025

### Test bitstreams – ALPHA

#### Test bitstreams #ALPHA_A_BBC

Bitstream file name: ALPHA_A_BBC_1.bit

Bitstream feature: Texture signal plus alpha plane encoded as auxiliary picture along with the associated SEI message. The following encoder parameter have been used to specify the information associated with the alpha channel:

- SEIAlphaCancelFlag           : 0

- SEIAlphaUseIdc               : 0

- SEIAlphaBitDepthMinus8       : 0

- SEIAlphaTransparentValue     : 0

- SEIAlphaOpaqueValue          : 255

- SEIAlphaIncrementFlag        : 0

- SEIAlphaClipFlag             : 1

- SEIAlphaClipType             : 0

The sequence as texture signal is the VenueVu5 described in document JCTVC-K0296. Please note the following copyright information:

The test sequence and all intellectual property rights therein remain the property of the owner below. This material can only be used for the purpose of academic research and development of standards. This material cannot be distributed with charge. The owner makes no warranties with respect to the material and expressly disclaims any warranties regarding its fitness for any purpose.

Owner of these sequences:

- Owner: British Broadcasting Corporation.

- Production: Crystal CG.

Output Layer Sets (OLS):

- OLS_0: Layer 0, (Main 4.1)

- OLS_1: Layer 0, (Main 4.1)

Layer 1, (Scalable-Main 4.1)

Number of layers: 2

Profile, tier, and level for each layer: (Num PTL 3)

- Profile L0: main

- Profile L1: main

- Profile L2: scalable-main

- Level   L0: 4.1

- Level   L1: 4.1

- Level   L2: 4.1

Picture size of each layer:

- Layer 0: 1920x1080, 8bit, color space BT.709

- Layer 1: 1920x1080, 8bit, monochrome

Frame rate of each layer:

- Layer 0: 30 Hz

- Layer 1: 30 Hz

SHM version:

8.0, development branch rev 1037

    - Test bitstreams – DEPTH

#### Test bitstreams #DEPTH_A_NOKIA

Bitstream file name: DEPTH_A_NOKIA_1.bit

Bitstream feature name: Depth auxiliary layers

Bitstream feature description:

Layers 0 and 1 are texture layers and layers 2 and 3 are associated auxiliary depth layers.

Coding structure: Low Delay P

Number of Frames: 4

Output Layer Sets:

- OLS_0 -   Layer: 0, PTL Idx: 1 (Main 3.1)

- OLS_1 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

- OLS_2 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

Layer: 2, PTL Idx: 2 (Scalable Main 4)

- OLS_3 -   Layer: 0, PTL Idx: 1 (Main 3.1)

Layer: 1, PTL Idx: 2 (Scalable Main 4)

Layer: 2, PTL Idx: 2 (Scalable Main 4)

Layer: 3, PTL Idx: 2 (Scalable Main 4)

Number of layers: 4

Profile, Tier, Level information: Num PTL = 3

- PTL_Idx 0 - Profile: MAIN [V1 Whole Bitstream PTL]

Level: 4.1

- PTL_Idx 1 - Profile: MAIN [Base layer PTL]

Level: 3.1

- PTL_Idx 2 - Profile: SCALABLE MAIN [Enhancement layer PTL]

Level: 4

Each layer resolution: 1024x768

Frame rate: 30 fps

SHM Version: SHM Dev branch, rev 1015

    - Test bitstreams – OVERLAY (Missing)

### Test bitstreams – LAYERS (Missing)

# Bitstream generation instructions

Volunteers should upload the bitstream and associated files in a zip archive.  Files should be uploaded to the following ftp site.

ftp://ftp3.itu.ch/jctvc-site/dropbox/

Please contact with teruhikos@jp.sony.com or jill@vidyo.com to get the access information to dropbox.

Once a bitstream is uploaded to the dropbox, send an email to teruhikos@jp.sony.com or jill@vidyo.com.

Then it will be moved to the downloading area of the ftp site:

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/draft_SHVC_conformance/

All files inside the zip archive (name.zip) should have the same base file name, with extensions as described in the list below. Each of the files is further described in the following sections.

- Bitstream file (name.bit)

- Description of the bitstream (name.txt)

- MD5 check sum of decoded YUV file for each layer (name-L#. md5)

- Output picture log for each output layer set (name-OLS#.opl)

The following naming convention should be used for the base name.

name = Feature_BistreamID_Source_Version

Feature is summarized in Section 2.

BitstreamID is a single capital letter, used when more than one bitstream is used for a particular feature.

Source is the name of the company who provided the bitstream.

Version is the version number of the file, in case a bitstream must be replaced because of an identified problem.

For example, if the archive is named "feature_A_company_1.zip", and the bitstream contains two layer sets and three layers, the files included in the zip archive would be named:

feature_A_company_1.bit

feature_A_company_1.txt

feature_A_company_1-OLS0.opl

feature_A_company_1-OLS1.opl

feature_A_company_1-L0.md5

feature_A_company_1-L1.md5

feature_A_company_1-L2.md5

## Bitstream file (name.bit)

Except for the hybrid scalability case, all layers are included within the same bitstream file.

Hybrid scalability case TBD.

## Description of the bitstream (name.txt)

In a short text file which describes the bitstream, the following information should be provided.

- Bitstream file name

- Explanation of bitstream features

- Number of output layer sets in the bitstream

- Number of layers present in the bitstream

- Profile, tier, and level for each layer in each output layer set in the bitstream

- Picture size of each layer

- Frame rate of each layer (if available)

- SHM version number used to generate the bitstream, if appropriate

Picture size and frame rate is necessary to check HRD conformance of the bitstreams.

If the SHM was used to generate the bitstream, the SHM version number used should be noted.

## MD5 file per layer containing the MD5 check sum of decoded YUV file for each layer (name-L#.md5)

An md5 file should be provided for each layer present in the bitstream, regardless of whether or not the layer is a target output layer of an output layer set. The name of the md5 file should contain the layer number of the layer, as represented by the nuh_layer_id value.

Each md5 file should contain the hash value for the decoder’s output YUV file for that layer (which contains all decoded pictures of the layer, in output order).  The md5 file should contain only the hash value, and should not contain any text remarks, which, if needed, can be provided in the bitstream description file.

The recommended md5sum tool on UNIX is described at this link, http://en.wikipedia.org/wiki/Md5sum.

## Output picture log file for each output layer set (name-OLS#.opl)

An output picture log file should be provided for each output layer set present in the bitstream. The name of the output picture log file should contain the output layer set number.

The SHM decoder can be configured to create output picture log files for a bitstream, by using a –confMode command line option. This feature is expected to be included in the SHM-8.0 release, and is now available in the SHM-dev branch.  For example, to decode the n-th output layer set of the bitstream bitstream.bit, along with generating the relevant log files, the following command line could be used:

TAppDecoder.exe –b bitstream.bit –confMode 1 –olsidx n

Additionally, the command line option “-confPrefix” could be used to prefix the names of the log files generated. The default name used would be "decodedBitstream", e.g. decodedBitstream-L#.yuv, decodedBitstream-OLS#.opl.

Each output picture log file contains one row for each output picture in the bitstream, in output order. If a picture is not a target output picture for a particular output layer set, or the value of the PicOutputFlag variable for a picture is equal to 0, the information for the picture should not be included in the output picture log file.

The following information is included in a row for each output picture:

- nuh_layer_id

- PicOrderCntVal

- MD5 checksum for the Y component

- MD5 checksum for the U component

- MD5 checksum for the V component

The first few lines of an example output picture log file for a 2-layer bitstream is shown below.

LayerId      POC    MD5

------------------------

0        0    MD5:d4a80b3da02ce37a0f9c74b84eebd191,4cccea85a761256502c2a0dd50e243d7,dc72e4796ab321b27a16a7fd316b9ddd

1        0    MD5:c48fc168d2052062a5370fd3d75ab510,f6b1bb462b32eadb9b70d98dc8caa40f,0cff1459777fb5ddfda6371eb14208a3

0        1    MD5:a7e7eff85f2773ff19a43e0ddcfa2aa3,2b083d69271697af4a780eb5ae70bd5a,70e1bca8703d5a8dc6a5be8dd5aed5d6

1        1    MD5:31db80f40e5d56743c8f87ae97954d39,20ddc453e7a9168f5d2cb48e545d9c2c,df53c5e1e6e88188a88a8feb13bcab2a

0        2    MD5:e2351d657bb1bfaf38bab96ecce6530e,6fc36992be946374bad85f6dba719dbb,10652af9aa60dad51700a5f760201419

1        2    MD5:221f765c2f80c5c6bfe91628b17e593f,7e89a200590e35391b74aff5073a1b64,69a1b29df26c7c3cfc861ba493cbdb19

0        3    MD5:f70d9bee7e314afb20581a7ab6d6481b,5808ba66486b1b96caea908353b38033,4c0c25773eed62547b320357adcd8323

1        3    MD5:6546b65ad49626c0e7380d2f9c71c4f3,fb3e56671740f88125c53af27ebb6ff2,28a943b8f4d0c84693a91f7da2d1f21c

0        4    MD5:e63fd13ce2708fcf9e3d9f1addd622e2,c60fcf34bd0dd5c449354e2b6faf2940,b0f236ad816d535037cb2fe5ffa6b72a

1        4    MD5:3b580b825e65d0fb4c4f8fd1812036c6,94eca9606fbc2f0529739475d5ba5a5f,7a37aa0e162b3fc988ef50c5187533cc

0        5    MD5:6c4e657076424cc390b0e5829157bef4,4e77418d2db800bcfd8ed26f042c6f72,c75cbf596dcbdeb8903d74d2d12e7ba2

1        5    MD5:b4e28798dfdf50b9012b9f9fa433b91a,632424ddb5f1ce9e80de6f1e58cd0d1f,ae63094bb9abef4f7204b0c51a283ce0

0        6    MD5:32e9c63a8bcc9fd8d4803d34427609a1,301dfae4b86d17651f4f165c0f361b53,21bbc1d163c8a6c57ae1b07ccb8dfe4f

1        6    MD5:b27a004c9190c627415a14c582820cbb,e28dac76acb8e91a371eab142664b95e,95c38cb60f3a98efbf855d8846852a57

0        7    MD5:b59b6f712251fb146de05236f3d62d69,5133f701cd84278c3a5000428ffc06db,7e925a90f137dab5552e4498026d6b6c

1        7    MD5:e2d75b80f2b4b30b5b534d3cc2e4af4c,24a413e09de9807e8180fdb9db5dc6d7,05c931b9051b8dc58374f2fbe3e0aa71

0        8    MD5:6abfa89ea2ab076668aae713ca8ad252,1fcf393e5648540e8f2abc7a5056f5c4,f6360d3839e4896266467b281c37f252

1        8    MD5:b3b01379ba08916ef6b1b35f7d9ad51c,7fa38083093d5a57cfca3680f35c9c93,3c4aa75b2f95d5ae40f24c14f677336f

0        9    MD5:992cd6d0574bf0026b477ae35062f20e,b820cd9b8874684dfb1b6c7f01b8142e,df535187e3be8eeb22e54ac43718af72

1        9    MD5:b3b01379ba08916ef6b1b35f7d9ad51c,c5e8f694e90712a783b09a7d4bd4dd1e,9899e7f2a7079f16637ac778be73a930