| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0021 |
| --- | --- |

| Title: | CE1: Summary report on chroma deblocking filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Alexis Michael Tourapis (primary) Woo-Shik Kim Krishna Rapaka Xiaoyu Xiu | Email: | atourapis@apple.com wooshikk@qti.qualcomm.com krapaka@qti.qualcomm.com Xiaoyu.Xiu@InterDigital.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document summarizes the Core Experiment 1 (CE1) on chroma deblocking filtering.

# Introduction

During the 21st JCT-VC meeting in Warsaw it was determined that a Core Experiment (CE 1) would be formed [1] to evaluate whether chroma deblocking could be improved in the context of the Screen Content extension [4] of HEVC [3]. The CE consisted of two tests. This contribution describes the test conditions used and the final test results achieved.

# Test conditions

The tests were conducted under the SCM-5.2 common test conditions specified in [2]. However, since the intention of the test was to mainly resolve issues with chroma deblocking on natural sequences, three additional sequences were included in the test and used primarily for subjective evaluations. The first two sequences, named FireEater and Tibul, were provided by Technicolor (Dolby SDR grade) and ere used for the 4:2:0 chroma format tests. FireEater was also used, after upconversion to 4:4:4 for the 4:4:4 tests as well. The third sequence, named CampFire Party, was provided by the Shanghai Jiao Tong University [5] and was used for the 4:4:4 chroma format tests only. This third sequence was tested after either scaling or cropping to a 1920x1080 resolution to reduce the workload of the experiments. For the final evaluation only the cropped version was used. Only Low Delay B tests were performed for these sequences to avoid temporal masking of artifacts.

Expert viewer subjective test analysis is also to be performed during the 22nd meeting in Geneva.

# Test descriptions and results

## Method 1: Chroma deblocking when Bs >0

- Proponent: Apple and Qualcomm, JCTVC-U0138

- Crosschecker: InterDigital

- Description:

In this mode chroma deblocking is performed for all edges that have a Bs value larger than 0. The same deblocking filter as is currently specified in the HEVC specification for edges with a Bs value equal to 2 is reused. This mode applies to all chroma formats.

- Results:

Objective results using this method commonly showed a loss in objective numbers, if this method is to be performed exclusively on an entire sequence. This was not unexpected given the characteristics of the chroma deblocker. However, it was also pointed out that if the scheme is combined in an RD optimal manner with the existing deblocking scheme, at least at the sequence level, then no loss, and even small gains could be achieved. No test combining the two methods at the picture level was performed.

The summary results for these tests are provided below:

- Lossy, 444, full-frame IBC

| 444, full-frame IBC | Method 1 |  |  | RD optimal Method 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All Intra | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.8% | 0.8% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.4% | 0.4% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.8% | 0.8% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.7% | 0.6% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | - |  |  |
| Dec Time[%] | 103% |  |  | - |  |  |

| 444, full-frame IBC | Method 1 |  |  | RD optimal Method 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Random Access | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.2% | 2.9% | 2.7% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.3% | 2.9% | 2.9% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.2% | 3.5% | 4.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.5% | -1.3% | 0.0% | -0.9% | -2.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.2% | 2.4% | 2.7% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 2.4% | 2.3% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.1% | 3.7% | 2.6% | 0.0% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -1.2% | -0.5% | 0.0% | -1.0% | -0.9% |
| Enc Time[%] | 107% |  |  | - |  |  |
| Dec Time[%] | 107% |  |  | - |  |  |

| 444, full-frame IBC | Method 1 |  |  | RD optimal Method 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Low Delay B | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 1.9% | 7.8% | 7.9% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 2.3% | 12.7% | 12.4% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 1.3% | 8.1% | 9.8% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 2.0% | -1.2% | -0.2% | -0.7% | -2.9% |
| YUV, text & graphics with motion, 1080p & 720p | 1.1% | 6.4% | 6.7% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.5% | 7.4% | 7.3% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.6% | 8.7% | 5.2% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | -0.1% | -1.0% | -0.2% | -0.1% | -1.1% | -1.2% |
| Enc Time[%] | 103% |  |  | - |  |  |
| Dec Time[%] | 105% |  |  | - |  |  |

- Lossy, 444, 4x1-CTU IBC

| 444, 4x1-CTU IBC | Method 1 |  |  | RD optimal Method 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All Intra | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.5% | 0.5% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.3% | 0.3% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.5% | 0.5% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.5% | 0.4% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 91% |  |  | - |  |  |
| Dec Time[%] | 91% |  |  | - |  |  |

| 444, 4x1-CTU IBC | Method 1 |  |  | RD optimal Method 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Random Access | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.2% | 2.6% | 2.4% | 0.00% | 0.00% | 0.00% |
| RGB, mixed content, 1440p & 1080p | 0.3% | 2.7% | 2.6% | 0.00% | 0.00% | 0.00% |
| RGB, Animation, 720p | 0.1% | 3.5% | 3.9% | 0.00% | 0.00% | 0.00% |
| RGB, camera captured, 1080p | 0.0% | 0.6% | -1.3% | -0.18% | -0.74% | -2.88% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 2.2% | 2.3% | 0.00% | 0.00% | 0.00% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 2.1% | 2.0% | 0.00% | 0.00% | 0.00% |
| YUV, Animation, 720p | 0.0% | 3.6% | 2.5% | 0.00% | 0.00% | 0.00% |
| YUV, camera captured, 1080p | -0.1% | -1.2% | -0.6% | -0.11% | -1.20% | -1.31% |
| Enc Time[%] | 91% |  |  | - |  |  |
| Dec Time[%] | 91% |  |  | - |  |  |

| 444, 4x1-CTU IBC | Method 1 |  |  | RD optimal Method 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Low Delay B | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 1.7% | 7.6% | 7.9% | 0.00% | 0.00% | 0.00% |
| RGB, mixed content, 1440p & 1080p | 2.2% | 12.5% | 12.2% | 0.00% | 0.00% | 0.00% |
| RGB, Animation, 720p | 1.4% | 8.0% | 9.8% | 0.00% | 0.00% | 0.00% |
| RGB, camera captured, 1080p | 0.2% | 2.0% | -1.2% | -0.03% | -0.84% | -2.09% |
| YUV, text & graphics with motion, 1080p & 720p | 0.9% | 6.3% | 6.5% | 0.00% | 0.00% | 0.00% |
| YUV, mixed content, 1440p & 1080p | 0.5% | 7.2% | 7.4% | 0.00% | 0.00% | 0.00% |
| YUV, Animation, 720p | 0.6% | 8.3% | 5.0% | 0.00% | 0.08% | -0.01% |
| YUV, camera captured, 1080p | -0.1% | -1.1% | -0.2% | -0.03% | -1.00% | -0.95% |
| Enc Time[%] | 94% |  |  | - |  |  |
| Dec Time[%] | 95% |  |  | - |  |  |

- Lossy, 420, full-frame IBC

| 420, full-frame IBC | Method 1 |  |  | RD optimal Method 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All Intra | G/Y | B/U | R/V | G/Y | B/U | R/V |
| YUV, TGM, 1080p & 720p | 0.0% | 0.6% | 0.7% | 0.00% | -0.02% | -0.03% |
| YUV, mixed, 1440p & 1080p | 0.0% | 0.4% | 0.4% | 0.00% | 0.02% | 0.00% |
| YUV, Animation, 720p & 768p | 0.0% | 0.1% | 0.1% | 0.00% | -0.01% | -0.01% |
| Enc Time[%] | 104% |  |  | - |  |  |
| Dec Time[%] | 100% |  |  | - |  |  |

| 420, full-frame IBC | Method 1 |  |  | RD optimal Method 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Random Access | G/Y | B/U | R/V | G/Y | B/U | R/V |
| YUV, TGM, 1080p & 720p | 0.1% | 2.3% | 2.3% | 0.00% | -0.01% | 0.00% |
| YUV, mixed, 1440p & 1080p | 0.0% | 1.7% | 2.1% | -0.01% | -0.02% | -0.01% |
| YUV, Animation, 720p & 768p | 0.1% | 6.8% | 6.9% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 92% |  |  | - |  |  |
| Dec Time[%] | 94% |  |  | - |  |  |

| 420, full-frame IBC | Method 1 |  |  | RD optimal Method 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Low Delay B | G/Y | B/U | R/V | G/Y | B/U | R/V |
| YUV, TGM, 1080p & 720p | 0.4% | 4.0% | 4.0% | 0.00% | -0.02% | -0.01% |
| YUV, mixed, 1440p & 1080p | 0.1% | 3.9% | 5.5% | 0.00% | 0.08% | -0.07% |
| YUV, Animation, 720p & 768p | 0.5% | 15.8% | 21.0% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 105% |  |  | - |  |  |
| Dec Time[%] | 104% |  |  | - |  |  |

- Additional Sequences using Low Delay B configuration

|  | Y | Cb | Cr |
| --- | --- | --- | --- |
| Tibul 4:2:0 | -0.2% | -3.8% | -3.0% |
| FireEater 4:2:0 | -0.1% | -2.3% | -1.5% |

## Method 2: Chroma deblocking as luma

- Proponent: Apple and Qualcomm, JCTVC-U0138

- Crosschecker: InterDigital

- Description:

In this mode chroma deblocking is performed using exactly the same process and filters as specified for the luma component. This mode only applies to 4:4:4 formats.

- Results:

Unlike method 1, method 2 demonstrated consistently objective gains over the anchors. This is likely due to the improved characteristics and adaptability of the luma deblocking filter. Sequence level RD optimal combination provided some small improvements in some scenarios. Figure 3-1 and Figure 3-2 demonstrated the subjective impact for a segment in sequence CampfireParty using Method 2.

The summary results for these tests are provided below:

- Lossy, 444, full-frame IBC

| 444, full-frame IBC | Method 2 |  |  | RD optimal Method 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All Intra | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.6% | -0.7% | 0.0% | -0.6% | -0.7% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.9% | -0.8% | 0.0% | -0.9% | -0.8% |
| RGB, Animation, 720p | 0.0% | -1.3% | -1.4% | 0.0% | -1.3% | -1.4% |
| RGB, camera captured, 1080p | 0.0% | -0.5% | -0.1% | 0.0% | -0.3% | -0.2% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.4% | -0.3% | 0.0% | -0.4% | -0.4% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.6% | -0.7% | 0.0% | -0.6% | -0.7% |
| YUV, camera captured, 1080p | 0.0% | -0.8% | -1.1% | 0.0% | -0.7% | -1.1% |
| Enc Time[%] | 105% |  |  | - |  |  |
| Dec Time[%] | 109% |  |  | - |  |  |

| 444, full-frame IBC | Method 2 |  |  | RD optimal Method 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Random Access | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.8% | -1.0% | -0.1% | -0.8% | -1.0% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -1.6% | -1.6% | -0.1% | -1.6% | -1.6% |
| RGB, Animation, 720p | -0.2% | -1.2% | -1.3% | -0.2% | -1.2% | -1.3% |
| RGB, camera captured, 1080p | -0.2% | -2.0% | -3.6% | -0.2% | -2.0% | -3.6% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.2% | 0.0% | 0.0% | -0.3% | -0.2% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.3% | -0.3% | -0.1% | -0.5% | -0.4% |
| YUV, Animation, 720p | -0.1% | 0.3% | 0.4% | 0.0% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | -0.2% | -2.7% | -2.0% | -0.2% | -2.6% | -2.2% |
| Enc Time[%] | 106% |  |  | - |  |  |
| Dec Time[%] | 107% |  |  | - |  |  |

| 444, full-frame IBC | Method 2 |  |  | RD optimal Method 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Low Delay B | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.3% | -0.5% | -0.8% | -0.3% | -0.6% | -0.9% |
| RGB, mixed content, 1440p & 1080p | -0.8% | -1.3% | -1.2% | -0.8% | -1.3% | -1.3% |
| RGB, Animation, 720p | -1.1% | 0.5% | 0.3% | -0.2% | -0.2% | -0.1% |
| RGB, camera captured, 1080p | -0.8% | -1.1% | -4.3% | -0.8% | -1.1% | -4.3% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.7% | 1.2% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | -0.2% | 1.4% | 1.9% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 4.1% | 3.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | -0.1% | -2.2% | -0.7% | -0.1% | -2.2% | -2.1% |
| Enc Time[%] | 103% |  |  | - |  |  |
| Dec Time[%] | 105% |  |  | - |  |  |

- Lossy, 444, 4x1-CTU IBC

| 444, 4x1-CTU IBC | Method 2 |  |  | RD optimal Method 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All Intra | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.7% | -0.8% | 0.0% | -0.7% | -0.8% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -1.0% | -1.0% | 0.0% | -1.0% | -1.0% |
| RGB, Animation, 720p | 0.0% | -1.3% | -1.4% | 0.0% | -1.3% | -1.4% |
| RGB, camera captured, 1080p | 0.0% | -0.5% | -0.1% | 0.0% | -0.5% | -0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.5% | -0.5% | 0.0% | -0.5% | -0.5% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.4% | -0.3% | 0.0% | -0.4% | -0.3% |
| YUV, Animation, 720p | 0.0% | -0.6% | -0.7% | 0.0% | -0.6% | -0.7% |
| YUV, camera captured, 1080p | 0.0% | -0.8% | -1.1% | 0.0% | -0.8% | -1.1% |
| Enc Time[%] | 90% |  |  | - |  |  |
| Dec Time[%] | 94% |  |  | - |  |  |

| 444, 4x1-CTU IBC | Method 2 |  |  | RD optimal Method 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Random Access | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.9% | -1.1% | -0.05% | -0.94% | -1.09% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -1.8% | -1.8% | -0.09% | -1.77% | -1.80% |
| RGB, Animation, 720p | -0.1% | -1.1% | -1.2% | -0.14% | -1.06% | -1.18% |
| RGB, camera captured, 1080p | -0.2% | -1.9% | -3.6% | -0.24% | -1.94% | -3.64% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.4% | -0.2% | -0.01% | -0.39% | -0.30% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.5% | -0.4% | -0.09% | -0.55% | -0.44% |
| YUV, Animation, 720p | 0.0% | 0.3% | 0.3% | 0.00% | -0.04% | -0.11% |
| YUV, camera captured, 1080p | -0.1% | -2.7% | -2.0% | -0.12% | -2.52% | -2.20% |
| Enc Time[%] | 89% |  |  | - |  |  |
| Dec Time[%] | 92% |  |  | - |  |  |

| 444, 4x1-CTU IBC | Method 2 |  |  | RD optimal Method 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Low Delay B | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.3% | -0.6% | -0.8% | -0.27% | -0.61% | -0.83% |
| RGB, mixed content, 1440p & 1080p | -0.7% | -1.4% | -1.3% | -0.68% | -1.36% | -1.34% |
| RGB, Animation, 720p | -1.0% | 0.5% | 0.3% | -0.13% | -0.18% | -0.14% |
| RGB, camera captured, 1080p | -0.8% | -1.0% | -4.2% | -0.80% | -1.01% | -4.24% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | 0.5% | 1.1% | -0.03% | -0.04% | -0.08% |
| YUV, mixed content, 1440p & 1080p | -0.2% | 1.2% | 1.8% | -0.06% | -0.06% | -0.06% |
| YUV, Animation, 720p | 0.0% | 4.0% | 2.8% | 0.00% | 0.00% | 0.00% |
| YUV, camera captured, 1080p | -0.1% | -2.4% | -0.6% | -0.13% | -2.29% | -1.99% |
| Enc Time[%] | 89% |  |  | - |  |  |
| Dec Time[%] | 92% |  |  | - |  |  |

- Additional Sequences using Low Delay B configuration

|  | Y | Cb | Cr |
| --- | --- | --- | --- |
| CampfireParty 4:4:4 | -0.3% | -0.3% | -1.4% |
| FireEater 4:4:4 | -0.6% | -4.5% | -4.7% |

Figure 3-1. Subjective Quality: CampfireParty 4:4:4 QP 32 - Anchor

Figure 3-2. Subjective Quality: CampfireParty 4:4:4 QP 32 – Method 2

- CE-related documents

JCTVC-V0071, “CE1: cross-verification of method 2 on 444 and method 1 on 420”

# References

- A.M. Tourapis, K. Rapaka, and X. Xiu (CE coordinators), “Description of Core Experiment 1 (CE1): Chroma deblocking filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-U1101, Jun. 2015.

- H. Yu, R. Cohen, K. Rapaka, and J. Xu (editors), “Common Test Conditions for Screen Content Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-U1015, Jul. 2015.

- ISO/IEC 23008:2014 Information technology -- High efficiency coding and media delivery in heterogeneous environments -- Part 2: High efficiency video coding, Second Edition

- R. Joshi, S. Liu, G. Sullivan, G. Tech, J. Xu, and Y. Ye, “High Efficiency Video Coding (HEVC) Screen Content Coding: Draft 4”, MPEG/JCTVC document M36870/JCTVC-U1005, Warsaw, PL, Jun. 2015

- SJTU 4K Video Sequences, http://medialab.sjtu.edu.cn/web4k/index.html