| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0033_r1 |
| --- | --- |

| Title: | Comparison of Compression Performance of HEVC Screen Content Coding Extensions Test Model 5 with AVC High 4:4:4 Predictive profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Gary J. Sullivan | Email: | libin@microsoft.com jzxu@microsoft.com garysull@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This contribution is a study of the relative objective (i.e. PSNR-based) compression performance of HEVC Screen Content Coding (SCC) Test Model 5 (SCM 5) and AVC High 4:4:4 Predictive Profile. It builds upon the prior work reported in JCTVC-G399, JCTVC-H0360, JCTVC-I0409, JCTVC-J0236, JCTVC-K0279, JCTVC-L0322, JCTVC-M0329, JCTVC-O0184, JCTVC-P0213, JCTVC-R0101, JCTVC-S0084, JCTVC-T0042, and JCTVC-U0051 – updating the results by using the latest available reference software (JM-19.0, HM-16.6+SCM-5.2), profile and test model designs, and SCC common test conditions (CTC) test sequences. The overall results indicate that for screen content CTC sequences, the HEVC SCC Test Model 5 improves quite substantially over JM-19.0. For example, for RGB text and graphics with motion (TGM) 1080p&720 sequences, HEVC SCC Test Model 5 saves 86%, 81%, and 78% bits for AI, RA and LB lossy coding over JM-19.0, respectively (the corresponding numbers are 86%, 80% and 78% in JCTVC-U0051, which compares HM-16.4+SCM-4.0 with JM-18.6, without the ChineseEditing sequence).

# Performance Comparison

We used HM-16.6+SCM-5.2 and JM-19.0 in the tests. The JM software was configured with the same coding structure and referencing structure as HM using the “HM-like” configurations. High 4:4:4 Predictive profile is used in JM. SCC CTC [13] test conditions (e.g. internal bit depth, intra period, QP setting, etc.) are followed in the comparison.

It should be noted that, for screen test sequences, SCM-5.2 improves quite a lot over JM-19.0. If we still use the same QP values to generate bitstreams and calculate BD-Rate, the PSNR overlapping range may be small (or even not overlapped). So we adjust the QP values to generate JM-19.0 bitstreams. For Desktop and Console, we use QP 12, 17, 22, 27 to generate JM-19.0 bitstreams. For FlyingGraphics, ChineseEditing, WebBrowsing, Map, Programming, SlideShow, BasketballScreen, MissionControl2, MissionControl3, and ChinaSpeed, we use QP 17, 22, 27, 32 to generate JM-19.0 bitstreams. For other sequences, we use the QP 22, 27, 32, 37 to generate JM-19.0 bitstreams.

The configuration files used to generate JM-19.0 bitstreams are provided in cfg.zip. The command lines used are something like

lencod_JM19.0 -d ..\..\cfg\encoder_JM_Intra_RExt.cfg -f ..\..\cfg\per-sequence-JM\WebBrowsing_RGB.cfg -p QPISlice=22 -p QPPSlice=23 -p QPBSlice=23 -p IntraPeriod=1 -p ReconFile="WebBrowsing_RGB_JM_Intra_RExt_JM19.0_Q22.yuv" -p OutputFile="WebBrowsing_RGB_JM_Intra_RExt_JM19.0_Q22.bin" -p FramesToBeEncoded=300 -p SourceBitDepthRescale=1 -p OutputBitDepthLuma=8 -p OutputBitDepthChroma=8 -p ProfileIDC=244

lencod_JM19.0 -d ..\..\cfg\encoder_JM_Intra_RExt.cfg -f ..\..\cfg\per-sequence-JM\WebBrowsing_444.cfg -p QPISlice=0 -p IntraPeriod=1 -p ReconFile="WebBrowsing_444_JM_Intra_RExt_JM19.0_Q0.yuv" -p OutputFile="WebBrowsing_444_JM_Intra_RExt_JM19.0_Q0.bin" -p FramesToBeEncoded=300 -p SourceBitDepthRescale=1 -p OutputBitDepthLuma=8 -p OutputBitDepthChroma=8 -p ProfileIDC=244 -p LosslessCoding=1

lencod_JM19.0 -d ..\..\cfg\encoder_JM_RA_B_RExt.cfg -f ..\..\cfg\per-sequence-JM\WebBrowsing_RGB.cfg -p QPISlice=32 -p QPPSlice=33 -p QPBSlice=33 -p IntraPeriod=32 -p ReconFile="WebBrowsing_RGB_JM_RA_B_RExt_JM19.0_Q32.yuv" -p OutputFile="WebBrowsing_RGB_JM_RA_B_RExt_JM19.0_Q32.bin" -p FramesToBeEncoded=300 -p SourceBitDepthRescale=1 -p OutputBitDepthLuma=8 -p OutputBitDepthChroma=8 -p ProfileIDC=244

lencod_JM19.0 -d ..\..\cfg\encoder_JM_RA_B_RExt.cfg -f ..\..\cfg\per-sequence-JM\WebBrowsing_444.cfg -p QPISlice=0 -p IntraPeriod=32 -p ReconFile="WebBrowsing_444_JM_RA_B_RExt_JM19.0_Q0.yuv" -p OutputFile="WebBrowsing_444_JM_RA_B_RExt_JM19.0_Q0.bin" -p FramesToBeEncoded=300 -p SourceBitDepthRescale=1 -p OutputBitDepthLuma=8 -p OutputBitDepthChroma=8 -p ProfileIDC=244 -p LosslessCoding=1 -p QPPSlice=0 -p QPBSlice=0 -p ExplicitHierarchyFormat=B3r0B1r0b0e0b2e0B5r0b4e0b6e0

lencod_JM19.0 -d ..\..\cfg\encoder_JM_LB_RExt.cfg -f ..\..\cfg\per-sequence-JM\WebBrowsing_RGB.cfg -p QPISlice=22 -p QPPSlice=23 -p QPBSlice=23 -p IntraPeriod=0 -p ReconFile="WebBrowsing_RGB_JM_LB_RExt_JM19.0_Q22.yuv" -p OutputFile="WebBrowsing_RGB_JM_LB_RExt_JM19.0_Q22.bin" -p FramesToBeEncoded=300 -p SourceBitDepthRescale=1 -p OutputBitDepthLuma=8 -p OutputBitDepthChroma=8 -p ProfileIDC=244

lencod_JM19.0 -d ..\..\cfg\encoder_JM_LB_RExt.cfg -f ..\..\cfg\per-sequence-JM\WebBrowsing_444.cfg -p QPISlice=0 -p IntraPeriod=0 -p ReconFile="WebBrowsing_444_JM_LB_RExt_JM19.0_Q0.yuv" -p OutputFile="WebBrowsing_444_JM_LB_RExt_JM19.0_Q0.bin" -p FramesToBeEncoded=300 -p SourceBitDepthRescale=1 -p OutputBitDepthLuma=8 -p OutputBitDepthChroma=8 -p ProfileIDC=244 -p LosslessCoding=1 -p QPPSlice=0 -p QPBSlice=0 -p ExplicitHierarchyFormat=B0r0B1r0B2r0

# Experiment Results

It should be noted that the encoding/decoding times, especially decoding times, are not so reliable – since the experiments were performed using a computing "cluster".

The results are shown in Tables 1-4, comparing SCM with JM. For the lossy coding results, the numbers are BD-Rate, and for lossless coding result, the numbers are bitrate change percentages. For both lossy and lossless coding, a negative number means the codec listed on the right is better.

Table 1 – JM-19.0 vs. HM-16.6+SCM-5.2 for SCC CTC sequences, lossy coding

Table 2 – JM-19.0 vs. HM-16.6+SCM-5.2 for SCC CTC 4:2:0 sequences, lossy coding

Table 3 – JM-19.0 vs. HM-16.6+SCM-5.2 for SCC CTC 4:4:4 sequences, lossless coding

Table 4 – JM-19.0 vs. HM-16.6+SCM-5.2 for SCC CTC 4:2:0 sequences, lossless coding

# References

- JCTVC-G399, “Comparison of Compression Performance of HEVC Working Draft 4 with AVC High Profile,” Geneva, CH, Nov. 2011.

- JCTVC-H0360, “Comparison of Compression Performance of HEVC Working Draft 5 with AVC High Profile,” San José, US, Feb. 2012.

- JCTVC-I0409, “Comparison of Compression Performance of HEVC Draft 6 with AVC High Profile,” Geneva, CH, 27 April – 7 May 2012.

- JCTVC-J0236, “Comparison of Compression Performance of HEVC Draft 7 with AVC High Profile,” Stockholm, SE, 11–20 July 2012.

- JCTVC-K0279, “Comparison of Compression Performance of HEVC Draft 8 with AVC High Profile and Performance of HM8.0 with Different Delay Characteristics,” Shanghai, CN, 10–19 Oct. 2012.

- JCTVC-L0322, “Comparison of Compression Performance of HEVC Draft 9 with AVC High Profile and Performance of HM9.0 with Temporal Scalability Characteristics,” Geneva, CH, 14–23 Jan. 2013.

- JCTVC-O0184, “Comparison of Compression Performance of HEVC Range Extensions Draft 4 with AVC High 4:4:4 Predictive profile,” Geneva, CH, 23 Oct. – 1 Nov. 2013.

- JCTVC-P0213, “Comparison of Compression Performance of HEVC Range Extensions Draft 5 with AVC High 4:4:4 Predictive profile,” Valencia, ES, 27 Mar. – 4 Apr. 2014.

- JCTVC-R0101, “Comparison of Compression Performance of HEVC 4:4:4 Range Extensions Test Model 7 and HEVC Screen Content Coding Extensions Test Model 1 with AVC High 4:4:4 Predictive profile,” Sapporo, JP, 30 June – 9 July 2014.

- JCTVC-S0084, “Comparison of Compression Performance of HEVC 4:4:4 Range Extensions Test Model 8 and HEVC Screen Content Coding Extensions Test Model 2 with AVC High 4:4:4 Predictive profile,” Strasbourg, FR, 17–24 Oct. 2014.

- JCTVC-T0042, “Comparison of Compression Performance of HEVC Test Model 16.2 and HEVC Screen Content Coding Extensions Test Model 3 with AVC High 4:4:4 Predictive profile,” Geneva, CH, 10–18 Feb. 2015.

- JCTVC-U0051, “Comparison of Compression Performance of HEVC Test Model 16.4 and HEVC Screen Content Coding Extensions Test Model 4 with AVC High 4:4:4 Predictive profile,” Warsaw, PL, 19–26 June 2015.

- JCTVC-U1015, “Common conditions for screen content coding tests,” Warsaw, PL, 19–26 June 2015.