| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0034 |
| --- | --- |

| Title: | Palette encoder improvements for the 4:2:0 chroma format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christophe Gisquet, Guillaume Laroche, Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, France | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon CRF |  |  |

_____________________________

# Abstract

During the 20th Meeting in Geneva, in February 2015, an extension of the palette-mode for the 4:2:0 chroma format has been adopted. It operates by actually encoding 4:4:4 content, then discarding the superfluous chroma samples. It is asserted that the encoder does not properly account for this when producing the mapping of pixels to palette entries, causing a substantial loss in coding efficiency. The present contribution therefore suggests to weight the 4:4:4 chroma samples according to their position relative to the 2x2 grid of samples and the chroma format in several computations. It is reported that the non-normative changes provides a DBR gain of up to 3.2%, 2.4% and 1.3% in luma for respectively the AI, RA and LDB lossy configuration.

# Description

## Decoding algorithm

The following description is extracted for the reader’s convenience from [1], although the proposal is similar in a number of ways to various other proposals from Qualcomm or InterDigital.

In the adopted implementation, a YUV420 palette coded CU is treated as YUV444 in the way that Chroma values in position 1, 2, 3 are padded with the Chroma value in position 0, as in Figure 1. Then the usual 4:4:4 palette derivation and coding processes are applied.

Figure 1 Pad position 1, 2, 3 Chroma value with Chroma value in position 0 for YUV420 CU

During the reconstruction, Chroma values in position 1, 2 and 3 are ignored, such that a YUV420 CU is reconstructed. In one approach, when Escape happens at positions 1, 2 and 3, the Chroma values are signalled, but discarded during the reconstruction process. As a result, the current palette syntax remains the same. In the other approach, when Escape happens at positions 1, 2 and 3, the Chroma values are not signalled as they will not be used in reconstruction process.

Furthermore, it was noted, e.g. in [2], that in addition the escape coding of a pixel may also entail signalling either Y only or Y,U,V depending to said position.

## Current encoder algorithms

These algorithms can be put into 3 main categories:

- Pixel mapping to a level (aka palettization);

- Palette entry estimation, e.g. through a K-means-like clustering algorithm [3];

- Palette optimization for prediction, e.g. by doing rate-distortion-optimized replacement of palette entries by palette predictor entries [4].

All of those involve measuring a distance (through the sum of absolute differences or squared differences over the component values) between a triplet and another. The encoder was modified for the 4:2:0 chroma format by using the same chroma sample values for all 4 pixels in the aforementioned 2x2 block.

This results in attributing a relevance to chroma samples which will actually be discarded on decoding. Obviously, this results in at least too large an emphasis put on chroma fidelity, and potentially incorrect selection.

## Of a solution

A first solution is to not take into account the discarded chroma samples values when computing a distance in either category. This gives for an L distance measure like SSE, and pixels P1, P2:

- D = L(P1[Y], P2[Y]) + L(P1[Cb], P2[Cb]) + L(P1[Cr], P2[Cr]) for pixel 0

- D = L(P1[Y], P2[Y]) for pixels 1, 2 and 3

It has been found that this is suboptimal and the formula should actually be:

L(P1[Y], P2[Y]) + wC * L(P1[Cb], P2[Cb]) + wC * L(P1[Cr], P2[Cr])

with wC 1 for non-discarded chroma samples, and a fairly small value for wC, e.g. 2-5.

Furthermore, instead of using the average (centroid) of the cluster for the level, a weighted average with similar concept of weighting the discarded chroma sample is best used.

## Limits of the proposal

The proposed solution represents only marginal modifications to try and (not) account for the discarded chroma samples. Better algorithms may be envisioned, but the current proposal only implements the above modifications

A solution for lossless was not studied, and as could be expected, the modifications to the common algorithms provide any benefit. Similar concepts could however be applied to the palette generation.

Different settings were studied, and although the cross-checked results only account for one scenario (solution active on iterative derivation of the palette and the pixel mapping), we provide in annex results for settings where the solution is active for all relevant algorithms (i.e. other palette derivations).

Minimal tests were performed by generating sequences with different chroma sample type. While this does have a small impact on the weighting, the fact that the kept chroma sample is always the top left has a stronger influence on the decoded result.

# Results

## 4:2:0, lossy configuration

## 4:2:0, lossless configuration

## 4:4:4, lossy configuration

## 4:4:4, lossless configuration

# Conclusion

The concept of applying weight to the discarded chroma is rather simple, and does not represent a big software modification, although it implies a systematic modification of code sometimes triplicated. In light of the coding gain improvement, it is suggested to adopt this non-normative change.

# Annex: results for complete activation, 4:2:0 lossy

The complete activation affects in addition the initial palette generation as well as the one where palette prediction is forced. The results are only marginally better, and whether they are a better trade-off is debatable.

# References

- JCTVC-T0072, “CE1-related: Palette Coding for non-444 format content,” J. Ye, S. Liu, S. Lei (MediaTek), Geneva, CH, Feb. 2015.

- JCTVC-T0053, “Non-CE1: Palette based coding for non-4:4:4 format,” J. Zhu, Z. Wang, J. Ye, (Fujitsu), Geneva, CH, Feb. 2015.

- JCTVC-P0119, “Non-RCE4: combination of palette coding improvements”, C. Gisquet, G. Laroche, T. Poirier, P. Onno (Canon), San José, US, Jan. 2014.

- JCTVC-R0086, “SCCE3 Test C.2: combination of palette coding tools”, C. Gisquet, G. Laroche, T. Poirier, P. Onno (Canon), Sapporo, JP, July 2014.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).