| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0036 |
| --- | --- |

| Title: | The need for correction or clarification of colour description semantics (especially for transfer_characteristics) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information / text bug report |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA | Tel: Email: | +1 425 703 5308 garysull@microsoft.com |
| Source: | Microsoft |  |  |

_____________________________

# Abstract

This contribution contains remarks on the specification of semantics for the video transfer characteristics syntax element in the VUI parameters of HEVC and in related standards including MPEG-2, AVC, and CICP. It suggests that action is needed to correct and clarify this part of the video coding standards, especially in regard to the specification for SMPTE ST 2084 "PQ" and SMPTE ST 428-1 "DCDM". The content of our current video coding standards is reported to be incorrect in regard to the recent additions, or at least to be inconsistent with the way other transfer characteristics are specified. It is suggested that the proper way to address the issue may involve changing what the video coding standards say about both these newly-supported SMPTE standards and the other transfer characteristics curves (and possibly for related parameters such as colour primaries and matrix coefficients).

# The current specification of colour description semantics

The colour description syntax in HEVC contains three VUI parameter syntax elements: colour_primaries, transfer_characteristics, and matrix_coeffs, which are interpreted together with the video_full_range_flag to describe what the decoded pictures represent in terms of colour.

The 8-bit syntax element "transfer_characteristics" in the colour description syntax is intended to indicate the non-linear transfer function. For this syntax element, the HEVC text says the following (subclause E.3.1, ITU published text of version 3, approval date 2015-04):

transfer_characteristics indicates the opto-electronic transfer characteristic of the source picture as specified in Table E.4 as a function of a linear optical intensity input Lc with a nominal real-valued range of 0 to 1.

The text of AVC (approval date 2014-02) has exactly the same phrasing.

Thus, the transfer_characteristics value is specified as an identification of an opto-electronic transfer function (OETF) for the capture of light at the source (i.e., a camera). In colour jargon, this seems to be what is known as a scene-referred expression.

# Light input (and encoding) versus display output (and decoding)

A key issue to notice is that the colour description semantics are written in a somewhat unusual way, relative to the general editorial principles followed in drafting these standards, as they are written from the perspective of the input to an encoder, whereas the actual intent is to guide the interpretation of the decoded pictures for display and other such purposes. The author's personal recollection is that at some point in time there was a NOTE in the text to try to explain and clarify that, but such a note does not seem to appear in the current published HEVC or AVC standards (nor was it found in some prior versions that were checked when writing this contribution).

Suggestion #1: It may be advisable to add a note or some other text to the standard to clarify that the intent of the text of the colour description semantics is to assist with the display rendering of decoded video, not to prescriptively specify the input to encoders.

# Proper interpretation

From a naïve perspective, one might think that the job of a post-decoding renderer – at least for one that is operating under nominal viewing conditions (e.g., in a reference viewing environment) is simply to invert the OETF in order to reproduce, on the display, the same intensity of light that was captured by the camera (in corresponding spatial positions). This, unfortunately, would be an incorrect understanding. Industry specifications for how to display video that was captured by a specified OETF are not ordinarily intended to invert that function. An example of this is that while ITU-R BT.709 [1] contains the specification of the OETF that is generally recommended for HDTV (standard dynamic range) video, the specification for the generally recommended display of such video is a different standard – namely, that is ITU-R BT.1886 [2]. That specification defines an electro-optical transfer function (EOTF) that is not an inversion of the OETF specified in ITU-R BT.709.

The cascade of the preferred OETF with the preferred EOTF is not intended as a unity pass-through function (even if viewing conditions are considered ideal). In contrast, the concept of an optical-to-optical transfer function (OOTF) has recently been discussed as a way to help clarify these matters (see, e.g., [3]). There is a tone-mapping concept involved in imagery production that is intended to provide a subjectively appropriate mapping from real-world light to the more limited domain of displayable rendered pictures.

In the past, this may not have been so important to consider in our video coding standards, as the display rendering process is outside the formal scope of these standards. However, our recent adoption of support for the SMPTE ST 2084 "PQ" [4] and SMPTE ST 428-1 "DCDM" [5] specifications changes that picture.

# The problem and the need for action

Those "in the know" about how to interpret video that uses SMPTE ST 2084 or SMPTE ST 428-1 certainly ignore what our standards exactly say about them and merely interpret the enumeration code values that we have assigned to them (the values 16 and 17) as an identification that the video content is intended to be interpreted according to SMPTE ST 2084 or SMPTE ST 428-1 – and indeed that is what they should do. However, that doesn't mean that what our standards say is correct. With the way we have described the values 16 and 17 into our standards, our text has become essentially self-inconsistent.

Whereas we provide the formulas of curves for SMPTE ST 2084 and SMPTE ST 428-1 that we describe as OETFs, that is not what they actually are. In the terminology of SMPTE ST 2084, what we have provided is the formula for the "inverse EOTF", not a proper OETF. Our video coding standards are thus providing an incorrect description of what a camera designed for use with SMPTE ST 2084 or SMPTE ST 428-1 should do.

SMPTE ST 2084 and SMPTE ST 428-1 ultimately express their formulas from the perspective of an EOTF, not an OETF. They specify how to display image content, not how to capture it with a camera. In colour jargon, these are output-referred specifications, not scene-referred specifications. They are more analogous to ITU-R BT.1886 than to ITU-R BT.709.

Our video coding specifications are thus incorrect when it comes to SMPTE ST 2084 and ST 428-1.

For values of transfer_characteristics other than the values 16 and 17, the curves we provide are the nominal specification of camera capturing behaviour. For these new values, they are not. The current text is simply incorrect.

In the distant past of a decade or more ago, it may have been adequate for display characteristics to be implicitly determined by the behaviour of the phosphors used in CRTs, but that changed when CRTs stopped being the generally expected display technology for video.

Suggestion #2: A simply way to fix the immediate problem would be to change our standards to say that the formulas for the two new transfer functions (the values 16 and 17) are expressing a function of a linear optical intensity for display output Ld with, rather than for camera-captured input Lc, while retaining the prior description for the others.

However, the proper way to address this is probably not just to modify what the text says for SMPTE ST 2084 and SMPTE ST 428-1.

Suggestion #3: It also seems desirable to clarify that the display process is generally not intended as an inversion of the capturing process and that there are industry practices for the display of images with other transfer characteristics equations as well. Adding bibliographic citations to such other specifications may be beneficial. It may even be advisable to add an additional column to the table for interpretation of the transfer_characteristics syntax element so that specifications for display can be listed in a separate column from specifications for camera capture. For most entries, we have definitions for camera capture; for two of them (SMPTE ST 2084 and ST SMPTE 428-1), we have definitions for display output; and for at least one (ITU-R BT.709), both types of specification exist.

# References

- Recommendation ITU-R BT.709-5 (2002), Parameter values for the HDTV standards for production and international programme exchange.

- Recommendation ITU-R BT.1886 (2011), Reference electro-optical transfer function for flat panel displays used in HDTV studio production

- ITU-R WP6C Temporary document 274, "Preliminary draft New Recommendation ITU-R BT.[HDR-TV]: Parameter values for high dynamic range television systems for production and international programme exchange", July 2015.

- SMPTE FCD ST 2084 (2014), High Dynamic Range Electro-Optical Transfer Function of Mastering Reference Displays.

- SMPTE ST 428-1 (2006), D-Cinema Distribution Master (DCDM) — Image Characteristics.