| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0040r1 |
| --- | --- |

| Title: | Performance of the SCM with the macro “SCM_U0095_FAST_INTRA_ACT” enabled |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yao-Jen Chang, 2Chun-Lung Lin, Jih-Sheng Tu and Ching-Chieh Lin 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-8774 1britpablo@itri.org.tw 2Chunlung@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

In the last meeting, JCTVC-U0095 proposed to disallow the TU splitting for the ACT-enabled intra prediction when the CU sizes are 64x64 and 32x32 in the encoder. It was reported that this method improves 4% saving of encoding time in AI-lossy condition on top of SCM 4.0. It was also commented that it is desirable to test this method on top of adoption of JCTVC-U0106 (TU-based ACT). Therefore, the final decision was to adopt the method of JCTVC-U0095 into the next version of SCM and set the macro of this method as OFF in default for CTC. To understand the performance of this method on top of JCTVC-U0106, this contribution sets the macro of this method “SCM_U0095_FAST_INTRA_ACT” as ON in SCM 5.2. The performance results show that, compared to the SCM 5.2 anchor, it is observed 4% saving of encoding time on average in AI-lossy condition no matter how the condition of IBC search range is set. Note that the number of the saving in SCM 5.2 is the same as the number of saving in SCM 4.0.

# Introduction

This contribution shows the performance results of the adopted method in JCTVC-U0095 [1] when its SW macro “SCM_U0095_FAST_INTRA_ACT” is set as ON in SCM 5.2 anchor. Note that the configuration parameter “CUNoSplitIntraACT” should be set as “true” accordingly to enable the method. Also, note that JCTVC-U0106 (TU-based ACT) [2] has been integrated into SCM 5.2 anchor.

# Simulation results

The method of JCTVC-U0095 adopted in the last meeting is tested on top of the SCM 5.2 anchor under the common test condition [3]. Table 1 shows the lossy coding results on the condition of full-frame IBC search and 444 chroma format. The results show that it saves 4% encoding time with no effect on the coding gain in AI condition. Table 2 shows the lossy coding results on the condition of constrained IBC search and 444 chroma format. The results show that it saves 4% encoding time with no effect on the coding gain in AI condition. The simulation results for the other cases can be found in the attached spreadsheets.

Table 1. Lossy results under the condition of full-frame IBC search and 444 chroma format

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | G/Y |  | B/U |  | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% |  | 0.0% |  | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% |  | 0.0% |  | 0.0% |
| RGB, Animation, 720p | 0.0% |  | 0.0% |  | 0.0% |
| RGB, camera captured, 1080p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% |  | 0.1% |  | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, Animation, 720p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, camera captured, 1080p | 0.0% |  | 0.0% |  | 0.0% |
| Enc Time[%] | 96% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |
|  | Random Access |  |  |  |  |
|  | G/Y |  | B/U |  | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% |  | 0.0% |  | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% |  | 0.0% |  | 0.0% |
| RGB, Animation, 720p | 0.0% |  | -0.1% |  | 0.0% |
| RGB, camera captured, 1080p | 0.1% |  | 0.0% |  | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% |  | 0.0% |  | -0.1% |
| YUV, Animation, 720p | 0.0% |  | 0.0% |  | 0.1% |
| YUV, camera captured, 1080p | 0.0% |  | 0.0% |  | 0.1% |
| Enc Time[%] | 99% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |
|  | Low delay B |  |  |  |  |
|  | G/Y |  | B/U |  | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% |  | -0.1% |  | -0.2% |
| RGB, mixed content, 1440p & 1080p | 0.0% |  | -0.2% |  | 0.1% |
| RGB, Animation, 720p | -0.1% |  | 0.2% |  | 0.0% |
| RGB, camera captured, 1080p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% |  | 0.0% |  | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% |  | -0.1% |  | -0.2% |
| YUV, Animation, 720p | 0.0% |  | -0.1% |  | 0.0% |
| YUV, camera captured, 1080p | 0.0% |  | 0.0% |  | 0.0% |
| Enc Time[%] | 100% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |

Table 2. Lossy results under the condition of constrained IBC search (1x4 CTUs) and 444 chroma format

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | G/Y |  | B/U |  | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% |  | 0.0% |  | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% |  | 0.0% |  | 0.0% |
| RGB, Animation, 720p | 0.0% |  | 0.0% |  | 0.0% |
| RGB, camera captured, 1080p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, Animation, 720p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, camera captured, 1080p | 0.0% |  | 0.0% |  | 0.0% |
| Enc Time[%] | 96% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |
|  | Random Access |  |  |  |  |
|  | G/Y |  | B/U |  | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% |  | 0.0% |  | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.1% |  | 0.0% |  | 0.0% |
| RGB, Animation, 720p | 0.1% |  | 0.1% |  | 0.1% |
| RGB, camera captured, 1080p | 0.1% |  | 0.1% |  | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% |  | 0.0% |  | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% |  | 0.1% |  | 0.1% |
| YUV, Animation, 720p | 0.0% |  | 0.2% |  | 0.0% |
| YUV, camera captured, 1080p | 0.0% |  | 0.0% |  | 0.0% |
| Enc Time[%] | 100% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |
|  | Low delay B |  |  |  |  |
|  | G/Y |  | B/U |  | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% |  | 0.0% |  | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.1% |  | 0.2% |  | 0.0% |
| RGB, Animation, 720p | -0.1% |  | -0.1% |  | -0.1% |
| RGB, camera captured, 1080p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% |  | 0.0% |  | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% |  | -0.3% |  | -0.2% |
| YUV, Animation, 720p | 0.0% |  | 0.3% |  | -0.1% |
| YUV, camera captured, 1080p | -0.1% |  | -0.1% |  | 0.0% |
| Enc Time[%] | 100% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |

# Conclusion

This contribution enables the macro “SCM_U0095_FAST_INTRA_ACT” to test the adopted method of JCTVC-U0095 on top of the SCM 5.2 anchor, which includes the TU-based ACT (JCTVC-U0106) also adopted in the last meeting. On top of the SCM 5.2 anchor it observes 4% saving of encoding time in AI-lossy condition. The number of the saving on top of the SCM 5.2 is the same as that on top of the SCM 4.0 anchor, where the TU-based ACT is not included yet. Due to the method irrelative to the TU-based ACT, it is suggested to set the macro “SCM_U0095_FAST_INTRA_ACT” as ON in default for CTC.

# Reference

- Y. J. Chang, C. L. Lin, J. S. Tu, C. C. Lin, “Non-CE: Encoder modification for intra prediction mode using adaptive colour transform,” 21th JCT-VC meeting, JCTVC-U0095, Warsaw, Poland, Jun. 2015.

- X. Xiu, Y. He, Y. Ye, “On signaling adaptive color transform at TU level,” 21th JCT-VC meeting, JCTVC-U0106, Warsaw, Poland, Jun. 2015.

- H. Yu, R. Cohen, K. Rapaka, J. Xu , “Common Test Conditions for Screen Content Coding,” 21th JCT-VC meeting, JCTVC-U1015, Warsaw, Poland, Jun. 2015.

# Patent rights declaration(s)

ITRI international may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)