| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0041 |
| --- | --- |

| Title: | Restriction on signalling for palette escape samples |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Rajan Joshi, Krishna Rapaka, Marta Karczewicz | Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the palette mode, escape samples are signalled with palette_escape_val syntax element. This syntax element is binarized using Exp-Golomb code of order 3 and there is no upper limit specified on the values that can be signalled in a bitstream, unlike, for example, with coefficient coding where the maximum coefficient value is specified per colour component. In this contribution, it is proposed to limit palette_escape_val per colour component based on the bitdepth of that colour component. Additional BD-rate results are presented for reusing the coeff_abs_level_remaining binarization for binarization of escape samples in lossy coding, it is ranged between -0.1 to 0.2 BD-rate luma saving across all test cases with full frame intra block copy.

# Introduction

Palette mode can have escape samples which are signalled with palette_escape_val syntax element. Exp-Golomb code of order 3 (JCTVC-U0052), is used in the binarization of this syntax element. This code can represent infinitely large numbers, so practical limits are required to be imposed on the signalled values in the bitstream.

It was also noticed that the role of escape samples is similar to that of transform skip. However the current treatment of escape samples is different from the transform skip samples. In this contribution, unifying the processing path (binarization, dequantization) of transform skip samples and escape samples is studied. In addition to the design unification, the limits on palette_escape_val syntax element can be automatically imposed by using maximum coefficient values CoeffMaxY and CoeffMaxC,

# Proposal

First part of this proposal is purely software contribution. It was noticed that in the SCM software, palette encode related functions are implemented in TComPrediction class which is shared with the decoder application, however the decoder does not need them. Also, it is different from encoder implementation for other modes where such encoder functions are implemented in TEncSearch class.

In this software contribution, palette encoder functions were moved to TEncSearch class and it is proposed to adopt this change into the SCM software. It was verified that this change provides identical results.

## Solution 1. Constraint based on bitdepth

In the first solution, it is proposed to put a constraint per colour component that palette_escape_val cannot be greater than (1<<(bitDepth + 1)) – 1, where bitDepth is the luma or chroma signalled bitdepth in the SPS and plus 1 is used to accommodate the accuracy increase associated with QP values smaller than 4. It was verified that the maximum codeword with Exp-Golomb of order 3 binarization with such restriction is not exceeding 32 bits.

This method provided identical results for the SCM software since encoder is not selecting escape values greater than the proposed restriction.

The advantage of this solution is that it requires minimal change to the semantics of palette_escape_val in the working draft.

palette_escape_val specifies the quantized escape coded sample value for a component.

The variable PaletteEscapeVal[ cIdx ][ xC ][ yC ] specifies escape value of a sample for which PaletteIndexMap[ xC ][ yC ] is equal to MaxPaletteIndex and palette_escape_val_present_flag is equal to 1. The array index cIdx specifies the colour component. The array indices xC, yC specify the location ( xC, yC ) of the sample relative to the top-left luma sample of the picture.

It is a requirement of bitstream conformance that the value of palette_escape_val shall be in the range of 0 (1<<(bitDepth + 1)) – 1, inclusive, where bitDepth is equal to BitDepthY for luma samples, and bitDepth is equal to BitDepthC for chroma samples.

## Solution 2. Align escape coding with transform skip

In this solution, the inverse quantization of the escape samples is completely aligned with the transform skip samples for lossy coding mode. It reuses coeff_abs_level_remaining binarization for palette_escape_val syntax element, which also imposes the constraint on the palette_escape_val value such that it is no greater than CoeffMaxY for luma samples and CoeffMaxC for chroma samples.

Generally speaking, escape coding can be split into three parts: dequantization, binarization, and limit on the signalled value. If there is a question of aligning with transform skip, various level of unification can be considered by aligning only some parts of the escape coding with transform skip coefficients as shown in Table 1.

|  | Dequantization | Binarization | Limits |
| --- | --- | --- | --- |
| Solution 1 | current method | current method (EG3) | (1<<(bitDepth + 1)) – 1 |
| Solution 2 | coefficient dequantization | coeff_abs_level_remaining | CoeffMaxY, CoeffMaxC |

Table 1. Escape coding steps

Solution 2 was implemented on top SCM-5.2 software by invoking transform skip forward and inverse quantization functions for escape samples. Test results based on CTC with full frame IBC 4:4:4: is summarized in below, and can be find in more detail in the accompanied Excel tables.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.1% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.1% | -0.2% |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.2% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.5% |
| YUV, Animation, 720p | -0.1% | 0.1% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 96% |  |  |

Working draft changes relatively to JCTVC-U1005-v2 are provided below.

8.4.4.2.7 Decoding process for palette mode

Inputs to this process are:

- a location ( xCb, yCb ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

- a variable cIdx specifying the colour component of the current block,

- two variables nCbSX and nCbSY specifying the width and height of the current block, respectively,

Output of this process is an array recSamples[ x ][ y ], with x = 0..nCbSX − 1, y = 0..nCbSY − 1 specifying reconstructed sample values for the block.

Depending on the value of cIdx, the variables nSubWidth and nSubHeight are derived as follows:

- If cIdx is equal to 0, nSubWidth is set to 1 and nSubHeight is set to 1,

- Otherwise, nSubWidth is set to SubWidthC and nSubHeight is set to SubHeightC.

The ( nCbSX x nCbSY ) block of the reconstructed sample array recSamples at location ( xCb, yCb ) is derived as follows:

For x = 0..nCbSX − 1, y = 0..nCbSY − 1, recSamples[ x ][ y ] is set as follows:

The variables xL and yL are derived as follows:

xL = palette_transpose_flag ? x * nSubHeight : x * nSubWidth	(8-1)

yL = palette_transpose_flag ? y * nSubWidth : y * nSubHeight	(8-2)

The variable bIsEscapeSample is derived as follows:

- If PaletteIndexMap[ xCb + xL ][ yCb + yL ] is equal to MaxPaletteIndex and palette_escape_val_present_flag is equal to 1, bIsEscapeSample is set equal to 1:

- Otherwise, bIsEscapeSample is set equal to 0.

- If bIsEscapeSample is equal to 0, the following applies:

recSamples[ x ][ y ] = CurrentPaletteEntries[ cIdx ][  PaletteIndexMap[ xCb + xL ][ yCb + yL ] ]	(8-3)

- Otherwise, if cu_transquant_bypass_flag is equal to 1, the following applies:

recSamples[ x ][ y ] = PaletteEscapeVal[ cIdx ][ xCb + xL ][ yCb + yL ]	(8-4)

- Otherwise (bIsEscapeSample is equal to 1 and cu_transquant_bypass_flag is equal to 0), transform process as specified in clause 8.6.2 is invoked with the luma location (xCb + xL, yCb + yL ), the variable trafoDepth set equal to 0, the variable cIdx, and the transform size nTbS set equal to nCbSX as inputs, and the output is an 1x1 array recSamples [ x ][ y ].

- Otherwise (bIsEscapeSample is equal to 1 and cu_transquant_bypass_flag is equal to 0), the following ordered steps apply:

- The derivation process for quantization parameters as specified in clause 8.6.1 is invoked with the location ( xCb, yCb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture.

- The quantization parameter qP is derived as follows:

- If cIdx is equal to 0,

qP = max( 0, Qp′Y )		(8-5)

- Otherwise, if cIdx is equal to 1,

qP = max( 0, Qp′Cb )		(8-6)

- Otherwise (cIdx is equal to 2),

qP = max(0, Qp′Cr )		(8-7)

- The variables bitDepth is derived as follows:

bitDepth = ( cIdx  = =  0 ) ? BitDepthY : BitDepthC	(8-8)

- The list levelScale[ ] is specified as levelScale[ k ] = { 40, 45, 51, 57, 64, 72 } with k = 0..5.

- The following applies

tmpVal = ( PaletteEscapeVal[ cIdx ][ xCb + xL ][ yCb + yL ] *

levelScale[ qP%6 ] )  <<  ( qP / 6 ) + 32 )  >>  6	(8-9)

recSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth) − 1, tmpVal )	(8-10)

The variable numComps is derived as follows:

numComps = ( ChromaArrayType  = =  0 ) ? 1 : 3	(8-11)

The variables PredictorPaletteSize and PredictorPaletteEntries are modified as follows:

for( i = 0; i < CurrentPaletteSize; i++ )

for( cIdx = 0; cIdx < numComps; cIdx++ )

newPredictorPaletteEntries[ cIdx ][ i ] = CurrentPaletteEntries[ cIdx ][ i ]

newPredictorPaletteSize = CurrentPaletteSize

for( i = 0; i < PredictorPaletteSize  &&  newPredictorPaletteSize < PaletteMaxPredictorSize; i++ )

if( !PalettePredictorEntryReuseFlag[ i ] ) {

for( cIdx = 0; cIdx < numComps; cIdx++ ) 	(8-12)

newPredictorPaletteEntries[ cIdx ][ newPredictorPaletteSize ] =

PredictorPaletteEntries[ cIdx ][ i ]

newPredictorPaletteSize++

}

for( cIdx = 0; cIdx < numComps; cIdx++ )

for( i = 0; i < newPredictorPaletteSize; i++ )

PredictorPaletteEntries[ cIdx ][ i ] = newPredictorPaletteEntries[ cIdx ][ i ]

PredictorPaletteSize = newPredictorPaletteSize

### 8.6.3 Scaling process for transform coefficients

…

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS – 1 or x = 0 and y = 0 when palette_mode_flag[ xTbY ][ yTbY] is equal to 1, the following applies:

–	The scaling factor m[ x ][ y ] is derived as follows:

–	If one or more of the following conditions are true, m[ x ][ y ] is set equal to 16:

–	scaling_list_enabled_flag is equal to 0.

–	transform_skip_flag[ xTbY ][ yTbY ] is equal to 1 and nTbS is greater than 4.

–	palette_mode_flag[ xTbY ][ yTbY] is equal to 1.

–	Otherwise, the following applies:

m[ x ][ y ] = ScalingFactor[ sizeId ][ matrixId ][ x ][ y ]	(8-13)

Where sizeId is specified in Error: Reference source not found for the size of the quantization matrix equal to (nTbS)x(nTbS) and matrixId is specified in Error: Reference source not found for sizeId, CuPredMode[ xTbY ][ yTbY ], and cIdx, respectively.

–	The scaled transform coefficient d[ x ][ y ] is derived as follows:

d[ x ][ y ] = Clip3( coeffMin, coeffMax, ( ( TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ][ x ][ y ] * m[ x ][ y ] *
			levelScale[ qP%6 ]  <<  (qP / 6 ) ) + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift )	(8-14)

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).