| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0042 |
| --- | --- |

| Title: | SPS and PPS palette predictor initialization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Rajan Joshi, Marta Karczewicz | Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the palette mode, palette predictor initializer can be signalled in SPS and PPS together, however only one initializer can be used at a time, also when SPS initializer is signalled it is not possible to disable initialization on picture basis. This contribution proposes the ability to disable palette predictor initialization by signalling palette predictor size equal to zero, and for more efficient signalling it is proposed to compose palette predictor initializer from the initializers signalled in both PPS and SPS sets.

# Problem Statement

In the current palette mode, palette predictor initializers can be signalled in PPS, SPS or both sets together, however only one palette initializer can be used at a time with the following rule.

- If pps_palette_predictor_initializer_present_flag is equal to 1, palette predictor is initialized from PPS,

- Otherwise if sps_palette_predictor_initializer_present_flag is equal to 1, palette predictor is initialized from SPS

- Otherwise, palette predictor is not initialized.

Also, as can be seen from the above rule, palette predictor is always initialized when sps_palette_predictor_initializer_present_flag is equal to 1 and cannot be turned off on picture basis, however other PPS initializers, which can be picture specific, are possible to be signalled.

It is also contradicts the initial adoption of palette predictor intialization, when there was only PPS initializer, which can be disabled on picture basis.

# Proposal

## Allow zero size PPS palette predictor initializer

To address the first problem, it is proposed to modify the PPS initializer signalling to have ability to signal zero size PPS initializer, in this case even when sps_palette_predictor_initializer_present_flag is equal to 1, it is possible to disable palette predictor initialization by signaling pps_palette_predictor_initializer_present_flag is equal to 1 with size zero.

The working draft text change is the following:

| pps_palette_predictor_initializer_present_flag | u(1) |
| --- | --- |
| if( pps_palette_predictor_initializer_present_flag ) { |  |
| pps_num_palette_predictor_initializer |  |
| if( pps_num_palette_predictor_initializer > 0 ) { |  |
| monochrome_palette_flag | u(1) |
| luma_bit_depth_entry_minus8 | ue(v) |
| if( !monochrome_palette_flag ) |  |
| chroma_bit_depth_entry_minus8 | ue(v) |
| pps_num_palette_predictor_initializer_minus1 | ue(v) |
| numComps = monochrome_palette_flag ? 1 : 3 |  |
| for( comp = 0; comp < numComps; comp++ ) |  |
| for( i = 0; i < pps_num_palette_predictor_initializer_minus1; i++ ) |  |
| pps_palette_predictor_initializers[ comp ][ i ] | u(v) |
| } |  |
| } |  |

pps_num_palette_predictor_initializer specifies the number of entries in the picture palette predictor initializer.

It is a requirement of bitstream conformance that the value of pps_num_palette_predictor_initializer_minus1 plus 1 is in the range of 0 to PaletteMaxPredictorSize, inclusive.

pps_palette_predictor_initializers[ comp ][ i ] specifies the value of the comp-th component of the i-th palette entry that is used to initialize the array PredictorPaletteEntries. For values of i in the range of 0 to pps_num_palette_predictor_initializer − 1,_minus1, inclusive, the value of the pps_palette_predictor_initializers[ 0 ][ i ] shall be in the range of 0 to ( 1  <<  BitDepthEntryY ) − 1, inclusive, and the values of pps_palette_predictor_initializers[ 1 ][ i ] and pps_palette_predictor_initializers[ 2 ][ i ] shall be in the range of 0 to ( 1  <<  BitDepthEntryC ) − 1, inclusive.

The initialization rule can be changed accordingly:

Outputs of this process are the initialized palette predictor variables PredictorPaletteSize and PredictorPaletteEntries.

The variable numComps is derived as follows:

numComps = ( ChromaArrayType  = =  0 ) ? 1 : 3	(9-1)

  - If pps_palette_predictor_initializer_present_flag is equal to 1, the following applies:

- PredictorPaletteSize is set equal to pps_num_palette_predictor_initializer_minus1 plus 1.

- The array PredictorPaletteEntries is derived as follows:

for( comp = 0; comp < numComps; comp++ )
	for( i = 0; i < PredictorPaletteSize; i++ )	(9-2)
		PredictorPaletteEntries[ comp ][ i ] = pps_palette_predictor_initializers[ comp ][ i ]

  - Otherwise (pps_palette_predictor_initializer_present_flag is equal to 0 ), if sps_palette_predictor_initializer_present_flag is equal to 1, the following applies:

- PredictorPaletteSize is set equal to sps_num_palette_predictor_initializer_minus1 plus 1.

- The array PredictorPaletteEntries is derived as follows:

for( comp = 0; comp < numComps; comp++ )
	for( i = 0; i < PredictorPaletteSize; i++ )	(9-3)
		PredictorPaletteEntries[ comp ][ i ] = sps_palette_predictor_initializers][ comp ][ i ]

  - Otherwise (pps_palette_predictor_initializer_present_flag is equal to 0 and sps_palette_predictor_initializer_present_flag is equal to 0), PredictorPaletteSize is set equal to 0 and PredictorPaletteSize is set equal to 0.

## Combined palette predictor initializer from PPS and SPS

In the second part of the contribution, it is proposed to combine palette predictor initializers from both PPS and SPS. In the combination, PPS entries are placed first followed by SPS entries. Of course, SPS entries can go first, but the rationale behind the proposed scheme is that PPS entries can be picture specific, rather than SPS entries that can represent the entire sequence characteristic. In PPS, only unique entries comparing to SPS entries can be signalled.

Also, the combined size of the PPS and SPS palette predictor entries shall be not greater than maximum palette predictor size, so the actual signalling is reduced comparing to the current scheme, where the repeated entries can be signalled in both PPS and SPS sets, since only one palette predictor initializer can be used.

This functionality is similar to the scaling list signalling, where the scaling list entry can be reused from the reference scaling list or default scaling list by signalling scaling_list_pred_mode_flag.

The combination of PPS and SPS palette predictor initializers was implemented on top of SCM-5.2, the PPS entries are signalled only if they are different from the SPS entries. Two tests were performed to evaluate this method. In the first test PPS and SPS palette predictor initialization was enabled in SCM-5.2 and this test results were compared against the proposed method.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.3% | -0.4% | -0.4% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | -0.1% | -0.3% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.5% | -0.5% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.3% | -0.4% |
| YUV, Animation, 720p | -0.1% | -0.8% | -0.7% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.2% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.1% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.6% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.3% | -0.2% |
| YUV, Animation, 720p | 0.3% | 0.1% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.5% | -0.6% | -0.6% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.6% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.5% | -0.4% |
| YUV, mixed content, 1440p & 1080p | -0.4% | -1.0% | -1.1% |
| YUV, Animation, 720p | 0.1% | 0.0% | -0.6% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 96% |  |  |

Test 1. SCM-5.2 PPS and SPS palette predictor initializers with full frame IBC 4:4:4

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.3% | -0.3% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | -0.1% | -0.3% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.6% | -0.4% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.4% | -0.4% |
| YUV, Animation, 720p | -0.1% | -0.7% | -0.6% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 90% |  |  |
| Dec Time[%] | 90% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.4% | -0.3% |
| RGB, mixed content, 1440p & 1080p | 0.1% | -0.2% | -0.1% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.3% | -0.2% | 0.2% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.4% | -0.4% |
| YUV, Animation, 720p | 0.1% | 0.0% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.5% | -0.6% | -0.6% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -0.5% | 0.0% |
| RGB, Animation, 720p | -0.1% | 0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.4% | -0.3% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.8% | -0.8% |
| YUV, Animation, 720p | 0.1% | -0.2% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 90% |  |  |
| Dec Time[%] | 91% |  |  |

Test 2. Proposed combination of PPS and SPS palette predictor initializers with full frame IBC 4:4:4

Number of saved PPS palette predictor entries by reusing the SPS entries and comparing it to the SCM-5.2 is summarized in the next table.

|  | All Intra |
| --- | --- |
|  | Saved entries |
| RGB, text & graphics with motion, 1080p & 720p | 56.9% |
| RGB, mixed content, 1440p & 1080p | 55.6% |
| RGB, Animation, 720p | 53.5% |
| RGB, camera captured, 1080p | 86.6% |
| YUV, text & graphics with motion, 1080p & 720p | 60.6% |
| YUV, mixed content, 1440p & 1080p | 58.9% |
| YUV, Animation, 720p | 59.3% |
| YUV, camera captured, 1080p | 62.3% |
| Average | 58% |
|  | Random Access |
|  | Saved entries |
| RGB, text & graphics with motion, 1080p & 720p | 79.9% |
| RGB, mixed content, 1440p & 1080p | 99.2% |
| RGB, Animation, 720p | 99.7% |
| RGB, camera captured, 1080p | 100.0% |
| YUV, text & graphics with motion, 1080p & 720p | 83.5% |
| YUV, mixed content, 1440p & 1080p | 99.7% |
| YUV, Animation, 720p | 99.9% |
| YUV, camera captured, 1080p | 100.0% |
| Average | 86% |
|  | Low delay B |
|  | Saved entries |
| RGB, text & graphics with motion, 1080p & 720p | 85.9% |
| RGB, mixed content, 1440p & 1080p | 98.1% |
| RGB, Animation, 720p | 97.0% |
| RGB, camera captured, 1080p | 100.0% |
| YUV, text & graphics with motion, 1080p & 720p | 87.9% |
| YUV, mixed content, 1440p & 1080p | 98.7% |
| YUV, Animation, 720p | 97.6% |
| YUV, camera captured, 1080p | 100.0% |
| Average | 90% |

Table 1. Saved PPS palette predictor entries relative to the number of PPS entries signaled in the SCM-5.2

Working draft changes are provided based on JCTVC-U1005-v2.

- 9.3.2.3 Initialization process for palette predictor entries

Outputs of this process are the initialized palette predictor variables PredictorPaletteSize and PredictorPaletteEntries.

The variable numComps is derived as follows:

numComps = ( ChromaArrayType  = =  0 ) ? 1 : 3	(9-4)

  - If pps_palette_predictor_initializer_present_flag is equal to 1 and sps_palette_predictor_initializer_present_flag is equal to 1, the following applies:

- PredictorPaletteSize is set equal to pps_num_palette_predictor_initializer_minus1 plus sps_num_palette_predictor_initializer_minus1 plus 2. It is a requirement of bitstream conformance that PredictorPaletteSize shall be less than or equal to PaletteMaxPredictorSize.

- The array PredictorPaletteEntries is derived as follows:

for( comp = 0; comp < numComps; comp++ )
	for( i = 0; i <= pps_num_palette_predictor_initializer_minus1; i++ )	
		PredictorPaletteEntries[ comp ][ i ] = pps_palette_predictor_initializers[ comp ][ i ]
	for( i = pps_num_palette_predictor_initializer_minus1 + 1; i < PredictorPaletteSize; i++ )	
		PredictorPaletteEntries[ comp ][ i ] = sps_palette_predictor_initializers[ comp ][ i ]

  - If pps_palette_predictor_initializer_present_flag is equal to 1, the following applies:

- PredictorPaletteSize is set equal to pps_num_palette_predictor_initializer_minus1 plus 1.

- The array PredictorPaletteEntries is derived as follows:

for( comp = 0; comp < numComps; comp++ )
	for( i = 0; i < PredictorPaletteSize; i++ )	(9-5)
		PredictorPaletteEntries[ comp ][ i ] = pps_palette_predictor_initializers[ comp ][ i ]

  - Otherwise (pps_palette_predictor_initializer_present_flag is equal to 0 ), if sps_palette_predictor_initializer_present_flag is equal to 1, the following applies:

- PredictorPaletteSize is set equal to sps_num_palette_predictor_initializer_minus1 plus 1.

- The array PredictorPaletteEntries is derived as follows:

for( comp = 0; comp < numComps; comp++ )
	for( i = 0; i < PredictorPaletteSize; i++ )	(9-6)
		PredictorPaletteEntries[ comp ][ i ] = sps_palette_predictor_initializers][ comp ][ i ]

  - Otherwise (pps_palette_predictor_initializer_present_flag is equal to 0 and sps_palette_predictor_initializer_present_flag is equal to 0), PredictorPaletteSize is set equal to 0 and PredictorPaletteSize is set equal to 0.

# Conclusion

Two modifications are proposed for palette predictor initialization: allow to signal zero palette predictor initializer size in PPS for the ability to disable palette predictor initialization on the picture basis, and combine palette predictor initializer from both PPS and SPS sets for signalling efficiency.

It is proposed to adopt both modifications into the SCC standard.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).