| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0043 |
| --- | --- |

| Title: | Restriction for maximum palette predictor size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Rajan Joshi, Marta Karczewicz | Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the current working draft, the restriction on the syntax element delta_palette_max_predictor_size, used in the derivation of the maximum palette predictor size, is set to a constant value equal to 64. If the signalled maximum palette size is less than 64, it is not possible to utilize maximum palette predictor size equal to 128, as used in CTC. It is proposed to modify the constraint on the delta_palette_max_predictor_size to (128 − palette_max_size) instead of a constant value of 64.

# Introduction and proposal

In the current working draft, the maximum palette predictor size is derived as follows

PaletteMaxPredictorSize = palette_max_size + delta_palette_max_predictor_size,

where palette_max_size and delta_palette_max_predictor_size shall be less than or equal to 64.

With the current delta_palette_max_predictor_size restriction, it is not possible to signal palette predictor size equal to 128 if the maximum palette size is less than 64. However, in SCM-5.2 and CTC maximum palette predictor size is set equal to 128.

It is proposed to modify delta_palette_max_predictor_size restriction as follows:

delta_palette_max_predictor_size shall be less than or equal to 128 − palette_max_size.

Working draft change is provided based on JCTVC-U1005-v2.

When an active SPS for the base layer has palette_mode_enabled_flag equal to 1, palette_max_size and delta_palette_max_predictor_size shall be less than or equal to 64, and delta_palette_max_predictor_size shall be less than or equal to 128 − palette_max_size.

# Patent rights declaration(s)

Qualcomm Incorporated does not have any current or pending patent rights relating to the technology described in this contribution.