| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0044 |
| --- | --- |

| Title: | Weighted prediction for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Rajan Joshi, Marta Karczewicz | Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this contribution, current picture is additionally added into the reference picture list 1 and weighted prediction is enabled for that added picture. Simulation results reportedly show BD-rate improvement for luma in the range of -0.1 to 0.7 for all intra coding with full frame intra block copy.

# Introduction

Last meeting normative disabling of weighted prediction for intra block copy (IBC) was adopted, however no results were shown about potential usefulness of the weighted prediction. This contribution attempts to study this question.

# Proposal

This contribution proposes to allow weighted prediction for intra block copy, and the following configuration is studied.

Additionally to the reference picture list 0, current picture is added as a last reference picture into reference picture list 1. It is done for all pictures for all intra coding configuration and only for IRAP pictures for RA and LB test conditions. The latter was done to minimize the overhead associated with increased number of reference pictures.

Weighted prediction is enabled only for the current reference picture in the reference picture list 1, and all pictures are coded with B-slices, however additional search for intra block copy is performed only for uni-direction-L1 motion vectors similarly to what is done for reference picture list 0, and bi-prediction for IBC was not checked at encoder.

In this contribution, only fixed offset with the value -8 is used for the weighted prediction. This offset can be treated as a compensation of the quantization errors happening in intra block copy coding.

More sophisticated weights assignment can be done based, for example, on the characteristics and pixel distribution for each picture, which potentially may further improve IBC prediction with weighted prediction and was not considered in this contribution.

To study the effect of only adding the current picture into the reference picture list 1, two tests based on SCM-5.2 under CTC with and without weighted prediction enabled were performed and are summarized in the tables below for full frame intra block copy 4:4:4. More detailed results can be found in the accompanied Excel tables.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 118% |  |  |
| Dec Time[%] | 95% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.0% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.3% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.2% | 0.0% |
| Enc Time[%] | 92% |  |  |
| Dec Time[%] | 92% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.1% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.1% | -0.2% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.4% | 0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.2% | 0.2% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.5% |
| YUV, Animation, 720p | -0.1% | 0.3% | 0.2% |
| YUV, camera captured, 1080p | 0.1% | 0.3% | 0.1% |
| Enc Time[%] | 93% |  |  |
| Dec Time[%] | 96% |  |  |

Test 1. Current picture added to the RefPicList1 without weighted prediction

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.6% | -0.6% | -0.6% |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.7% | -0.6% | -0.6% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 157% |  |  |
| Dec Time[%] | 107% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -0.2% | -0.1% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.2% | 0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.2% | 0.0% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 114% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.3% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.3% | 0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.2% | 0.3% | 0.4% |
| YUV, mixed content, 1440p & 1080p | 0.2% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.1% | 0.3% | 0.0% |
| YUV, camera captured, 1080p | 0.1% | 0.4% | 0.2% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 132% |  |  |

Test 2. Current picture added to the RefPicList1 with weighted prediction

Working draft change is provided based on JCTVC-U1005-v2.

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( ChromaArrayType != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) |  |
| if( PicOrderCnt( RefPicList0[ i ] ) != PicOrderCnt( CurrPic ) ) |  |
| luma_weight_l0_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) |  |
| if( PicOrderCnt(RefPicList0[ i ]) != PicOrderCnt( CurrPic ) ) |  |
| chroma_weight_l0_flag[ i ] | u(1) |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) { |  |
| if( luma_weight_l0_flag[ i ] ) { |  |
| delta_luma_weight_l0[ i ] | se(v) |
| luma_offset_l0[ i ] | se(v) |
| } |  |
| if( chroma_weight_l0_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l0[ i ][ j ] | se(v) |
| delta_chroma_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| if( slice_type = = B ) { |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) |  |
| if( PicOrderCnt(RefPicList0[ i ]) != PicOrderCnt( CurrPic ) ) |  |
| luma_weight_l1_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) |  |
| if( PicOrderCnt(RefPicList0[ i ]) != PicOrderCnt( CurrPic ) ) |  |
| chroma_weight_l1_flag[ i ] | u(1) |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) { |  |
| if( luma_weight_l1_flag[ i ] ) { |  |
| delta_luma_weight_l1[ i ] | se(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_weight_l1_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l1[ i ][ j ] | se(v) |
| delta_chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |

# Patent rights declaration(s)

Qualcomm Incorporated does not have any current or pending patent rights relating to the technology described in this contribution.