| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0045 |
| --- | --- |

| Title: | TMVP constraint for intra block copy with constrained intra prediction enabled |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Rajan Joshi, Krishna Rapaka, Marta Karczewicz | Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the current intra block design, when constrained intra prediction is enabled, TMVP motion vector candidate is not inserted into the candidate list, making it different from inter prediction and this change can be considered as a block level change, contrary to the intra block copy design principle to not have such modifications. It is proposed to replace normative disabling of the TMVP when constrained intra prediction is enabled with bitstream constraint.

# Problem statement

When constrained intra prediction (CIP) is enabled, temporal motion vector predictor (TMVP) is not added into the candidate list. The following specification language in the derivation process for temporal luma motion vector prediction is applied:

- Otherwise, if the reference picture is the current picture and constrained_intra_pred_flag is equal to 1, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

This check is not present in HEVC v1 and can be treated as a low level change, since the same candidate list derivation cannot be shared between inter and intra blokc copy (IBC). In the latter case, the TMVP candidate, treated as unavailable, is replaced with another MV candidate and the candidate list would be different from the inter one.

However, TMVP cannot be used for IBC since the motion vector information is coming from the temporal reference picture and this prediciton propogation is not allowed for CIP.

# Proposal

It is proposed to replace normative disabling of the TMVP candidate for IBC when CIP is enabled with the encoder constraint that such a candidate cannot be signalled in the bitstream.

It is a requirement of bitstream conformance that if the reference picture is the current picture and constrained_intra_pred_flag is equal to 1, TMVP candidate shall not be signalled and used for intra block copy prediction.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).