| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0046r1 |
| --- | --- |

| Title: | On the CU-level escape flag in the palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yao-Jen Chang, 2Jih-Sheng Tu, 3Chun-Lung Lin, Ching-Chieh Lin 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-8774 1britpablo@itri.org.tw 2sunrisejstu@itri.org.tw 3Chunlung@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

In the current Screen Content Coding, the CU-level escape flag palette_escape_val_present_flag is parsed if the condition “CurrentPaletteSize != 0” is true. This condition check has been adopted into the HEVC SCC standard since the 19th JCT-VC meeting. The reason to adopt this condition check was that there was a logic bug in the HEVC SCC Draft Text 1 when palette size is equal to zero and palette_escape_val_present_flag is parsed as zero. However, in the latest specification draft (HEVC SCC Draft Text 4), if the palette size is zero, no logic bug is occurred no matter what the value of palette_escape_val_present_flag is parsed as zero or one. Therefore, the condition check “CurrentPaletteSize != 0” is redundant in the latest HEVC SCC Draft Text. To reduce this redundant condition check and make the syntax simpler and cleaner, this contribution proposes to remove the condition check “CurrentPaletteSize != 0” while parsing palette_escape_val_present_flag.

# Problem statement

In the current design [1], the CU-level escape flag palette_escape_val_present_flag is parsed if the condition “CurrentPaletteSize != 0” is true; otherwise, palette_escape_val_present_flag is inferred as one. This scheme is proposed in JCTVC-S0105/S0110/S0173 due to the reason that if palette_escape_val_present_flag and palette size are both zero, a logic bug occurs in the HEVC SCC Draft Text 1 [2]. The flag palette_index_idc can not be correctly parsed in this case, and neither do the value of PaletteRun and the flag copy_above_palette_indices_flag. However, in the current HEVC SCC Draft Text 4 [1], if the palette size is equal to zero, these syntax valuables will be set as default values:

    - copy_above_palette_indices_flag inferred as 0

    - palette_index_idc inferred as 0

    - PaletteRun inferred as

It implies that no logic bug is occurred when palette size is equal to zero and palette_escape_val_present_flag is parsed as zero. Therefore, it doesn’t matter whether to check the palette size or not before parsing palette_escape_val_present_flag.

# Proposed method

In the proposed modification, the “if” statement is removed while parsing palette_escape_val_present_flag. The syntax change is shown as follows:

| palette_coding( x0, y0, nCbS ) { | Descriptor |
| --- | --- |
| …… |  |
| if( CurrentPaletteSize != 0 ) |  |
| palette_escape_val_present_flag | ae(v) |
| …… |  |

The proposed change doesn’t affect the performance results in the common test condition [3]. Therefore, this contribution skips to demonstrate the simulation results.

# Conclusion

This contribution proposes a syntax modification for the CU-level escape flag. The proposed syntax modification can remove one condition check associated with the palette size, and make the syntax simpler and cleaner.

# Reference

- R. Joshi, J. Xu, S. Liu, Y. Ye, "High Efficiency Video Coding (HEVC) Screen Content Coding: Draft 4", 21st JCT-VC meeting, JCTVC-U1005, Poland, Warsaw, July 2015.

- R. Joshi, J. Xu, “HEVC Screen Content Coding Draft Text 1,” 18th JCT-VC meeting, JCTVC-R1005, Sapporo, Japan, Jun. 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding,” 21th JCT-VC meeting, JCTVC-U1015, Warsaw, Poland, Jun. 2015.

# Patent rights declaration(s)

ITRI international may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)