| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0047 |
| --- | --- |

| Title: | On the parsing process for the palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yao-Jen Chang, 2Chun-Lung Lin, Ching-Chieh Lin and Jih-Sheng Tu 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-8774 1britpablo@itri.org.tw 2Chunlung@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

There are parsing dependencies between the flags palette_predictor_run, num_signalled_palette_entries, num_signalled_palette_entries and palette_escape_val_present_flag. This contribution removes the dependencies by grouping the flags with longest delays together, i.e., palette_predictor_run and num_signalled_palette_entries. Without the dependencies between flags, it can increase the flexibility of the implementation on the parsing process of the palette syntax. The simulation results show that there is no change on coding performance compared with the SCM 5.2 anchor under the common test conditions.

# Introduction

In the current Screen Content Coding, there are parsing dependencies during the parsing of the palette coding. The parsing process of the palette coding described in Sub-section 7.3.8.13 in the SCC text [1] is shown in Fig. 1. The flag num_signalled_palette_entries is dependent on the flag palette_predictor_run, the flag palette_entry is dependent on the flag num_signalled_palette_entries, and the flag palette_escape_val_present_flag is dependent on the flag num_signalled_palette_entries. Suppose the maximum palette size and the maximum palette predictor size can support the sizes to 63 and 128, respectively. The number of bits for num_signalled_palette_entries is between 1 and 10, inclusively. The flag palette_escape_val_present_flag will only need 1 bit. However, the worst cases for palette_predictor_run and palette_entry would require 128 bits and 1890 bits, respectively. Therefore, these dependencies will cause a serious long delay on implementation. This contribution proposes to remove the dependency by grouping the flags with longest delays, i.e., palette_predictor_run and palette_entry. Note that palette_predictor_run and palette_entry are independent to each other if palette_entry is parsed prior to palette_predictor_run.

| Palette_coding( x0, y0, nCbS ) { | Descriptor |
| --- | --- |
| palettePredictionFinished = 0 |  |
| NumPredictedPaletteEntries = 0 |  |
| for( predictorEntryIdx = 0; predictorEntryIdx < PredictorPaletteSize && !palettePredictionFinished && NumPredictedPaletteEntries < palette_max_size; predictorEntryIdx++ ) { |  |
| palette_predictor_run | ae(v) |
| if( palette_predictor_run != 1 ) { |  |
| if( palette_predictor_run > 1 ) |  |
| predictorEntryIdx += palette_predictor_run – 1 |  |
| PalettePredictorEntryReuseFlag[ predictorEntryIdx ] = 1 |  |
| NumPredictedPaletteEntries++ |  |
| } else |  |
| palettePredictionFinished = 1 |  |
| } |  |
| if( NumPredictedPaletteEntries < palette_max_size ) |  |
| num_signalled_palette_entries | ae(v) |
| numComps = ( ChromaArrayType = = 0 ) ? 1 : 3 |  |
| for( cIdx = 0; cIdx < numComps; cIdx++ ) |  |
| for( I = 0; I < num_signalled_palette_entries; i++ ) |  |
| palette_entry | ae(v) |
| if( CurrentPaletteSize != 0 ) |  |
| palette_escape_val_present_flag | ae(v) |
| if( MaxPaletteIndex > 0) { |  |
| num_palette_indices_minus1 | ae(v) |
| … |  |

Fig. 1. The syntax of the palette coding (extracted from the HEVC SCC Draft Text 4 [1])

# Proposed method 1: Change of the parsing process of the palette table

This contribution proposes to remove the dependency between num_signalled_palette_entries and palette_predictor_run by deleting the condition “NumPredictedPaletteEntries < palette_max_size”. Then, it further proposes to put the flags with the longest delays together: palette_predictor_run and palette_entry. Because there is no dependency between palette_predictor_run and palette_entry if palette_entry is parsed prior to palette_predictor_run, they can be parsed concurrently and thus improve the flexibility of the implementation on the parsing process. The detail of the proposed parsing process is as shown in Fig. 2. Note that the words with yellow background are the proposed change compared to the original syntax in Sub-section 7.3.8.13 in the SCC text [1].

| Palette_coding( x0, y0, nCbS ) { | Descriptor |
| --- | --- |
| num_signalled_palette_entrie | ae(v) |
| numComps = ( ChromaArrayType = = 0 ) ? 1 : 3 |  |
| for( cIdx = 0; cIdx < numComps; cIdx++ ) |  |
| for( i = 0; i < num_signalled_palette_entries; i++ ) |  |
| palette_entry | ae(v) |
| palettePredictionFinished = 0 |  |
| NumPredictedPaletteEntries = 0 |  |
| for( predictorEntryIdx = 0; predictorEntryIdx < PredictorPaletteSize && !palettePredictionFinished && NumPredictedPaletteEntries < palette_max_size && num_signalled_palette_entrie < palette_max_size; predictorEntryIdx++ ) { |  |
| palette_predictor_run | ae(v) |
| if( palette_predictor_run != 1 ) { |  |
| if( palette_predictor_run > 1 ) |  |
| predictorEntryIdx += palette_predictor_run − 1 |  |
| PalettePredictorEntryReuseFlag[ predictorEntryIdx ] = 1 |  |
| NumPredictedPaletteEntries++ |  |
| } else |  |
| palettePredictionFinished = 1 |  |
| } |  |
| if( NumPredictedPaletteEntries < palette_max_size ) |  |
| num_signalled_palette_entries | ae(v) |
| numComps = ( ChromaArrayType = = 0 ) ? 1 : 3 |  |
| for( cIdx = 0; cIdx < numComps; cIdx++ ) |  |
| for( i = 0; i < num_signalled_palette_entries; i++ ) |  |
| palette_entry | ae(v) |
| if( CurrentPaletteSize != 0 ) |  |
| palette_escape_val_present_flag | ae(v) |
| … |  |

Fig. 2. The proposed method 1

# Proposed method 2: Change of the parsing process of the palette table and the CU-level escape flag

In JCTVC-V0046 [2], the redundant condition check “CurrentPaletteSize  !=  0” is removed to make the syntax simpler and cleaner while parsing palette_escape_val_present_flag. On top of this change, this contribution further proposes to move the flag palette_escape_val_present_flag to the beginning of the palette syntax for removal of the dependency of palette_escape_val_present_flag. The detail of the proposed parsing process is as shown in Fig. 3. Note that the words with yellow background are the proposed change compared to the original syntax in Sub-section 7.3.8.13 in the SCC text [1].

| Palette_coding( x0, y0, nCbS ) { | Descriptor |
| --- | --- |
| palette_escape_val_present_flag | ae(v) |
| num_signalled_palette_entrie | ae(v) |
| numComps = ( ChromaArrayType = = 0 ) ? 1 : 3 |  |
| for( cIdx = 0; cIdx < numComps; cIdx++ ) |  |
| for( i = 0; i < num_signalled_palette_entries; i++ ) |  |
| palette_entry | ae(v) |
| palettePredictionFinished = 0 |  |
| NumPredictedPaletteEntries = 0 |  |
| for( predictorEntryIdx = 0; predictorEntryIdx < PredictorPaletteSize && !palettePredictionFinished && NumPredictedPaletteEntries < palette_max_size && num_signalled_palette_entrie < palette_max_size; predictorEntryIdx++ ) { |  |
| palette_predictor_run | ae(v) |
| if( palette_predictor_run != 1 ) { |  |
| if( palette_predictor_run > 1 ) |  |
| predictorEntryIdx += palette_predictor_run − 1 |  |
| PalettePredictorEntryReuseFlag[ predictorEntryIdx ] = 1 |  |
| NumPredictedPaletteEntries++ |  |
| } else |  |
| palettePredictionFinished = 1 |  |
| } |  |
| if( NumPredictedPaletteEntries < palette_max_size ) |  |
| num_signalled_palette_entries | ae(v) |
| numComps = ( ChromaArrayType = = 0 ) ? 1 : 3 |  |
| for( cIdx = 0; cIdx < numComps; cIdx++ ) |  |
| for( i = 0; i < num_signalled_palette_entries; i++ ) |  |
| palette_entry | ae(v) |
| if( CurrentPaletteSize != 0 ) |  |
| palette_escape_val_present_flag | ae(v) |
| … |  |

Fig. 3. The proposed method 2

# Simulation results

The proposed methods are simulated on top of the SCM 5.2 anchor under the common test condition [3].  Table 1 shows the lossy coding results of Method 1 on the condition of full-frame IBC search and 444 chroma format. Table 2 shows the lossy coding results of Method 2 on the condition of constrained IBC search and 444 chroma format. Full results can be found in the attached spreadsheets (to be updated for those data unavailable now). The results show that the proposed changes have no effect on the coding gain.

Table 1. Lossy results under the condition of full-frame IBC search and 444 chroma format for Method 1

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | G/Y |  | B/U |  | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% |  | 0.0% |  | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% |  | 0.0% |  | 0.0% |
| RGB, Animation, 720p | 0.0% |  | 0.0% |  | 0.0% |
| RGB, camera captured, 1080p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, Animation, 720p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, camera captured, 1080p | 0.0% |  | 0.0% |  | 0.0% |
| Enc Time[%] | 100% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |
|  | Random Access |  |  |  |  |
|  | G/Y |  | B/U |  | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% |  | 0.0% |  | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% |  | 0.0% |  | 0.0% |
| RGB, Animation, 720p | 0.0% |  | 0.0% |  | 0.0% |
| RGB, camera captured, 1080p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, Animation, 720p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, camera captured, 1080p | 0.0% |  | 0.0% |  | 0.0% |
| Enc Time[%] | 99% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |
|  | Low delay B |  |  |  |  |
|  | G/Y |  | B/U |  | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% |  | 0.0% |  | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% |  | 0.0% |  | 0.0% |
| RGB, Animation, 720p | 0.0% |  | 0.0% |  | 0.0% |
| RGB, camera captured, 1080p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, Animation, 720p | 0.0% |  | 0.0% |  | 0.0% |
| YUV, camera captured, 1080p | 0.0% |  | 0.0% |  | 0.0% |
| Enc Time[%] | 100% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |

Table 2. Lossy results under the condition of full-frame IBC search and 444 chroma format for Method 2

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | G/Y |  | B/U |  | R/V |
| RGB, text & graphics with motion, 1080p & 720p |  |  |  |  |  |
| RGB, mixed content, 1440p & 1080p |  |  |  |  |  |
| RGB, Animation, 720p |  |  |  |  |  |
| RGB, camera captured, 1080p |  |  |  |  |  |
| YUV, text & graphics with motion, 1080p & 720p |  |  |  |  |  |
| YUV, mixed content, 1440p & 1080p |  |  |  |  |  |
| YUV, Animation, 720p |  |  |  |  |  |
| YUV, camera captured, 1080p |  |  |  |  |  |
| Enc Time[%] |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |
|  | Random Access |  |  |  |  |
|  | G/Y |  | B/U |  | R/V |
| RGB, text & graphics with motion, 1080p & 720p |  |  |  |  |  |
| RGB, mixed content, 1440p & 1080p |  |  |  |  |  |
| RGB, Animation, 720p |  |  |  |  |  |
| RGB, camera captured, 1080p |  |  |  |  |  |
| YUV, text & graphics with motion, 1080p & 720p |  |  |  |  |  |
| YUV, mixed content, 1440p & 1080p |  |  |  |  |  |
| YUV, Animation, 720p |  |  |  |  |  |
| YUV, camera captured, 1080p |  |  |  |  |  |
| Enc Time[%] |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |
|  | Low delay B |  |  |  |  |
|  | G/Y |  | B/U |  | R/V |
| RGB, text & graphics with motion, 1080p & 720p |  |  |  |  |  |
| RGB, mixed content, 1440p & 1080p |  |  |  |  |  |
| RGB, Animation, 720p |  |  |  |  |  |
| RGB, camera captured, 1080p |  |  |  |  |  |
| YUV, text & graphics with motion, 1080p & 720p |  |  |  |  |  |
| YUV, mixed content, 1440p & 1080p |  |  |  |  |  |
| YUV, Animation, 720p |  |  |  |  |  |
| YUV, camera captured, 1080p |  |  |  |  |  |
| Enc Time[%] |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |

# Conclusion

This contribution removes the parsing dependencies between the first four flags of the palette syntax. Without the parsing dependencies, it can increase the flexibility of the implementation on the parsing process of the palette syntax. The simulation results show that the proposed changes have no effect on the coding gain. It is suggested to adopt the proposed changes to the next version of the SCC text.

# Reference

- R. Joshi, S. Liu, G. J. Sullivan, J. Xu, Y. Ye, “HEVC Screen Content Coding Draft Text 4,” 21th JCT-VC meeting, JCTVC-U1005, Warsaw, Poland, Jun. 2015.

- Y. J. Chang, J. S. Tu, C. L. Lin, C. C. Lin, “On the CU-level escape flag in the palette mode,” 22th JCT-VC meeting, JCTVC-V0046, Geneva, Switzerland, Oct. 2015.

- H. Yu, R. Cohen, K. Rapaka, J. Xu , “Common Test Conditions for Screen Content Coding,” 21th JCT-VC meeting, JCTVC-U1015, Warsaw, Poland, Jun. 2015.

# Patent rights declaration(s)

ITRI international may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)