| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0048r2 |
| --- | --- |

| Title: | On bi-prediction restriction when intra block copy is enabled |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Rapaka V. Seregin R. Joshi M. Karczewicz | Tel: Email: | 858-658-2042 krapaka@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes methods to improve bi-prediction restriction when intra block copy is enabled. During the 21st JCTVC meeting in Warsaw, JCTVC-U0078 was adopted to apply 8x8 bi-prediction restriction when IBC is enabled (i.e curr_pic_as_ref_enabled_flag is equal to 1) or adaptive motion vector resolution is enabled (i.e use_integer_mv equal to 1). This method ensures that the worst case memory bandwidth (BW) per sample does not exceed the HEVC v1 worst case BW requirement. In this contribution it is proposed to relax existing 8x8 bi-prediction restriction by applying it only when both MVs of an 8x8 bi-predicted block are not integer-pel, or that MVs are not equal, or they are not from the same reference picture. It is observed that worst case memory BW remain within the limits of HEVC v1. It is reported that the proposed approach provides objective bit rate reductions of 0.5 % and 0.3% for Random access 720p Animation RGB and YUV categories, respectively over SCM-5.0 anchor. It is reported that the proposed approach provides objective bit rate reductions of 0.6 % and 0.6% for Low delay B 720p Animation RGB and YUV categories, respectively over SCM-5.2 anchor.

In the revision r1 of this document, the complete experimental results are uploaded. Also, previous full frame IBC results are modified by removing encoder optimizations.

In the revision r2 of this document, the presentation deck is uploaded and minor typos have been corrected.

In the revision r3 of this document, an issue related to spec and software mismatch is reported.

# Technical Description

In this contribution, it is proposed to improve the current bi-prediction restriction for intra block copy as follows.

The 8x8 bi-prediction is restricted when all of the following conditions are true:

- IBC is enabled for the current block based on slice, picture or sequence level IBC control

- use_integer_mv is equal to zero for the current slice

- Both MVs of an 8x8 bi-predicted block are

  - not integer-pel (fractional-pel accuracy)

  - not equal

  - not from the same reference picture

It is observed that with the above conditions the SCC worst case memory BW is within the limits of HEVC v1. The memory analysis numbers based on the JCTVC-U0175 are shown in the section 4 of this document.

# Results

Experimental results for random access and low delay lossy configuration are provided for the proposed method implemented over SCM5.2 under SCC test conditions. The simulation results of lossless are attached as a part of this proposal.

The simulation times are not accurate as heterogeneous clusters were used for the simulations. The proposed method does not impact All intra coding test configuration from CTC.

Table 1: Proposed method over SCM 5.2 anchor (Full frame IBC 4:4:4)

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.2% |
| RGB, Animation, 720p | -0.5% | -0.6% | -0.6% |
| RGB, camera captured, 1080p | -0.2% | -0.3% | -0.2% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.3% | -0.2% |
| YUV, Animation, 720p | -0.3% | -0.3% | -0.4% |
| YUV, camera captured, 1080p | -0.2% | -0.1% | -0.2% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.2% | -0.3% |
| RGB, mixed content, 1440p & 1080p | -0.6% | -0.5% | -0.5% |
| RGB, Animation, 720p | -0.6% | -0.7% | -0.7% |
| RGB, camera captured, 1080p | -0.3% | -0.3% | -0.3% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.2% | -0.1% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.4% | -0.6% |
| YUV, Animation, 720p | -0.4% | -0.5% | -0.4% |
| YUV, camera captured, 1080p | -0.1% | -0.1% | -0.2% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2: Proposed method over SCM 5.2 anchor (local search IBC 4:4:4)

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.2% |
| RGB, Animation, 720p | -0.5% | -0.6% | -0.6% |
| RGB, camera captured, 1080p | -0.2% | -0.2% | -0.2% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.1% |
| YUV, Animation, 720p | -0.3% | -0.3% | -0.4% |
| YUV, camera captured, 1080p | -0.1% | -0.1% | -0.2% |
| Enc Time[%] | 83% |  |  |
| Dec Time[%] | 84% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.3% | -0.2% | -0.2% |
| RGB, mixed content, 1440p & 1080p | -0.6% | -0.5% | -0.4% |
| RGB, Animation, 720p | -0.6% | -0.8% | -0.8% |
| RGB, camera captured, 1080p | -0.2% | -0.3% | -0.3% |
| YUV, text & graphics with motion, 1080p & 720p | -0.3% | -0.2% | -0.3% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.3% | -0.4% |
| YUV, Animation, 720p | -0.3% | -0.4% | -0.4% |
| YUV, camera captured, 1080p | -0.1% | -0.2% | -0.2% |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 92% |  |  |

Table 3: Proposed method over SCM 5.2 anchor (full frame IBC 4:2:0)

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.2% | -0.1% |
| YUV, Animation, 720p | -0.2% | -0.4% | -0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| YUV, text & graphics with motion, 1080p & 720p | -0.2% | -0.1% | -0.3% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.1% | -0.1% |
| YUV, Animation, 720p | -0.2% | 0.1% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 103% |  |  |

# Specification changes

inter_pred_idc[ x0 ][ y0 ] specifies whether list0, list1, or bi-prediction is used for the current prediction unit according to Table 7-1. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

Table 7-1 – Name association to inter prediction mode

| inter_pred_idc | Name of inter_pred_idc |  |
| --- | --- | --- |
|  | ( nPbW + nPbH ) != 12 | ( nPbW + nPbH ) = = 12 |
| 0 | PRED_L0 | PRED_L0 |
| 1 | PRED_L1 | PRED_L1 |
| 2 | PRED_BI | Na |

When inter_pred_idc[ x0 ][ y0 ] is not present, it is inferred to be equal to PRED_L0.

The variable EightbyEightBiPredhasnoIntegerMv  is derived as EightbyEightBiPredhasnoIntegerMv  = !((mvL0 & 0x3 == 0) || (mvL1&0x3 == 0))

The variable EightbyEightBiPrednotIdenticalMv is derived as EightbyEightBiPrednotIdenticalMv = !((mvL0 == mvL1) && (POC corresponding to refIdxL0 and refIdxL1 point to the same picture))

When inter_pred_idc[ x0 ][ y0 ] is equal to 2, nPbW is equal to 8, nPbH is equal to 8, and either of CurrPicInList0Flag and CurrPicInList1Flag is equal to 1, and EightbyEightBiPredhasnoIntegerMv equal to 1 and EightbyEightBiPrednotIdenticalMv is equal to 1  the variable EightbyEightBiPredInUseforCurrPic is set equal to 1.

# Worst Case memory BW analysis

| Worst case BW analysis for 4:4:4 (per sample memory access) |  |  |
| --- | --- | --- |
| HEVC V1 | Proposed method with IBC enabled | Proposed method with IBC disabled |
| 23.375 | 21.75 | 23.375 |
| 30.750 | 28.25 | 30.750 |
| 25.000 | 23 | 25.000 |
| 33.000 | 30 | 33.000 |
| 31.500 | 28 | 31.500 |

| Worst case BW analysis for 4:2:0 (per sample memory access) |  |  |
| --- | --- | --- |
| HEVC V1 | Proposed method with IBC enabled | Proposed method with IBC disabled |
| 14.375 | 13.125 | 14.375 |
| 18.000 | 17.75 | 18.000 |
| 15.500 | 14 | 15.500 |
| 19.500 | 19 | 19.500 |
| 20.000 | 17.5 | 20.000 |

# On SW and WD mismatch

In this section, a mismatch between WD and the software is reported related to adoption of JCTVC-U0078 that was discovered during an offline discussion in the 22nd JCTVC meeting in Geneva. The SCM 5.2 (as well as CE software) performs 8x8 bi-to-uni conversion of the final candidate list when 8x8 bi-prediction restriction is applied. This is in similar lines to HEVC v1 which also converts bi-to-uni for 8x4 bi-prediction merge candidates. The WD text specification, however, does not apply bi-to-uni conversion of merge candidates resulting in mismatch with SCM 5.2.

We would like to bring this issue to the attention of experts to decide which way to proceed

- Align WD with software :- Bi-to-Uni conversion of merge candidates

- Align Software with WD: - No conversion and disallow usage of the merge candidate.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).