| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0049r1 |
| --- | --- |

| Title: | On intra block copy merge vector handling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Rapaka V. Seregin R. Joshi M. Karczewicz | Tel: Email: | 858-658-2042 krapaka@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes methods to handle of fractional-pel accuracy merge candidates for intra block copy (IBC). In the current WD, the accuracy of luma motion vectors is restricted to integer-pel. However, it is possible based on the current WD to have merge candidates in the final list that have fractional-pel accuracy (for example temporal merge candidate). In this contribution two solutions are proposed to handle these merge vectors. It is observed that the proposed approach does not have any performance change comparing to SCM 5.2 under common test conditions.

In the revision 1 of this package, presentation deck has been uploaded.

- Problem

In the current WD, the IBC luma block vectors are restricted to integer precision accuracy, however merge MV candidate may have fractional-pel accuracy, for example TMVP candidate. Current WD does not prohibit this case, so it is possible to signal fractional MV for IBC.

# Proposed method

In this section, two methods are proposed to address the problem of fractional MV for IBC. It is observed that the proposed approach does not have any performance change comparing to SCM 5.2 under common test conditions since the current picture is not added into the reference picture list having reference index equal to zero and TMVP enabled.

## Rounding the merge candidates for IBC

## In this method, it is proposed to round the merge vectors when the merge candidate corresponds to intra block copy mode (i.e reference picture is the current picture) in the similar way as done when use_integer_mv is equal to 1. The proposed specification changes (highlighted in yellow) are:

##### 8.5.3.2.2 : Derivation process for luma motion vectors for merge mode

The following assignments are made with N being the candidate at position merge_idx[ xOrigP ][ yOrigP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xOrigP ][ yOrigP ] ] ) and X being replaced by 0 or 1:

refIdxLX = refIdxLXN		(8-1)

predFlagLX = predFlagLXN		(8-2)

    - When use_integer_mv_flag is equal to 0 and the reference picture is not the current picture

mvLX[ 0 ] = mvLXN[ 0 ]		(8-3)

mvLX[ 1 ] = mvLXN[ 1 ]		(8-4)

    - Otherwise ( use_integer_mv_flag is equal to 1 or the reference picture is the current picture )

mvLX[ 0 ] = ( mvLXN[ 0 ]  >>  2 )  <<  2	(8-5)

mvLX[ 1 ] = ( mvLXN[ 1 ]  >>  2 )  <<  2	(8-6)

## Encoder restriction to discard fractional-pel merge candidates for IBC

## In this method, it is proposed to add an encoder restriction to not include the fractional-pel accuracy merge vectors in the final list when the merge candidate corresponds to intra block copy mode (i.e reference picture is the current picture). It can be formulated as follows.

## It is a requirement of bitstream conformance that bitstream shall not have a merge_idx associated with non-integer MV when reference picture is the current picture.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).