| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0052 |
| --- | --- |

| Title: | HDR-10 update |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chad Fogg MovieLabs 475 Sansome Street, Suite 740 San Francisco, CA 94111 | Email: | chadfogg@gmail.com |
| Source: | Motion Picture Laboratories, Inc. |  |  |

_____________________________

# Abstract

Minor corrections to JCTVC-U0045 are included in this document.  For reference, an updated version of U0045 (revision 3) is also attached to the JCTVC-V0052 .zip archive.

# Introduction

A corrected version of section 3.2, description of x265 use, in JCT-VC U0045 is described below:

To obtain a copy of the latest source code [8]:

hg clone https://bitbucket.org/multicoreware/x265

$ cd x265/build/linux

$ ./make-Makefiles.bash

When running the ./make-Makefiles.bash script, be sure to set “HIGH_BIT_DEPTH = On” to enable 10-bit (HEVC Main10) encodes.  Finally, make the project:

$ make

The following command line arguments are needed to signal HDR-10 combination in x265:

--input-depth 10 --colorprim bt2020 --transfer smpte-st-2084 --colormatrix bt2020nc –chromaloc 2 –P main10

optional parameters for ST 2086 (using the Table 4 example), MaxFALL, MaxCLL:

--master-display "G(13250,34500)B(7500,3000)R(34000,16000)WP(15635,16450)L(40000000,47) "

--max-cll "4000,0"

The content level example above assumes MaxCLL (Maximum Content [pixel] Light Level) =4000 candelas/m2, while MaxFALL (Maximum Frame Average Light Level) is set to 0 to illustrate a case where the value is unknown or not measured (0 is reserved in both MaxCLL and MaxFALL in the CEA 861.3 specification to indicate that the value is not specified).  Example calculation of MaxCLL and MaxFALL measures are provided in JCTVC-T0101 [9]

A complete x265 command line example:

x265 --colorprim bt2020 --transfer smpte-st-2084 --colormatrix bt2020nc --chromaloc 2  \

--master-display "G(13250,34500)B(7500,3000)R(34000,16000)WP(15635,16450)L(40000000,47)"\

--max-cll "4000,0" \

--profile main10 --level-idc 5.0 --fps 24 --input-res 1920x1080 --input-depth 10 \

--input src.yuv –o bitstream.hevc

x265 currently only outputs raw HEVC elementary bitstreams. The mp4box [11] utility can wrap the stream in a .mp4 file container required by some players:

mp4box -add bitstream.hevc:FMT=HEVC -fps 24 -new bitstream.mp4