| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0053r2 |
| --- | --- |

| Title: | Best practices for HDR/WCG video workflow and coding for Ultra HD Blu-ray |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chad Fogg, Jim Helman MovieLabs 475 Sansome Street, Suite 740 San Francisco, CA 94111 | Email: | chadfogg@gmail.com jhelman@movielabs.com |
| Source: | Motion Picture Laboratories, Inc. |  |  |

_____________________________

# Abstract

# Introduction

MovieLabs is publishing a “best practices” guide to authoring HDR/WCG content, with examples for Ultra HD Blu-ray format, at its website http://www.movielabs.com  starting in October 2015.  An excerpt is included in this document, JCTVC-V0053r1.  A slide presentation with additional detail accompanies version 2 of the zip file archive associated with this document.

# Summary

An abridged version of MovieLabs [1] of best HDR workflow and coding practices is described in this document that recommends filters from [2] and encoding parameters that meet Ultra HD Blu-ray requirements [3].

# Pre-processing

A minimum of 9-tap (including zeros) filter is recommended from [2] for the conversion of 4:4:4 to 4:2:0 samples.  The design of the filter should provide a balance of sharpness and low ripple energy, such as the annotated Matlab magnitude-phase chart below from Figure 1-6 in [2]:

# Encoding parameters

The following x265 parameters are recommended for Blu-Ray 3.0 disc authoring. The full list of x265 parameters are given at the following URL:

http://x265.readthedocs.org/en/default/cli.html

To signal HDR10:

--range limited

--colorprim	bt2020             			[can also be specified as: --colorprim 9]

--transfer smpte-st-2084     			[can also be specified as:  --transfer 16]

--colormatrix bt2020nc     			[can also be specified as:  --colormatrix 9]

--chromaloc 2

The quality efficiency of the veryslow mode is recommended.

--preset veryslow

If the content contains film grain:

--tune film

If the content contains significant film grain and/or noise, disable the Sample Adaptive Offset in-loop filter tool:

--no-sao

For HDR-10 mandatory layer bitstreams:

--input-depth 10

Resolution is either 1920x1080 or 3840x2160:

--input-res 1920x1080

or

--input-res 3840x2160

HEVC Main10 requires 4:2:0 color space.

--input-csp i420

If frame rate is fractional, the frame rate must use ratios defined by the general constraints section 3.1 in [3]. For example, fractional 24 fps (24000/1001) must be set as:

--fps 24000/1001

Ultra HD Blu-ray is strictly progressive:

--no-interlace

Ultra HD Blu-ray only allows HEVC bitstreams to be coded in “High10 profile”

--profile main10

Due to 100 mbps buffering requirements, all Ultra HD Blu-ray mandatory layer UHD bitstreams must  specify level 5.1 high tier:

--level-idc 5.1

--high-tier

Due to semantic restriction that no chroma non-intra deblocking in HEVC [2], it is highly recommended to set Qp chroma offset for both chroma channels to at least -6 for both Cb and Cr chroma planes:

--cbqpoffs -6

--crqpoffss -6

To meet the 100 mbps for UHD 60 fps content, it may be necessary to enable full RDO and full tool complexity. Most of these modes are already enabled by the “veryslow” pre-set or generic default settings of x265:

--rd 6

--ctu 64

--min-cu-size 8

--rect

--amp

--limit-refs 3

--no-early-skip

--no-fast-intra

--b-intra

--no-t-skip-fast

--tu-intra-depth 4

--tu-inter-depth 4

--no-tskip

--rdpenalty 0

--max-tu-size 32

--max-merge 5

--temporal-mvp

--weightp

--weightb

--rc-lookahead  [frame rate]

--b-adapt 2

--b-pyramid

--qg-size 16

--cu-tree

Adaptive quantization is necessary tool to achieve transparency 100 mbps target of Blu-Ray.

--rdoq-level 2

Multipass coding provides better visual consistency, especially when the content is straining the bitrate limits of the disc buffer:

During the first pass:

--analysis-mode 1   --pass 1

During the second pass:

--analysis-mode 2  --pass 2

If quality targets are not being met within the limits of the 100 mbps Ultra HD Blu-ray buffer, it may be necessary to apply pre-processing to the video signal to reduce noise and/or film grain.  An alternative fast method is also available within x265.  Experimental values of approximately 10 have been found to be fairly non-destructive to image detail:

--nr-intra [value]

--nr-inter [value]

Other tools that can help lower bitrate in heavy motion scenes at the expense of computation time are..

Larger search range, for example +/- 128 pixels instead of the default 57:

--merange 128

Full motion search:

--me full

Full subpixel refinement:

--subme 7

Slow first pass:

--slow-firstpass

If significant posterization appears in flat areas of the picture, it may be useful to enable strong intra smoothing and boost visual rate distortion optimization to favour flat, dark areas more strongly than textured, busy picture areas:

--strong-intra-smoothing

--aq-mode 3

Additional Ultra HD Blu-ray specific restrictions:

--min-keyint  [frame rate]

--vbv-bufsize 100000

--vbv-maxrate 100000

--annexb

--no-repeat-headers

--aud

--hrd

# References

[1] http://www.movielabs.com

[2] JCTVC-U0041
http://phenix.int-evry.fr/jct/doc_end_user/current_document.php?id=10045

[3] Blu-ray Disc Association, White Paper: Blu-Ray Disc Read-Only Format (Ultra HD Blu-ray) Audio Visual Application Format Specifications for BD-ROM 3.0 (July 2015):
http://www.blu-raydisc.com/en/Technical/TechnicalWhitePapers/BDROM.aspx