| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0055 |
| --- | --- |

| Title: | Comments on Hybrid Log-Gamma (HLG) video transfer functions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chad Fogg MovieLabs 475 Sansome Street, Suite 740 San Francisco, CA 94111 | Email: | chadfogg@gmail.com |
| Source: | Motion Picture Laboratories, Inc. |  |  |

_____________________________

# Abstract

Two High Dynamic Range (HDR) video transfer functions based on the Hybrid Log Gamma (HLG) formula standardized recently as ARIB 67 (JCTVC-U0032). The first version described in that document (now documented as transfer_characteristics code point 18 in  the latest HEVC version 3 draft, JCTVC-U1005) is limited to 1200 cd/m2 at unity. The second transfer function extendeds the first by combining a “system gamma” tone mapping step that effectively extends the range of the first transfer function to, for example, the 10,000 cd/m2 range of SMPTE ST 2084 (PQ) transfer function (transfer_characteristics code point 16).  The authors of this input document argue that the transfer function must supply information necessary to unambiguously translate each sample code level (1024 in the case of  BitDepth = 10) to intensity values that can be directly imported into ACES, among other systems, whose color space is defined in terms of linear light.

# Summary

Further consideration of transfer functions for inclusion into the HEVC specification should include sufficient information, such as utilization of available metadata, to unambiguously convert sample integer levels to linear light intensity levels.  For example, metadata such as luminance at unity [1], “system gamma”, and ambient light levels [2] may be required in order to perform this mapping.

# References

[1] JCTVC-T0103

http://phenix.int-evry.fr/jct/doc_end_user/current_document.php?id=9872

[2] JCTVC-U0112

http://phenix.int-evry.fr/jct/doc_end_user/current_document.php?id=10116