| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0056 |
| --- | --- |

| Title: | On intra block copy signalling and constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Shan Liu Shawmin Lei | Email: | xiaozhong.xu@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

Two IBC related high level syntax changes are proposed in this document. In item one, the constraint of 8x8 bi-prediction mode usage is modified such that the presence of two versions of current picture is taken into consideration. With the proposed change, 8x8 bi-prediction mode is not allowed only when there are two versions of current decoded picture, together with other existing conditions such as the use of current picture as a reference picture, and the presence of fractional motion vector resolution; In item two, currently, the signalling of the use of current picture as a reference picture is handled at both sequence and picture level. It is proposed to remove the SPS flag sps_curr_pic_ref_enabled_flag, which is used to signal the use of current picture as a reference picture at sequence level. With the proposed change, the use of current picture as a reference picture is signalled at picture level, using the flag pps_curr_pic_ref_enabled_flag.

# Introduction

When current picture is used as a reference picture, the unfiltered reconstructed pixels are used for block compensation. In order to do this, these unfiltered samples need to be written to the storage memory. The writing operations require extra memory access bandwidth as compared to HEVC v1. With these extra writing, the worse case memory bandwidth access may be increased to a number beyond the case in HEVC v1. In order to reduce the worst case memory bandwidth, 8x8 bi-prediction mode is disallowed in some circumstances [1]. Currently, if current picture is used as a reference picture and integer motion resolution is not used, then 8x8 bi-prediction mode is not allowed for the current slice. This constrain avoid the worst base memory access bandwidth to be beyond HEVC v1 only when the two versions of current decoded picture are different. The usage of loop filters should be taken into account for this constrain.

In the current screen content coding specification [2], current picture is used as a reference picture. To enable the use of this tool, there are two different signaling flags. One is at the sequence level, where a flag sps_curr_pic_ref_enabled_flag is used. When sps_curr_pic_ref_enabled_flag is equal to 1, for each picture, another flag pps_curr_pic_ref_enabled_flag is used to signal if the current picture may use itself a reference picture. The picture level signaling of IBC usage appears to be enough.

# Proposed methods

Item 1: modification of the constraint on the use of 8x8 bi-prediction mode

Method 1: use picture level decision

A.3.7     Screen content coding extensions profiles

……

–           For each B slice, when use_integer_mv_flag is equal to 0 and TwoVersionsOfCurrDecPicFlag is equal to 1, the value of EightbyEightBiPredInUseforCurrPic shall be equal to 0.

……

–           When a B slice has use_integer_mv_flag equal to 0 and TwoVersionsOfCurrDecPicFlag is equal to 1, the variable EightbyEightBiPredInUseforCurrPic shall not be equal to 1 after operation of the decoding process for the slice.

……

Method 2: use slice level decision

A variation of this item would be, a similar TwoVersionsOfCurrDecSliceFlag can be derived and used to replace TwoVersionsOfCurrDecPicFlag for the decision, as the decision level for 8x8 biprediction here is on slice level, instead of picture level. TwoVersionsOfCurrDecSliceFlag is derived as follows:

TwoVersCurrDecSliceFlag = pps_curr_pic_as_ref_enabled_flag  &&

( slice_sao_luma_flag  | |  slice_sao_chroma_flag | | ! slice_deblocking_filter_disabled_flag )

OR,

TwoVersCurrDecSliceFlag = ( CurrPicInList0Flag | | CurrPicInList1Flag )  &&

( slice_sao_luma_flag  | |  slice_sao_chroma_flag | | ! slice_deblocking_filter_disabled_flag )

A.3.7     Screen content coding extensions profiles

……

–           For each B slice, when use_integer_mv_flag is equal to 0 and TwoVersionsOfCurrDecSliceFlag is equal to 1, the value of EightbyEightBiPredInUseforCurrPic shall be equal to 0.

……

–           When a B slice has use_integer_mv_flag equal to 0 and TwoVersionsOfCurrDecSliceFlag is equal to 1, the variable EightbyEightBiPredInUseforCurrPic shall not be equal to 1 after operation of the decoding process for the slice.

……

Item 2: removal of sequence level flag for the use of current picture as a reference picture

##### 7.3.2.2.3 Sequence parameter set screen content coding extension syntax

| sps_scc_extension( ) { | Descriptor |
| --- | --- |
| sps_curr_pic_ref_enabled_flag | u(1) |
| palette_mode_enabled_flag | u(1) |
| …… |  |

##### 7.4.3.2.3 Sequence parameter set screen content coding extension semantic

sps_curr_pic_ref_enabled_flag equal to 1 specifies that a picture in the CVS may be included in a reference picture list of a slice of the picture itself. sps_curr_pic_ref_enabled_flag equal to 0 specifies that a picture referring in the CVS is never included in a reference picture list of a slice of the picture itself. When not present, the value of sps_curr_pic_ref_enabled_flag is inferred to be equal to 0.

##### Picture parameter set screen content coding extension semantics

pps_curr_pic_ref_enabled_flag equal to 1 specifies that a picture referring to the PPS may be included in a reference picture list of a slice of the picture itself. pps_curr_pic_ref_enabled_flag equal to 0 specifies that a picture referring to the PPS is never included in a reference picture list of a slice of the picture itself. When not present, the value of pps_curr_pic_ref_enabled_flag is inferred to be equal to 0.

It is a requirement of bitstream conformance that when sps_curr_pic_ref_enabled_flag is equal to 0, the value of pps_curr_pic_ref_enabled_flag shall be equal to 0.

……

A.3.2 Main profile

……

- Active SPSs for the base layer shall have transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, implicit_rdpcm_enabled_flag, explicit_rdpcm_enabled_flag, extended_precision_processing_flag, intra_smoothing_disabled_flag, high_precision_offsets_enabled_flag, persistent_rice_adaptation_enabled_flag, cabac_bypass_alignment_enabled_flag, sps_curr_pic_ref_enabled_flag, palette_mode_enabled_flag, motion_vector_resolution_control_idc, and intra_boundary_filtering_disabled_flag, when present, equal to 0 only.

……

- Active PPSs for the base layer shall have pps_curr_pic_ref_enabled_flag, when present, equal to 0 only.

……

    - Main 10 profile

……

- Active SPSs for the base layer shall have transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, implicit_rdpcm_enabled_flag, explicit_rdpcm_enabled_flag, extended_precision_processing_flag, intra_smoothing_disabled_flag, high_precision_offsets_enabled_flag, persistent_rice_adaptation_enabled_flag, cabac_bypass_alignment_enabled_flag, sps_curr_pic_ref_enabled_flag, palette_mode_enabled_flag, motion_vector_resolution_control_idc, and intra_boundary_filtering_disabled_flag, when present, equal to 0 only.

……

- Active PPSs for the base layer shall have pps_curr_pic_ref_enabled_flag, when present, equal to 0 only.

……

A.3.4 Main Still Picture profile

- ……

- Active SPSs for the base layer shall have transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, implicit_rdpcm_enabled_flag, explicit_rdpcm_enabled_flag, extended_precision_processing_flag, intra_smoothing_disabled_flag, high_precision_offsets_enabled_flag, persistent_rice_adaptation_enabled_flag, cabac_bypass_alignment_enabled_flag, sps_curr_pic_ref_enabled_flag, palette_mode_enabled_flag, motion_vector_resolution_control_idc, and intra_boundary_filtering_disabled_flag, when present, equal to 0 only.

……

- Active PPSs for the base layer shall have pps_curr_pic_ref_enabled_flag, when present, equal to 0 only.

……

A.3.5 Format range extensions profiles

……

- Active SPSs for the base layer shall have separate_colour_plane_flag, cabac_bypass_alignment_enabled_flag, sps_curr_pic_ref_enabled_flag, palette_mode_enabled_flag, motion_vector_resolution_control_idc, and intra_boundary_filtering_disabled_flag, when present, equal to 0 only.

……

- Active PPSs for the base layer shall have pps_curr_pic_ref_enabled_flag, when present, equal to 0 only.

……

A.3.6 High throughput profiles

……

- Active SPSs for the base layer shall have separate_colour_plane_flag, sps_curr_pic_ref_enabled_flag, palette_mode_enabled_flag, motion_vector_resolution_control_idc, and intra_boundary_filtering_disabled_flag, when present, equal to 0 only.

……

- Active PPSs for the base layer shall have pps_curr_pic_ref_enabled_flag, when present, equal to 0 only.

……

A.4.2 Profile-specific level limits for the video profiles

……

where MaxLumaPs is specified in Table A.4, and maxDpbPicBuf is equal to 6 for all profiles where the value of sps_curr_pic_ref_enabled_flag is required to be equal to 0 and 7 for all Screen-Extended profiles where the value of sps_curr_pic_ref_enabled_flag is not required to be equal to 0 and 6 for all other profiles.

……

# Conclusions

It is proposed in this contribution to modify the signalling of the use of current picture as a reference picture such that only picture level signalling is necessary. It is also proposed in this document that when two versions of current picture (or slice) are identical, there is no need to constrain the use of 8x8 bi-prediction mode.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- K. Rapaka, V. Seregin, R. Joshi, T. Hsieh, M. Karczewicz, “CE2 : Test 5 on intra block copy constraints on prediction,” Document of Joint Collaborative Team on Video Coding, JCTVC-U0078, Warsaw, PL, 19–26 Jun. 2015.

- R. Joshi, S. Liu, G. J. Sullivan, J. Xu, Y. Ye, HEVC Screen Content Coding Draft Text 4, Document of Joint Collaborative Team on Video Coding, JCTVC-U1005, Warsaw, PL, 19–26 Jun. 2015.