| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0057 |
| --- | --- |

| Title: | DPB considerations when current picture is a reference picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Shan Liu Shawmin Lei | Email: | xiaozhong.xu@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current SCC specification, the current decoded picture prior to the loop filtering operations is used as a reference picture. This reference picture is also put and managed in the decoded picture buffer (DPB), together with other decoded pictures. Some modifications are proposed in this contribution to take into account the presence of this reference picture in DPB. Firstly, the total number of decoded pictures, including the current decoded picture, shall not exceed the decoded picture buffer size; secondly, when the maximum allowed DPB size is equal to 1, the unfiltered version of current decoded picture cannot be used as a reference picture; thirdly, when no loop filters are used, the unfiltered version of current decoded picture does not exist. The specification that uses “unfiltered version of current decoded picture as a reference picture” needs to be modified to remove potential ambiguity in this case.

# Introduction

In this contribution, the term “filtered version” of current decoded picture refers to the current decoded picture after the invocation of loop filtering operations; the term “unfiltered version” of current decoded picture refers to the current decoded picture prior to the invocation of loop filtering operations.

In the current specification [1], the decoded picture buffer is assigned as follows:

- One empty picture buffer is allocated to store the filtered version of current decoded picture

- If TwoVersionsOfCurrDecPicFlag  is equal to 1, another empty picture buffer is allocated to store the unfiltered version of current decoded picture

- The rest of DPB is used to store other reference pictures or previously decoded pictures for output

With the adoption of putting the unfiltered version of current picture into DPB [2], the following issues need to be addressed.

Firstly, the total number of decoded pictures, including the current picture, shall not exceed the decoded picture buffer size. However, when the unfiltered version of current picture is put in DPB, the constraint of total number has not yet been updated in the specification in some bitstream conformance conditions.

Secondly, when the maximum allowed DPB size is equal to 1, the filtered version of current decoded picture will fill this picture buffer. The unfiltered version of current decoded picture cannot be used as a reference picture as there is no space in DPB to store this picture.

Thirdly, the filtered version of current decoded picture is put in DPB and marked as “used for short-term reference” upon the completion of decoding current picture. On the other hand, the unfiltered version of current picture is used as a reference picture. It is put into DPB and marked as “used for long-term reference”. One issue is that when both versions of current decoded picture are identical due to the absence of loop filters usage, there is only one version of current decoded picture, or, in other words, the unfiltered version of current decoded picture does not exist. However, in the specification, the “filtered version” of current decoded picture is not assigned as a reference picture instead in this case. Some modification of text is necessary to remove the ambiguity this case as well.

# Proposed methods

Item 1:

When calculating the number of reference pictures allowed considering the DPB capability, the variable TwoVersionsOfCurrDecPicFlag should be taken into account. Corresponding text changes are as follows:

In 7.4.7.1     General slice segment header semantics

num_long_term_pics specifies the number of entries in the long-term RPS of the current picture that are directly signalled in the slice header. When not present, the value of num_long_term_pics is inferred to be equal to 0.

When nuh_layer_id is equal to 0, the sum of NumNegativePics[ CurrRpsIdx ], NumPositivePics[ CurrRpsIdx ], num_long_term_sps, TwoVersionsOfCurrDecPicFlag, and num_long_term_pics shall be less than or equal to sps_max_dec_pic_buffering_minus1[ sps_max_sub_layers_minus1 ].

Item 2:

When the maximum DPB size for the current layer is 1 (sps_max_dec_pic_buffering_minus1 [TemporalId] = 0), IBC should not be enabled. Corresponding text changes are as follows:

Method 1:

7.4.3.3.3   Picture parameter set screen content coding extension semantics

pps_curr_pic_ref_enabled_flag equal to 1 specifies that a picture referring to the PPS may be included in a reference picture list of a slice of the picture itself. pps_curr_pic_ref_enabled_flag equal to 0 specifies that a picture referring to the PPS is never included in a reference picture list of a slice of the picture itself. When not present, the value of pps_curr_pic_ref_enabled_flag is inferred to be equal to 0.

It is a requirement of bitstream conformance that when sps_curr_pic_ref_enabled_flag is equal to 0, the value of pps_curr_pic_ref_enabled_flag shall be equal to 0.

The variable TwoVersionsOfCurrDecPicFlag is derived as follows:

TwoVersionsOfCurrDecPicFlag = pps_curr_pic_ref_enabled_flag  &&
	( sample_adaptive_offset_enabled_flag  | |  !pps_deblocking_filter_disabled_flag  | |	(7-1)
	deblocking_filter_override_enabled_flag )

It is a requirement of bitstream conformance that if sps_max_dec_pic_buffering_minus1[ TemporalId ] is equal to 0 and nuh_layer_id is equal to 0, the variable TwoVersionsOfCurrDecPicFlag shall be 0.

7.4.7.1 General slice segment header semantics

slice_type specifies the coding type of the slice according to Table 7-1.

Table 7-1 – Name association to slice_type

| - slice_type | - Name of slice_type |
| --- | --- |
| - 0 | - B (B slice) |
| - 1 | - P (P slice) |
| - 2 | - I (I slice) |

When nal_unit_type has a value in the range of BLA_W_LP to RSV_IRAP_VCL23, inclusive, i.e., the picture is an IRAP picture, nuh_layer_id is equal to 0, and pps_curr_pic_ref_enabled_flag is equal to 0, slice_type shall be equal to  2.

When sps_max_dec_pic_buffering_minus1[ TemporalId ] is equal to 0, pps_curr_pic_ref_enabled_flag is equal to 0,  and nuh_layer_id is equal to 0, slice_type shall be equal to 2.

Method 2:

In 7.4.3.3.3        Picture parameter set range extension semantics

pps_curr_pic_ref_enabled_flag equal to 1 specifies that a picture referring to the PPS may be included in a reference picture list of a slice of the picture itself. pps_curr_pic_ref_enabled_flag equal to 0 specifies that a picture referring to the PPS is never included in a reference picture list of a slice of the picture itself. When not present, the value of pps_curr_pic_ref_enabled_flag is inferred to be equal to 0.

It is a requirement of bitstream conformance that when sps_curr_pic_ref_enabled_flag is equal to 0, the value of pps_curr_pic_ref_enabled_flag shall be equal to 0.

It is a requirement of bitstream conformance that when sps_max_dec_pic_buffering_minus1[ TemporalId ] is equal to 0 and nuh_layer_id is equal to 0, pps_curr_pic_ref_enabled_flag shall be 0.

Item 3:

When no loop filters are used, there is only one version of current picture. There is no storage in DPB created for unfiltered version of current picture. There is a need to modify the spec so that when current picture is used as a reference picture, it is referred to the picture buffer used for storing the current picture after (instead of prior to) the invocation of loop filtering. Corresponding text changes are as follows:

8.1.3     Decoding process for a coded picture with nuh_layer_id equal to 0

- The processes in clauses 8.4, 8.5, 8.6, and 8.7 specify decoding processes using syntax elements in all syntax structure layers. It is a requirement of bitstream conformance that the coded slices of the picture shall contain slice segment data for every coding tree unit of the picture, such that the division of the picture into slices, the division of the slices into slice segments, and the division of the slice segments into coding tree units each forms a partitioning of the picture. Decoded sample values of the current picture after the invocation of the in-loop filter process as specified in clause 8.7 are stored in the picture storage buffer for the current picture. When TwoVersionsOfCurrDecPicFlag is equal to 0 and pps_curr_pic_as_ref_enabled_flag is equal to 1, the current picture after the invocation of the in-loop filter process as specified in clause 8.7 is marked as “used for long-term reference”. and not marked as "used for long-term reference". When TwoVersionsOfCurrDecPicFlag is equal to 1, the decoded sample values of the current picture immediately before the invocation of the in-loop filter process as specified in clause 8.7 are stored in the picture storage buffer for the current picture and marked as "used for long-term reference".

- After all slices of the current picture have been decoded, the current decoded picture, after the invocation of the in-loop filter process as specified in clause 8.7, is marked as "used for short-term reference".

In Annex C 3.4 Current decoded picture marking and storage

When TwoVersionsOfCurrDecPicFlag is equal to 1 or when pps_curr_pic_as_ref_enabled_flag is equal to 0, the current decoded picture after the invocation of the in-loop filter process as specified in clause 8.7 is stored in the DPB in an empty picture storage buffer, the DPB fullness is incremented by one, and this picture is marked as "used for short-term reference"; otherwise, when TwoVersionsOfCurrDecPicFlag is equal to 0 and pps_curr_pic_as_ref_enabled_flag is equal to 1, the current decoded picture after the invocation of the in-loop filter process as specified in clause 8.7 is stored in the DPB in an empty picture storage buffer, the DPB fullness is incremented by one, and this picture is marked as "used for long-term reference". After all slices of the current picture have been decoded, the current decoded picture, after the invocation of the in-loop filter process as specified in clause 8.7, is marked as "used for short-term reference".

When TwoVersionsOfCurrDecPicFlag is equal to 1, the current decoded picture before the invocation of the in-loop filter process as specified in clause 8.7 is stored in the DPB in an empty picture storage buffer, the DPB fullness is incremented by one, and this picture is marked as "used for long-term reference".

NOTE – Unless more memory than required by the level limit is available for storage of decoded pictures, decoders should start storing decoded parts of the current picture into the DPB when the first slice segment is decoded and continue storing more decoded samples as the decoding process proceeds.

# Conclusions

In this contribution, several DPB related issues are discussed when IBC is used and the unfiltered version of current picture is put in to DPB. It is proposed that some corresponding constraints and operations need to be updated to incorporate this extra reference picture.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- R. Joshi, S. Liu, G. J. Sullivan, J. Xu, Y. Ye, HEVC Screen Content Coding Draft Text 4, Document of Joint Collaborative Team on Video Coding, JCTVC-U1005, Warsaw, PL, 19–26 Jun. 2015.

- X. Xu, S. Liu, C. Liu, S. Lei, Y.-K. Wang, K. Rapaka, V. Seregin  “On storage of filtered and unfiltered current decoded pictures” Document of Joint Collaborative Team on Video Coding, JCTVC-U0181, Warsaw, PL, 19–26 Jun. 2015.