| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 201 | Document: JCTVC-V0058 |
| --- | --- |

| Title: | Intra block copy constraints for non-444 video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Xiaozhong Xu, Shan Liu, Ching-Yeh Chen, Yu-Wen Huang, Shawmin Lei MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Tzu-Der Chuang +886-3-5670766 ext. 24602 {peter.chuang, xiaozhong.xu, shan.liu, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In SCM-5.2, to reduce the worst case memory bandwidth of single burst memory, the 8x8 bi-prediction block is disallowed when IBC is used and use_integer_mv_flag is equal to 0 for the current slice. However, the multiple burst DDR/DDR2 SDRAMs are usually used to store the reference pictures. With the multi-burst memories, the required memory bandwidth of the 4x8 uni-prediction block with fractional MV in HEVC-SCC is larger than the worst case memory bandwidth in HEVC in 420 video. In this contribution, two encoder constraints are proposed. In method-1, the 4x8 uni-prediction block with fractional MV-x or fractional MV-y is disallowed in bitstream for non-444 video. In method-2, the 4x8 uni-prediction block with fractional MV-y is disallowed in bitstream for non-444 video. The worst case bandwidth is reduced to be less than 95.8% of the worst case memory bandwidth in HEVC. It is reported that for YUV text & graphics with motion sequences in lossy condition, 0.2% and 0.4% BD-rate increases are shown under RA and LB configuration for method-1 respectively, and 0.1% and 0.3% BD-rate increases are shown under RA and LB configuration for method-2 respectively.

# Problem statement

In HEVC Screen Content Coding extension, the Intra Block Copy (IBC) mode is introduced to improve the coding efficiency. In IBC, the unfiltered reconstructed pixels (before deblocking) of the current picture are used to predict the current PU. The unfiltered reconstructed pixels of the current picture are usually stored in the off-chip memory, e.g. SDRAM or DDR SDRAM. The additional data write bandwidth is required for IBC mode. The detail bandwidth requirement is analyzed in JCTVC-U0175 [1]. Both memory read and write are taken into account in the bandwidth calculation. For memory read, bandwidth is calculated as number of reference pixels fetched from the memory based on some memory patterns such as 4x2 etc. It is assumed that memory can be accessed and read based on the memory patterns, the chroma storage is not interleaved and only integer number of blocks can be fetched.

Let’s denote a block size to be compensated as M×N, memory pattern as m×n, and L is the interpolation filter length (L =8 for Luma, L=4 for chroma).

For each pixel within M×N block the number of pixels per fetched from the memory can be calculated as follows.

For memory write, both the filtered and unfiltered reconstructed pixels are written to memory for SCM 4.0.

In 4:2:0 chroma format, the chroma is considered to be interleaved as recommended in JCTVC-C007 and JCTVC-L0440. This methodology as described in these documents were used for AHGs in HEVC v1 and SHVC memory BW analysis. For each pixel within M×N luma block the number of pixels per fetched from the memory for chroma is calculated as follows.

In order to reduce the memory bandwidth requirement, an encoder constraint is proposed in JCTVC-U0078 [2] that the bi-prediction for 8x8 block is disallowed in bitstream when the IBC mode is used and the use_integer_mv_flag is 0. If the memory pattern is 4x1, 8x1, 4x2, or 8x2 and single burst memory access is used, in 444 chroma format, the worst case bandwidth in SCC is reduced to 91-93% of the worst case memory bandwidth in HEVC. In 420 chroma format, the worst case bandwidth in SCC is reduced to 91-93% of the worst case memory bandwidth in HEVC. However, in practical video codec architecture design, the multiple burst memories, such as the DDR/DDR2 SDRAM, are usually used to store the reference pictures. The minimum burst-length in DDR SDRAM is 2, and the minimum burst-length in DDR2 SRAM is 4. For DDR-series memories, the data address for a burst usually aligned to 128-bits, 256-bits, or 512-bits, which means the memory patterns of 16x1, 32x1, 16x2, 32x2 blocks are need to be considered. Table 1 shows the worst case memory bandwidth of 8x8 bi-prediction block with fractional MV and 4x8 uni-prediction block with fractional MV in HEVC-SCC with different memory pattern. The detail bandwidth calculation can be found in the attached bandwidth calculation spreadsheet with this proposal. The required bandwidth of the 4x8 uni-prediction block with fractional MV is the same as the required bandwidth of 8x8 bi-prediction block with fractional MV in 420 video. Therefore, for these pratically used memory patterns, with the restriction of disabling 8x8 bi-prediction block, the worst case bandwidth of HEVC-SCC in 420 video is still larger than the worst case bandwidth of HEVC.

Table 1. Worst case memory bandwidth of different PU in different memory configuration in 420 chroma format

| Memory pattern | Total Per pixel access |  |  |  |
| --- | --- | --- | --- | --- |
|  | HEVC | HEVC-SCC 8x8 bi-pred. with fractional MV | HEVC-SCC 4x8 uni-pred. with fractional MV | HEVC-SCC 4x16+12x16 bi-pred. with fractional MV |
| 16x1 (4x1 burst 4, 8x1 burst 2) | 23.5 | 25 (106.4%) | 25 (106.4%) | 22.52 (95.8%) |
| 32x1 (4x1 burst 8, 8x1 burst 4) | 45.5 | 47 (103.3%) | 47 (103.3%) | 36.305 (79.8%) |
| 16x2 (4x2 burst 4, 8x2 burst 2) | 25.5 | 27 (105.9%) | 27 (105.9%) | 23.625 (92.6%) |
| 32x2 (4x2 burst 8, 8x2 burst 4) | 49.5 | 51 (103.0%) | 51 (103.0%) | 38.265 (77.3%) |

# Proposed encoder constraint

In this contribution, two encoder constraints are proposed for non-444 video.

## Method-1: disable 4x8 uni-prediction PU when one of the MV-x and MV-y is fractional MV

It is proposed that the 4x8 uni-prediction PU with fractional MV-x or fractional MV-y is disallowed in bitstream when the chroma format is non-444. With this encoder constraint, the 4x16+12x16 bi-prediction blocks will be the worst case. The worst case bandwidth is smaller than that in HEVC.

## Method-2: disable 4x8 uni-prediction PU when MV-y is fractional MVs

When the memory patterns of 16x1, 32x1, 16x2, 32x2 blocks are considered, the reference block height has more impact than the reference block width in bandwidth requirement. The reference block height is equal to PU height + interpolation filter tap length – 1 when the MV-y is fractional, and the reference block height is equal to PU height when the MV-y is integer. In our method-2, it is proposed that the 4x8 uni-prediction PU with fractional MV-y is disallowed in bitstream when the chroma format is non-444. The worst case bandwidth is smaller than that in HEVC.

# Experimental results

The proposed method is integrated on SCM-5.2 [3]. All tests were conducted under the common test condition (CTC) [4] with the full picture IBC search range in 420 video. Table 2 and Table 3 show the results of the proposed method-1 encoder constraint for lossy and lossless coding respectively. For YUV text & graphics with motion sequences in lossy condition, 0.2% and 0.4% BD-rate increases are shown under RA and LB configuration respectively. Table 4 and Table 5 show the results of the proposed method-2 encoder constraint for lossy and lossless coding respectively. For YUV text & graphics with motion sequences in lossy condition, 0.1% and 0.3% BD-rate increases are shown under RA and LB configuration respectively.

# Conclusions

This contribution proposes two encoder constraints to reduce the worst case bandwidth. In method-1, the 4x8 uni-prediction block with fractional MV-x or fractional MV-y is disallowed in bitstream in non-444 video. In method-2, the 4x8 uni-prediction block with fractional MV-y is disallowed in bitstream in non-444 video. The worst case bandwidth of HEVC-SCC in 420 video is reduced to be less than 95.8% of the worst case memory bandwidth in HEVC. It is reported that for YUV text & graphics with motion sequences in lossy condition, 0.2% and 0.4% BD-rate increases are shown under RA and LB configuration for method-1, and 0.1% and 0.3% BD-rate increases are shown under RA and LB configuration for method-2.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- K. Rapaka, “BoG report on worst case memory bandwidth assessment,” Document of Joint Collaborative Team on Video Coding, JCTVC-U0175, June 2015.

- K. Rapaka, V. Seregin, R. Joshi, T. Hsieh, M. Karczewicz, “CE2 : Test 5 on intra block copy constraints on prediction,” Document of Joint Collaborative Team on Video Coding, JCTVC-U0078, June 2015.

- SCM-5.2, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.2+SCM-5.2/

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-U1015, June 2015.

Table 2.  Lossy results of the proposed method-1 in 420 format and full frame Intra Block Copy search

Table 3. Lossless results of the proposed method-1 in 420 format and full frame Intra Block Copy search

Table 4.  Lossy results of the proposed method-2 in 420 format and full frame Intra Block Copy search

Table 5. Lossless results of the proposed method-2 in 420 format and full frame Intra Block Copy search