| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0059-v2 |
| --- | --- |

| Title: | Suggested text specification and software fixes for constrained intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan Joshi, Vadim Seregin, Krishna Rapaka, Ye-Kui Wang, Marta Karczewicz | Email: Tel. | rajanj@qti.qualcomm.com +1-858-658-4511 |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

During the 21st JCT-VC meeting in Warsaw, the BoG report on constrained intra prediction for IBC unification (JCTVC-U0178) noted that both the software and draft text specification had some deficiencies with respect to constrained intra prediction (CIP). The SCM-5.2 software allows the use of intra and IBC reference samples for intra prediction and intra block copy prediction. However it is asserted that SCM draft text 4 (JCTVC-U1005) is incomplete on allowing use of IBC reference samples for constrained intra prediction. This document proposes modifications to SCM draft text 4 to align it with SCM-5.2 software for constrained intra prediction. Additionally, it is asserted that two adoptions from the 21st JCT-VC meeting in Warsaw related to IBC chroma interpolation and allowing the current picture to appear in both list 0 and list 1 affect constrained intra prediction. The document proposes changes to both SCM-5.2 and SCM draft text 4, in light of these adoptions. An alternative based on HEVC Range Extensions draft text specification 6 (JCTVC-P1005_v4) is also presented.

# Introduction

During the 21st JCT-VC meeting in Warsaw, the BoG report on constrained intra prediction for IBC unification (JCTVC-U0178) noted that both the software and draft text specification had some deficiencies with respect to constrained intra prediction (CIP). The SCM-5.2 software allows the use of intra and IBC reference samples for intra prediction and intra block copy prediction. However SCM draft text 4 (JCTVC-U1005) is incomplete on allowing use of IBC reference samples for constrained intra prediction, where this possibility is only mentioned in the semantics of constrained_intra_pred_flag. This document proposes modifications to SCM draft text 4 to align it with SCM-5.2 software for constrained intra prediction.

Several adoptions during the 21st JCT-VC meeting in Warsaw also have an impact on constrained intra prediction. They are:

- JCTVC-U0104: Allow the current picture to appear in both list 0 and list 1.

- JCTVC-U0077 A2 / JCTVC-U0080 / JCTVC-U0103: Chroma interpolation for IBC blocks for non-4:4:4 colour formats.

For example, for a bi-predictive block to be classified as "intra block copy prediction" block, it is not sufficient that only one of the reference indices points to the current picture. Similarly, if one or more samples used for chroma interpolation use inter prediction, that block should not be used as reference block when CIP is enabled. Fixes to text specification and software are proposed to take these two items into account.

An alternative based on HEVC Range Extensions draft text specification 6 (JCTVC-P1005_v4) is also presented.

# Proposal

## Text specification fixes

The proposed text specification is provided in the accompanying file. There are 3 main changes:

- A definition is added for 'intra block copy prediction'. A note is also added to clarify that when a block has multiple motion vectors and reference indices and each reference index points to the current picture, the block is said to be coded using intra block copy prediction.

- The semantics for constrained_intra_pred_flag are modified to allow, when constrained_intra_pred_flag is equal to 1, the use of decoded samples coded using intra prediction as well as intra block copy prediction in both intra prediction and intra block copy prediction.

- A requirement for bitstream conformance is added. The requirement states that all the decoded samples for forming the prediction for an intra block copy prediction block shall be coded using intra prediction or intra block copy prediction. In the case of fractional motion vectors for non-4:4:4 formats, the samples needed for interpolation have to satisfy this criterion as well.

## Software fixes

- Currently, the function isIntraBC checks whether the reference picture is the current picture only for list 0. But in case of bi-predictive blocks, this decision should be based on applying this check to both list 0 and list 1.

- Currently, when constrained intra prediction is enabled, the software only allows decoded samples that are coded using intra prediction or intra block copy prediction to be used as reference samples for intra prediction and intra block copy prediction. But it does not take the samples needed for chroma interpolation into account.

# Alternative solution based on Range Extension draft text

In HEVC Range Extensions draft text specification 6 (JCTVC-P1005_v4) a slight variation is present. Similar to what is currently in the software, intra reference samples from IBC blocks may be used for intra prediction when constained_intra_pred_flag is equal to 1. However in this case, it is left up to the encoder to make sure that in regions where error propagation is to be controlled through the use of constrained intra, IBC blocks do not refer to samples coded using inter prediction.

The relevant text from JCTVC-P1005_v4 related to CIP is quoted below:

constrained_intra_pred_flag equal to 0 specifies that intra prediction allows usage of residual data and decoded samples of neighbouring coding blocks coded using either intra or inter prediction modes. constrained_intra_pred_flag equal to 1 specifies constrained intra prediction, in which case the general intra sample prediction process only uses residual data and decoded samples from neighbouring coding blocks coded using intra prediction modes.

NOTE 5 – Samples reconstructed from available intra blocks with intra_bc_flag equal to 1 are considered as available for the purposes of constrained intra prediction. However, no restriction is placed on the origin of reference samples used by intra blocks with intra_bc_flag equal to 1. Encoders that employ constrained intra prediction for the purpose of error propagation control should, in regions where error propagation is to be controlled through the use of constrained intra prediction, ensure that intra blocks with intra_bc_flag equal to 1 only use - from neighbouring coding blocks coded using intra prediction modes.

For this alternative, the suggested text specification changes and software changes are as follows:

## Text specification fixes

No changes to the text specification are necessary in this case, although definition for 'intra block copy prediction' may still be added for clarity.

- A definition is added for 'intra block copy prediction'. A note is also added to clarify that when a block has multiple motion vectors and reference indices and each reference index points to the current picture, the block is said to be coded using intra block copy prediction.

## Software fixes

- Currently, the function isIntraBC checks whether the reference picture is the current picture only for list 0. But in case of bi-predictive blocks, this decision should be based on applying this check to both list 0 and list 1.

- Currently, when constrained intra prediction is enabled, the software should be changed to allow the use of decoded inter samples for intra block copy prediction while taking into account refresh strategy and regions where error propagation is to be controlled. If the use of decoded samples for intra block copy prediction needs to be limited to samples coded using intra prediction or intra block copy prediction, the samples needed for chroma interpolation should be taken into account.

# Conclusion

Two alternatives are presented for fixing the text specification and the software in case contsrained_intra_pred_flag is equal to 1. When constrained_intra_prediction_flag is equal to 1, both alternatives allow the use of decoded samples coded using intra prediction as well as intra block copy prediction for intra prediction. In the first alternative, it is also normative that when constrained_intra_prediction_flag is equal to 1, all the decoded samples necessary for intra block copy prediction shall be coded using intra prediction or intra block copy prediction. Whereas in the second alternative, enforcement of this criterion is encouraged but left to the encoder.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).