| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0060-v2 |
| --- | --- |

| Title: | Analysis of palette run coding binarization and suggested editorial improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Editorial improvements |  |  |
| Author(s) or Contact(s): | Rajan Joshi, Marta Karczewicz, Wei Pu, Vadim Seregin, Krishna Rapaka | Email: Tel. | rajanj@qti.qualcomm.com +1-858-658-4511 |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In HEVC SCC draft text 4 (JCTVC-U1005), the PaletteRun is specified using syntax elements palette_run_msb_id_plus_1 and palette_run_refinement_bits. It is asserted that the binarization of PaletteRun has similarities to the binarization of coeff_abs_level_remaining, and LastSignificantCoeffX (or Y). It is proposed to rename the syntax elements palette_run_msb_id_plus_1 and palette_run_refinement_bits to palette_run_prefix and palette_run_suffix. Editorial improvements to semantics are proposed as well.

# Proposed editorial improvements

In HEVC SCC draft text 4 (JCTVC-U1005), the PaletteRun is specified using syntax elements palette_run_msb_id_plus_1 and palette_run_refinement_bits. Table 1 shows the binarization of PaletteRun. For convenience, it is assumed that MaxPaletteRun is large (> 64). The binarization of PaletteRun is a concatenation of binary strings for palette_run_msb_id_plus_1 and palette_run_refinement_bits.

Table 1: Binarization of PaletteRun in HEVC SCC draft text 4

| PaletteRun | binarization |
| --- | --- |
| 0 | 0 |
| 1 | 10 |
| [2, 3] | 110x |
| [3, 7] | 1110xx |
| [8, 15] | 11110xxx |
| ... |  |

When PaletteRun is equal to 2 or greater, the binarization of PaletteRun is a concatenation of truncated unary code for 2,with cMax = 2 (11 shown in red) and truncated exponential Golomb code with Golomb parameter equal to 1 and cMax equal to MaxPaletteRun − 2 (shown in green). For PaletteRun less than 2, the binarization of palette run is truncated unary. It should be noted that truncated unary is a special case of truncated Rice codes with cRiceParam = 0. For PaletteRun binarization, The switching point is at 2.

As a comparison, consider the binarization of coeff_abs_level_remaining for cRiceParam = 0. This is shown in Table 2.

Table 2: Binarization of coeff_abs_level_remaining for cRiceParam = 0

| coeff_abs_level_remaining | binarization |
| --- | --- |
| 0 | 0 |
| 1 | 10 |
| 2 | 110 |
| 3 | 1110 |
| [4, 5] | 11110x |
| [6, 9] | 111110xx |
| [10, 17] | 1111110xxx |
| ... |  |

For coeff_abs_level_remaining greater than or equal to 4, the binarization is a concatenation of truncated unary code for 4 with cMax = 4 (1111 shown in red) and exponential Golomb code with Golomb parameter equal to 1. In this case, the switching point is 4.

When extended_precision_processing_flag is equal to 1, truncated form of exponential Golomb code is used. Ignoring differences in truncation, the only other difference in the binarizations of PaletteRun and coeff_abs_level_remaining is the switching point. In JCTVC-S0039, the existing binarization of PaletteRun was expressed as a concatenation of Golomb Rice and exponential Golomb codes.

However, it is difficult to use the binarization framework of coeff_abs_level_remaining for the case of PaletteRun. This is because in case of coeff_abs_level_remaining, all the bins are coded in bypass mode. Whereas in case of PaletteRun binarization, up to the first 5 bins may be context-coded with the restriction that the bins from the suffix part of exponential Golomb code are always coded in bypass mode. Hence it is better to combine the truncated unary part and the prefix of the exponential Golomb code into a single syntax element. This results in the syntax element palette_run_msb_id_plus_1. But the semantics of this syntax element are problematic when it has the value zero. The syntax and semantics can be simplified by renaming the syntax elements palette_run_msb_id_plus_1 and palette_run_refinement_bits to palette_run_prefix and palette_run_suffix.

This is similar to the variable LastSignificantCoeffX and the corresponding syntax elements last_sig_coeff_x_prefix and last_sig_coeff_x_suffix. In that case, the suffix is absent when LastSignificantCoeffX is less than 4.

In both cases, the prefix is coded using truncated Rice code (cRiceParam = 0) and the suffix is coded using fixed length code. The only difference is that in case of paletteRun, when the value of palette_run_prefix is the maximum possible, the suffix is coded using truncated binary (TB) binarization.

# Draft text specification

Draft text specification is provided in the accompanying file.

# Patent rights declaration(s)

Qualcomm Incorporated does not have any current or pending patent rights relating to this contribution.