| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0061r1 |
| --- | --- |

| Title: | Simplification for the index of the MSB in the paletteRun binarization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yao-Jen Chang, 2Ching-Chieh Lin, 3Chun-Lung Lin and Jih-Sheng Tu 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-8774 1britpablo@itri.org.tw 2JackLin@itri.org.tw 3Chunlung@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

In JCTVC-U0109, the context mode for the 4th bin and the 5th bin of palette_run_msb_id_plus1 is replaced by the bypass mode. There were some comments: 1) Performance results for another modification, modifying one context-coded bin in palette_run_msb_id_plus1 as one bypass-coded bin, were not provided. 2) No analyzing results were provided to evaluate whether the method hav measurable benefit in the overall complexity of the decoding process. In this contribution, some evaluation results are provided to clarify the proposed method. First, the BD-rate results on top of SCM 5.2 are provided when modifying N context-coded bins in palette_run_msb_id_plus1 as N bypass-coded bins, where N couset as 1, 2, 3 andor 4 in s contribution. Secondly, the analyzing results are provided to show how large number of the context-coded bins modified as the bypass-coded bins are over the number of all context-coded bins in all coding modes of HEVC SCC. It is reported that, when changing 3 context-coded bins to 3 bypass-coded bins, the BD-rate increases are 0.0-0.3% with average smaller than 0.1%, and the percentage of the modified context-coded bins over all context-coded bins in all coding modes can be up to 10.5%.

# Introduction

In SCM 5.2, the palette mode signals the palette run type flag, palette index, palette run value and escape values for index map coding [1]. COPY_INDEX_MODE and COPY_ABOVE_MODE are two possible run types of the palette run type flag. Two syntax elements palette_run_msb_id_plus1 and palette_run_refinement_bits are signaled to represent the palette run value. The first five bins of the syntax element palette_run_msb_id_plus1 are coded by context mode, and the ohters are coded by bypass mode. The derivation process can be found in 9.3.4.2.8. The variable ctxInc for palette_run_msb_id_plus1 is shown as follows:

| binIdx | 0 | 1 | 2 | 3 | 4 | > 4 |
| --- | --- | --- | --- | --- | --- | --- |
| palette_run_type_flag = COPY_ABOVE_MODE | 5 | 6 | 6 | 7 | 7 | bypass |
| palette_run_type_flag = COPY_INDEX_MODE | 0,1,2 | 3 | 3 | 4 | 4 | bypass |

In JCTVC-U0109 [4], the context mode for the 4th bin and the 5th bin of palette_run_msb_id_plus1 is replaced by the bypass mode, and the modification is as shown below:

| binIdx | 0 | 1 | 2 | > 2 |
| --- | --- | --- | --- | --- |
| palette_run_type_flag = COPY_ABOVE_MODE | 5 | 6 | 7 | bypass |
| palette_run_type_flag = COPY_INDEX_MODE | 0,1,2 | 3 | 4 | bypass |

There were some comments from participants for this contribution: 1) Performance results for another modification, modifying one context-coded bin in palette_run_msb_id_plus1 as one bypass-coded bin, were not provided. 2) No analyzing results were provided to evaluate whether the methods have measurable benefit in the overall complexity of the decoding process. Therefore, this contribution provides several experiment results to clarify the proposed method.  upe pdwa

# Different modifications of the proposed method

The basic concept of the proposed method in JCTVC-U0109 is to modify some context-coded bins of palette_run_msb_id_plus1 as bypass-coded bins. To study the method, this contribution provides four modifications according to the proposed method on top of SCM 5.2.

Modification 1: Modifying 1 context-coded bin of palette_run_msb_id_plus1 to be bypass-coded bin.

| binIdx | 0 | 1 | 2 | 3 | > 3 |
| --- | --- | --- | --- | --- | --- |
| palette_run_type_flag = COPY_ABOVE_MODE | 5 | 6 | 7 | 7 | bypass |
| palette_run_type_flag = COPY_INDEX_MODE | 0,1,2 | 3 | 4 | 4 | bypass |

Modification 2: Modifying 2 context-coded bins of palette_run_msb_id_plus1 to be bypass-coded bins.

| binIdx | 0 | 1 | 2 | > 2 |
| --- | --- | --- | --- | --- |
| palette_run_type_flag = COPY_ABOVE_MODE | 5 | 6 | 7 | bypass |
| palette_run_type_flag = COPY_INDEX_MODE | 0,1,2 | 3 | 4 | bypass |

Modification 3: Modifying 3 context-coded bins of palette_run_msb_id_plus1 to be bypass-coded bins.

| binIdx | 0 | 1 | > 1 |
| --- | --- | --- | --- |
| palette_run_type_flag = COPY_ABOVE_MODE | 6 | 7 | bypass |
| palette_run_type_flag = COPY_INDEX_MODE | 0,1,2,3 | 4,5 | bypass |

Modification 4: Modifying 4 context-coded bins of palette_run_msb_id_plus1 to be bypass-coded bins.

| binIdx | 0 | > 0 |
| --- | --- | --- |
| palette_run_type_flag = COPY_ABOVE_MODE | 7 | bypass |
| palette_run_type_flag = COPY_INDEX_MODE | 0,1,2,3,4,5,6 | bypass |

rbi fifh  binsf pa

|  |  |  | 2 |
| --- | --- | --- | --- |
|  |  | , |  |

- Experimental results

The proposed modifications are -ested under the common test conditions on top of SCM 5.2 [2][3]. Table 1 ahows the all-intra lossy aesults for different modifications fn the condition oftfull frame IBC search area. One can refer to the spreadsheets appended with the package of this contribution for more details. The results show that Modification 1 and Modification 2 only 0.0-0.1% coding change for the classes of text & graphics with motion (TGM) and mixed content (MC), Modification 3 only 0.1-0.3% coding change for the classes of TGM and MC, and Modification 4 has 0.2-0.7% coding loss for the classes of TGM and MC. As for the classes of animation content (AC) and camera captured (CC), there is no any impact to the coding performance in every modification.

Table 1. Y-BD-rate results for different modifications

|  | Y-BD-rate results for modifications |  |  |  |
| --- | --- | --- | --- | --- |
|  | Mod. 1 | Mod. 2 | Mod. 3 | Mod. 4 |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.1% | 0.1% | 0.5% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.1% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.3% | 0.7% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.1% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% | 100% | 100% | 100% |
| Dec Time[%] | 100% | 100% | 100% | 101% |

To clarify whether the method has measurable benefit in the overall complexity of the decoding process, Table 2 demonstrates the occurring rate of the modified context-coded bins of palette_run_msb_id_plus1 over all context-coded bins in all coding modes of HEVC SCC.

Table 2. Occurring rate of the modified context-coded bins of palette_run_msb_id_plus1 over all context-coded bins in all coding modes

|  | 5th bit (Mod. 1) | 4th – 5th bit (Mod. 2) | 3rd – 5th bit (Mod. 3) | 2nd – 5th bit (Mod. 4) |
| --- | --- | --- | --- | --- |
| RGB, text & graphics with motion, 1080p | 1.4% | 4.0% | 7.8% | 13.1% |
| RGB, text & graphics with motion, 720p | 0.6% | 1.8% | 3.6% | 6.3% |
| YUV, text & graphics with motion, 1080p | 2.0% | 5.4% | 10.5% | 17.6% |
| YUV, text & graphics with motion, 720p | 1.0% | 2.6% | 5.2% | 9.0% |
| Average | 1.2% | 3.4% | 6.8% | 11.5% |

# Conclusion

This contribution proposes to clarify the method in the contribution JCTVC-U0109. The experiment results shown that under 0.2% coding change on average there is a huge impact to the overall HEVC SCC codec, up to 17.6% context-coded bin saving. According to the results provided, it is suggested to adopt the method for further improving the HEVC SCC.

# References

- R. Joshi, S. Liu, G. J. Sullivan, J. Xu, Y. Ye, “HEVC Screen Content Coding Draft Text 4,” 21th JCT-VC meeting, JCTVC-U1005, Warsaw, Poland, Jun. 2015.

- H. Yu, R. Cohen, K. Rapaka, J. Xu , “Common Test Conditions for Screen Content Coding,” 21th JCT-VC meeting, JCTVC-U1015, Warsaw, Poland, Jun. 2015.

- SCM 5.2, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.4+SCM-5.2/

- Y. J. Chang, C. L. Lin, C. C. Lin, J. S. Tu, W. Digonnet, “Non-CE1: Simplification for parsing the index of the most significant bit in the paletteRun binarization,” 21th JCT-VC meeting, JCTVC-U0109, Warsaw, Poland, Jun. 2015.

# Patent rights declaration(s)

ITRI international may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).