| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0061 |
| --- | --- |

| Title: | Simplification for the index of the MSB in the paletteRun binarization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yao-Jen Chang, 2Ching-Chieh Lin, 3Chun-Lung Lin and Jih-Sheng Tu 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-8774 1britpablo@itri.org.tw 2JackLin@itri.org.tw 3Chunlung@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

In the palette mode, non-escape indices are reconstructed by parsing the palette run type flag, the palette index and the palette run value. The parsing syntax of the palette run value is composed of palette_run_msb_id_plus1 and palette_run_refinement_bits. The first five bins of palette_run_msb_id_plus1 are coded by context mode, and the other bins are coded by bypass mode. To reduce the context-coded bins in the current palette mode, this contribution proposes to simplify the derivation process of palette_run_msb_id_plus1 by replacing the context mode with the bypass mode for some bins. It is reported the BD-rate increases are 0-0.x% with average smaller than 0.x%, when changing N context-coded bins to bypass-coded bins. In addition, it is also reported that the percentage of the reduced context-coded bins over all context-coded bins in all coding modes can be up to xx%.

# Introduction

In SCM-5.2, the palette mode signalls the palette run type flag, palette index, run value and escape values for index map coding [1].  COPY_INDEX_MODE and COPY_ABOVE_MODE are the two run types in the current design. Two syntax palette_run_msb_id_plus1 and palette_run_refinement_bits are signalled to represne the palette run value. The first five bins of the syntax element palette_run_msb_id_plus1 are coded by context bins, and the ohters are coded by bypass mode. The derivation process can be found in 9.3.4.2.8. The variable ctxInc for palette_run_msb_id_plus1 is shown as follows:

| binIdx | 0 | 1 | 2 | 3 | 4 | > 4 |
| --- | --- | --- | --- | --- | --- | --- |
| palette_run_type_flag = COPY_ABOVE_MODE | 5 | 6 | 6 | 7 | 7 | bypass |
| palette_run_type_flag = COPY_INDEX_MODE | 0,1,2 | 3 | 3 | 4 | 4 | bypass |

If the two binary bins are continuously parsed by CABAC context mode, they must be parsed one-by-one due to the update process of the occurring probability of the bin value. When the number of bins of palette_run_msb_id_plus1 is larger than or equal to 5, the decoder requires at least five stages to parse first five bins (one-by-one) of palette_run_msb_id_plus1 for hardware implementation.

# Proposed method

In order to reduce the context-coded bins, this contribution proposes to replace the context mode for the fourth and fifth bins of palette_run_msb_id_plus1 by the bypass mode. Thus the proposed derivation process of the variable ctxInc for palette_run_msb_id_plus1 is as follows:

| binIdx | 0 | 1 | 2 | >2 |
| --- | --- | --- | --- | --- |
| palette_run_type_flag = COPY_ABOVE_MODE | 5 | 6 | 7 | bypass |
| palette_run_type_flag = COPY_INDEX_MODE | 0,1,2 | 3 | 4 | bypass |

# Experimental results

The proposed method is integrated into the SCM-5.2 reference software and tested under the common test condition [2][3]. Table 1 and Table 2 show the lossy and lossless results for the full frame IBC search area. The details for the other experiment results with different test conditions are accompanied with this document. It is observed that the average BD-rate increase is smaller than 0.x%.

(TBD)

# Conclusion

This contribution proposes to use bypass mode to code the fourth and fifth bins of the syntax element palette_run_msb_id_plus1. This modification can simplify the derivation process for palette_run_msb_id_plus1 and reduce the context-coded bin about xx%. According to the experimental results, there is no significant gain loss for the lossy test condition under the full frame IBC search area. It is recommended to adopt this contribution.

# References

- R. Joshi, S. Liu, G. J. Sullivan, J. Xu, Y. Ye, “HEVC Screen Content Coding Draft Text 4,” 21th JCT-VC meeting, JCTVC-U1005, Warsaw, Poland, Jun. 2015.

- H. Yu, R. Cohen, K. Rapaka, J. Xu , “Common Test Conditions for Screen Content Coding,” 21th JCT-VC meeting, JCTVC-U1015, Warsaw, Poland, Jun. 2015.

- SCM-5.2, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.4+SCM-5.2/

- Y. J. Chang, C. L. Lin, C. C. Lin, J. S. Tu, W. Digonnet, “Non-CE1: Simplification for parsing the index of the most significant bit in the paletteRun binarization,” 21th JCT-VC meeting, JCTVC-U0109, Warsaw, Poland, Jun. 2015.

# Patent rights declaration(s)

ITRI international may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).