| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0062 |
| --- | --- |

| Title: | HEVC corrigendum: On parsing of bitstream partition nesting SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Hendry Ye-Kui Wang | Email: | 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-651-4722 fhendry@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the bitstream partition nesting SEI message syntax, the syntax element num_seis_bsp_minus1 is signalled after the byte alignment bits. It is reported that this signalling could lead to situations where the parsing of the subsequent sei_message( ) syntax structures is incorrect. This document provides two signalling approaches to address the signalling of the bitstream partition nesting SEI message – the first includes changing the coding of num_seis_bsp_minus1 from ue(v) coding to u(8) coding; the second approach proposes signalling of num_seis_bsp_minus1 before the byte-alignment bits. It is asserted that both the approaches can fix the parsing issue; the authors prefer the first approach.

# Introduction

## The syntax structure of the bitstream partition nesting SEI message is given below:

| bsp_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| sei_ols_idx | ue(v) |
| sei_partitioning_scheme_idx | ue(v) |
| bsp_idx | ue(v) |
| while( !byte_aligned( ) ) |  |
| bsp_nesting_zero_bit /* equal to 0 */ | u(1) |
| num_seis_in_bsp_minus1 | ue(v) |
| for( i = 0; i <= num_seis_in_bsp_minus1; i++ ) |  |
| sei_message( ) |  |
| } |  |

The syntax element num_seis_in_bsp_minus1 was added in the Sapporo meeting (JCTVC-R0221) to prevent limitations on the extensibility of the bitstream partition nesting SEI message and also to avoid limitations on the position of the SEI message in the SEI NAL unit. The addition of the syntax element enabled the extensibility feature of the sei_payload( ) syntax to be applicable to the bitstream partition nesting SEI message.

The position of the addition of the syntax element, however, may cause a parsing error of the SEI message. When the number of bits to code num_seis_in_bsp_minus1 is not a multiplication of 8, the first sei_message nested in a bitstream partition nesting SEI message would not start at a byte-aligned location; consequently, the checking more_data_in_payload(), that is performed at the end of each nested sei_message(), cannot return FALSE so that it will cause the parser to read more data beyond payload_bit_equal_to_one. This issue was reported in JCT-VC reflector and was agreed to be a bug in the Specification.

After some study, it was concluded that some change in the syntax table is unavoidable for fixing this issue.

# Proposal

Two alternative changes are proposed to fix the issue discussed. The authors prefer the first approach of changing the coding of num_seis_in_bsp_minus1 to u(8) instead of ue(v).

## Approach 1

In this approach, the coding of num_seis_in_bsp_minus1 is changed from ue(v) to u(8). Using 8 bits is sufficient as the value range of the syntax element is 0 to 63, inclusive. This effectively makes the starting point of the first sei_message( ) structure at a byte-aligned position. All deletions to the Spec are highlighted with red strikethrough, and additions to the Spec are highlighted in yellow.

### Syntax of bitstream partition nesting SEI message

| bsp_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| sei_ols_idx | ue(v) |
| sei_partitioning_scheme_idx | ue(v) |
| bsp_idx | ue(v) |
| while( !byte_aligned( ) ) |  |
| bsp_nesting_zero_bit /* equal to 0 */ | u(1) |
| num_seis_in_bsp_minus1 | u(8)ue(v) |
| for( i = 0; i <= num_seis_in_bsp_minus1; i++ ) |  |
| sei_message( ) |  |
| } |  |

### Semantics of bitstream partition nesting SEI message

No changes to the semantics is needed.

## Approach 2

In this approach, the syntax element num_seis_in_bsp_minus1 is moved to be signalled immediately after bsp_idx and before the byte-alignment syntax elements bsp_nesting_zero_bit. All deletions to the Spec are highlighted with red strikethrough, and additions to the Spec are highlighted in yellow.

### Syntax of bitstream partition nesting SEI message

| bsp_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| sei_ols_idx | ue(v) |
| sei_partitioning_scheme_idx | ue(v) |
| bsp_idx | ue(v) |
| num_seis_in_bsp_minus1 | ue(v) |
| while( !byte_aligned( ) ) |  |
| bsp_nesting_zero_bit /* equal to 0 */ | u(1) |
| num_seis_in_bsp_minus1 | ue(v) |
| for( i = 0; i <= num_seis_in_bsp_minus1; i++ ) |  |
| sei_message( ) |  |
| } |  |

### Semantics of bitstream partition nesting SEI message

No changes to the semantics is needed (except swapping of the positions of the semantics of the syntax elements num_seis_in_bsp_minus1 and bsp_nesting_zero_bit).

# Patent rights declaration(s)

The authors of this contribution are not aware of current or pending patent rights regarding the specific modifications proposed in this contribution. However, Qualcomm Incorporated may have current or pending patent rights relating to other related aspects of the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).