| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0064 |
| --- | --- |

| Title: | Clarifications on the semantics of CRI SEI message and its usage for HDR/WCG video compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. K. Ramasubramonian J. Sole D. Rusanovskyy D. Bugdayci S. Lee M. Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-5804 aramasub@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes two clarifications to the semantics of the colour remapping information (CRI) SEI message in HEVCv2. The clarifications are related to the input signal to the CRI SEI and to the processing order when more than one CRI SEI is associated with a current picture. The document also describes how the CRI SEI can be used as a range adjustment tool to improve the coding efficiency of HDR video content. It is argued that the suggested changes allow this type of application of the SEI with no impact on its current usage.

# Two clarifications on the CRI SEI

The colour remapping information (CRI) SEI message defined in HEVC conveys information that is used to map pictures in one colour space to another. The syntax of the CRI SEI message includes three parts: a first look-up table (Pre-LUT), followed by a 3x3 matrix indicating colour remapping coefficients, and followed by a second look-up table (Post-LUT). For each color component, e.g. R, G, and B or Y, Cb, and Cr, independent LUT is defined for both, Pre-LUT and Post-LUT.

The CRI SEI message also includes a syntax element colour_remap_id, different values of which may be used to indicate different purposes of the SEI message.

This section proposes two changes to clarify the usage of the CRI SEI.

## Input Samples

The semantics of the SEI message is unclear in the interpretation as to where in a hypothetical processing flow of video decoding system the information contained in the SEI message can be applied. For example, it is mentioned in the semantics that the SEI message may be applied directly to the decoded samples. However, it is unclear as to whether this message can be applied to the decoded samples produced as result of applying of one or more processes to the decoded samples, such as inverse quantization, up-sampling from 4:2:0 to 4:4:4, or after performing a colour space conversion of the decoded samples. The following clarification note (in highlight) could be introduced:

The colour remapping information SEI message provides information to enable remapping of the reconstructed colour samples of the output pictures. The colour remapping information may be applied directly to the decoded sample values, regardless of whether they are in the luma and chroma domain or the RGB domain. The colour remapping information may also be applied to the decoded samples after the application of one or more processes. The colour remapping model used in the colour remapping information SEI message is composed of a first piece-wise linear function applied to each colour component (specified by the “pre” set of syntax elements herein), a three by-three matrix applied to the three colour components, and a second piece-wise linear function applied to each colour component (specified by the “post” set of syntax elements herein).

## Multiple CRI SEI messages

The specification of the CRI SEI message includes the syntax element colour_remap_id to indicate the purpose of the SEI message. However, its semantics do not specify processing of concurrent or sequential CRI SEI messages with different IDs, and whether such concurrency or sequence of CRI SEI messages may be carried and/or applied. Even if multiple CRI messages are included, it is unclear in which order these SEI messages are to be applied. The following changes could be introduced to clarify this aspect:

colour_remap_id contains an identifying number that may be used to identify the purpose of the colour remapping information. The value of colour_remap_id shall be in the range of 0 to 232 − 2, inclusive.

Values of colour_remap_id from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of colour_remap_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all colour remapping information SEI messages containing a value of colour_remap_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, and bitstreams shall not contain such values.

NOTE 1 – The colour_remap_id can be used to support different colour remapping processes that are suitable for different display scenarios. For example, different values of colour_remap_id may correspond to different remapped colour spaces supported by displays.

When more than one SEI message is associated with the current picture, and each such SEI message has a different value of colour_remap_id, the colour remapping information of one SEI message is applied after the application of colour remapping information of another SEI message. The order in which the SEI messages are to be applied may be determined by the application. When not specified, the colour remapping information is applied in increasing order of the values of colour_remap_id of the colour remapping information SEI messages associated with the current picture.

# Example of usage: Range Adjustment for HDR compression

HDR/WCG video compression can be improved by adjusting the distribution of the input values to the codec by the dynamic range adjustment (DRA) method. The method DRA could be seen as an additional module to the HDR10 system that can be implemented with the CRI SEI. Several configurations can be envisaged, from low-complexity to SDR backwards compatible to higher efficiency, and all of them may be realized with the signalling mechanisms already standardized in HEVCv2.

As means of example, a simple scaling of the chroma samples that improves HDR performance can be signalled by means of the CRI SEI. In this simple configuration, the DRA applies a scaling factor to the 10-bit Cb/Cr values (figure 1) before the HEVC encoder. At the decoder, the inverse scaling is applied to recover the original range of the chroma samples.

The chroma scaling method via CRI SEI has been implemented on the top of HM16.2. The performance is compared against HDR10 using 9 sequences. Objective metrics (tPSNRY, deltaE) are provided in table 1. Subjectively, the approach shows less chroma artifacts at same bitrates.

Figure 1. CRI SEI can be configured to apply a scaling to the chroma components.

| Row Labels | Average of BD-tPSNR-X | Average of BD-tPSNR-Y | Average of BD-tPSNR-Z | Average of BD-tPSNR-XYZ | Average of BD-PSNR_DE |
| --- | --- | --- | --- | --- | --- |
| Tibul_2020 | -6.53% | 1.78% | -16.87% | -5.72% | -20.73% |
| Market_2020 | -2.16% | 2.36% | -9.43% | -3.94% | -49.94% |
| FireEater_2020 | -14.11% | -0.82% | -18.89% | -11.87% | -29.65% |
| AutoWelding | 7.32% | 7.25% | 6.32% | 6.96% | -26.41% |
| BikeSparkle | -3.09% | 3.68% | -13.85% | -5.24% | -40.32% |
| ShowGirl | -0.90% | 5.84% | -7.56% | -1.05% | -33.93% |
| MagicHour_hdr | -3.70% | 2.94% | -9.18% | -5.03% | -30.29% |
| WarmNight_hdr | -5.78% | 3.36% | -7.46% | -4.17% | -32.43% |
| BalloonFestival | -7.33% | 5.61% | -10.81% | -6.27% | -37.85% |
| Average | -4.03% | 3.56% | -9.75% | -4.04% | -33.50% |

Table 1. Objective metrics improvements of chroma scaling using CRI SEI. The reference is MPEG CfE anchor.

# Conclusions

HEVCv2 provides means of improving the HDR coding performance beyond encoder modifications thanks to the CRI SEI. This SEI can be configured beyond its initial intended usage to adjust the range of the HDR data in order to, for example, provide higher coding efficiency. Two clarifications are proposed to allow this and other usages of CRI SEI. Backward compatibility to the HEVCv2 CRI SEI is kept.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).