| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0065 |
| --- | --- |

| Title: | Further redundancy removal for coding palette index map |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Tzu-Der Chuang, Yu-Wen Huang, Shawmin Lei MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shih-Ta Hsiang +886-3-5670766 ext. 24685 {shih-ta.hsiang, Peter.Chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the modifications to further remove redundancy for coding the palette index map. The proposed modifications remove possible redundant information and improve the worst-case number of the coded bins for coding syntax element num_palette_indices_minus1. This contribution also proposes the corrected formula for deriving the variable PaletteMaxRun to further remove bit redunacy. The proposed method for coding num_palette_indices_minus1 reportedly achieves average Luma BD-rate savings 0.0%, 0.0%, and 0.1% for lossy coding YUV, text & graphics with motion, 1080p & 720p sequences for the AI, RA, LB settings, respectively, under the common test conditions. The proposed modification for deriving the variable PaletteMaxRun reportedly achieves average Luma BD-rate savings 0.0%, 0.0%, and 0.1% for lossy coding YUV, text & graphics with motion, 1080p & 720p sequences for the AI, RA, LB settings, respectively, under the common test conditions.

# Proposed modifications

Coding syntax element num_palette_indices_minus1

The current method for the binarization of the syntax element num_palette_indices_minus1 re-uses the existing method for the binarization of the syntax element coeff_abs_level_remaining. The Rice parameter is derived as follows:

cRiceParam = 3 + ( ( MaxPaletteIndex + 1 )  >>  3 ),

where the variable MaxPaletteIndex specifies the maximum possible value for a palette index for the current coding unit (CU). In this way, the value of resulting parameter cRiceParam increases with the variable MaxPaletteIndex, leading to a large number of the coded bins. Under the worst case, the maximal number of the coded bins is equal to 12, 20, 36, and 132 for max palette size equal to 63, 127, 255, 1024 (32x32), respectively.

In the current binarization method, the parameter cRiceParam indicates the number of the fixed-length suffix bits. However, the value of the syntax element num_palette_indices_minus1 cannot be greater than the total number of the pixles in the current CU, indicated by the variable CUSize. Thus, we need no more than Log2(CUSize) bits for representing syntax element in a fixed-length code, or no more than 6, 8, 10 bits for 8x8, 16x16, and 32x32 CU, respectively. The current method will contain redundant 0 bits in the most significant part of the suffix bin string when the Rice parameter cRiceParam is greater than Log2(CUSize).

In the proposed method, the Rice parameter cRiceParam is derived as follows:

cRiceParam = Log2(CUWidth) + ( ( MaxPaletteIndex + 1 )  >>  3 ),

where the variable CUWidth indicates the width of the current CU. If the derived value of the Rice parameter cRiceParam is greater than or equal to Log2(CUSize), the syntax element is binarized by the fixed-length binarization process with the bit length equal to Log2(CUSize). Otherwise, the current binarization process is still applied.

Deriving variable PaletteMaxRun

The SCC draft utilizes the variable PaletteMaxRun to represents the maximum possible value for variable PaletteRun, derived as follow:

- PaletteMaxRun = nCbS * nCbS − PaletteScanPos − 1 − remainingNumIndices

The proposed modification further considers the possible final run with copy_above_indices_for_final_run_flag equal to 1 and the variable PaletteMaxRun is derived by

- PaletteMaxRun = nCbS * nCbS − PaletteScanPos − 1 − remainingNumIndices–-copy_above_indices_for_final_run_flag

# Experimental results

The proposed method has been integrated into the SCC reference software SCM-5.2. All the tests were conducted according to the common test conditions specified in [1].

Table 1 summarizes the lossy simulation results of the proposed method for coding the syntax element num_palette_indices_minus1 using full-frame IBC search. Table 2 summarizes the lossy simulation results of the proposed method for deriving variable PaletteMaxRun using full-frame IBC search. Please note the comparisons on the run time results are not accurate in both tables.

Table 1: Lossy BD bitrate results of the proposed method for coding num_palette_indices_minus1 versus the SCM-5.2 anchor for lossy coding, FF-IBC.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 128% |  |  |
| Dec Time[%] | 129% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.2% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 122% |  |  |
| Dec Time[%] | 125% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | 0.0% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.0% | 0.1% |
| RGB, Animation, 720p | -0.1% | 0.2% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.2% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.1% | -0.5% |
| YUV, Animation, 720p | 0.0% | 0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 127% |  |  |
| Dec Time[%] | 126% |  |  |

Table 2: Lossy BD bitrate results of the proposed method for deriving variable PaletteMaxRun versus the SCM-5.2 anchor for lossy coding, FF-IBC.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | 0.0% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.2% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.5% | -0.6% |
| YUV, Animation, 720p | 0.0% | 0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-U1015, 21st Meeting: Warsaw, PL, 19–26 June 2015.