| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0075 |
| --- | --- |

| Title: | Signalling of updated video regions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | D. Marchya Y.-K. Wang M.M.K. Amara Venkata R. Joshi S. Kottilingal | Tel: Email: | +1-858-845-1472 dmarchya@quicinc.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes a new SEI message, named updated regions SEI message, to indicate regions covered by one or more rectangles that have updated decoded sample values relative to the previous picture in output order, while other regions remain unchanged. It is asserted that the indication can be used by the display subsystem at the decoder side to perform partial composition for display.

It is suggested to consider adding this SEI message to both HEVC and AVC.

# Introduction

During screen sharing, screen recording, or wireless display mirroring, often only the composed UI layers of the source display subsystem is typically encoded as video bitstream and transmitted. The UI layers tend to have only one or more small updated regions in many instances, while other regions remain unchanged. This bitstream is decoded and displayed at the destination. Additional overlays may be composed on the decoded output at the destination display subsystem.

Smart display panels are capable of composing partial frames. This capability can be used to compose only updated regions of a video frame. Instead of always composing the full video layer, which leads to inefficient utilization of hardware resources, partial composition for display can save bandwidth between the decoded picture buffer and the display, and there can be less clock voting, which means less power consumption.

If the information about source updated regions is signalled to the decoder side, the destination display subsystem can use it to perform partial display frame composition.

# Proposal

A picture level SEI message, named updated regions SEI message, is proposed to convey the information of updated regions. The syntax and semantics of the proposed SEI message are as follows:

| updated_regions( payloadSize ) { | Descriptor |
| --- | --- |
| updated_regions_cancel_flag | u(1) |
| if( !updated_regions_cancel_flag ) { |  |
| updated_region_cnt_minus1 | ue(v) |
| for( i = 0; i <= updated_region_cnt_minus1; i++ ) { |  |
| updated_region_left_offset[ i ] | ue(v) |
| updated_region_top_offset[ i ] | ue(v) |
| updated_region_width_offset[ i ] | ue(v) |
| updated_region_height_offset[ i ] | ue(v) |
| } |  |
| updated_regions_persistence_flag | u(1) |
| } |  |
| } |  |

The updated regions SEI message indicates the rectangular regions, in the associated picture, in which the samples have different decoded sample values than the collocated samples in the previous picture in output order. The samples of the associated picture that are not in the indicated rectangular regions have the same decoded sample values as the collocated samples in the previous picture in output order.

updated_regions_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous updated regions SEI message in output order that applies to the current layer. updated_regions_cancel_flag equal to 0 indicates that updated regions information follows.

updated_region_cnt_minus1 specifies the number of updated rectangular regions that are specified by the updated regions SEI message. The value of updated_region_cnt_minus1 shall be in the range of 0 to 15, inclusive.

updated_region_left_offset[ i ], updated_region_top_offset[ i ], updated_region_width[ i ] and updated_region_height[ i ], specify, as unsigned integer quantities in units of sample spacing relative to the luma sampling grid, the location of the i-th updated rectangular region.

The value of updated_region_rect_left_offset[ i ] shall be in the range of 0 to pic_width_in_luma_samples − 1, inclusive. The value of updated_region_top_offset[ i ] shall be in the range of 0 to pic_height_in_luma_samples − 1, inclusive. The value of updated_region_width[ i ] shall be in the range of 1 to pic_width_in_luma_samples − updated_region_left_offset[ i ], inclusive. The value of updated_region_height[ i ] shall be in the range of 1 to pic_height_in_luma_samples − updated_region_top_offset[ i ], inclusive.

The i-th rectangular updated region is specified, in units of sample spacing relative to a luma sampling grid, as the region with horizontal coordinates from updated_region_left_offset[ i ] to pic_width_in_luma_samples − updated_region_right_offset[ i ] − 1 and with vertical coordinates from updated_region_rect_top_offset[ i ] to pic_height_in_luma_samples − pan_scan_rect_bottom_offset[ i ] − 1, inclusive.

updated_regions_persistence_flag specifies the persistence of the updated regions SEI message for the current layer.

updated_regions_persistence_flag equal to 0 specifies that the updated regions information applies to the current decoded picture only.

Let picA be the current picture. updated_regions_persistence_flag equal to 1 specifies that the updated regions information persists for the current layer in output order until any of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a updated regions SEI message and applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).