| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0078 |
| --- | --- |

| Title: | Improvement of coding efficiency for rate control under the constraint of HRD |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong-Jo Ahn Xiangjian Wu Donggyu Sim Hochan Ryu | Tel: Email: | +82-2-940-5470 yongjoahn@kw.ac.kr xjw0118@kw.ac.kr dgsim@kw.ac.kr hc.ryu@digitalinsights.co.kr |
| Source: | KWU (Kwangwoon University), Digital Insights Co. |  |  |

_____________________________

# Abstract

This contribution proposes a way to improve usage of coded picture buffer (CPB) and coding performance of the target bit saturation method, which was proposed in JCTVC-U0132. To prevent CPB from overflow and underflow, the target bit saturation method was proposed in JCTVC-U0132 adjusts the target bit for a picture by using the fixed lower and upper bound of CPB fullness when it is expected to occur CPB overflow or underflow. In the performance evaluation of the contribution, some degradations of coding efficiency were observed for a few sequences. In this contribution, an improvement of the target bit saturation method is presented to achieve coding gain and to improve usages of CPB under the constraint of hypothetical reference decoder (HRD). To improve the coding efficiency of rate control under the constraint of HRD, this contribution proposes a target bit saturation for each picture using adaptive lower bound instead of a fixed lower bound. To determine the adaptive lower bound (L) for preventing CPB from underflow, target bit for the next coded picture is estimated and considered. If the target bit for the next coded picture is expected to be high, the lower bound is set to relatively high. Otherwise, the lower bound is set to relatively low. It can guarantee to prevent CPB from underflow with adaptively using of CPB fullness.

# Introduction

To prevent coded picture buffer (CPB) from overflow and underflow, JCTVC-U0132 [1] proposed a target bit saturation method under the constraint of hypothetical reference decoder (HRD) at the previous meeting. The proposed method controls the occupancy of CPB between 10% and 90% of the CPB size for lower and upper bounds. The proposed method performs target bit saturation to prevent CPB from overflow and underflow with 0.82% BD-BR loss and 0.04% rate control accuracy decrement. However, it is observed that some degradations of coding efficiency in particular sequences, for example, ‘Traffic’, ‘ParkScene’, and ‘BQSquare’. The reason of degradations is that conventional rate control algorithm [2] without any consideration for the constraint of HRD allocates an extremely high target bit for I-pictures (key pictures). The rate control with the target bit saturation method (JCTVC-U0132) can allocate a proper target bit for key pictures for the constraint of HRD as shown in Fig. 1; however, it causes propagation of coding performance from degraded reference pictures.

Figure 1. Target bit allocations for key picture without and with consideration for the constraint of HRD

# Proposed techniques

## Conventional target bit saturation with a fixed bound

Figure 2. Illustration of CPB fullness with parameters (R, B, F)

Figure 2 shows an example of coded picture buffer (CPB) with the hypothetical reference decoder (HRD) parameters. R means the transmission bit rate, B is the CPB size, and F is the CPB fullness. In addition, this contribution denote that i is the picture number, bi is the amount of bits for the i-th picture, and f means a frame rate. A rate control algorithm should guarantee to avoid CPB overflow and underflow under the constraint of HRD. Conditions of CPB overflow and underflow can be written by

(1)

,		                    	   (2)

respectively. Note that Fi is the CPB fullness of the i-th picture and bi is the actual bits of the i-th picture. When the CPB occupancy is full, but the amount of bits for a picture is relatively small, CPB overflow occurs. When the amount of bits for a picture is larger than current CPB fullness, CPB underflow occurs.

In JCTVC-U0132, the target bit saturation method in picture-level was proposed to prevent CPB from underflow and overflow. This method tries to make the occupancy of CPB stay between 10% and 90% of the CPB size for lower and upper bound. Especially, target bit allocation of key pictures is related to the CPB underflow under the constraint of HRD. In JCTVC-U0132, the CPB underflow is checked by a condition with the fixed lower bound and denoted by

.	               		(3)

Note that tbi is the target bit of the i-th picture estimated by the rate control method in HM. If the expected buffer fullness after encoding the i-th picture is smaller than the lower bound, re-estimated target bit of the i-th picture, tb'i, is set to max(200, Fi-B×0.1). Finally, the saturated target bit becomes input for estimating lambda and QP of the rate control method in HM. Figure 3 shows examples of CPB underflow and target bit saturation with the fixed lower bound.

| (a) | (b) |
| --- | --- |

Figure 3. Examples of (a) CPB underflow and (b) target bit saturation with the fixed lower bound

## Proposed target bit saturation with an adaptive lower bound

To improve the coding efficiency of rate control under the constraint of HRD, this contribution proposes a target bit saturation for each picture using adaptive lower bound instead of a fixed lower bound. To determine the adaptive lower bound (L) for preventing CPB from underflow, target bit for the next coded picture is estimated and considered. If the target bit for the next coded picture is expected to be high, the lower bound is set to relatively high. Otherwise, the lower bound is set to relatively low. It can guarantee to prevent CPB from underflow with adaptively using of CPB fullness. The proposed adaptive lower bound of i-th picture (Li) is calculated by

(4)

where TBGOP is the target bit for current group of picture (GOP), CBGOP is coded bit of current GOP, wi is weighted parameter for i-th picture, and N is number of pictures in GOP. Target bit for i-th picture, tbi, is calculated by the conventional R-lambda rate control of HM and it is denoted by

.      	              	(5)

In Eq. (4), (CBGOP+tbi) means the expected bit after encoding the i-th picture, and the proposed adaptive lower bound of i-th picture is derived from the estimated target bit for the next, i+1-th, picture at the current, i-th, picture time. In case of the last picture in GOP, the proposed adaptive lower bound is derived by the estimated target bit of the first picture in current GOP because rate control of HM can not calculate the target bit for the next, i+1-th, picture at the current, i-th, picture time.

In this contribution, the CPB underflow is checked by a condition with the adaptive lower bound and it is written by

.		                		(6)

where tbi is the target bit of the i-th frame estimated by the rate control method in HM. If the buffer fullness is smaller than the proposed adaptive lower bound (Li), re-estimated target bit of i-th picture, tb'i, is set to max(200, Fi-Li). Finally, the saturated target bit becomes input for estimating lambda and QP of the rate control method in HM. In our implementation, the rate control algorithm itself has not been altered. Figure 4 shows an example of target bit saturation with the proposed adaptive lower bound to prevent CPB from underflow.

Figure 4. Example of target bit saturation with the proposed adaptive lower bound

# Performance evaluation

The proposed method was evaluated under the common test conditions (CTC) for HEVC [3], and it is integrated on HM-16.6 [4]. For performance evaluation, two scenarios are considered. The first scenario assumes that CPB size is set to half of a target bitrate and initial CPB fullness is set to 90% of the CPB size. That is, the initial CPB delay is 0.45 sec. For the second scenario, CPB size is set to the target bitrate and initial CPB fullness is set to 75% of CPB size. That is, the initial CPB delay is 0.75 sec.

Figure 5 and 8 show the CPB fullness with respect to frame index with the rate control of HM without the target bit saturation. As shown in Fig. 5 and 6, CPB overflow and underflow are observed. Figure 6 and 9 show the CPB fullness of the rate control with the target bit saturation using the fixed lower bound. In addition, Fig. 7 and 10 show the CPB fullness of the rate control with the target bit saturation using the proposed adaptive lower bound. As shown in figures, CPB overflow and underflow are not observed with the target bit saturation method. Especially, Fig. 7 and 10 show that the target bits for key pictures with the proposed algorithm are relatively higher, compared to those with the fixed lower bound.

| (a) | (b) |
| --- | --- |

Figure 5. CPB fullness with respect to frame index without target bit saturation (Scenario 1: B=0.5R, F=0.9B)

| (a) | (b) |
| --- | --- |

Figure 6. CPB fullness with respect to frame index with the target bit saturation using the fixed lower bound (Scenario 1: B=0.5R, F=0.9B)

| (a) | (b) |
| --- | --- |

Figure 7. CPB fullness with respect to frame index with the target bit saturation using the proposed adaptive lower bound (Scenario 1: B=0.5R, F=0.9B)

| (a) | (b) |
| --- | --- |

Figure 8. CPB fullness with respect to frame index without target bit saturation (Scenario 2: B=R, F=0.75B)

| (a) | (b) |
| --- | --- |

Figure 9. CPB fullness with respect to time index with the target bit saturation using the fixed lower bound (Scenario 2: B=R, F=0.75B)

| (a) | (b) |
| --- | --- |

Figure 10. CPB fullness with respect to time index with the target bit saturation using the proposed adaptive lower bound (Scenario 2: B=R, F=0.75B)

Table 1 and 3 show BD-BR and accuracy of rate control with the target bit saturation method using the proposed adaptive lower bound compared to the rate control without any target bit saturation method in scenario 1 and scenario 2, respectively. In addition, Table 2 and 4 show BD-BR and accuracy of rate control with the the target bit saturation method with the proposed adaptive lower bound compared to rate control with the target bit saturation using the fixed lower bound in scenario 1 and 2, respectively. In Table 1 to 4, delta bitrate error, ΔE, is denoted by

(5)

where TB is the target bitrate and AB is the actual bitrate. In performance evaluation, the proposed method performs target bit saturation to avoid CPB overflow and underflow with 0.58% BD-BR loss and 0.01% rate control accuracy decrement in scenario 1. In addition, the proposed method can yield an additional average coding gain of 0.15% BD-BR against the target bit saturation with the fixed lower bound.

Table 1. RD performance of the proposed target bit saturation method compared to rate control of HM without any target bit saturation method (Scenario 1: B=0.5R, F=0.9B)

|  | Random access |  |  |  | Low-delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | ΔE | Y | U | V | ΔE |
| Class A | 2.05 | 5.01 | 5.27 | -0.00 |  |  |  |  |
| Class B | 1.71 | 5.18 | 5.13 | -0.01 | -0.22 | 2.49 | 3.18 | -0.02 |
| Class C | 0.01 | 0.73 | 0.44 | 0.01 | 0.03 | 0.94 | 1.01 | 0.00 |
| Class D | 0.28 | 1.39 | 1.44 | 0.10 | 0.17 | 1.48 | 1.79 | 0.05 |
| Class E |  |  |  |  | 0.65 | 2.30 | 2.76 | 0.01 |
| Overall | 1.05 | 3.20 | 3.19 | 0.02 | 0.10 | 1.82 | 2.21 | 0.00 |

Table 2. RD performance of the proposed target bit saturation method compared to rate control of HM with the target bit saturation using the fixed lower bound (Scenario 1: B=0.5R, F=0.9B)

|  | Random access |  |  |  | Low-delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | ΔE | Y | U | V | ΔE |
| Class A | -0.43 | -0.70 | -0.57 | -0.00 |  |  |  |  |
| Class B | 0.11 | 0.86 | 0.79 | 0.08 | -0.08 | -0.29 | -0.35 | 0.00 |
| Class C | -0.09 | -0.29 | -0.30 | 0.00 | -0.01 | -0.01 | -0.03 | -0.01 |
| Class D | -0.35 | -0.49 | -0.82 | -0.06 | 0.02 | 0.41 | -0.31 | -0.00 |
| Class E |  |  |  |  | -0.52 | -1.24 | -1.74 | -0.00 |
| Overall | -0.17 | -0.09 | -0.16 | 0.01 | -0.12 | -0.22 | -0.52 | -0.00 |

Table 3. RD performance of the proposed target bit saturation method compared to rate control of HM without any target bit saturation method (Scenario 2: B=R, F=0.75B)

|  | Random access |  |  |  | Low-delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | ΔE | Y | U | V | ΔE |
| Class A | 0.00 | 0.85 | 0.95 | 0.00 |  |  |  |  |
| Class B | 0.06 | 0.60 | 0.35 | -0.12 | -0.07 | 0.18 | 0.31 | -0.00 |
| Class C | -0.03 | -0.21 | -0.13 | -0.01 | 0.01 | -0.06 | 0.07 | 0.00 |
| Class D | 0.05 | 0.37 | 0.50 | 0.03 | 0.02 | 0.41 | 0.61 | 0.00 |
| Class E |  |  |  |  | -0.08 | -0.30 | 0.23 | 0.00 |
| Overall | 0.02 | 0.41 | 0.41 | -0.03 | -0.03 | 0.09 | 0.31 | 0.00 |

Table 4. RD performance of the proposed target bit saturation method compared to rate control of HM with the target bit saturation using the fixed lower bound (Scenario 2: B=R, F=0.75B)

|  | Random access |  |  |  | Low-delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | ΔE | Y | U | V | ΔE |
| Class A | -0.18 | -0.44 | -0.42 | 0.00 |  |  |  |  |
| Class B | 0.06 | 0.60 | 0.35 | -0.12 | -0.07 | 0.18 | 0.31 | 0.00 |
| Class C | -0.03 | -0.21 | -0.13 | -0.01 | 0.01 | -0.06 | 0.07 | 0.00 |
| Class D | 0.05 | 0.37 | 0.50 | 0.03 | 0.02 | 0.41 | 0.61 | 0.00 |
| Class E |  |  |  |  | 0.00 | 0.00 | 0.00 | 0.00 |
| Overall | -0.02 | 0.11 | 0.09 | -0.03 | -0.02 | -0.15 | -0.27 | -0.00 |

# Conclusion

In this contribution, a way to improve usage of CPB and coding efficiency of the target bit saturation method is proposed. To improve coding efficiency, this contribution proposes a target bit saturation for each picture using the adaptive lower bound instead of the fixed one. The proposed method performs target bit saturation under the constraint of HRD with 0.58% BD-BR loss and 0.01% rate control accuracy decrement, compared to rate control of HM without any consideration for the constraint of HRD. In addition, the proposed method can yield an additional average coding gain of 0.15% BD-BR against the target bit saturation with the fixed lower bound.

# References

- B. Li, H. Li, L. Li, and J. Zhang, “Rate control by R-lambda model for HEVC,” JCTVC-K0103, 11th JCT-VC meeting, Shanghai, CN, Oct. 2012.

- Y.-J. Ahn, X. Wu, W. Lim, and D. Sim, “Target bits saturation to avoid CPB overflow and underflow under the constraint of HRD,” JCTVC-U0132, 21st JCT-VC meeting, Warsaw, PL, June 2015.

- F. Bossen, “Common HM test conditions and software reference configurations,” JCTVC-L1100, 12th JCT-VC meeting, Geneva, CH, Jan. 2013.

- HEVC reference model (HM) 16.6, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.6

# Patent rights declaration(s)

KWU and Digital Insights Co. do not have any current or pending patent rights relating to the technology described in this contribution.