| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0080 |
| --- | --- |

| Title: | Palette encoder improvements for the 4:2:0 chroma format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Christophe Gisquet, Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, France | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon CRF |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results related to contribution JCTVC-V0042. It proposes a new initialization process where the slice’s PPS and SPS initializers are combined to generate the slice’s palette predictor initializer. The verification work was performed by using the software provided by the proponent. As conclusion, the BD rate results and the memory consumption figures reported in this crosscheck match to those provided in the contribution JCTVC-V0042.

# Summary of JCTVC-V0042 proposal

## Scheme as of SCC Draft 4

Syntax was first proposed in [1] to signal at the PPS level palette entries used to initialize at the slice level the palette predictor. The primary intent for this proposal was coding efficiency when using multiple slices per image, and was tested in a 1500-byte-slice scenario.

It was refined in [2] to allow signalling at the SPS level: indeed, if for some reason, a new PPS need to be transmitted in order to change some parameters, this requires sending again the palette predictor initializer in this slice. Furthermore, it was demonstrated to show some benefit in e.g. the RA scenario of the Common Test Conditions [3].

## Normative changes

There are 2 items:

- Allowing a PPS-level zero-sized initializer, indicating that the palette predictor for the slice is not initialized and should remain empty;

- It proposes to initialize, if not marked as empty, the palette predictor by first putting the PPS entries then the SPS entries, so as to reduce the PPS-level signalling of duplicated entries. This entails putting restrictions on the number of the PPS and SPS entries

## Implementation

There are two main parts:

- At the encoder and decoder, performing the normative process above;

- At the encoder, pruning PPS entries by comparing them to the SPS entries.

Because there is an upper-limit on the number of entries in the palette predictor, the encoder signals less entries in the SPS so as to make room for the PPS entries.

## Results reported

The proposal (“test2”) requires activating both SPS- and PPS-level signaling. It therefore uses as an anchor (“test1”) SCM5.2 having those activated, following the CTCs, which differs from the tests performed in [1] (which used slices) and [2] (which compared PPS-only and SPS-only signaling).

An additional result reported is the number of PPS entries saved. This is different from the total number of entries for 2 reasons:

- V0042 restricts itself to using fewer SPS entries, so this would not compare exactly the same things;

- The argument can be made that, for a long, repetitive sequence, the number of SPS entries becomes negligible compared to the entries sent through PPSes.

# Results

## 4:2:0, lossy configuration

### Test 1

### Test 2

## 4:2:0, lossless configuration

### Test 1

### Test 2

## 4:4:4, lossy configuration

### Test 1

### Test 2

We have also gathered the statistics for PPS entries:

Basically, AI scenarios really need being refreshed frequently, while RA/LDB ones need either a longer period or not refresh at all. This confirms the results of [2], where it was asserted sending the initializer entries at the SPS level is often enough in the CTCs.

## 4:4:4, lossless configuration

### Test 1

### Test 2

Similarly, PPS entries statistics:

It is interesting to notice that in lossless, the statistics are not that different from the lossy 4:4:4 scenarios.

# References

- JCTVC-T0048, “Non-CE1: On palette prediction for slices,” C. Gisquet, T. Poirier, G. Laroche, P. Onno (Canon), Geneva, CH, Feb. 2015.

- JCTVC-U0084, “On palette predictor initialization for Screen Content Coding,” J. Ye, S. Liu, S. Lei (MediaTek), Warsaw, PL, June. 2015.

- JCTVC-U1015, “Common Test Conditions for Screen Content Coding,” H. Yu, R. Cohen, K. Rapaka, J. Xu, Warsaw, PL, June. 2015.