| CE/AHG relation Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0083 |
| --- | --- |

| Title: | SJTU Test Sequences for video coding development |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Song | Email: | song_li@sjtu.edu.cn |
| Source: | Institute of Image Communication & Network Engineering Shanghai Jiao Tong University |  |  |

_____________________________

# Abstract

This document presents a set of 15 video sequences (3480x2160, 30 fps, BT.709, 4:4:4 10-bit and 4:2:0 8bit) for standardization development and coding efficiency evaluation of HEVC and Beyond.

# Sequences Description

In this contribution we introduce a dataset of 4:4:4 and 4:2:0 4K@30fps video sequences with diverse characteristics that can be used for video standardization/development activities. Considering the significance of video content in the relevant researches, the factors such as image texture, image detail, movement speed of the object in the image, light intensity, and the camera lens stretching, panning are taken into account and thus the dataset can represent a wide variety of content types.

All of the UHD video sequences included in the dataset were shot using Sony F65 camera [1] designed for premium cinema productions recording. The raw data of the images quantified with 10 bit were exported at resolution of 3840x2160 and frame rate of 30fps with DPX format. The DPX image files were combined and converted into uncompressed YUV files using FFMPEG software. Both YUV 4:4:4 color sampling, 10 bits per sample and YUV 4:2:0 color sampling, 8 bits per sample formats are offered in our dataset.

All video sequences presented in the dataset are of 300 frames or 10s.

The relevant technical details of the recording devices and processing are also provided [2].

Table 1: Summary of the SJTU 4K video sequences dataset

| Scenario | Description | Thumbnail |
| --- | --- | --- |
| Bund Nightscape | The nightscape of the gallery of International Architecture in Bund |  |
| Campfire Party | The flames before the crew of the NERC-DTV in a campfire party |  |
| Construction Field | The excavator arms in a construction area |  |
| Fountains | The artificial fountains jetting water into the air in front of a tall building |  |
| Library | The sparsely moving students in the front of the new library |  |
| Marathon | The early stages of 2012 Shanghai International Marathon Race |  |
| Runners | The runners in midway of 2012 Shanghai International Marathon Race |  |
| Residential Building | A residential building with dense texture of the outside wall and intricate lines |  |
| Rush Hour | The students on the ways to canteen or dormitory after classes |  |
| Tall Buildings | Skyscraper in Lujiazui, Pudong New District in Shanghai |  |
| Traffic and Building | Cars on the road and distant buildings |  |
| Traffic Flow | The roadways with the automobiles moving in a different direction |  |
| Tree Shade | The shade under a large tree |  |
| scarf | The vivid exhibition of an accessory shop |  |
| Wood | A wood in the campus of SJTU with sunlight piercing through |  |

- Copyright information

The test sequences(SJTU-4K) and all intellectual property rights therein remain the property of Institute of Image Communication & Network Engineering, Shanghai Jiao Tong University.

The following uses are allowed for the SJTU-4K sequences:

- Can be published in technical papers.

- Can be played at technology research and development events.

- Can be used by Standards committees. (e.g., ITU, MPEG, VQEG,).

The following uses are not allowed for the SJTU-4K sequences:

- Do not publish snapshots in product brochures.

- Do not redistribute video with a commercial product.

- Do not use in television shows, commercials, or movies.

- References

- Sony F65, https://pro.sony.com/bbsc/ssr/product-F65RSPAC1/

- L. Song, X. Tang, W. Zhang, X. Yang, P. Xia, The SJTU 4K Video Sequence Dataset, the Fifth International Workshop on Quality of Multimedia Experience (QoMEX2013), Klagenfurt, Austria, July 3rd-5th, 2013