| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0085 |
| --- | --- |

| Title: | Comments on contribution V0055 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Matteo Naccari Andrew Cotton Centre House, Block D 56 Wood Lane, W12 7SB London | Tel: Email: | +44 (0)303 0409640 matteo.naccari@bbc.co.uk andrew.cotton@bbc.co.uk |
| Source: | BBC R&D |  |  |

_____________________________

# Abstract

This contribution provides comments and a reply to the points raised in document JCTVC-V0055. The comments made in this contribution relate to an incorrect interpretation of the values associated with the Hybrid Log Gamma (HLG), (ARIB STD-B67) OETF and points out how to perform the conversion from HLG to ACES colour space.

# Introduction

Contribution V0055 summarises the Hybrid Log Gamma (HLG) transfer function and requires some additional information on how the values derived after applying HLG are converted to the ACES colour space. It should be noted that HLG as standardised in ARIB STD-B67 defines an Opto-Electro Transfer Function (OETF) which is dimensionless, i.e. it maps linear light to code words written on to the associated image file. The text in the abstract of V0055 is therefore incorrect as it attaches units to the concepts of OETF. Moreover, the second transfer function mentioned in the abstract refers to the Opto-Optical Transfer Function (OOTF), a system gamma function which is part of the Electro-Optical Transfer Function (EOTF) and implemented in the display. Since ARIB STD-B67 defines the OETF only, it should be noted that there is only one HLG transfer function which corresponds to entry 18 of the HEVC VUI.

# Converting from hybrid log gamma to ACES colour space

V0055 also requires comments on how to “unambiguously translate each sample code level to intensity values that can be directly imported into ACES”. Just like the ACES colour space, HLG defines a scene referred system whose end-to-end processing chain is depicted in Figure 1. As may be noted, the OOTF is part of the display. Thus in order to convert from the HLG scene-referred image data to the ACES scene-referred colour space, one simply applies the inverse of the HLG OETF to recreate the linear scene light signal. That scene light signal can then be directly imported in to ACES, with an appropriate colour space conversion.

It should be noted that conversion from display referred signals such as ST2084 to the scene referred ACES system is actually rather more complex, as the OOTF for the mastering environment and display must first be removed from the signal.

Figure 1: End-to-end chain associated with the HLG transfer function.