| CE/AHG relation Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0086 |
| --- | --- |

| Title: | B-Com Test Sequences for video coding development |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Felix Henry, Xavier Ducloux Thibaud Biatek Jean-Yves Aubié | Email: | felix.henry@b-com.com xavier.ducloux@b-com.com thibaud.biatek@b-com.com jean-yves.aubie@b-com.com |
| Source: | b<>com |  |  |

_____________________________

# Abstract

The Institute of Research & Technology b<>com is willing to provide five of its UltraHD video contents for the purpose of development, testing and promulgation of video coding standards. This document gives a description of these video contents and compression performance with HEVC.

# Description of test material

The test set is composed of five UltraHD (3840x2160) sequences in RGB 4:4:4 10 bits linear DPX format or YUV 4:4:4 10 bits format. Four of them are shot at one frame rate and the double frame rate with two different cameras fixed on a same rig. Therefore the two versions of the same content at different frame rate are not exactly the same in terms of framing.

| Sequence | Camera | Frame rate | Aperture | Color primaries | Length |
| --- | --- | --- | --- | --- | --- |
| CatRobot | Sony F65 | 60 fps | 180° | 709 | 10 s |
| Drums | Red Dragon Epic | 50 fps | 180° | 709 | 10 s |
|  | Red Mysterium | 100 fps | 180° | 709 | 10 s |
| FreeSardines | Red Dragon Epic | 60 fps | 180° | 709 | 10 s |
|  | Red Dragon Epic | 120 fps | 180° | 709 | 10 s |
| Katana | Red Dragon Epic | 50 fps | 180° | 709 | 10 s |
|  | Red Mysterium | 100 fps | 90° | 709 | 10 s |
| Rowing | Red Dragon Epic | 60 fps | 180° | 709 | 10 s |
|  | Red Dragon Epic | 120 fps | 180° | 709 | 10 s |

Table 1 – Test set

They have been shot for the purpose of compression research. They cover a large variety of content with studio or external shooting and contain challenging video characteristics for codecs, like:

- camera pan,

- non-rigid object motion, irregular motion trajectories,

- challenging texture structures,

- saturated colour,

- large contrast/dynamic range.

| CatRobot |  |
| --- | --- |
| Drums |  |
| FreeSardines |  |
| Katana |  |
| Rowing |  |

# Compression characteristics

The sequences described in the Table 1 have been first converted from DPX to YUV 4:4:4 10 bits format, using FFMPEG [1] with the following parameters:

./FFMPEG/ffmpeg -start_number xx –i name.dpx -pix_fmt yuv444p10le

-frames xxx name.yuv

They have been converted to YUV 4:2:0 10 bits format, using ChromaConvert tool of HDRTools V0.9  toolset [2].

They have been encoded using the HEVC HM 16.5 reference software [3] in RA10 and LDB10 configurations. For each sequence and configuration, four QP values 22-27-32-37 have been tested. We have used the configuration files provided on Aachen server, with deletion of “DeblockingFilterControlPresent” parameter in order to make it compliant to HM-16.5.

The intra period in RA configuration has been set to :

- 48 for 50 fps content,

- 64 for 60 fps content,

- 96 for 100 fps content,

- 120 for 120 fps content.

The R-D curves are respectively provided in Figure 1 and 2 for LDB10 and RA10 configurations. These curves represent in X-axis the log(Bitrate) value and in Y-axis the Y-PSNR value. The interpolated curves have been computed as described in [4].

Figure 1: Low-Delay B 10bit

Figure 2: Random-Access 10bit

We can observe that the proposed content offer a large range of encoding complexity, with a quality varying from 48 to 32dB on the bitrates test range. The increase of distortion in function of QP is significant.

The viewing of sequences shows strong visible artefacts for all the sequences at QP 37:

- CatRobot: blocks on flat areas (dark table, shadows of scarves on the wall), visible degradations on scarves texture and on flying sheets of the book,

- Drums: visible artefacts on the wall behind the metallic fence,

- FreeSardines: blocks on the waterfall, encoding activity on the rocks,

- Katana: encoding activity on the wall, loss of details on the face, ringing effects on the face contours,

- Rowing: visible artefacts on coxswain contours.

# Conclusion

We think that the proposed test material has challenging characteristics for video compression and leave place for improvement in the next video coding development. We are pleased to make this test material available for the purpose of development, testing and promulgation of video coding standards.

# Allowance of usage

Please note that this test material remains the property of the Institute of Research & Technology b<>com and is solely provided “as is" to ITU-T SG16 Q6 and ISO/IEC JTC1/SC29/WG11, on a non-transferable and non-exclusive basis, and solely for the purpose of development, testing and promulgation of video coding standards. Any disclosure to third parties, exhibition, broadcast, distribution, public display or other exploitation of the test material is strictly prohibited.

# References

- https://www.ffmpeg.org/

- http://wg11.sc29.org/svn/repos/Explorations/XYZ/HDRTools/tags/0.9

- HM-16.5, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.5/

- G. Bjøntegaard, “VCEG-M33: Calculation of Average PSNR Differences Between RD-Curves“, April 2001.