| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0094 |
| --- | --- |

| Title: | Advanced SCC tool using Pseudo 2D String Matching (P2SM) integrated into HM16.6 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kailun Zhou, Liping Zhao, Tao Lin 1239 Siping Ave, Shanghai 200092, China | Tel: Email: | lintao@tongji.edu.cn |
| Source: | Tongji University |  |  |

_____________________________

# Abstract

This document proposes an SCC tool using Pseudo 2D String Matching (P2SM) integrated into HM16.6. Using SCM5.2 as anchor, the coding results for YUV TGM AI lossy coding are reported as -4.7%, -4.2%, -4.2%. Encoding time ratio is 84%, that is 16% decrease from SCM5.2.

# Introduction

The most important characteristic of screen content is its diversity and comprehensiveness. There are usually many many matching patterns in typical screen content. The shapes and colors of matching patterns are also extremely diverse and comprehensive. For example, the shapes range from simple ones such as squares, rectangles, triangles, circles, polygons, crescents, diamonds of different sizes to complex ones such as all kinds of mathematic curves, geometric shapes, and fonts of different typeface, size, weight, slope, width, and special effect.

IBC is efficient to code matching patterns of a few fixed sizes with rectangle or square shapes in a picture, but is not flexible enough to code matching patterns of varying sizes from a few pixels to a few thousands of pixels with a variety of shapes. The palette coding technique (PLT) can code matching patterns inside a CU using two types of intra-CU pixel-index string matching, but it cannot exploit non-local matching patterns out of a CU. Therefore, IBC and PLT only partially explore the pattern matchability in screen content.

On the other hand, pseudo 2D string matching (P2SM) technique can fully explore the pattern matchability in screen content by using flexible pseudo 2D strings as basic matching unit and allowing reference strings in any pre-determined valid region inside a picture.

# Proposal

The posposed P2SM has 3 submodes:

1) Normal mode which is equivalent to old ISC tool [3] and PPC tool [4].

2) SRB (Secondary Reference Buffer) only (SRBonly) mode which has all advantages of PLT and more.

3) Big string (BS) mode which has all advantages of IBC and more. In BS mode, a CU has only one string or two equal-size strings in either horizontal scan or vertical scan. String matching residuals are coded in BS mode.

The reference string range is full frame. String size (the number of pixel) is not limited in 2 CTUs (the current CTU and the left CTU). String size must be equal to or greater than 32 pixels outside of 2 CTUs. Therefore, the memory bandwidth demand of P2SM is almost the same as IBC. Further more, as a very trivial and common practice in IC design, 2 CTUs of reference buffer are implemented in SRAM. Therefore, the memory bandwidth is never an issue in P2SM.

# Experimental Results

CTC [2] is used and the anchor is HM16.6+SCM5.2 [1].

| Anchor: HM16.6+SCM5.2 FF IBC | All Intra |  |  |
| --- | --- | --- | --- |
| Tested HM16.6+P2SM | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | #VALUE! | #VALUE! | #VALUE! |
| RGB, mixed content, 1440p & 1080p | #VALUE! | #VALUE! | #VALUE! |
| RGB, Animation, 720p | #VALUE! | #VALUE! | #VALUE! |
| RGB, camera captured, 1080p | #VALUE! | #VALUE! | #VALUE! |
| YUV, text & graphics with motion, 1080p & 720p | -4.7% | -4.2% | -4.2% |
| YUV, mixed content, 1440p & 1080p | -1.4% | -1.0% | -1.5% |
| YUV, Animation, 720p | #VALUE! | #VALUE! | #VALUE! |
| YUV, camera captured, 1080p | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 84% |  |  |
| Dec Time[%] | 98% |  |  |

See attached “JCTVC-V0094-SCM5.2_vs_HM16.6+P2SM_444_lossy.xlsm” for details.

# Conclusion

P2SM provides a single tool for SCC and outperforms HM16.6+SCM5.2 with a encoding time reduction. In particular, as shown in a related document [5], for screen texts rendered using anti-aliasing techniques such as Clear Type to achieve clear and smooth display on an LCD monitor, P2SM can obtain 39% BD-rate reduction using SCM5.2 as anchor. Furthermore, also shown in [5], for a translucently blended screen content sequence, P2SM can obtain 25% BD-rate reduction using SCM5.2 as anchor.

Text rendering using anti-aliasing techniques such as Clear Type and translucently blended image display such as Aero visual effects are two of the most important and widely used computer raphics display features. They provide excellent user visual experiences, but also make SCC much difficult and increase bitrate significantly in cloud-computing. That is the reason why people usually turn off Clear Type and Aero in remote desktop.

Therefore, we would recommend:

- To evaluate the proposed P2SM tool for possible adoption. Considering anti-aliasing rendered clear and smooth text display is probably the most important aspect of computer display and SCC, we believe P2SM will significantly enhance the future market acceptance and expand the total available market of HEVC and beyond.

- To cerate a branch of JEM for P2SM software.

Adoption of P2SM is the only way to make SCC standard a real superior and competitive standard product in the real market of emerging cloud-computing and interactive TV.

# References

- JCT-VC, “HM-16.6+SCM-5.2,” https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.6+SCM-5.2/

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” JCTVC-U1015, Warsaw, June. 2015.

- Liping Zhao, Kailun Zhou, Shuhui Wang, and Tao Lin, “Non-CE3: Improvement on Intra String Copy,” JCT-VC Doc JCTVC-T0139, Feb. 2015.

- Tao Lin, Kailun Zhou, Liping Zhao, “Non-CE1: Enhancement to palette coding by palette with pixel copy (PPC) coding,” JCTVC-U0116, Warsaw, June. 2015.

- Liping Zhao, Jing Guo, Tao Lin, “Significantly improving coding performance of Clear Type texts and translucently blended screen content by P2SM,” JCT-VC Doc JCTVC-V0095, Oct. 2015.

# Patent rights declaration(s)

Tongji University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).