| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0095 |
| --- | --- |

| Title: | Significantly improving coding performance of Clear Type texts and translucently blended screen content by P2SM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liping Zhao, Jing Guo, Tao Lin 1239 Siping Ave, Shanghai 200092, China | Tel: Email: | lintao@tongji.edu.cn |
| Source: | Tongji University |  |  |

_____________________________

# Abstract

This document reports coding performance improvement achieved by Pseudo 2D String Matching (P2SM) technique for a class of screen content including Clear Type texts and translucently blended screen content, proposes to put special emphasis on intra picture coding performance when evaluating SCC tools, and proposes to re-evaluate P2SM or its simplified version ISC. Using SCM5.2 as anchor, it is reported that P2SM can achieve 39% BD-rate reduction for a typical screen snapshot with clear and smooth texts rendered by Clear Type, a widely used anti-aliasing technique for LCD display.

# Introduction

In a majority of SCC applications such as cloud computing, cloud TV, cloud-mobile computing, remote desktop, human-computer interaction is involved and crisp system response time (such as the time from a mouse move by a user to the curor move seen on the screen by the user) is absoluted required. In such applications, coding latency should be less than 1-2 frame period. Therefore, peak intra-picture bitrate is the most important factor to achieve excellent user experiences.

A very large percentage of typical screen content is text. Clear and smooth text display is a top requirement. Therefore, text display oriented anti-aliasing techniques such as Clear Type have been developed and widely applied in text rendering to achieve clear and smooth text display on an LCD screen. The following two screen captures have the same texts. The left one is rendered with Clear Type off and the right one is rendered with Clear Type on. The right one looks much better than the left one.

Part of the two screen captures are magnified 8 times and shown in the following two images. A 8x8 CU is shown in red box. It is obvious that when Clear Type is off , the CU can be coded very efficiently by PLT using a few index runs. On the other hand, when Clear Type is on, PLT needs to use many very short runs to code the CU and is very inefficient, while P2SM can easily find two reference strings, one from a nearby “9” and the other from a nearby “8”, to code the CU very efficiently.

Another class of commonly seen screen content that is hard to code is translucently blended sharp and crisp images such as texts, lines, and squares. One example is a bitstream analyzer display window that shows three layers of images translucently blended: 1) the original image being coded, 2) the CU structures of the coded bitstream, 3) coding modes such as IBC or PLT or Intra in different colors. The following is a snapshot of a bitstream analyzer display window.

.

As illustrated in these two types of screen content, there are usually many many matching patterns in typical screen content. The shapes and colors of matching patterns are extremely diverse and comprehensive. The shapes range from simple ones such as squares, rectangles, triangles, circles, polygons, crescents, diamonds of different sizes to complex ones such as all kinds of mathematic curves, geometric shapes, and fonts of different typeface, size, weight, slope, width, and special effect.

P2SM [5] can fully explore the pattern matchability in screen content by using flexible pseudo 2D strings as basic matching unit and allowing reference strings in any pre-determined valid region inside a picture

# Proposal

- Since peak intra-picture bitrate is one of the most crucial factors to achieve excellent:user experiences in human-computer interaction, it is proposed to put special emphasis on intra-picture, i.e. single frame (snapshot) coding performance when evaluating SCC tools.

- Since P2SM can achieve significant coding performance improvement (up to 40%) for at least a class of typical, common, and very important screen content such as Clear Type texts and translucently blended screen content, it is proposed to re-evaluate P2SM [5] or its simplified version ISC [3] or PPC [4].

# Experimental Results

All experiments use the test configuration and encoder settings of [2]. Anchor is HM16.6+SCM5.2 FF IBC. Tested is HM16.6+P2SM.

We coded two 1920x1080 screen snapshots:

1) a snapshot of spreadsheet with Clear Type off, see BDrateSpreadsheet_ClearTypeOff.bmp/rgb/yuv

2) a snapshot of spreadsheet with Clear Type on, see BDrateSpreadsheet_ClearTypeOn.bmp/rgb/yuv

The coding results are as follows.

|  |  | HM16.6+SCM5.2 |  |  |  | HM16.6+P2SM |  |  |  | BD-rate(anchor_t1) (piecewise cubic) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RGB | QPISlice | kbps | G/Y psnr | B/U psnr | R/V psnr | kbps | G/Y psnr | B/U psnr | R/V psnr | G/Y | B/U | R/V |
| Clear | 22 | 42988.80 | 55.85 | 54.58 | 54.26 | 23219.52 | 56.39 | 54.93 | 55.25 | -39.0% | -39.0% | -39.4% |
| Type | 27 | 39561.12 | 48.76 | 48.20 | 47.47 | 23030.40 | 49.67 | 48.80 | 47.69 |  |  |  |
| on | 32 | 33651.36 | 43.16 | 39.57 | 39.88 | 22401.12 | 44.61 | 42.52 | 41.56 |  |  |  |
| rgb | 37 | 26773.44 | 37.05 | 32.82 | 32.65 | 21138.24 | 38.57 | 35.17 | 36.07 |  |  |  |
| Clear | 22 | 18322.08 | 64.90 | 64.13 | 64.22 | 16665.60 | 67.27 | 65.09 | 65.06 | -9.7% | -9.6% | -9.6% |
| Type | 27 | 18225.60 | 59.56 | 58.30 | 59.03 | 16451.52 | 61.88 | 59.69 | 59.68 |  |  |  |
| off | 32 | 18369.60 | 53.61 | 51.31 | 52.48 | 16422.72 | 56.78 | 55.04 | 55.31 |  |  |  |
| rgb | 37 | 17861.76 | 48.05 | 46.22 | 46.44 | 16811.52 | 51.64 | 49.18 | 49.35 |  |  |  |
| YUV |  |  |  |  |  |  |  |  |  |  |  |  |
| Clear | 22 | 42533.28 | 54.09 | 54.42 | 53.25 | 22932.96 | 54.31 | 54.72 | 53.19 | -36.5% | -37.6% | -37.3% |
| Type | 27 | 37433.28 | 47.97 | 46.62 | 46.24 | 22209.12 | 47.98 | 46.24 | 46.65 |  |  |  |
| on | 32 | 31604.16 | 42.40 | 40.09 | 39.97 | 21205.92 | 43.31 | 40.17 | 39.47 |  |  |  |
| yuv | 37 | 24961.44 | 35.76 | 34.71 | 34.86 | 19398.24 | 35.68 | 34.77 | 34.70 |  |  |  |
| Clear | 22 | 18143.52 | 63.73 | 64.55 | 65.36 | 16947.84 | 65.92 | 67.04 | 66.82 | -4.7% | -5.0% | -4.8% |
| Type | 27 | 18092.16 | 57.86 | 57.45 | 59.41 | 17120.16 | 60.04 | 60.48 | 60.28 |  |  |  |
| off | 32 | 17691.84 | 52.30 | 53.54 | 53.37 | 17069.28 | 53.64 | 55.40 | 55.54 |  |  |  |
| yuv | 37 | 17564.16 | 46.56 | 48.76 | 47.55 | 16916.64 | 48.03 | 51.27 | 49.08 |  |  |  |

It can be seen that in Clear Type off case, SCM5.2 can compress the picture well. The resulting bitrate (at 60fps) is about 18Mbps and the PSNR is close to 65dB for qp=22. On the other hand, in Clear Type on case,  SCM5.2 is inefficient. The resulting bitrate is about 43Mbps and the PSNR is around 55dB for qp=22. In contrast, P2SM can do a much better job, especially in Clear Type on case. The BD-rate reduction is 39% and 37% for rgb and yuv, respectively, in Clear Type on case. P2SM also achieves BD-rate reduction of 10% and 5% for rgb and yuv, respectively, in Clear Type off case

We also capture the following two test sequences:

- ClearTypeSpreadsheet. The content is three BD-rate calculation spreadsheet windows side by side overlapped by RD-curve charts with a series of common actions like resizing and moving windows and charts.

- BitstreamAnalyzer. The content is a bitstream analyzer display window that shows three layers of images translucently blended: 1) the original image being coded, 2) the CU structures of the coded bitstream, 3) coding modes such as IBC or PLT or Intra in different colors. The following is a snapshot of a bitstream analyzer display window.

The coding results are as follows.

|  |  | Anchor (HM16.6+SCM5.2) |  |  |  | Tested1 (HM16.6+P2SM) |  |  |  | BD-rate(anchor_t1) (piecewise cubic) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QPISlice | kbps | G/Y psnr | B/U psnr | R/V psnr | kbps | G/Y psnr | B/U psnr | R/V psnr | G/Y | B/U | R/V |
| ClearTypeSpreadsheet_1920x1080_30_8bit_rgb | 22 | 21796.54 | 55.96 | 54.83 | 54.90 | 12623.31 | 57.18 | 55.49 | 55.61 | -35.4% | -35.6% | -35.7% |
|  | 27 | 19947.49 | 48.91 | 48.34 | 47.76 | 12375.97 | 50.39 | 48.97 | 48.40 |  |  |  |
|  | 32 | 17031.30 | 43.09 | 39.74 | 40.02 | 11896.68 | 44.52 | 42.45 | 41.85 |  |  |  |
|  | 37 | 13548.83 | 37.14 | 33.09 | 32.85 | 11050.06 | 38.34 | 35.28 | 35.70 |  |  |  |
| ClearTypeSpreadsheet_1920x1080_30_8bit_yuv | 22 | 21498.13 | 54.21 | 54.64 | 53.46 | 12417.83 | 54.96 | 54.81 | 53.29 | -33.7% | -34.3% | -34.1% |
|  | 27 | 18974.96 | 47.89 | 46.79 | 46.21 | 11865.44 | 47.98 | 46.71 | 46.72 |  |  |  |
|  | 32 | 15916.55 | 42.19 | 40.43 | 40.16 | 11185.37 | 43.12 | 40.49 | 39.84 |  |  |  |
|  | 37 | 12584.67 | 35.12 | 34.93 | 34.67 | 10061.64 | 35.65 | 34.85 | 34.74 |  |  |  |
| BitstreamAnalyzer_1920x1080_60_8bit_rgb | 22 | 109898.49 | 54.32 | 52.55 | 52.18 | 79028.47 | 54.39 | 52.83 | 52.43 | -24.4% | -25.5% | -25.8% |
|  | 27 | 97088.61 | 49.02 | 47.63 | 47.35 | 70573.62 | 48.95 | 48.25 | 47.57 |  |  |  |
|  | 32 | 82217.59 | 43.29 | 41.54 | 41.43 | 63991.64 | 43.71 | 43.06 | 42.67 |  |  |  |
|  | 37 | 63253.20 | 37.34 | 34.25 | 33.93 | 56385.21 | 38.22 | 36.20 | 37.10 |  |  |  |
| BitstreamAnalyzer_1920x1080_60_8bit_yuv | 22 | 103904.31 | 54.56 | 53.38 | 52.41 | 73506.61 | 54.48 | 53.62 | 52.10 | -23.8% | -24.4% | -22.7% |
|  | 27 | 90888.02 | 49.22 | 47.51 | 47.25 | 66642.27 | 49.29 | 47.63 | 47.03 |  |  |  |
|  | 32 | 75440.01 | 42.64 | 41.85 | 42.20 | 58868.49 | 42.73 | 41.76 | 41.90 |  |  |  |
|  | 37 | 56478.22 | 35.94 | 36.83 | 35.68 | 48702.62 | 36.41 | 37.16 | 35.65 |  |  |  |

P2SM achieves about 35% and 25% BD-rate reduction for the two sequences, respectively.

# Conclusion

P2SM [5] provides a single tool for SCC and outperforms HM16.6+SCM5.2 with a encoding time reduction. In particular, for screen texts rendered using anti-aliasing techniques such as Clear Type to achieve clear and smooth display on an LCD monitor, P2SM can obtain 39% BD-rate reduction using SCM5.2 as anchor. Furthermore, for a translucently blended screen content sequence, P2SM can obtain 25% BD-rate reduction using SCM5.2 as anchor.

Text rendering using anti-aliasing techniques such as Clear Type and translucently blended image display such as Aero visual effects are two of the most important and widely used computer graphics display features. They provide excellent user visual experiences, but also make SCC much difficult and increase bitrate significantly in cloud-computing. That is the reason why people usually turn off Clear Type and Aero in remote desktop.

Coding latency has to be less than 1-2 frame period to achieve crisp system response time in cloud-computing applications such as remote desktop. So, peak intra-picture bitrate is the most important factor to achieve excellent user experiences in human-computer interaction.

Therefore, we would recommend:

- To put special emphasis on peak intra picture coding bitrate when evaluating SCC tools

- To evaluate P2SM [5] for adoption to SCC draft to make SCC a real superior and competitive standard product in the real market of emerging cloud-computing and interactive TV.

- To cerate a branch of JEM for P2SM software.

# References

- JCT-VC, “HM-16.6+SCM-5.2,” https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.6+SCM-5.2/

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” JCTVC-U1015, Warsaw, June. 2015.

- Liping Zhao, Kailun Zhou, Shuhui Wang, and Tao Lin, “Non-CE3: Improvement on Intra String Copy,” JCT-VC Doc JCTVC-T0139, Feb. 2015.

- Tao Lin, Kailun Zhou, Liping Zhao, “Non-CE1: Enhancement to palette coding by palette with pixel copy (PPC) coding,” JCTVC-U0116, Warsaw, June. 2015.

- Kailun Zhou, Liping Zhao, Tao Lin, “Advanced SCC tool using Pseudo 2D String Matching (P2SM) integrated into HM16.6,” JCT-VC Doc JCTVC-V0094, Oct. 2015.

# Patent rights declaration(s)

Tongji University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).