| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct 2015 | Document: JCTVC-V0096 |
| --- | --- |

| Title: | Status report of rate control on 4k videos |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jiangtao Wen Meiyuan Fang Ziyu Wen Tsinghua University Beijing, China | Email: | jtwen@tsinghua.edu.cn fmy13@mails.tsinghua.edu.cn wen-zy13@mails.tisnghua.edu.cn |
| Source: | Tsinghua University |  |  |

_____________________________

# Abstract

This document reports the results of rate control on 4k videos. It is noted that rate control on 4k videos underperforms the cqp mode with a similar bitrate by 12~15% on average and up to 25%.

# Introduction

Rate control is widely used with the development of the HEVC standard. And we performed it on 4k videos. The sequences that are used in our experiments are:

| Name | Resolution | Frame rate | #Frames |
| --- | --- | --- | --- |
| Cactus | 3840x2160 | 24 | 336 |
| Clownlogoed * | 3840x2160 | 30 | 640 |
| Coastguard | 3840x2160 | 24 | 240 |
| Cobra * | 3840x2160 | 30 | 353 |
| Foreman | 3840x2160 | 24 | 370 |
| Mobile | 3840x2160 | 24 | 355 |
| News | 3840x2160 | 24 | 257 |
| Susielogoed * | 3840x2160 | 30 | 588 |
| Suzie | 3840x2160 | 24 | 355 |

The videos marked with * are from Harmonic, others are from Elemental.

- Experimental Results

The experiments are performed based on HM-16.0. We use the bitrate of cqp mode (QP = 22, 27, 32, 37) as the target bitrate of the rate control mode. Table 1 shows the R-D performance with the Random Access Main configuration. Table 2 shows the R-D performance with the Low Delay Main configuration, and Table 3 shows the R-D performance with the Low Delay P Main configuration.

Table 1  BD-BR in the case of Random Access

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8 bit) | 5.0% | -2.2% | -0.6% |
| Class B | 4.3% | -0.8% | -2.1% |
| Class C | -0.5% | -5.4% | -5.2% |
| Class D | 1.1% | 0.3% | -0.4% |
| Class 4K | 12.8% | -4.6% | -2.8% |
| Overall | 6.2% | -2.9% | -2.5% |
|  | 5.9% | -2.6% | -2.1% |

Table 2  BD-BR in the case of Low Delay

|  | Low delay B Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8 bit) |  |  |  |
| Class B | 5.8% | -8.9% | -10.8% |
| Class C | -1.4% | -12.2% | -11.1% |
| Class D | -0.1% | -9.8% | -9.2% |
| Class 4K | 15.0% | -6.8% | -6.8% |
| Overall | 7.2% | -8.8% | -8.9% |
|  | 6.9% | -8.3% | -8.3% |

Table 3  BD-BR in the case of Low Delay P

|  | Low delay P Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8 bit) |  |  |  |
| Class B | 4.4% | -10.5% | -12.8% |
| Class C | -2.7% | -14.8% | -13.8% |
| Class D | -1.2% | -12.9% | -12.3% |
| Class 4K | 12.0% | -7.4% | -8.6% |
| Overall | 5.2% | -10.5% | -11.1% |
|  | 5.0% | -9.9% | -10.5% |