| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct. 2015 | Document: JCTVC-V0097 |
| --- | --- |

| Title: | A new SCC test sequence with ClearType text rendering for consideration |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jing Guo, Liping Zhao, Tao Lin 1239 Siping Ave, Shanghai 200092, China | Tel: Email: | lintao@tongji.edu.cn |
| Source: | Tongji University |  |  |

_____________________________

# Abstract

This contribution proposes a new test sequence for the SCC CTC. This sequence consists of content with three BD-rate calculation spreadsheet windows side by side overlapped by RD-curve charts with a series of common actions like resizing and moving windows and charts. It is reported that the content represents a typical and common SCC application scenario where a larg portion of the screen is texts rendered using anti-aliasing (sub-pixel rendering) techniques such as ClearType.This type of content exhibits very different coding characteristics when compared with the existing SCC test sequences.

# Introduction

The 100th frame of this new sequence is shown in Figure 1 below. This new sequence is available for review in this meeting.

Figure 1 100th frame of ClearTypeSpreadsheet, 1920x1080, 30 fps

The rgb version sequence name is ClearTypeSpreadsheet_1920x1080_30_8bit_rgb.rgb. The majority of content in this sequence shows numbers in spreadsheets. Three BD-rate calculation spreadsheet windows are side by side and overlapped by RD-curve charts with a series of common actions like resizing and moving windows and charts.

This sequence has 300 frames. The original RGB sequence was generated by capturing the raw 24-bit RGB signals from a computer screen to hard drive. The screen display format was set to 1920x1080 at 30fps.

# Coding characteristics evaluation

A very large percentage of typical screen content is text. Clear and smooth text display is a top requirement. Therefore, text display oriented anti-aliasing (sub-pixel rendering) techniques such as Clear Type have been developed and widely applied in text rendering to achieve clear and smooth text display on an LCD screen. The following two screen captures have the same texts. The left one is rendered with Clear Type off and the right one is rendered with Clear Type on. The right one looks much better than the left one.

Part of the two screen captures are magnified 8 times and shown in the following two images. A 8x8 CU is shown in red box. It is obvious that when Clear Type is off , the CU can be coded very efficiently by PLT using a few index runs. On the other hand, when Clear Type is on, PLT needs to use many very short runs to code the CU and is very inefficient. Therefore, new coding tools are needed to code this type of screen content efficiently.

We coded two 1920x1080 screen snapshots captured from the same scenario and using the same environment and setting as ClearTypeSpreadsheet_1920x1080_30_8bit_rgb.rgb

1) a snapshot of spreadsheet with Clear Type off

2) a snapshot of spreadsheet with Clear Type on

The coding results using HM16.6+SCM5.2 are as follows.

| QP | ClearType off |  |  |  | ClearType On |  |  |  | BD-rate |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22 | 18322.08 | 64.90 | 64.13 | 64.22 | 42988.80 | 55.85 | 54.58 | 54.26 | 126.98% | 123.69% | 125.55% |
| 27 | 18225.60 | 59.56 | 58.30 | 59.03 | 39561.12 | 48.76 | 48.20 | 47.47 |  |  |  |
| 32 | 18369.60 | 53.61 | 51.31 | 52.48 | 33651.36 | 43.16 | 39.57 | 39.88 |  |  |  |
| 37 | 17861.76 | 48.05 | 46.22 | 46.44 | 26773.44 | 37.05 | 32.82 | 32.65 |  |  |  |

It can be seen that when turing on Clear Type, BD-rate increases about 125%, i.e., bitrate is more than double..

Therefore, ClearType like text rendering, which is a very common practice in modern computing devices including smartphones and tablets, significantly increases bandwidth demand in cloud-computing and remote desktop like applications. This is a very challenging issue that advanced SCC technology needs to solve.

The proposed test sequence is intended to provide a proper means for evaluating ClearType coding performance of SCC tools in HEVC and beyond.

# Recommendation

It is recommended to add this new sequence in SCC CTC.

# References

- JCT-VC, “HM-16.6+SCM-5.2,” https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.6+SCM-5.2/

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” JCTVC-U1015, Warsaw, June. 2015.

# Conditions of Use – Copyright and Release Notes

The sequence has been generated by Tongji University, and all intellectual property rights remain with Tongji University. The material shall be used for research, development, experiment, publication, and demo of video coding technologies purposes only under the following conditions of use: 1) do not alter any picture of the test sequences except to resize and/or clip it; 2) do not use only portions of any picture alone; 3) any publication or demo using the test sequence must mention or display the acknowledgement of “by courtesy of Tongji University”.

This copyright and permission notice shall be duplicated whenever the data are copied or distributed.

The material cannot be distributed with charge. Tongji University makes no warranties with respect to the material and expressly disclaims any warranties regarding its fitness for any purpose. Unless the above conditions are agreed to by the recipient, no permission is granted for any use, copying, modification, and distribution of the data. By using the sequence the user agrees to the conditions of this copyright and disclaimer.

# Availability of the Sequences

Tongji University agrees to donate the test sequence to the video coding community and to upload the test sequence to JCTVC website under the Condition of Use set forth on 5 of this document. Contact email: vlsi@tongji.edu.cn.