| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Geneva, CH, 15–21 Oct 2015 | Document: JCTVC-V0099 |
| --- | --- |

| Title: | BVI_Texture UHD 120fps test sequences for HEVC and beyond |  |  |
| --- | --- | --- | --- |
| Status: | input to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Miltiadis Alexios Papadopoulos, Fan Zhang, Dimitris Agrafiotis, David Bull Visual Information Laboratory, University of Bristol, UK Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Email: Email: | miltiadis.alexios.papadopoulos@bristol.ac.uk ohm@ient.rwth-aachen.de |
| Source: |  |  |  |

_____________________________

# Abstract

The Visual Information Laboratory of UoB, in coordinated effort under the Marie Skłodowska-Curie Actions ITN project PROVISION, is offering this contribution consisting of 20 UHD (3840x2160) high frame rate (120fps) source sequences of high static and dynamic texture content to MPEG and VCEG as testing material for evaluating and developing current state-of-the-art (i.e. HEVC) and future video coding technologies. The sequences are part of the BVI_Texture database as presented for ICIP 2015 [1]. This document includes information about the sequences and supports their suitability for evaluating the efficiency of coding standards.

# Overview of sequences

Table 1 lists specifications of the sequences we are providing. All the sequences were shot on a Red EPIC Mysterium-X camera in 3840 x 2160 resolutioon, 120 fps, using file format REDcode 10:1 with a fully open shutter (360°). They were color graded on the REDcolor3 colorspace, REDgamma3 gamma space and exported as TIFF 16bit files using the software REDCINE-X PROFESSIONAL Build 33.1. Finally, they were subsampled to 4:2:0 and packed as .yuv files using ffmpeg version N-67742-g3f07dd6 by enabling the flag –pix_fmts yuv420p. The sequences can also be made available as BT.709 4:4:4, 10 bit.

| Resolution | 3840 x 2160 |
| --- | --- |
| Bit Depth | 8 |
| Frame rate | 120 fps |
| Length | 1200 frames |
| Colour format | BT. 601 YCbCr 4:2:0 |

Table 1 Specification of proposed test sequences

Table 2 provides the name of the sequence, a sample screenshot and a brief description of the sequence’s content. The sequences are divided into 3 classes indicated by the letters A, B and C, offering sequences of static, mixed and dynamic textured content. In all of the sequences but sequence A4 the camera is stationary in terms of translation. Camera motion is experienced through panning and tilting of the tripod’s head. A PSNR vs Bitrate chart is also provided for every sequence, after encoding a down-sampled version of it in full-HD at 60fps with HEVC HM16.2, low-delay configuration, QPoffset=0 and QP=5,10,15,20,27,32,37,42,45,50.

| ID | Sequence | Screenshot | Description of content | PSNR vs Bitrate on HM 16.2 |
| --- | --- | --- | --- | --- |
| A1 | Bookcase |  | The sequence exhibits fast and random panning and tilting of the camera in front of a bookcase. It combines high temporal and spatial information while featuring a lot of camera motion blurring. |  |
| A2 | CarpetCircleFast |  | A carpet of neutral colors covers the whole frame in an attempt to create a scene consisting entirely of one homogenous static textured surface. The camera is rapidly rotated on the pitch and yaw axis, following a circular pattern. |  |
| A3 | CarpetPanAverage |  | Same as A2 except for the cameras rotation, which is limited to panning sideways at an average speed. |  |
| A4 | CarpetSlowTrans |  | The prop used is the same as A2 and A3. The color grading and lighting is slightly different, the camera is placed much closer to the carpet and it is slowly translated to the right while fixed on a horizontal slider. This is the only sequence where the camera rig experiences translation. |  |
| A5 | PaintingTilting |  | The sequence includes a shot of a fake painting of what appears to be a field of flowers and other plants. It is the highest indexed sequence that the database offers in terms of spatial information. The camera is tilting slowly downwards. |  |
| A6 | PaperStatic |  | This is a static shot of a piece of thin yellow paper being blown by a fan operating outside of the frame. This sequence was composed while trying to create material of low spatial and temporal information, with tiny bends and reflections on the paper itself. |  |
| A7 | PondDragonflies |  | This is a natural colorful scene of a small pond filled with water lilies, grass and flying blue dragonflies. The camera is stationary. There is apparent movement in the foreground grass and the dragonflies passing by. |  |
| A8 | Squirrel |  | This sequence consists mostly of green grass and bushes. At the middle of the frame there is a small squirrel. The camera is stationary. |  |
| B1 | BallUnderWater |  | This sequence was shot underwater using a water tank with some light shining from the top. Inside the water a red ball gets tossed around by a hand, which disturbs the calm surface of the water, creating diagonal streams of light. The camera is stationary. |  |
| B2 | BricksBushesStatic |  | Bricked structures are dominant in this sequence with a few bushes covering them while a gentle wind blows on a sunny day. The camera is stationary. |  |
| B3 | BricksLeaves |  | On this part of a bricked wall there is a sudden avalanche of fallen leaves. The camera is stationary. |  |
| B4 | Flowers2 |  | In this shot, the foreground is nicely separated from the background, which is mostly covered by greenery. In the foreground, colorful flowers are shuffled around by the wind. The camera is stationary. |  |
| B5 | PlasmaFree |  | This shot combines the dynamic texture of a plasma toy ball in front of a completely black static background. The camera is stationary. |  |
| B6 | TreeWills |  | This sequence features a great tree in front of Wills memorial building in Bristol, while a strong wind moves its leaves around. The sky is covered with white clouds. Towards the end of the clip, a passing cloud causes a dramatic change in lighting. The camera is stationary. |  |
| C1 | BubblesClear |  | This underwater shot exhibits air bubbles rising in a side lighted water tank. The camera is stationary. |  |
| C2 | CalmingWater |  | This shot of a small pond is dominated by water gradually reaching its calm state. The camera is stationary. |  |
| C3 | DropsOnWater |  | Unlike C2, this shot of the pond shows the calmness of the muddy waters being disturbed by drops of water. The camera is stationary. |  |
| C4 | LampLeaves |  | This shot shows part of a bricked wall covered in thick bushes, being shifted by a strong wind. The wind also bends slightly a nearby lamp post. The camera is stationary. |  |
| C5 | SmokeClear |  | A thick cloud of steam covers most of the frame in this otherwise dark framed sequence. The camera is stationary. |  |
| C6 | Sparkler |  | A sparkler is lit outside in front of the camera during the night. The camera is tilting slowly downwards to keep up with the sparkler’s rate of burning. |  |

Table 2 Description of sequence attributes and sample screenshots

The coverage of the feature descriptors SI/TI (Spatial Information/Temporal Information) for the source sequences in Table 2 using their full-HD 60fps down-sampled version is illustrated in Figure 1 below:

Figure 1 SI/TI characterization according to [2] (left) and [3] (right)

- Usage terms

UoB is willing to offer the sequences under the following terms:

We are however open to agree with other terms allowing usage for non-commercial purposes, if desirable within a unified test set.

Individuals and organizations extracting sequence from this archive agree that the sequence and all intellectual property rights therein remain the property of the Visual Information Laboratory of University of Bristol, UK. This material may only be used for the purpose of developing, testing and promulgating technology standards, and for scientific purposes. This material cannot be distributed with charge. The owner makes no warranties with respect to the material and expressly disclaims any warranties regarding its fitness for any purpose.

# References

| [1] | M. A. Papadopoulos, F. Zhang, D. Agrafiotis and D. Bull, "A video texture database for perceptual compression and quality assessment," in ICIP, Quebec, 2015. |
| --- | --- |
| [2] | P910, ITU-T Recommendation ITU-R, "Subjective video quality assessment methods for multimedia applications," 1999. |
| [3] | S. Winkler, "Analysis of public image and video databases for quality assessment," Selected Topics in Signal Processing, IEEE Journal of, vol. 6, no. 6, pp. 616--625, 2012. |