| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0012 |
| --- | --- |

| Title: | JCT-VC AHG report: SHVC software development (AHG12) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Ad-hoc group report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Yong He G. Barroux | Email: | vseregin@qti.qualcomm.com Yong.He@InterDigital.com guillaume.b@jp.fujitsu.com |
| Source: | AHG chairs |  |  |

_____________________________

# Abstract

This report summarizes activities of the AHG12 on SHVC software development between 22th and 23th JCT-VC meetings.

# Introduction

| SHVC software development (AHG12) (jct-vc@lists.rwth-aachen.de) - Prepare SHM 11.x software (based on HM 16.x) for experimentation. - Generate anchors and templates based on common test conditions. - Discuss and identify additional issues related to SHVC software. | V. Seregin, Y. He, G. Barroux (co-chairs) | N |
| --- | --- | --- |

The latest software version is SHM-11.0.

SHM software can be downloaded at https://hevc.hhi.fraunhofer.de/svn/svn_SHVCSoftware/tags/

The software issues can be reported using bug tracker https://hevc.hhi.fraunhofer.de/trac/shvc

# Software versions

The latest version is SHM-11.0 and it was released with JCTVC-V1013 for DAM.

SHM-11.0 is based on HM-16.7 and includes newly adopted profiles for scalable monochrome and 4:4:4 chroma colour format coding. The following software improvements have been additionally made:

- Fixes for tickets ## 90, 93, 94, 96, 97

- Reduce amount of software changes relative to HM for potential software merging

After adoption of the new profiles, SHM software supports higher than 10 bitdepth input. For such high bitdepth, macro RExt__HIGH_BIT_DEPTH_SUPPORT should be enabled for better performance.

Anchor data and templates have been generated based on common test conditions JCTVC-Q1009 and attached to this report.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall (Test vs single layer) | 12.8% | 14.8% | 14.6% | 10.5% | 9.7% | 9.3% |  |  |  |
| Overall (Ref vs single layer) | 12.8% | 14.8% | 14.5% | 10.5% | 9.7% | 9.3% |  |  |  |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 98.7% |  |  | 87.8% |  |  |  |  |  |
| Dec Time[%] | 85.0% |  |  | 77.7% |  |  |  |  |  |
| Enc Mem[%] | 99.5% |  |  | 100.9% |  |  |  |  |  |
| BL Match | Not matched |  |  | Not matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.0% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.1% | 0.0% |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.1% | 0.0% |
| Overall (Test vs single layer) | 18.9% | 31.1% | 32.0% | 15.9% | 26.3% | 28.7% | 14.3% | 29.8% | 34.4% |
| Overall (Ref vs single layer) | 19.0% | 31.0% | 31.9% | 16.0% | 26.3% | 28.5% | 14.3% | 29.8% | 34.4% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 102.3% |  |  | 100.5% |  |  | 104.5% |  |  |
| Dec Time[%] | 87.2% |  |  | 83.7% |  |  | 93.5% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.0% |  |  | 100.0% |  |  |
| BL Match | Not matched |  |  | Not matched |  |  | Not matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  | 0.0% | -0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% | 0.1% | 0.1% | 0.2% |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% | 0.1% | 0.0% | 0.1% |
| Overall (Test vs single layer) | 28.3% | 38.7% | 39.9% | 24.7% | 33.0% | 35.5% | 24.3% | 34.8% | 39.5% |
| Overall (Ref vs single layer) | 28.3% | 38.8% | 39.8% | 24.7% | 32.9% | 35.8% | 24.2% | 34.8% | 39.3% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% | 0.1% | 0.0% | 0.2% |
| Enc Time[%] | 107.1% |  |  | 101.4% |  |  | 103.1% |  |  |
| Dec Time[%] | 90.3% |  |  | 86.0% |  |  | 89.7% |  |  |
| Enc Mem[%] | 100.0% |  |  | 99.9% |  |  | 100.0% |  |  |
| BL Match | Not matched |  |  | Not matched |  |  | Not matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% |  |  |  | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | -0.3% |
| Overall (Test vs Ref) | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | -0.2% |
| Overall (Test vs single layer) | 26.5% | 37.8% | 39.4% | 22.8% | 32.6% | 35.4% | 23.4% | 34.7% | 39.1% |
| Overall (Ref vs single layer) | 26.5% | 37.7% | 39.3% | 22.8% | 32.5% | 35.6% | 23.4% | 34.6% | 39.4% |
| EL only (Test vs Ref) | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | -0.2% |
| Enc Time[%] | 101.7% |  |  | 98.0% |  |  | 98.9% |  |  |
| Dec Time[%] | 87.5% |  |  | 86.2% |  |  | 90.7% |  |  |
| Enc Mem[%] | 100.0% |  |  | 99.8% |  |  | 100.0% |  |  |
| BL Match | Not matched |  |  | Not matched |  |  | Not matched |  |  |

Table 1. SHM-11.0 against SHM-10.0 under common test conditions (HEVC base layer)

# Development plan and recommendations

- Continue to develop reference software based on SHM-11.0 and improve its quality.

- Fix open tickets.