| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0014 (also, ISO/IEC JTC1/SC29/WG11 mxxxxx) |
| --- | --- |

| Title: | Report of MPEG AHG on HDR and WCG Video Coding |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Information |  |  |  |
| Author(s) or Contact(s): | C. Fogg (Movielabs) E. Francois (Technicolor) W. Husak (Dolby) A. Luthra (ARRIS Group, Inc.) | Email: Email: Email: Email: | Chad.Fogg@gmail.com Edouard.Francois@technicolor.com wjh@dolby.com Ajay.Luthra@arris.com |  |
| Source: | AHG |  |  |  |

_____________________________

# Abstract

This document provides the report of the MPEG AHG on HDR and WCG Video Coding.

# Introduction

An adhoc group on HDR and WCG (N15622) was established in the 113th meeting of WG11 held in Geneva, Switzerland in October 2015 with the following mandates:

- Continue refinements of requirements and use cases

- Discuss and update the exploratory test model (H2M) and common test conditions

- Discuss and update the optimized anchors from CE1

- Review results of CEs by an interim meeting, and suggest refinements of the work plan towards the February meeting

An email reflector xyz@lists.uni-klu.ac.at was set up and hosted by Alpen-Adria-Universität Klagenfurt and industry experts were invited to join via http://lists.uni-klu.ac.at/mailman/listinfo/xyz and participate in this activity. As of February 3, 2016 there were 459 email addresses on this reflector’s list. Several emails were exchanged on various topics related to the mandates. Eight separate sub-reflectors were also set up associated with eight categories of the Core Experiments identified in Geneva (N15622).

The AHG had one face-to-face meeting in Vancouver from Jan 12-14, 2016.  As this activity of HDR/WCG video coding was planned to be made a part of JCTVC starting from February 19, 2016, the 2nd face to face meeting of the AHG, tentatively planned to start on February 19, 2016, was not held.

# Report of Activity since Vancouver Meeting

CE1

- Jan 22 - CE1 anchor package (software and xls results) redelivered to CE1 reflector by CE1 coordinators

- Jan 27 – updated CTCs package sent to xyz reflector by CTCs document authors

CE2

Activities related to ETM_RC reference software

- Jan 21 – delivery of ETM_RC_r1 software by ETM CE1 anchor package (software and xls results) to CE2 reflector by CE2 coordinators  - also uploaded to MPEG ftp site

- Jan 27 – delivery of ETM_RC_r1 bitrates matches results for reshape_setting 0 and 1, Standard Range

- Jan 28 - delivery of ETM_RC_r1 bitrates matches results for reshape_setting 2, with corresponding cfg files, Standard Range

- Jan 28 – cross-checking from Samsung of md5 sums from reshaper, for reshape_setting 0,1,2 done

CE2 experiments status

| Planed sub-test | Proponents | Soft | Results |
| --- | --- | --- | --- |
| CE2.a-1 luma ATF with LCS (m37245) | Qualcomm |  |  |
| CE2.a-2 reshaping (m37267) Jointly with CE2.a-2, CE2.c, CE2.d, CE2.e-3 | Dolby, Interdigital | Feb 02 | Feb 05 |
| CE2.e-3 automatic selection of ETM parameters |  |  |  |
| CE2.c Chroma QP offset (m37179) |  |  |  |
| CE2.d DeltaQP adjustment for Luma |  |  |  |
| CE2.b-1 Cross-channel modulation (m37088 and m37285) | Technicolor | Feb 02 | Feb 05 |
| CE2.a-3 adaptive transfer function (m37091) | Arris | Feb 04 | Feb 07 |
| CE2.b-2 SDR backward compatibility study (m37092) |  |  |  |
| CE2.c Chroma QP offset (m37179) |  |  |  |
| CE2.d DeltaQP adjustment for Luma |  |  |  |
| CE2.c Chroma QP offset (m37179) | Qualcomm | Feb 02 | Feb 05 |
| CE2.e-1 Enhancement of ETM | Qualcomm |  |  |
| CE2.e-2 harmonization of luma and chroma reshaping (m37332) | Arris | withdrawn | withdrawn |

CE3

- Jan 27 - c-code for HDR-VQM delivered by Samsung  uploaded on MPEG ftp site

CE5

CE5 experiments status

| Planed sub-test | Proponents | Soft | Results |
| --- | --- | --- | --- |
| CE5-3.1 - Y"u"v" | Philips | Feb 08 |  |
| CE5-3.2 - Y'FbFr | FastVDO | Feb 08 | Feb 08 |
| CE5-3.3 - Constant Luminance Approach | Qualcomm |  | Feb 08 |

CE6

- Feb 01 – updated CE6 status sent by CE6 coordinators

CE6 experiments status

| Planed sub-test | Proponents | Soft | Results |
| --- | --- | --- | --- |
| CE6-4.2 - Color Enhancement (m37072) | InterDigital | Feb 02 | Feb 02 |
| CE6-4.3 -Dynamic Range Adaptation and Re-saturation (m36266) | Philips | Feb 02 |  |
| CE6-4.6a -Technologies based upon m37285: m36266, when using an SEI message, described in m37285, | Philips | Feb 02 |  |
| CE6-4.6b -Technologies based upon m37285: the test model, with usage of SEI | Technicolor | Feb 02 | Feb 05 |
| CE6-4.7 -Gamut mapping | Technicolor | withdrawn | withdrawn |

CE7

- Feb 03 – EXR test picture sent by Arris (3 bars with same RGB ratios but different luminance) to CE7 reflector

- Feb 01 – md5sums for CE7.2a and CE7.2b sent by BBC to CE7 reflector

- updated CE6 status sent by CE6 coordinators

CE8

There has been a number of discussions on the xyz reflector regarding the conversion process required to convert an SDR 2020 content to SDR 709 in order to display it on SDR 709 display. A related document has been initiated (on Jan 24 – JCTVC-Wxxxx_conversionSDR709toSDR2020_v0.doc). An update based on these discussions will be delivered to bJCTVC San Diego meeting. The purpose of the document is to describe the recommended processes for BT.709 4:2:0 Y’CbCr to BT.2020 4:2:0 Y’CbCr conversion and for BT.2020 4:2:0 Y’CbCr to BT.709 4:2:0 Y’CbCr conversion, in the context of the JCTVC experiments on HDR with SDR backward compatibility.

# Report of Vancouver Meeting

Logistics:

01/12 meeting is at UBC.  01/13-14 meeting is at Telus downtown Vancouver.

Documents reviewed:

| JCTVC | MPEG | Source | Title |
| --- | --- | --- | --- |
| W0032 | m37539 | InterDigital | Encoder optimization for HDR/WCG coding |
| W0031 | m37536 | CE2 chairs | Description of the reshaper parameters derivation process in ETM reference software |
| W0024 | m37537 | CE4 chairs | Report of HDR CE4 |
| W0025 | m37538 | CE5 chairs | Report of HDR CE5 |
| W0034 | m37542 | Philips | Report on CE6 |
| W0033 | m37541 | BBC | Partial results for Core Experiment |
| W0101 | m37535 | BBC | Hybrid Log-Gamma HDR |
| W0035 | m37543 | Arris | Some observations on visual quality of Hybrid Log-Gamma (HLG) TF processed video (CE7) |
| ? | m37541 | BBC | Partial results for Core Experiment 7 |
| ? | ? | Technicolor | SDR quality evaluation of HLG |
| W0036 | m37602 | Philips | Report on CE7 xCheck and Viewing |

CE1

There was a teleconference on 12/01/15.  The anchor (super anchor) is currently at version V3.2.  The new anchors currently have adaptive QP, chroma QP offset and new rate points.  VUI signaling and deblocking filtering have been optimized and added.  V3.1 is the latest version with all technologies and V3.2 removes some non-essential data to make the file smaller.  The AHG will review the material for recommendation at this meeting.

There is a parallel fork with two technologies from V3.2 named V3.0(1) and V3.0(2).  For the interim meeting, we will compare the following:

V2.1 and V3.2 – V2.1 is a known quantity form the last face to face meeting.

V3.2 and V3.0(1) – fork from V3.0

V3.2 and V3.0(2) – fork with additional technologies

Current timeline is 01/13 is software is available and 01/20 is the date for completion of crosscheck and final decision.

During the viewing session on Tuesday, viewers of CE1 anchors favored the clips without chroma Qp offset proposed in w15794

After the AHG reviewed the content, the group has decided to use anchor V3.2.

CE2

CE2 ETM experiments can be categorized as follows:

| Mode | Use case | Luma | Chroma mode | Mode |
| --- | --- | --- | --- | --- |
| 0 | Single-layer HDR | 8 piecewise polynomial segments. | Intra plane | 1 piece-wise linear segment each for Cb, Cr. |
| 1 | Single-layer SDR backward compatible. |  | Cross plane | 8 piece-wise linear segments each for Cb,Cr. |
| 2 |  |  |  | 8 piece-wise linear segments shared by Cb,Cr. |

The CE2 software V8.1 (called ETM) was delivered on 12/12/15.  The description was delivered on 01/11/16 to the MPEG and JCT repositories as m37536 and JCTVC-W0031 respectively.  There are some bugs that are being examined that seem to be mismatches between Windows and Linux versions.  The images seem to be visually fine.

The current ETM software is expected to remain stable until the San Diego meeting.  There are expectations to deliver bug fixes in time for San Diego.  There is a desire to deliver an updated clean version of the ETM by 01/20/16.  The updated version will contain bug fixes, cleaned code and an automatic tuner of Mode 0.  Final delivery of CE2 software will occur by 02/01/16 although additional inputs can be made through contributions.  With respect to the automatic tuner, the discussion will occur via reflector and final decision at the San Diego JCT-VC meeting by 02/26/16.

There was a discussion on which pixel range to be used for the codings.  A short debate took place over whether video in future practice will be full range, “standard range” (aka. “narrow range” as it is called in SMPTE RP 2077, or “legal range” in other documents), or something closer to “SDI” range (also defined in SMPTE RP 2077).

There are several recommendations that define standard range from SMPTE, ITU and EBU.  There is a distinction that the bitstream has to be SMPTE range in order to enable backward compatibility modes although the reshaper output may not be bounded by this constraint.

For comparison to CE1, point B in the diagram above should be SMPTE range since the anchor is SMPTE range.  For backward compatibility, point A should be SMPTE range due to being compliant with current SDR distribution.  This leads the group to conclude that both A and B should be SMPTE range for backward compatibility (BC) testing.

For CE2 mode 0, the system will be tested against CE1.  For mode 1/2, the system will be test against CE1 for HDR and the same stream will be tested against two SDR references.  This will be further explored in CE8.

CE3

Several objective metric software packages have been completed including VQM, VIF and VDP2.  VQM executable is available via MPEG ftp.  Samsung has ported VQM to C++ and it has been inserted into HDRTools but not publicly released.  Samsung will kindly release the VQM code to AHG participants in order to verify performance.  Upon successful verification, Samsung will release the code widely to MPEG and VCEG.  UBC has indicated VIF C++ package would be released by the San Diego meeting.  Alan Chalmers has exported the VDP2 Matlab code to Windows and linux is expected to be completed by late January 2016.  It is expected to be released widely by the San Diego meeting.  The Dolby VDP2 implementation has been verified by Arris.  A request was made to Dolby to examine release of their C++ implementation ASAP.

EPFL has completed a subjective evaluation of the CFE anchors and will have an input contribution for the San Diego meeting.  EPFL will release the MOS scores for independent cross correlation to the objective metrics.  UBC also has MOS scores that they are willing to release.

CE4

The primary effort is to investigate whether reliable tests can be conducted on consumer monitors.  The interim meeting will have an initial viewing of content.  The group has not developed a list of parameters as yet.

Elena Alshina presented a document on Samsung’s experiences using a consumer monitor.  Using the J9500, there is a limit to the resolution, frame rate and bit rate combinations.  The data rate is limited to 80 Mbps through the USB port which is the only input for HDR at this point.  There were comments from the floor that the tests need to be repeatable and that people that have used the USB port had issues with frame rate reliability.  HDMI 2.0a support is expected by the San Diego meeting.

The CE4 coordinators expect the CE to last well beyond the next meeting.

CE5

The primary effort is to investigate alternative color matrices and sampling filters.  There are no results for this meeting but there will be results for the next meeting.

CE6

The primary effort is to investigate additional post processing.  There was substantial discussion on what the target is and how it might be implemented. The JCT Chairs clarified that the larger group is tasked with investigating technology rather than deciding where and how it is implemented into the standard.  The current feeling of the group is the boundary between CE2 and CE6 is the 4:2:0 and 4:4:4 interface point.  A thought that has come from the opening discussion is the group should think through the CE structure coming out of the San Diego meeting.

CE7

An overview of the motivation for development of HLG was given to the group by BBC.  There was substantial discussion on the impact and interactions of various elements of the OOTF.  BBC presented a document which was uploaded to the MPEG and JCT repositories as m37535 and JCTVC-W0037 respectively.  The utility of modifying the system gamma to change the peak brightness was discussed.  BBC reported on a subjective test where viewers modified the system gamma by a factor in order to adapt the signal from one peak brightness to another.  The value is dependent on the source and target peak brightness values.

Only 2-3% of TV programs go through a reference display environment.

Only one of 24 in-house studios surveyed by BBC followed BT 2035 lighting environment recommended practice.

Action item: BBC will send ITU-R WP6C contribution on system gamma design for monitors to the MPEG / JCT  HDR adhoc group.

Supplemental information relayed verbally by speaker during presentation of the slides in JCTVC-W0037 / m37537:

On slide 13 of m37535_r2.pdf (“OOTF reference must be Adapted for Other Environments and Displays) “:

- BBC adjusts on luminance (Y) rather than R,G,B, since luminance companding produces more natural looking results.

- Surround and peak brightness of display are essential parameters in the compading algorithm.

- In BBC’s experience, the OOTF does not depend upon content — only viewing environment (peak display luminance and surround/ambient luminance)

- Literature that Andrew Cotton surveyed suggests small specular highlights determine adaptation of the eye.

- scene-referred signals can be processed with or without artistic adjustments. The adjustments could even be performed in the camera.

- In traditional SDR, gamma 2.4 is applied in the display in dim environments, while gamma 2.2 in brighter environments.

- In a production environment, such as a studio set, the camera operator adjusts aperture to align image scanned off a grey card to map code 940 coming out of the camera to peak white, code 64 to black, mid-grey etc.

- Essential definition: display referred codes represent light emitted by the display.

- OOTF: only depends on display peak luminance and surrounding light.

- For SDR,  the presenter confirmed a statement made by an audience member that traditional SDR OOTF is the concatenation of BT.709 gamma 2.2 and BT.1886 gamma  2.4 with a,b parameters in BT.1886 representing viewing monitor. (BT.2035 is the viewing environment adjustment for traditional SDR).

slide 20 of m37535_r2.pdf (“For HLG, OOTF_reference & OOTF_adjust Combined into a Single Function”): no metadata is needed in HLG since display environment accounted for in OOTFadjust stage in this slide.

slide 21 of m37535_r2.pdf (“Hybrid Log-Gamma OETF Similar to SDR OETF”):

- NHK desired a higher break point (x-axis linear light at 1.0).  Originally, BBC had a lower breakpoint.

- Mistake pointed out by audience: scaling on the x-axis should be 12 rather than 3

- Presenter made comment that [10-bit] HLG encompasses 17 f-stops of dynamic range.

- In March 2015, the 1.6 system gamma recommendation was determined using non-calibrated Sim2 displays.

- Also discovered that the historical assumption that a 100 lux display has a system gamma of 1.2 is incorrect.

- System gamma 1.45 turned out to be better for 4000 nits Sim2 when experiments were later re-run.

slide 22 of m37535_r2.pdf (“Role of the Opto-Electric Transfer Function”):

- Presenter believes the hybrid log-gamma curves are a good match to the human visual system (HVS);  PQ is closer to HVS.

- Banding artifacts are similar between PQ and HLG at 4000 nits.

slide 23 of m37535_r2.pdf (“For “Display Referred” Systems (e.g. SMPTE ST 2084) OOTF is Effectively Part of the OETF ”):

- In display referred systems, the EOTF is fixed.

- In scene-referred systems, the OOTF is part of the EOTF.

slide 24 of m37535_r2.pdf (“End-to-End Both HDR Systems Identical in Reference Environment “):

- point of the slide is to show that both display-referred and scene-referred produce the same looking image on the right.

- Audience member points out that the curve heavily depends on content.

- Presenter suggests that the monitor on slide 24 is: “relative scene-referred.”

slide 25 of m37535_r2.pdf (“Trnscoding from ST 2084 to HLG Therefore Straightforward, but BBC Believes Requires Implicit or Explicit Metadata “):

- this slide shows how BBC conducted their CfE submission.

- the metadata element in that slide is 4000 nits. max. display luminance. (the SMPTE ST 2086 element)

Slide 26 of m37535_r2.pdf (“When Converting from Display- to Scene-Referred it may be Beneficial to Apply Noise Modifier “):

- BT.1886 (display transfer function) mismatch with BT.709 (camera transfer function) tends to attenuate noise in blacks.

- Cameras try to follow the linear portion of the BT.709 transfer function to reduce noise in the same way.

Slide 28 of m37535_r2.pdf (“HLG OOTF_mastering Will Impact the SDR Compatible Image”): important that the HDR display has correct OOTF, otherwise artistic adjustments will be needed. (re-grading)

slide 29 of m37535_r2.pdf (“OOTF Adjustments for Non-Reference TV Production/Consumer Displays & Environments “): the system gamma of 1.2 for SDR does not fit the graph.  This cause the earlier recommendation of system gamma 1.6 to be wrong.

Slide 30 of m37535_r2.pdf (“OOTF for “Reference” Display and Environment”):

- when averaging viewer preferences, system gamma averages 1.26 to 1.50.

- with a system gamma of 1.6, the SDR pictures were too bright.

Slide 31of m37535_r2.pdf  (“Preliminary Informal Results for OOTF to Deliver Best SDR”):

- this slide shows how BBC arrived at system gamma 1.45.

- lower gamma produces more natural appearance, while higher gamma tends to be more pleasing.

- the test from slide 29 was conducted with Sim2 and PRM-4220 and OLED @ 1000 nits.

- for OLED, derived a system gamma value of 1.27.

Slide 33 of m37535_r2.pdf (“Different Brightness Images Require Different Gamma, Solved by Reference Levels”):

- Full explanation of slide 33 is in ITU-R WP6C document that BBC will distribute to this adhoc

- This graph shows adjustment is independent of OOTF.

- Lars H. @ SVT found it to be a powerful tool in March 2015.

- Some mismatches between different areas of the Sim2 were noted due to non-uniform MTF and backlight across screen.

- When the experiment was repeated on prototype OLED, BBC found uniformity across screen.

- Gamma 1.45 * 0.82 (when display luminance is 1000 nits).

- The tone mapping algorithm follows the basic steps: RGB —> XYZ —> Y u’ v’   modify the Y only —> RGB. (the only changing constant luminance) shown in the next slide.

- BBC studies found system gamma needs to be reduce when ambient lighting level is higher.

- ITU-R haven’t worked out what the reference level should be.

- 77% of non-linear signal (10-bit code 740) is BBC’s current reference (diffuse) white level.

- darker images need darker gamma,  brighter images need higher system gamma.

- Manish sat in front of Sim2 and adjusted pictures.  He didn’t want too much clipping, and wanted similar brightness between pictures.

m37535_r2.pdf : slide 35 is an approximation of a 3D LUT that implements slide 34 algorithm.

Arris presented a document on the backward compatibility of HLG.  The document was uploaded to the MPEG and JCT repositories as m37543 and JCTVC-W0035 respectively.  Arris presented images that showed CE7.1b looked noticeably washed out and suggested focusing on CE7.1a.  CE7.1a uses gamma processing in the Yuv domain and CE7.1b uses gamma processing in the RGB domain.  It was reported that there was a change in hue when the HLG processed video is shown on a legacy SDR TV. It needs to be further investigated.

Technicolor presented an informal document on the backward compatibility of HLG – specifically whether the compatibility is content dependent.  The test was similar to BBC’s where viewers were asked match SDR and HDR by adjusting the system gamma.  Technicolor showed selected ranges from gamma 1.2 to 1.6.  In the discussions, BBC stated that gamma 1.6 is no longer the recommended value.

Philips presented a document (m37602) describing their crosscheck of CE7.1 and CE7.2.  The document verified the work of Technicolor and Arris describing the content as washed out.  CE7.2 had better results than CE7.1 using a system gamma of 1.2.

It was noted that it is important to have consistency of settings of the different displays used for the test of SDR BC. There it will be good to have at least one professional SDR monitor.

It was also noted that the conversion from SDR 2020 to 709 could be achieved by HDRTools with a 2 step approach: conversion from SDR2020 to EXR2020, then conversion of EXR2020 to SDR709. Philips will make a fix (not yet commited) to enable direct conversion of SDR2020 to SDR709.

CE8

An overview of backward compatibility testing without a reference (BC-Lite) was given.  Vittorio Baroncini was not present and so Touradj stepped in for the overview.  The original plan was to have a dry run at this meeting.  Vittorio presented a document at the Geneva meeting with a single test comparing HDR and derived SDR known in the CE8 document as CE8.1.  There were many questions concerning the validity of the test.

A second proposed test (CE8.2) would have two steps.  The first step is expert selection of representative SDR grades to be used as the high example for subjective testing.  The testing will be informal expert tests.  The second step uses naïve viewers to provide MOS scores when comparing the high example, a potential low example and the derived (extracted) BC-Lite candidate.  An issue was discussed where the usage of compressed bit streams to generate the BC-Lite signal needs to be agreed upon.

For the next meeting, the CE expects to have a more fleshed out test and a dry run to select high and low examples.

Expected Recommendations:

- Recommendation of the anchor going into San Diego meeting.

- CE2 will use SMPTE range at point “A” and point “B” for compatibility testing.

List of Anchor Choices:

|  | Code/settings released | Version number | Comment |
| --- | --- | --- | --- |
|  |  | v1.0 | CfE anchor |
|  | Oct 30 | v2.0 | CE1.a + CE1.b + HM16.7 + VUI/SEI-fixes + integer QP |
| current anchor -> | Nov 10 | v2.1 | New sequences, split sequences, new rate points |
| proposal | Nov 21 | v3.0 proposal | Includes luma-average controlled adaptive QP |
|  | Dec 6 | v3.1 | Bugfix |
|  | Jan 3 | v3.2 | Simplification (0.2% improvement in bits) |
| proposal | Nov 21 | v3.0 proposal | Includes luma-average controlled adaptive QP |
| Interdigital 1 | Dec 6 | v3.0 + (1) | Chroma QP offset adjustment + Deblocking parameter selection optimization |
| Interdigital 2 | Jan 3 | v3.0 + (2) | Deblocking parameter selection optimization |

Comments on CE4 demo:

- Another target is to be able to accurately rank artifacts and also be able to detect relative differences.

- The artifacts should be similar and in the same locations between professional and consumer monitors.

- There may be more artifacts in the consumer monitor but all care should be taken to ensure the additional artifacts are not due to the consumer monitor.

- There were general comments that the video playback on the TV did not appear to be HDR.

- The group should test on more than one consumer device.

- The group should explore getting 1000 nit content.

- Evidence also needs to be shown on all MPEG sequences.

Acknowledgement

We are thankful to University of British Columbia and Telus for hosting the meeting. We are thankful to Panos Nasiopoulos and Mahsa Pourazad for their excellent organization of the meeting. We are very thankful to Ibrahim Gedeon, CTO of TELUS, for kindly sponsoring the social event. We are also thankful to Ronan Boitard, Stelios Ploumis, Maryam Azimi, Pedram Mohammadi, Hamid Tohidypour, Bambang A.B. Sarif , Basak Oztas Yoldemir for providing the technical support and all the Digital Multimedia Lab students who helped in smooth operation of the meeting.

Vancouver Meeting attendees

| Panos Nasiopoulos | University of British Columbia (UBC) |
| --- | --- |
| Mahsa T. Pourazad | TELUS & UBC |
| Ajay Luthra | Arris |
| Pankaj Topiwala | FastVDO |
| Joel Sole | Qualcomm |
| Walt Husak | Dolby |
| Peng Yin | Dolby |
| Chad Fogg | MovieLabs |
| Sebastien Lasserre | Technicolor |
| Edouard Francois | Technicolor |
| Elena Alshina | Samsung |
| Jens Ohm | RWTH |
| Jacob Ström | Ericsson |
| Tom Paridaens | University of Ghent |
| Koohyar Minoo | Arris |
| Wiebe de Haan | Philips |
| Robert Brondijk | Philips |
| Alan Chalmers | University of Warwick |
| Louis Kerofsky | Interdigital |
| Arianne Hinds | CableLabs |
| Touradj Ebrahimi | EPFL |
| Alan Stein | Technicolor |
| Matteo Naccari | BBC |
| Andrew Cotton | BBC |
| Hyun Mook Oh | LGE |
| Leon van de Kerkhof | Philips |
| Yasser F. Syed | Comcast |
| Gary Sullivan | Microsoft |
| Andrew Segall | Sharp Labs |