| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0022r1 |
| --- | --- |

| Title: | HDR CE2: Core Experment 2 Report |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | D.Rusanovskyy (Qualcomm), E. Francois (Technicolor), L. Kerofsky (InterDigital), T. Lu (Dolby), K. Minoo (Arris) | Tel: Email: |  |
| Source: | CE2 coordinators |  |  |

_____________________________

# Abstract

The exploratory experiment 2 (CE2) was established at the 113th MPEG meeting (N15800) with investigate HDR video coding technologies operating in 4:2:0 YCbCr NCL fixed point domain. Detailed CE2 description and planned sub-tests was provided in ISO/IEC JTC1/SC29/WG11 document N15795. There are 6 cross-checked input contribution reporting results of 10 sub-tests and 5 CE2 related contributions.

# Introduction

The HDR CE2 was established on the 113th MPEG meeting with following mandates:

- Develop fully automated encoder/reshaper algorithm as a reference for CE2

- Further investigate improvements over the reference code base (H2M)

- Report results to interim F2F meeting, Jan.2016

- Refine CE description if necessary by a resolution of F2F meeting of Jan.2016

- Report results of CE to JCT meeting, Feb.2016

As a result of CE2 activity all given mandates have been accomplished in time. The Exploration Test Model (ETM) with automated encoder side algorithms was released on 12/19/2015, SW description and simulation results has been presented on 01/12/2016.

Preliminary CE2 report was presented in January 2016, F2F meeting. The CE2 description was updated based on the outcome of the F2F meeting. This document reports CE2 activity which followed the F2F meeting.

# Report

HDR CE2 activity has been followed by 22 experts from 11 companies. CE2-related communication has been conducted through xyz-ee2@lists.aau.at reflector. SW and documents distribution has been conducted as through a participants mail list as well as through the MPEG FTP site.

The list of CE2 reports and CE2-related submissions to San Diego JCTVC meeting is provided in Table 1:

Table 1 List of CE2-reports and CE2 related submissions

| JCTVC doc | Title | Source | Sub-test |
| --- | --- | --- | --- |
| W0022 | HDR CE2 report (This document) | Coordinators |  |
| W0033 | HDR CE2: report of CE2.b-1 experiment (reshaper setting 1) | Technicolor | CE2.b-1 |
| W0084 | HDR CE2: CE2.a-2, CE2.c, CE2.d and CE2.e-3 | Dolby, InterDigital | CE2.a-2, CE2.c, CE2.d, C2.e-3 |
| W0093 | HDR CE2: Report of CE2.a-3, CE2.c and CE2.d experiments (for reshaping setting 2) | Arris | CE2a-3, CE2.c, CE2.d |
| W0094 | HDR CE2: Report of CE2.b-2, CE2.c and CE2.d experiments (for reshaping setting 2) | Arris | CE2.b-2, CE2.c, CE2.d |
| W0097 | HDR CE2: CE2.c-Chroma QP offset study report | Qualcomm | CE2.c |
| W0101 | HDR CE2: Report on CE2.a-1 LCS | Qualcomm | CE2.a |
| W0068 | CE2-related: Adaptive Quantization-Based HDR video Coding with HEVC Main 10 Profile | ZTE |  |
| W0071 | CE2-related: Adaptive PQ: Adaptive Perceptual Quantizer for HDR video Coding with HEVC Main 10 Profile | ZTE |  |
| W0085 | HDR CE2 related: Further Improvement of JCTVC-W0084 | Dolby, InterDigital |  |
| W0089 | HDR CE2-related: some experiments on reshaping with input SDR | Technicolor |  |
| W0100 | HDR CE2-related: Results for combination of CE1 (anchor 3.2) and CE2 | Qualcomm |  |
| W0112 | HDR CE2: cross-check report of JCTVC-W0084 | Technicolor |  |
| W0113 | HDR CE2: cross-check report of JCTVC-W0101 | Technicolor |  |
| W0093 | HDR CE2: cross-check report of JCTVC-W0033 | Dolby |  |
| W0094 | HDR CE2: cross-check report of JCTVC-W0097 | Dolby |  |
| W0101 | HDR CE2 related: cross-check report of linear luma re-shaper | Samsung |  |
| W0128 | HDR CE2: Cross-check of JCTVC-W0084 (combines solution for CE2.a-2, CE2.c, CE2.d and CE2.e-3) | Qualcomm |  |
| W0130 | Crosscheck for further improvement of HDR CE2 (JCTVC-W0085) | Intel |  |

## Exploration Test Model validation:

The following CE2 activities have been conducted to validate the ETM reference software

- Jan 21 – delivery of ETM_RC_r1 software (software and xls results) to CE2 reflector and MPEG ftp site.

- Jan 27 – delivery of ETM_RC_r1 bitrates matches results for reshape_setting 0 and 1, Standard Range

- Jan 28 - delivery of ETM_RC_r1 bitrates matches results for reshape_setting 2, with corresponding cfg files, Standard Range

- Jan 28 – cross-checking from Samsung of md5 sums from reshaper, for reshape_setting 0,1,2 done

## Planned CE2 sub-tests and evaluation procedure:

CE2-related technologies proposed to Geneva MPEG meeting were planned for study under following subtests, see Table 2. Detailed description of each of the test is provided in MPEG document N15795.

Table 2. Planned CE2 sub-tests

| Planed sub-test | Proponents |
| --- | --- |
| CE2.a-1 luma ATF with LCS (m37245) | Qualcomm |
| CE2.a-2 reshaping (m37267) | Dolby |
| CE2.a-3 adaptive transfer function (m37091) | Arris |
| CE2.b-1 Cross-channel modulation (m37088 and m37285) | Technicolor |
| CE2.b-2 SDR backward compatibility study (m37092) | Arris |
| CE2.c Chroma QP offset (m37179) | Qualcomm, Arris |
| CE2.d DeltaQP adjustment for Luma | Arris |
| CE2.e-1 Enhancement of ETM | Qualcomm |
| CE2.e-2 harmonization of luma and chroma reshaping (m37332) | Arris |
| CE2.e-3 automatic selection of ETM parameters | Interdigital |

Proponents of each test were expected to utilize the ETM as a code based and conduct simulation under the AhG CTC specified in N15793. Simulation results to be assessed and reported against the CE1 anchor v3.2. Non-formalized, free style visual quality assessment are expected to be conducted by a cross-checker.

## CE2 sub-tests status:

Sub-tests conducted under the CE2 plan is listed in Table 3. Description of the each sub-tests is provided below.

Table 3. Conducted CE2 sub-tests

| Reported sub-tests | Proponents | Cross-checker | Notes | Report Doc. | Cross-check report |
| --- | --- | --- | --- | --- | --- |
| CE2.a-1 luma ATF with LCS (m37245) | Qualcomm | Technicolor |  | W0101 | W0113 |
| CE2.a-2 reshaping (m37267) Jointly with CE2.a-2, CE2.c, CE2.d, CE2.e-3 | Dolby, Interdigital | Qualcomm Technicolor |  |  |  |
| CE2.e-3 automatic selection of ETM parameters |  |  | Merged with CE2.a-2 | W0084 | W0128 W0112 |
| CE2.c Chroma QP offset (m37179) |  |  | Merged with CE2.a-2 |  |  |
| CE2.d DeltaQP adjustment for Luma |  |  | Merged with CE2.a-2 |  |  |
| CE2.b-1 Cross-channel modulation (m37088 and m37285) | Technicolor | Dolby |  | W0033 | W0093 |
| CE2.a-3 adaptive transfer function (m37091) | Arris |  | Combined with CE2.c and CE2.d | W0093 |  |
| CE2.b-2 SDR backward compatibility study (m37092) | Arris |  | Combined with CE2.c and CE2.d | W0094 |  |
| CE2.c Chroma QP offset (m37179) | Qualcomm | Dolby |  | W0097 | W0094 |
| CE2.e-1 Enhancement of ETM | Qualcomm | Arris |  | Withdrawn | W0130 |
| CE2.e-2 harmonization of luma and chroma reshaping (m37332) | Arris |  |  | Withdrawn |  |

## CE2 sub-tests summary:

W0033:	CE2.b-1 experiment (reshaper setting 1)

Backward compatible configuration of ETM, reshaper setting 1 (cross component YCbCr processing)

Algorithms of luma and chroma reshapers derivation

No changes to the syntax or decoding process of the ETM reported

These experiments relate to SDR backward compatible configuration, and focus on reshape setting 1 that involves cross-plane color reshaping. The experiments only involve the reshaping algorithm, and does not require any change in the current ETM syntax and inverse reshaping process. The modified reshaper tuning algorithm for reshape setting 1 is proposed to be adopted in the ETM. It is reported that the proposed changes improves the reshaped SDR content quality (in terms of visual rendering of contrast and colors), while preserving the compression performance.

W0084:	Combined solution for CE2.a-2, CE2.c, CE2.d and CE2.e-3

- Non backward compatible configuration of ETM, reshaper setting 0 (independent component YCbCr processing)

- Algorithm for luma reshaper derivation and automatic update mechanism

- HM encoder optimization

  - DeltaQP adjustment for luma

  - ChromaQP offset adjustment for chroma

  - Change to deblocking filter param derivation

- No changes to the syntax or decoding process of the ETM reported

This proposal reports a combination of CE2 subtests: CE2.a-2 on luma forward reshaping improvement, CE2.c on chromaQPOffset, CE2.d on DeltaQP adjustment for luma, and CE2.e-3 on automatic selection of ETM parameters. The experiment is to test non-normative luma reshaping improvement as well as joint optimization of reshaper and encoder. The encoder optimization methods include chromaQPOffset, luma DeltaQP adjustment, and deblocking filter parameter selection.

W0093 HDR CE2: Report of CE2.a-3, CE2.c and CE2.d experiments (for reshaping setting 2)

- Backward compatible configuration of ETM, reshaper setting 2 (cross component YCbCr processing)

- Algorithms for Adaptive Transfer Function (ATF) derivation for luma and chroma

- HM encoder optimization

  - DeltaQP adjustment for luma

- No changes to the syntax or decoding process of the ETM reported

This document reports the results of experiments for CE2.a-3 in conjunction with encoding optimization [1] techniques which have been studied under CE2.c and CE2.d categories. The combined experiment was conducted to measure the performance of an alternative transfer function (instead of conventional PQ TF) as well as joint optimization of the reshaper and encoder, specifically when the reshaping setting is in mode 2. Informal subjective evaluations were conducted for both the reconstructed SDR and HDR sequences. It is asserted that compared to the version 3.2 of anchor, the proposed approach provides similar perceptual quality of the reconstructed HDR signal while supporting SDR backward compatibility.

W0094 HDR CE2: Report of CE2.b-2, CE2.c and CE2.d experiments (for reshaping setting 2)

- Backward compatible configuration of ETM, reshaper setting 2 (cross component YCbCr processing)

- Algorithms for Adaptive Transfer Function (ATF) derivation for luma and chroma

- HM encoder optimization

  - DeltaQP adjustment for luma

  - ChromaQP offset adjustment for chroma

- No changes to the syntax or decoding process of the ETM reported

This document reports a combination of CE2 subtests: CE2.b-2 on SDR backward compatibility, CE2.c on chromaQPOffset, and CE2.d on DeltaQP adjustment for luma. The experiment was conducted to study bitstream SDR backward compatibility as well as joint optimization of the reshaper and encoder. Subjective evaluation was conducted on a SIM2. It is asserted that compared to anchor v3.2, the proposed joint optimization of reshaper and encoder provides similar perceptual quality while supporting SDR backward compatibility.

W0097 HDR CE2: CE2.c-Chroma QP offset study report

- Non backward compatible configuration of ETM, reshaper setting 0 (independent component YCbCr processing)

- Algorithm for chroma ATF derivation

- Tested in 2 frameworks:

  - Reference ETM software

  - Software of the W0084 (alternative luma reshaper)

- HM encoder optimization:

  - CE1 anchor 3.2 default settings

- No changes to the syntax or decoding process of the ETM reported

This document reports results of CE2.c sub-test which studies harmonization of chroma QP offset utilized in CE1 anchor and CrCb processing of the ETM. As a result of the study, modification of encoder side algorithm is proposed to the ETM. Proposed in this document solution employs an independent multirange ATF (reshaper) for each color component. Proposed solution provides objective gain as well as improvement in visual quality over reference ETM design as well as over the CE1 anchor. Proposed multirange ATF was also integrated and tested in software of W0084.

W0101 HDR CE2: Report on CE2.a-1 Luma based Chroma Scaling

- Non backward compatible configuration of ETM, reshaper setting 1 (cross-component YCbCr processing)

- Algorithm for chroma ATF derivation

- No changes to the syntax or decoding process of the ETM reported

This document reports the results of the study of CE2.a-1 on luma-based chroma scaling (LCS). The chroma sample values are scaled by values that are dependent on the co-sited luma sample values, and the scale values are computed based on a function that increases with luma values. The document reports objective evaluation on the tested configuration.

## CE2 – related summary:

W0068	CE2-related: Adaptive Quantization-Based HDR video Coding with HEVC Main 10 Profile

- Non-ETM framework

The perceptual quantization (PQ) for HDR video coding has perceptual uniformity in the luminance range with the modest bit-depth. However, the dynamic range of HDR videos is not fully utilized in PQ, especially PQ for chrominance channels. Thus, there exist the wasted dynamic ranges in PQ which cause detail loss and color distortions. In this contribution, adaptive quantization-based HDR video coding is proposed. Adaptive mapping of sample values with bit depths of 16 bit and 10 bit is established to perform conversion based on cumulative distribution function (CDF), i.e. adaptive quantization, instead of linear mapping. First, CDF is derived from the histogram of each picture. Then, adaptive quantization is performed based on CDF. The metadata for adaptive conversion are coded for performing the inverse conversion at destination. Experimental results demonstrate that the proposed method achieves average 1.7 dB gains in tPSNR XYZ and 2 dB gains in PSNR_DE over Anchor. Subjective comparisons show that the proposed method preserves image details while reducing color distortion.

W0071	CE2-related: Adaptive PQ: Adaptive Perceptual Quantizer for HDR video Coding with HEVC Main 10 Profile

- Non-ETM framework

Proposed is an adaptive transform function based perceptual quantizer (PQ) for HDR videos, called adaptive PQ. Adaptive PQ is to solve the problem that PQ is not adaptive to HDR content because of its using a fixed mapping function from luminance to luma. By introducing a ratio factor derived from the luminance information of HDR content, adaptive PQ maps luminance to luma adaptively according to the content. Experimental results demonstrate that adaptive PQ achieves average bit-rate reductions of 3.51% in tPSNR XYZ and 5.51% in PSNR_DE over PQ.

W0085 HDR CE2 related: Further Improvement of JCTVC-W0084

This proposal reports a further improvement of JCTVC-W0084. The improvement is solely on luma DeltaQP adjustment.

W0089 HDR CE2-related: some experiments on reshaping with input SDR

Backward compatible configuration of ETM, reshaper setting 1 (cross component YCbCr processing)

Algorithms of luma and chroma reshapers derivation

No changes to the syntax or decoding process of the ETM reported

This document reports preliminary experiments on ETM with dual SDR/HDR grading. In this configuration, a graded SDR version as well as the usual graded HDR version are given as inputs to the reshaper. The reshaping process uses the input SDR as reference and generates reshaping data to match as much as possible the reshaped input HDR to the input SDR. In the experiments, reshape setting 1 configuration is used (cross-plane chroma reshaping). It is reported that these preliminary experiments tend to show that ETM models are able to properly reproduce an SDR intent.

W0100 Results for combination of CE1 (anchor 3.2) and CE2

- Non backward compatible configuration of ETM, reshaper setting 0 (independent component YCbCr processing)

- HM encoder optimization

  - CE1 default anchor settings

  - Additional DeltaQP adjustment for luma

  - Additional ChromaQP offset adjustment for chroma

- No changes to the syntax or decoding process of the ETM reported

This document reports results of combining the software of anchor 3.2 that is the outcome of CE1 and the ETM that is the outcome of CE2. Results for the direct combination of the CE1 and CE2 software are provided. It is noted that CE1 and CE2 algorithms have some degree of overlap, so the direct combination could be improved by a proper adjustment of the parameters. Results obtained by adjusting the parameters are also provided.

# Conclusions

The exploratory experiment 2 (CE2) was established at the 113th MPEG meeting (N15800) with investigate HDR video coding technologies operating in 4:2:0 YCbCr NCL fixed point domain. Detailed CE2 description and planned sub-tests was provided in ISO/IEC JTC1/SC29/WG11 document N15795. There are 6 cross-checked input contribution reporting results of 10 sub-tests and 5 CE2 related contributions. It is proposed to review CE2 and CE2 related during this meeting.