| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0026 |
| --- | --- |

| Title: | Report of HDR CE6 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | R. Brondijk, Philips Innovation Services HTC 37 6.022 5656 AE Eindhoven S. Lasserre, Y. He, D. Rusanovskyy (coordinators) |  | Tel: +31 646945350 Email: Robert.Brondijk@philips.com |
| Source: | Philips, Technicolor, InterDigital, Qualcomm |  |  |

_____________________________

# Abstract

In HDR CE6, various HDR post-processing technologies are explored that would require only a non-normative SEI message to be supported by an HEVC Main 10 decoder.

# Introduction / Problem Statement

This document describes the core experiment CE6 on HDR post processing approaches. The CE relates to HDR and WCG video coding technologies and focuses on the reconstruction of HDR video with the output decoded video from an HEVC Main 10 decoder.

In this CE, only solutions that operate as pre-processing to the encoder and post-processing to the main10 decoder are considered.

The tested technologies may require metadata which is signaled through SEI and is not utilized in a normative reconstruction process of the decoder. Post-processing may be performed on decoded samples in 4:2:0 representation and/or on samples in 4:4:4 representation. In the spirit of being non-normative, the proposals in the CE should produce a "viewable" picture even if the metadata, if any, is lost.

The CE6 Mandates are:

- Execute the core experiments defined in this document

- Report on possible solutions including meta-data

- Description of the processing using the meta-data

# List of Participants

| Participants | Affiliation | Email |
| --- | --- | --- |
| K. Minoo | Arris | koohyar.minoo@arris.com |
| J. Ström | Ericsson | jacob.strom@ericsson.com |
| Y. He | InterDigital | yuwen.he@interdigital.com |
| L. Kerofsky | InterDigital | louis.kerofsky@interdigital.com |
| Y. Ye | InterDigital | yan.ye@interdigital.com |
| R. Brondijk | Philips | robert.brondijk@philips.com |
| L. van de Kerkhof | Philips | leon.van.de.kerkhof@philips.com |
| R. Goris | Philips | rocco.goris@philips.com |
| J. Sole Rojals | Qualcomm | joels@qti.qualcomm.com |
| D. Rusanovskyy | Qualcomm | dmytror@qti.qualcomm.com |
| E. Alshina | Samsung | elena.alshina@gmail.com |
| E. Francois | Technicolor | Edouard.Francois@technicolor.com |
| S. Lasserre | Technicolor | sebastien.lasserre@technicolor.com |
| Taoran Lu | Dolby | tlu@dolby.com |
| Peng Yin | Dolby | pyin@dolby.com |

# The Basic System

The basic system is shown below. Modified HM software, based on HM 16.7, used to create the anchor will be used in this CE as a code base. The changes introduced to the HM codec shall be limited to signaling mechanism by means of SEI messages. Tested technology will be compared against the super anchor results. The output of the HEVC decoder may be SDR compatible.

# Summary of planned CEs under CE6

## Reshaper (m37064)

This test is the SDR Backwards Compatible configuration of the technology proposed in m37064. The technology is based on dynamic range adjustment and provides a method for a guided, end-user side dynamic range conversion. Parameters of the dynamic range adjustment are proposed to be conveyed to the decoder through an SEI message.

  - Color Enhancement (m37072)

The color enhancement in m37072 will be tested in YCbCr color space and 4:2:0 chroma format and the parameters for color enhancement are signaled in SEI message. The color enhancement filtering information is derived at the encoder automatically.

## Dynamic Range Adaptation and Re-saturation (m36266)

Study of the quality of "maxRGBY" based transfer function combined with re-saturation and automatic parameter generation.

## Continuation of previous CE2.2c (m37230)

Finalize the previous core experiment for the remaining sequences.

## Combined DRA, Re-saturation and Color Processing (m37245, m37230)

This experiments tests combination of tools proposed in m37245 and m37230. Combined solution would utilize dynamic range adaptation, based on application of 1D-mapping functions to the YCbCr and/or RGB components, and re-saturation and color processing proposed in m37230. Additional color processing specified in m37245 can be applied. The metadata associated to these 1D-functions, modeled by piece-wise linear models with signaling presented in m37245, are conveyed via an SEI.

## Technologies based upon m37285

This experiment is based on m37285. This contribution combines two technologies: dynamic range adaptation, based on application of 1D-mapping functions to the YCbCr and/or RGB components, and color correction consisting of the modulation of the CbCr and/or RGB components samples by a 1D scaling function depending on the Y component sample and signaled as presented in m37245. The metadata associated to these 1D-functions, modeled by piece-wise linear models, are conveyed via an SEI.

The following experiment will study the different implementations of the technologies:

- The performance of the Philips system, m36266, when using an SEI message, described in m37285, that describes a LUT instead of parameters,

- Implementation based on the test model, with usage of SEI instead of SPS/PPS signaling,

- Alternative SEI message based solution combining multiple technologies.

The following experiments in this category are under consideration:

- m36266: SDR mode, ATF and color correction applied to RGB 4:4:4

- m37064, configurations 1 and 2: HDR-mode, ATF applied to CbCr 4:2:0, then to RGB 4:4:4

- m37064, configuration 3: HDR-mode, ATF applied to YCbCr 4:2:0

- m37064, configuration 4: SDR-mode, ATF applied to YCbCr 4:2:0

- m37088: SDR-mode, color correction then ATF applied to YCbCr 4:2:0. Additional process, not included in the CTM, applies to complete the color reconstruction to YCbCr PQ CL.

- m37285: SDR-mode, color correction then ATF applied to YCbCr 4:2:0

Combinations/synergies with the test model will be explored.

## Gamut mapping (m37368)

This experiment will evaluate the gamut mapping technology proposed in m37368. The technology aims at performing a gamut mapping from one given large color gamut to another small color gamut, on the preprocess side, and the inverse gamut mapping on the post-process side. It is based on the specification of a preserved color gamut, inside the small color gamut, in which the color will remain unchanged. Samples with colors located inside the large color gamut and outside the small color gamut are mapped into the small color gamut. Metadata related to the process are conveyed in a SEI message that is used at the decoder side to regenerate a content in the large color gamut.

## HDR Adaptation (m37091, m37092)

Study of Post inv-quantization scaling and parametric power (gamma) function and PQ function as explained in m37091 and m37092.

# Test Conditions

## Test sequences

Common Test Conditions for HDR/WCG Video Coding Experiments specified in document N15793 of the AhG are to be used in this test.

## Anchors

The super anchors from CE1 will be used for this test. The reporting of the object metric will be by comparing the proponents generated EXR against the original EXR, or, if not available, the agreed set of EXRs (nearly) losslessly generated from the TIFFs by HDRTools.

If the original content is in TIFF, then a proponent may choose to start pre-processing from either the TIFF or the agreed set of EXRs that are generated from the TIFF by HDRTools. If the original content is in EXR, then the proponent shall start pre-processing from these EXRs.

## Software

The HM software that will be used to create the super anchors will be used to encode and decode the video stream and the metadata stream. The proponents shall deliver versions able to encode and decode the added metadata stream. The used HM software shall be sent to all CE6 participants.

The participants shall extend HDRTools with their software to describe their post- and pre- processing and creation and utilization of any VUI and, or SEI messages used. The reading and writing of VUI and SEI may be performed via files or parameters. It will be assumed that the files created or used, during the pre- and post-processing can be passed to the HM. In other words, the post processor need not rely on the HM to produce the files needed for decoding, so the post-processor may read the files created by the pre-processor

## Reported results

Results will be reported in two spreadsheets attached to the documents that describe the tests in more detail. The first spreadsheet shall contain bit rate and objective measurement values for each test point. This objective metric template has been updated with the metrics being studied in m37365. The second spreadsheet shall contain the md5sums related to the CE test.

Also the number of bits needed in SEI messages will be reported.

# List of performed Experiments and Cross Checkers

The following table lists the CE6 tests that have been performed with reference to the numbering in section 4 indicating Proponent and Cross Checker.

| CE Test | Contribution | Summary | Proponent | Cross Checker |
| --- | --- | --- | --- | --- |
| 4.61 | JCTVC-W0103 | Technology based on m37064: SDR Backward Compatible configuration of Dynamic Range Adjustment with signaling of parameters for HDR reconstruction through SEI message. | Qualcomm | Philips JCTVC-W0109 BBC JCTVC-W0125 |
| 4.2 | JCTVC-W0034 | Color Enhancement (m37072) | InterDigital | Samsung JCTVC-W0123 |
| 4.3 | JCTVC-W0063 | Dynamic Range Adaptation and Re-saturation (m36266) | Philips | Technicolor (version 1) JCTVC-W0114 Qualcom (version 2) |
| 4.6a | JCTVC-W0063 | Technologies based upon m37285: m36266, when using an SEI message described in m37285 | Philips | FastVDO JCTVC-W0073 |
| 4.6b | JCTVC-W0059 | Technologies based upon m37285: the test model, with usage of SEI | Technicolor | Philips JCTVC-W0064 |

# List of Related Contributions

| JCT-VC number | MPEG number | Created | First upload | Last upload | Title | Authors |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| JCTVC-W0026 | m37738 | 2016-02-10 00:03:57 | 2016-02-10 00:04:26 | 2016-02-11 09:10:11 | Report of HDR Core Experiment 6 | Robert Brondijk, Sebastien Lasserre, Dmytro Rusanovskyy, yuwen he |  |
| JCTVC-W0034 | m37614 | 2016-02-03 20:34:39 | 2016-02-09 21:50:15 | 2016-02-09 21:50:15 | CE-6 test4.2: Color enhancement | Y. He, L. Kerofsky, Y. Ye (InterDigital) |  |
| JCTVC-W0059 | m37693 | 2016-02-09 09:13:38 | 2016-02-10 18:57:43 | 2016-02-16 22:55:39 | HDR CE6: report of CE6-4.6b experiment (ETM using SEI message) | E. Francois, C. Chevance, Y. Olivier (Technicolor) |  |
| JCTVC-W0060 | m37694 | 2016-02-09 09:28:36 | 2016-02-09 10:25:19 | 2016-02-09 10:25:19 | Re-shaper syntax extension for HDR CE6 | Hyun Mook Oh, Jong-Yeul Suh |  |
| JCTVC-W0063 | m37698 | 2016-02-09 12:42:40 | 2016-02-09 22:04:01 | 2016-02-09 22:04:01 | HDR CE6: Core Experiments 4.3 and 4.6a: Description of the Philips CE system in 4:2:0 and with automatic reshaper parameter derivation | Wiebe de Haan, Robert Brondijk, rocco goris, Rene van der Vleuten |  |
| JCTVC-W0064 | m37699 | 2016-02-09 12:51:14 | 2016-02-09 14:58:17 | 2016-02-09 15:06:44 | HDR CE6-related: Cross Check of CE6.46b | Robert Brondijk, Wiebe de Haan |  |
| JCTVC-W0073 | m37708 | 2016-02-09 14:22:29 | 2016-02-17 18:21:13 | 2016-02-17 18:21:13 | HDR CE6: Cross-check of 6.46a | W. Dai, M. Krishnan, P. Topiwala (FastVDO) |  |
| JCTVC-W0103 | m37746 | 2016-02-10 08:22:29 | 2016-02-10 10:11:34 | 2016-02-10 10:11:34 | HDR CE6: Test 4.1 Reshaper from m37064 | D. B. Sansli, A. K. Ramasubramonian, D. Rusanovskyy, J. Sole, M. Karczewicz (Qualcomm) |  |
| JCTVC-W0109 | m37765 | 2016-02-11 09:12:58 |  |  | CE6-related: xCheck of CE6 4.1 | Robert Brondijk |  |
| JCTVC-W0114 | m37859 | 2016-02-15 14:59:56 | 2016-02-19 15:47:00 | 2016-02-19 15:47:00 | HDR CE6: crosscheck of CE6.4.3 (JCTVC-W0063) | C. Chevance, Y. Olivier (Technicolor) |  |
| JCTVC-W0123 | m38008 | 2016-02-17 06:58:56 | 2016-02-17 10:50:39 | 2016-02-17 10:50:39 | HDR CE6: cross-check report of test4.2: Color enhancement (JCTVC-W0034) | E.Alshina(Samsung) |  |
| JCTVC-W0125 | m38039 | 2016-02-17 19:56:41 | 2016-02-17 19:59:11 | 2016-02-17 19:59:11 | CE6-related: Cross check report for Test 4.1 Reshaper from m37064 | M. Naccari (BBC) |  |

# Original timeline

10/23/2015	Meeting ends

11/27/2015	Refine test plan and test description, and distributed over reflector

02/01/2016	Software solutions for CE6 tests to be provided to CE6 participants

02/04/2016	CE6 test simulation results to be provided to CE6 participants

02/09/2016	CE6 test results and text description of the experiment to be reported

02/19/2016	CE6 test cross-check results to be reported