| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0027 (also, ISO/IEC JTC1/SC29/WG11 m37681) |
| --- | --- |

| Title: | Report of HDR Core Experiment 7 on investigating the visual quality of HLG generated HDR and SDR video |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Information |  |  |  |
| Author(s) or Contact(s): | A. Luthra (ARRIS Group, Inc.) E. Francois (Technicolor) L. van de Kerkhof (Philips) | Email: Email: Email: | Ajay.Luthra@arris.com Edouard.Francois@technicolor.com leon.van.de.kerkhof@philips.com |  |
| Source: | CE7 AHG |  |  |  |

_____________________________

# Abstract

This document provides the report of the Core Experiment 7 investigating the visual quality of Hybrid Log-Gamma (HLG) generated HDR and SDR video.

# Introduction

The exploratory experiment 7 (CE7) was established at the 113th MPEG meeting (N15800) with the purpose of understanding various techniques used to generate video using Hybrid Log-Gamma (HLG) transfer function and investigating the visual quality of the video.

Hybrid Log-Gamma (HLG) Transfer Function (TF) is designed to possibly provide backward compatibility with the “legacy” SDR systems. At this stage, in MPEG’s work, the “legacy” systems are taken to be the SDR systems not needing color gamut conversion (i.e., for example, not needing BT.2020 to BT.709 color gamut conversion) but needing only the dynamic range conversion. If the legacy SDR TV cannot process BT.2020 container then it is assumed the BT.2020 container is removed externally and the video is provided with appropriate format. The details of the experiments that were conducted is provided in ISO/IEC JTC1/SC29/WG11 N15800 [1].

# Report

Initial version of N15800 lacked specific details. Those details were provided and uploaded as the second version of N15800. Various issues associated with HLG were discussed on the email reflector as well as during the HDR/WCG AHG’s face to face meeting in Vancouver in Jan 2016.

An overview of the motivation for development of HLG was given to the group by BBC during the Vancouver meeting and uploaded as ISO/IEC JTC1/SC29/WG11 m37535 (JCTVC-W0037).  There was substantial discussion on the impact and interactions of various elements of the OOTF.  The utility of modifying the system gamma to change the peak brightness was discussed.  BBC reported on a subjective test where viewers modified the system gamma by a factor in order to adapt the signal from one peak brightness to another.  The value is dependent on the source and target peak brightness values. It was observed that only 2-3% of TV programs go through a reference display environment and only one of 24 in-house studios surveyed by BBC followed BT 2035 lighting environment recommended practice.

BBC was requested to send ITU-R WP6C contribution on system gamma design for monitors to the MPEG/JCT HDR adhoc group.

Arris presented a document during the Vancouver meeting on the backward compatibility of HLG.  The document was uploaded to the MPEG and JCT repositories as ISO/IEC JTC1/SC29/WG11 m37543 (JCTVC-W0035) respectively.  Arris presented images that showed CE7.1b looked noticeably washed out and suggested focusing on CE7.1a.  CE7.1a uses gamma processing in the Yuv domain and CE7.1b uses gamma processing in the RGB domain.  It was also reported that there was a change in hue when the HLG processed video is shown on a legacy SDR TV. The amount of change in hue was observed to be a function of the brightness of the object. Its impact was that as an object moved from less bright area to brighter area, the displayed color changed. It was further investigated. To be able to test it in more controllable environment, Arris generated color bars that showed the similar behavior of change in the hue depending on the brightness of the object as seen in other video sequences. The EXR file consisting of those color bars were also provided on the CE7 reflector. The results are reported in ISO/IEC JTC1/SC29/WG11 m37543 (JCTVC-W0035).

Technicolor presented an informal document during the Vancouver meeting on the backward compatibility of HLG – specifically whether the compatibility is content dependent. The document was later completed and uploaded as document JCTVC-W0110. The test, done using BT.709 container, was similar to BBC’s where viewers were asked match SDR and HDR by adjusting the system gamma. Technicolor showed selected ranges from gamma 1.2 to 1.6. Technicolor reported that that the system gamma value for an optimal SDR rendering is content dependent, and not only dependent on the peak brightness and viewing environment. Bright content would require smaller gamma values (close to 1.2) while dark content would need large gamma values (above 1.4). In the discussions, BBC stated that gamma 1.6 is no longer the recommended value. The second round of test done using BT.2020 container confirmed the previous observations.

Philips presented a document (ISO/IEC JTC1/SC29/WG11 m37602) during the interim meeting describing their crosscheck of CE7.1 and CE7.2. The document confirmed the work of Technicolor and Arris. For 7.1.b, the SDRs look washed out and there are noticeable color tone changes. The 7.1.a SDRs look better than those of 7.1.b, however their colors are often too saturated (reds are too red, greens too green and blues too blue). The compression results vary: for some sequences, 7.1.b has fewer issues and for others 7.1.a have fewer issues. At the lower gammas of CE 7.2 the oversaturation is less noticeable, and the errors in luminance and color are smaller. Cross-checking of 7.2 experiments are provided by BBC in JCTVC-W0043.

It was also noted during the Vancouver meeting that the conversion from SDR 2020 to 709 could be achieved by HDRTools with a 2 step approach: conversion from SDR2020 to EXR2020, then conversion of EXR2020 to SDR709. Philips made a fix to enable direct conversion of SDR2020 to SDR709. There are still ongoing discussions on the reflector to clarify how the conversion from SDR2020 to SDR709 has to be applied. An input document (JCTVC-W0046) is provided as one of the suggested starting point, with the target of providing a recommendation by the end of the JCTVC meeting, for further visual quality evaluations of the SDR produced by the various studied solutions (see CE8 report for in-depth coverage of the topic of SDR backward compatiblity systems evaluation).

# Contributions list

Results report

- JCTVC-W0061/m37695 - CE7: Results Core Experiment 7.1 test a and b, M. Naccari, A. Cotton, M. Pindoria

- JCTVC-W0079/m37717 - HDR CE7-related: Additional results for Experiment 7.2a, M. Naccari, M. Pindoria, A. Cotton (BBC)

Comments/Observations

- JCTVC-W0035/m37543 - Some observations on visual quality of Hybrid Log-Gamma (HLG) TF processed video (CE7), A. Luthra, D. Baylon, K. Minoo, Y. Yu, Z. Gu (Arris)

- JCTVC-W0110/m37771 - HDR CE7: Comments on visual quality and compression performance of HLG for SDR backward compatible HDR coding system, E. Francois, Y. Olivier, C. Chevance (Technicolor)

- JCTVC-W0119/m37979 - Some considerations on hue shifts observed in HLG backward compatible video, M. Pindoria, M. Naccari, T. Borer, A. Cotton (BBC)

Cross-checking

- JCTVC-W0043/m37630 - CE7: Cross-check report for Experiment 7.2, M. Naccari, M. Pindoria (BBC)

- JCTVC-W0067/m37702 - HDR CE7: xCheck 1a 1b and Core Experiment 2a and 2b, Robert Brondijk, Wiebe de Haan, Rene van der Vleuten, Rocco Goris

Others

- JCTVC-W0046 /m37680 - Recommended conversion process of YCbCr 4:2:0 10b SDR content from BT.709 to BT.2020 color gamut, E. Francois, K. Minoo, R. van de Vleuten, A. Tourapis

# References

- A. Luthra, E. Francois, L. van de Kerkhof, “Core Experiment 7 on  investigating how to generate compressed HDR/SDR video using HLG and HEVC”, ISO/IEC JTC1/SC29/WG11 N15800, October 2015, Geneva, Switzerland.