| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0042r1 |
| --- | --- |

| Title: | SCC encoder improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yao-Jen Chang, Po-Han Lin, 2Chun-Lung Lin, Jih-Sheng Tu, and Ching-Chieh Lin 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan (R.O.C.) | Tel: Email: | +886-3-591-8774 1britpablo@itri.org.tw 2Chunlung@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

To reduce the SCM encoding time in 444 chroma format, JCTVC-U0095 proposed to disallow the TU splitting processes for the intra prediction mode enabling adaptive colour transform when the CU sizes are 64x64 and 32x32. The proposed method was adopted into SCM anchor in the last Geneva meeting for substantial encoder improvement in 444 chroma format. However, the TU splitting processes for the intra prediction mode in non-444 chroma formats are also a huge computational cost. This contribution proposes to apply the same method which disallows TU splitting processes to the intra prediction mode in non-444 chroma formats. Compared to the SCM6.0 anchor, it is reported that the proposed encoder can save 3% encoding time in average with 0.1 gain on the AI-lossy conditions in 420 chroma format. This contribution also re-tests the performance of the adopted method in JCTVC-U0095 when it is applied to different sizes of CUs for the intra prediction mode enabling adaptive colour transform in 444 chroma format.

# Introduction

In JCTVC-U0095 [1] and JCTVC-V0040, it was proposed to disallow the TU splitting processes of the 64x64 and 32x32 CU for the intra prediction mode enabling adaptive colour transform (ACT) [2], and was adopted into the latest SCM for its substantial improvement on encoding time. However, this method is only applied to 444 chroma format. This contribution proposes to apply the same method of JCTVC-U0095 to the intra prediction mode in non-444 chroma formats.

# Proposed coding method

The adopted method in JCTVC-U0095 is applied to non-444 chroma formats for the same encoder concerns. Specifically, the proposed encoder modification for non-444 chroma formats disables the TU splitting of the intra prediction mode when the CU sizes are 64x64 and 32x32. It decreases the encoding time.

# Simulation results

The proposed encoder modification is simulated on top of the SCM 6.0 anchor under the common test condition [3]. Table 1 are the BD-rate results of the proposed encoder compared to the anchor under the conditions of AI-lossy and 420 chroma format. The results show that it saves about 3% encoding time with 0.1% coding gain.

Table 1. AI-lossy results for the proposed encoder in 420 chroma format

# Supplemental experiments

The adopted method in JCTVC-U0095 is to disallow the TU splitting processes for the intra prediction mode enabling adaptive colour transform when the CU sizes are larger than or equal to NxN, where N is set as 32. This contribution further tests the adopted method under different sizes N on top of the SCM6.0 anchor. Table 2 shows the AI-lossy results when the adopted method in JCTVC-U0095 is disabled. Table 3 shows the AI-lossy results when N is set as 64 for the adopted method in JCTVC-U0095. Table 4 shows the AI-lossy results when N is set as 16 for the adopted method in JCTVC-U0095. Table 5 shows the AI-lossy results when N is set as 8 for the adopted method in JCTVC-U0095. The simulation results show that the adopted method in JCTVC-U0095 can further save 3% and 5% for N = 16 and N = 8, respectively.

Table 2. AI-lossy results when the adopted method in JCTVC-U0095 is disabled

Table 3. AI-lossy results when N is set as 64 for the adopted method in JCTVC-U0095

Table 4. AI-lossy results when N is set as 16 for the adopted method in JCTVC-U0095

Table 5. AI-lossy results when N is set as 8 for the adopted method in JCTVC-U0095

# Conclusion

This contribution proposes to improve the encoding complexity for SCM in non-444 chroma formats by disallowing the TU splitting of the intra prediction mode in 64x64 and 32x32 CUs. The results report that 3% encoding time saving can be achieved with 0.1% coding gain. It is recommended to adopt this method to speed-up the SCM in non-444 chroma formats. Furthermore, this contribution tests the adopted method of JCTVC-U0095 under different CU sizes on top of the SCM6.0 anchor. The simulation results show that the adopted method in JCTVC-U0095 can further save 2% and 5% with minor gain changes when more CU sizes are applied.

# Reference

- Y. J. Chang, C. L. Lin, J. S. Tu, C. C. Lin, “Non-CE: Encoder modification for intra prediction mode using adaptive colour transform,” 21st JCT-VC meeting, JCTVC-U0095, Poland, Warsaw, July 2015.

- R. Joshi, S. Liu, G. J. Sullivan, Y.-K. Wang, J. Xu, Y. Ye, "HEVC Screen Content Coding Draft Text 5", 22nd JCT-VC meeting, JCTVC-V1005, Geneva, CH, Oct. 2015.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding,” 21th JCT-VC meeting, JCTVC-U1015, Warsaw, Poland, Jun. 2015.

# Patent rights declaration(s)

ITRI international may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)