| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0045 |
| --- | --- |

| Title: | Hybrid Log Gamma observations |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Fogg 475 Sansome St. #740 San Francisco, CA 94111 USA | Email: | chadfogg@gmail.com |
| Source: | Motion Picture Laboratories Inc. |  |  |

_____________________________

# Abstract

This input contribution document examines the impact of conversion between HLG and PQ signals, and tries to identify the operating sweet spot range of each. While the HLG coded video signal is scene referred, the non-linear slope of the non-linear output of HLG by definition does not uniformly quantize linear input level levels, thus implying some perceptual-bias towards darker pixels.  One goal of this study is to find the perceptually optimal display range of HLG graded video signals, or confirm that the display gamma adjustment renders any perceptual quantization effect irrelevant within a range of concern, such as 400 to 4000 nits. To test this, the BT.HDR DNR [1] formulae for mapping scene relative linear light to display linear light are applied. Although is expected to be less optimal than tone mapping provided in ACES and proprietary tools, proponents of HLG state the Note 5e display gamma to be a reasonable proxy for display-embedded tone mapping (OOTF).  By contrast, PQ signals are display referred, but in practice displays most also tone map PQ signals if the display characteristics differ significantly from the mastering monitor characteristics that the PQ signal was originally graded upon, or if the PQ signal utilizes code level combinations outside those that can be directly displayed. Observations are noted of these converted video clips, sent to the Samsung JS9500 as HLG and PQ 10-bit integer signals in BT.2020 primaries color space (4:4:4 R'G'B' or 4:2:2 Y'C'BC'R).  Observations noted from viewing on Sony BVM-X300 professional OLED studio reference monitor may be provided later. An emerging explanation of the observations is that HLG behaves like a display-referred transfer system within an operating sweet-spot near 1000 nits, and superior performance to PQ in the 10 to 40 nits sub-rage.

# Introduction

The conversion scenarios here using the BT.HDR formulae [1] for display rendering are summarized in the following mapping scenario table:

|  | Dynamic Range |  |  |
| --- | --- | --- | --- |
| Color Gamut | 1. Reduce | 2. Keep | 3. Increase |
| A. Reduce | BT.HDR to BT.709 (SDR “backward-compatible” output) | BT.2020 to BT.709 | N/A |
| B. Keep | BT.HDR to BT.2020 (UK backward-compatible scenario with UHD phase 1) | BT.HDR to BT.HDR (PQ  HLG) | BT.2020 to BT.HDR |
| C. Increase | N/A | BT.2020 to BT.HDR | BT.709 to BT.HDR |

Mapping scenario 1A (e.g. BT.HDR to BT.709) and 1B (BT.HDR to BT.2020 and BT.709/HDR to BT.709/SDR) is the focus of the JCT Core Experiment (CE) 7.1.a and 7.1.b, whose results are presented in [2].  CE 7.1.a follows similar OOTF formulae provided in BT.HDR [1].

The main focus of this document is the highlighted yellow box (center) in the above table: scenario 2B.

Though HLG can be “stretched” through display system gamma adjustment (BT.HDR DNR Note 5e) , it is expected that HLG has a sweet spot range that is lower overall than the range of PQ. Contrast sensitivy functions predict that 10-bit HLG on a 1000 nits monitor will render fewer district contrast steps and dark, but more steps in mid-range bright pixels. Since both HLG and PQ are 10-bit signals in this scenario, each transfer function system must have differing code level densities, and thus different perceptual quantization performance at different positions along their respect curves.

.

## Predicting results form theory

Display optimal ranges might be predicted from the BT.HDR DNR equations for reference OOTF, which are a composite function of the EOTF and its inverse EOTF with an adjustment step in between that accounts for the display characteristics (maximum and minimum display luminance).

Table 5 Note 5b the relationship between input linear light (E), normalized real-value signal to the range [0.0, 1.0], and the non-linear HLG output signal E´ according to the formula:

E'=OETF E=3E0≤E≤112a⋅lnE−b+c112<E

where:  a= 0.17883277, b= 0.02372241, c= 1.00429347

Figure 1-1  HLG OETF

Meanwhile, Note5e computes system gamma according to the function, where Lw is the nominal peak (maximum) luminance of the display in cd/m2:

γ=1.2+0.42Log10LW/1000

Producing a familiar-shaped curve:

Figure 1-2 Note 5e: Display system gamma

The HLG Reference OOTF converts scene linear light (normalized in Note 5g to the real-value signal range [0.0, 1.0]) to display linear light in cd/m2 as a function of realive scene linear light and constants: Lw (display peak luminance) and Lb (display luminance for black) and system gamma (calculated according to BT.HDR DNR Note 5e).   The portion above 100 nits is show in the first figure below, and the portion below 100 nits in the figure that follows:

Figure 1-3 HLG OOTF (above 100 nits)

Note that the UHDA profile here was adjusted to Lw=1200 nits (peak luminance), otherwise at Lw=1000 nits it was found to be fully superimposed with the BVM-X300 curves in the figure above (100 to 4000 nits) and below (0 to 100 nits).

| Profile | Peak luminance (nits) | Black (nits) | System gamma (Note 5e) |
| --- | --- | --- | --- |
| UHDA LCD (modified) | 1200 | 0.05 | 1.233256 |
| UHDA OLED | 540 | 0.0005 | 1.087605 |
| EDR | 334.0485 | 0.2 | 1 |
| Pulsar | 4000 | 0.0047 | 1.452865 |
| BVM-X300 | 1000 | 0.0001 | 1.2 |
| UHDA LCD | 1200 | 0.05 | 1.233256 |

Figure 1-4  HLG OOTF (below 100 nits)

The PQ OOTF from BT.HDR DNR Table 4 predicts a nearly 1:1 relationship between scene-referred input and display-referred output.

| Reference PQ OOTF | FD = OOTF[E] = G1886 [G709[E]] where E = {Rs, Gs, Bs; Ys; or Is} is the signal determined by scene light and scaled by camera exposure E’ is a non-linear representation of E FD is the luminance of a displayed linear component (RD, GD, BD; YD; or ID) The values E, Rs, Gs, Bs, Ys, Is are in the range [0:1] FD = G1886 [ G709[E]] = G1886 E’ E’ = G709[E] = 1.099 (59.5208 E)0.45 – 0.099 for 1 > E > 0.0003024 = 267.84 E for 0.0003024 ≥ E ≥ 0 FD = G1886[E’] = 100 E’2.4 |
| --- | --- |

Figure 1-5  BT.HDR DNR Table 4 extract

Producing this LogX-axis/LogY-axis graph relationship between the EOTF and OOTF.  The effect of the PQ OOTF appears to be designed to supress noise in dark pixels.

It is not clear at this time whether the G1886  function is in a future edition of BT.HDR meant to include the black lift and gamma adjustment of BT.1886 informative Appendix 1 that would be analogous to BT.HDR DNR Note 5e for HLG.

Figure 1-6  BT.1886 Appendix 1

## Predicting optimal performance range from Contrast Sensitivity Function

BT.HDR DNR Hybrid Log Gamma reference EOTF (using range in Note 5g) includes display-specific gamma, and thus the UHDA [0.05, 1000] nits and UHDA [0.0005, 540] nits OLED produces different CSF plots

| Figure 1-7 UHD Alliance OLED profile (0.0005 to 540 nits, display gamma = 1.087605) |  |
| --- | --- |
| Figure 1-8 UHD Alliance LCD profile (0.05 to 1000 nits, display gamma = 1.2) |  |
| Figure 1-9 UHDA OLED profile (0.0005 to 540 nits, display gamma = 1.087605) |  |
| Figure 1-10 UHDA LCD profile (0.05 to 1000 nits, display gamma = 1.2) |  |

# PQ to HLG

Figure 2-11 PQ 4000 nits Market converted to HLG

Coded level histogram of PQ:

Code level histogram of HLG (after conversion from PQ):

Conversion steps for PQ to HLG above are:

# The need for roll-off

[ Picture and histogram comparison between ACES roll off tone map vs. BT.HDR ]

# Converting SDR to HLG

## The question of diffuse white

# References

[1] ITU-R WP6C “DRAFT NEW RECOMMENDATION ITU-R BT.[HDR-TV]: Image parameter values for high dynamic range television for use in production and internal programme exchange” , ITU-R Question 142/6, Document 6/39 , 9 February 2016.

[2]  JCTVC-V0027, “Report of HDR Core Experiment 7: On the visual quality of HLG generated HDR and SDR video”,  23rd Meeting: San Diego, USA, 19–26 February 2016.