| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0050 |
| --- | --- |

| Title: | Overview of ICtCp |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jaclyn Pytlarz 432 Lakeside Drive, Sunnyvale, CA 94085, USA | Tel: Email: | +1 408 330 3399 Jaclyn.Pytlarz@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This informational contribution presents an overview of Constant Intensity ICTCP signal format of Rec. ITU-R BT.[HDR-TV].

# Introduction

The legacy Y’C’BC’R non-constant luminance format is a colour-opponent based encoding scheme (in which signals are interpreted based on colour differences in an opposing manner) intended to separate luma from chroma information for the purposes of chroma subsampling (i.e., 4:2:2 and 4:2:0). High dynamic range and wide colour gamut content reveal the limitations of existing colour encoding methods. Errors that were previously small with standard dynamic range can become magnified. BT.2020 provides an alternative to Y’C’BC’R, i.e., the Y’CC’BCC’RC constant luminance format. This format resolves the issue of chroma leakage into the Y’ luma signal, but does not solve the problem of luminance contamination of the C’BC and C’RC components. Recommendation ITU-R BT.[HDR-TV] [1] provides an alternative method for colour difference encoding called constant intensity, which is based on IPT colour space [2] developed by Ebner and Fairchild.

# Non-Constant Luminance (NCL) Y’C’BC’R

Y’C’BC’R is widely used for standard dynamic range content and requires a specific conversion based on the primaries being encoded and decoded. Recommendation ITU-R BT.[HDR-TV] specifies PQ as a non-linearity to be used with the BT.2020 colour primaries. While Y’C’BC’R performs satisfactorily in many cases, some limitations have emerged for its use in high dynamic range wide colour gamut scenarios:

–	Quantisation distortions due to bit depth limitations with the increased colour volume.

–	Chroma subsampling distortions due to a perceptually uneven distribution of code words.

–	Colour volume mapping distortions due to incorrectly predicted hue and luminance.

–	Error propagation from chroma to luma channels.

The constant luminance method specified in BT.2020 helps reduce the last of these, but this solution is not being widely adopted because the benefits are considered modest and entail some additional complexity.

# Constant Intensity ICTCP Encoding

An alternative to Constant Luminance (CL) Y’CC’BCC’RC is the Constant Intensity (CI) ICTCP colour representation. Like Y’C’BC’R, ICTCP is a colour-opponent based encoding scheme intended to separate luma from chroma information. CI offers the same benefit as CL in that the chroma channels are lacking luminance, but ICTCP has the advantage that the lines of constant hue are straighter, and the MacAdam’s ellipses are more circular. The CI neutral (grey) axis is encoded with the PQ or HLG non-linearity to match the human visual system, and to optimize it for high dynamic range signal encoding. The alternative 3x3 colour matrices used to generate the colour difference channels have been optimized [3] for the human visual system perception of HDR and WCG. The in-camera encoding and in-display decoding steps for ICTCP are identical to those for NCL Y’C’BC’R, so ICTCP is compatible with that hardware.

## Constant Intensity ICTCP Encoding

Figure 1 and 2 are the conversion steps needed to get from camera linear RGB sensor signals into Y’C’BC’R and into ICTCP. Note that the matrix coefficients are decimal values that differ very slightly from the values shown in Recommendation ITU-R BT.[HDR-TV]; the values shown in in the Recommendation should be used in actual implementations.

Figure 1 Camera RGB Conversion To Y’C’BC’R

Figure 2 Camera RGB Conversion to ICTCP

## Advantages of Constant Intensity ICTCP

The specific design of the Constant Intensity colour space provides several benefits versus the 
Non-Constant Luminance colour space when used with the PQ or HLG non-linearity to provide HDR.

### Achromatic Channel

The achromatic axis of Y’C’BC’R (Y’ encoded in PQ or HLG) does not fully decorrelate luminance from colour. Therefore distortions introduced into the chroma channels can propagate to luminance where they become much more noticeable. As shown in Figure 3 the achromatic axis of ICTCP (I) corresponds very closely with Luminance (where Luminance is a weighted sum of linear R,G,B). This is an indicator of how well ICTCP separates luma from chroma information. This reduces errors that can be introduced when spatially sub-sampling the chroma components compared to conventional Non-Constant Luminance encoding. The axes in Figure 3  are from zero to full scale in PQ space. (The luminance errors shown for Y’C’BC’R are not as large for legacy systems using standard dynamic range with gamma encoding.)

Figure 3 Luminance Correlation

### Quantization to Limited Bit-Depth

Figure 4 shows the worst case visual colour difference between chroma channel code values (using ∆E2000) at various luminance levels. 10-bit ICTCP provides an approximately 1.5 bit colour difference improvement over 10-bit Y’C’BC’R. At less than an average of 1.0 ∆E above the visual difference threshold, use of ICTCP significantly decreases visible distortions thus enabling excellent colour performance with 10-bit encoding.

Figure 4 Maximum Colour Deviation at Various Bit-Depths

### Uniformity and Hue Linearity:

A colour space is hue linear when the hue remains constant as saturation or intensity are changed. Hue linearity is important during any interpolation such as colour volume mapping, chroma subsampling, and blending/fading. Y’C’BC’R has large deviations (see Figure 5) that cause hue shifts with highly saturated colours. ICTCP was designed to minimize deviation from lines of constant hue thereby reducing hue shifts. In addition, ICTCP has a more uniform distribution of colours. This improves efficiency, reduces worst case quantization and interpolation errors.

Purple Hue Shift 

Original in Y’C’BC’R 

Lightness Change 

Original in ICTCP 

Constant Blue Hue

Constant Lightness 

Figure 5 Blue Hue Linearity Comparison (using PQ)

If the Constant Luminance (CL) format specified in Recommendation ITU-R BT.2020 is applied to HDR, the Y’CC’BCC’RC representation introduces additional (over NCL) errors in skin tones.  The blue is significantly improved versus Non-Constant Luminance (but still contains errors) and CL has significantly worse errors in the red and green regions (see Figure 6). (The BT.2020 CL coefficients were designed for use with the SDR camera characteristic, and thus were not optimized for use in HDR.)

Figure 6 Constant Luminance vs. Constant Intensity Hue Linearity Comparison (PQ)

### Colour Sub-Sampling:

Figure 7 shows a practical example of a colour sub-sampling distortion due to Non-Constant Luminance encoding. Two very similar colours with a ∆E2000 of 0.1 were sub-sampled to 10bits 4:2:0 in Y’C’BC’R and ICTCP and reconstructed. Due to the poor decorrelation between Y’ of Y’C’BC’R and Luminance (Y), errors introduced in chroma during sub-sampling spread to the luminance and became more visible with a ∆E2000 of 4.0. Constant Intensity ICTCP has a higher tolerance for chroma error and the colours remain indistinguishable with a ∆E2000 of 0.2.

Figure 7 Colour Sub-Sampling Distortions Based on Correlation with Luminance

# Reference

- Draft Rec. ITU-R BT.[HDR-TV], " Image parameter values for high dynamic range television for use in production and international programme exchange ", Feb. 5, 2016.

- F. Ebner, M. Fairchild, “Derivation and Modelling of Hue Uniformity”. Rochester Institute of Technology. July 1998.

- J. Froehlich, et al., “Encoding Color Difference Signals for High Dynamic Range and Wide Gamut Imagery”, Color and Imaging Conference, Volume 2015, Number 1, October 2015, pp. 240-247(8).